3GPP TSG-RAN WG2 Meeting #128											    R2-2409661

Orlando, USA, November 18th – 22th, 2024

Source: 	Xiaomi

Title:  	Discussion on asymmetric DL sTRP and UL mTRP

Agenda Item:	8.12.2

Document for:	Discussion and Decision

# Introduction

According to the LS from RAN1 in [1], RAN1 asks RAN2 to design the pathloss offset configuration via RRC and the pathloss offset update via MAC CE.

Agreement For the association between PL offset and joint/UL TCI state, support the following - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s). “MAC-CE impact: MAC-CE updates the PL offset value(s) configured in one or more joint TCI states or UL TCI states”

The corresponding RAN1 agreements related to the asymmetric DL sTRP and UL mTRP can also be found in the Annex. In this contribution, we provide our understandings on the potential RAN2 specification impacts of the asymmetric DL sTRP and UL mTRP.

# Discussion

Figure 1: asymmetric DL sTRP/UL mTRP

According to the Rel-19 MIMO WID, the standard work for asymmetric DL sTRP/UL mTRP deployment as illustrated above (i.e. TRP-1 with both UL and DL and TRP-2 with only UL) is expected as the followings:

Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

## RACH

According to the RAN1 agreements, the pathloss offset is indicated via the PDCCH-order RACH procedure.

Agreement Support applying PL offset on PDCCH-order PRACH towards a UL TRP in FR1. - Note: The DL reference timing determination for PDCCH-order PRACH transmission to an UL TRP is still based on the DL RS defined in current RAN4 specification - Above is subject to a separate UE capability signaling Agreement For indicating PL offset for PDCCH-order PRACH, introduce a new 1-bit DCI field in DCI format 1_0: - This DCI field exists when the corresponding RRC parameter (which is a new RRC used to configure the presence of this 1-bit DCI field) is enabled and at least one TCI state is configured with PL offset. - When one joint/UL TCI state is indicated in Rel-17 unified TCI, - the bit field index 0 of this field indicates that PL offset is not included in the PRACH transmission power calculation - the bit field index 1 of this field indicates that the PL offset associated with the indicated TCI state is included in the PRACH transmission power. - FFS: Whether the bit field can be used to indicate other information - FFS: When two joint/UL TCI states are indicated in Rel-18 unified TCI Agreement For indicating PL offset for PDCCH-order PRACH, when two joint/UL TCI states are indicated in Rel-18 unified TCI, down-select one from the following Alts for the 1-bit DCI field in DCI 1_0 which indicates the application of PL offset on PRACH transmission: - Alt2: - the bit field index 0 of this field indicates that the PL offset associated in the first indicated joint/UL TCI state is included in the PRACH transmission power calculation - the bit field index 1 of this field indicates that the PL offset associated in the second indicated joint/UL TCI state is included in the PRACH transmission power calculation. - FFS: Whether a restriction that only one of the indicated joint/UL TCI states can be configured with PL offset is needed. FFS: If other information can be indicated by this same 1-bit DCI field for the PDCCH-order PRACH transmission

According to the RACH procedure as quoted below, the RSRP of the downlink pathloss reference is used for the selection of different RACH procedures, NUL and SUL, and different PRACH resources.

TS 38.321 - rsrp-ThresholdSSB: an RSRP threshold for the selection of the SSB for 4-step RA type. If the Random Access procedure is initiated for beam failure recovery, rsrp-ThresholdSSB used for the selection of the SSB within candidateBeamRSList refers to rsrp-ThresholdSSB in BeamFailureRecoveryConfig IE; - rsrp-ThresholdCSI-RS: an RSRP threshold for the selection of CSI-RS for 4-step RA type. If the Random Access procedure is initiated for beam failure recovery, rsrp-ThresholdCSI-RS is equal to rsrp-ThresholdSSB in BeamFailureRecoveryConfig IE; - msgA-RSRP-ThresholdSSB: an RSRP threshold for the selection of the SSB for 2-step RA type; - rsrp-ThresholdSSB-SUL: an RSRP threshold for the selection between the NUL carrier and the SUL carrier; - msgA-RSRP-Threshold: an RSRP threshold for selection between 2-step RA type and 4-step RA type when both 2-step and 4-step RA type Random Access Resources are configured in the UL BWP; - rsrp-ThresholdMsg1-RepetitionNum2: an RSRP threshold for Msg1 repetition with repetition number 2 (see clause 5.1.1b); - rsrp-ThresholdMsg1-RepetitionNum4: an RSRP threshold for Msg1 repetition with repetition number 4 (see clause 5.1.1b); - rsrp-ThresholdMsg1-RepetitionNum8: an RSRP threshold for Msg1 repetition with repetition number 8 (see clause 5.1.1b); - rsrp-ThresholdMsg3: an RSRP threshold for Msg3 repetition (see clause 5.1.1b);

Regarding the calculation of the pathloss for the two TRPs of asymmetric DL sTRP/UL mTRP, the UE would have the followings:

- PathlossUL_DL_TRP = PowerTx_UL_DL_TRP - RSRPPathloss_Reference

- PathlossUL _TRP

  - = PathlossUL_DL_TRP + Pathloss_Offset

  - = PowerTx_UL_DL_TRP – (RSRPPathloss_Reference - Pathloss_Offset)

  - = PowerTx_UL_DL_TRP – RSRPUL_TRP

Thus, the virtual RSRP (i.e. RSRPUL_TRP reflecting the uplink coverage of the UL-only TRP) of the UL_TRP would be as follows (assuming that the UL_TRP is changed to UL_DL_TRP with the TRP transmission power of PowerTx_UL_DL_TRP):

- RSRPUL_TRP = RSRPPathloss_Reference - Pathloss_Offset

As the PDCCH-order indicating pathloss offset is for the PRACH transmission to the UL-only TRP, in order to ensure the uplink coverage for the PRACH transmission to the UL-only TRP, the RSRP for the selection of the different RACH procedures, NUL and SUL, and different PRACH resources towards the UL-only TRP should combine the RSRP measured for the pathloss reference and the pathloss offset.

Proposal 1: When PDCCH-order indicates the pathloss offset, the RSRP used for the selection of RACH procedure combines both the RSRP measured for the pathloss reference and the pathloss offset.

- RSRPUL_TRP = RSRPPathloss_Reference - Pathloss_Offset

## PHR

According to the WID [2] as quoted below, only one pathloss reference is configured for both UL-only TRP and UL+DL TRP:

- Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

According to the RAN1 LS in [1], the pathloss offset is configured via RRC and updated via MAC CE.

Agreement For the association between PL offset and joint/UL TCI state, support the following - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s). “MAC-CE impact: MAC-CE updates the PL offset value(s) configured in one or more joint TCI states or UL TCI states”

According to the PHR procedure in 38.321, when the path loss change above phr-Tx-PowerFactorChange, the UE triggers PHR.

### 5.4.6 Power Headroom Reporting … A Power Headroom Report (PHR) shall be triggered if any of the following events occur: - phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one RS used as pathloss reference for one activated Serving Cell of any MAC entity of which the active DL BWP is not dormant BWP since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission;

Regarding the calculation of the pathloss for the two TRPs of asymmetric DL sTRP/UL mTRP, the UE would have the followings:

- PathlossUL_DL_TRP = PowerTx_UL_DL_TRP - RSRPPathloss_Reference

- PathlossUL _TRP = PathlossUL_DL_TRP + Pathloss_OffsetRRC + Pathloss_OffsetMAC_CE

Figure 2: Calculation of path loss change

For the UL_DL_TRP, the path loss change is due to the change of the DL measurement of the pathloss reference (i.e. RSRPPathloss_Reference).

For the UL_TRP, the pathloss change can be impacted by the DL measurement of the pathloss reference (i.e. RSRPPathloss_Reference), the RRC configuration of pathloss offset (i.e. Pathloss_OffsetRRC), and the MAC CE update of pathloss offset (i.e. Pathloss_OffsetMAC_CE). From our understanding, all the parameters impacting the pathloss calculation/change should be counted for the triggering of the PHR, so that the network can get the latest UE UL Tx power status in time.

Proposal 2: The PHR is triggered, when the path loss for the UL-only TRP has changed more than phr-Tx-PowerFactorChange due to:

- The RRC configuration of pathloss offset

- The MAC CE update of pathloss offset

# Conclusion

According to the above analysis, we have the following proposals:

Proposal 1: When PDCCH-order indicates the pathloss offset, the RSRP used for the selection of RACH procedure combines both the RSRP measured for the pathloss reference and the pathloss offset.

- RSRPUL_TRP = RSRPPathloss_Reference - Pathloss_Offset

Proposal 2: The PHR is triggered, when the path loss for the UL-only TRP has changed more than phr-Tx-PowerFactorChange due to:

- The RRC configuration of pathloss offset

- The MAC CE update of pathloss offset

# Reference

[1] R1-2407285, LSin, from RAN1, “LS to RAN2 on RRC and MAC CE impacts for Rel-19 NR MIMO Ph5”

[2] RP-242394, Samsung, “WID revision: NR MIMO Phase 5”

# Annex

## RAN1 agreements

### RAN1#116

Agreement

For the asymmetric DL sTRP/UL mTRP deployment scenarios, support to associate a UL TCI state with a PL offset:

- When a UL TCI state associated with a PL offset is applied for the PUSCH/PUCCH/SRS transmission, the UE shall calculate the Tx power of the PUSCH/PUCCH/SRS based on the DL PL RS and PL offset associated with this UL TCI state.

  - Reuse the legacy uplink power control formulation by replacing legacy PL with UL PL which is derived from the DL PL RS and the PL offset.

  - FFS: The UE can update UL PL in a way that new UL PL = current UL PL + an update delta indicated by the NW.

- Note: it does not intend to increase the number of maintained PLs per cell.

- FFS: whether to support associating joint TCI state (if supported) with a PL offset.

Further study whether/how to apply a PL offset on PDCCH-order PRACH transmission too.

- FFS: how to determine the Tx beam of PRACH towards UL TRP

- Note: this does not imply to support 2 TA for single-DCI based system.

Agreement

To facilitate the asymmetric DL sTRP/UL mTRP deployment scenarios, support configuring two closed-loop PC adjustment states for SRS in one CC, both of which are separate from that of the PUSCH.

Agreement

Study how to indicate TPC command for those two SRS CLPC adjustment states through DCI when the UE is configured two SRS CLPC adjustment states, down-select from the following options:

- Option 1: enhance the legacy DCI format 2_3 of higher layer parameter srs-TPC-PDCCH-Group = typeA;

- Option 2: enhance the legacy DCI format 2_3 of higher layer parameter srs-TPC-PDCCH-Group = typeB;

- Option 3: enhance the legacy DCI format 2_3 of higher layer parameter srs-TPC-PDCCH-Group = typeA and typeB;

- Option 4: enhance DCI format 1_1 and/or 0_1 to indicate TPC for SRS CLPC adjustment states

- Option 5: enhance the legacy DCI format 2_3 by introducing a new Type for higher layer parameter srs-TPC-PDCCH-Group

- Option 6: new DCI format to indicate TPC for SRS CLPC adjustment states

- Other options are not precluded.

For the Options1, 2, 3 and 5, consider at least the following Alts as possible examples:

- Alt1: In DCI format 2_3, add one additional TPC command for each CC configured with two SRS CLPC adjustment states,

  - the first TPC command is associated with the first SRS CLPC adjustment state and the second TPC command is associated with the second SRS CLPC adjustment state.

- Alt2: Introduce one 1-bit closed-loop-indicator field for each TPC command in DCI format 2_3

  - This 1-bit closed-loop-indicator indicates the first SRS CLPC adjustment state or the second SRS CLPC adjustment state.

- Alt3: use two different TPC-SRS-RNTIs for DCI format 2_3:

  - DCI format 2_3 with CRC scrambled with the first TPC-SRS-RNTI and the second TPC-SRS-RNTI indicates the TPC command for the first and second SRS CLPC adjustment state, respectively.

- Alt4: Implicit method:

Agreement

To support two SRS CLPC adjustment states, study and possibly down-select at least one from the following Alts:

- Alt1: SRS CLPC adjustment state is associated with SRS resource set

- Alt2: When the parameter srs-PowerControlAdjustmentStates is set to 'separateClosedLoop', closedLoopIndex-r17 in the TCI state indicates one of the SRS CLPC adjustment states

- Alt3: Add one extra parameter in P0AlphaSet-r17 of TCI state to indicate one of those two SRS CLPC adjustment states

- Alt4: SRS CLPC adjustment state is associated with SRS resource usage type

Note: Other alternatives are not precluded

Agreement

Down-select one from the following alternatives:

- Alt1: Use only RRC to update the PL offset associated with the UL TCI state

- Alt2: In addition to RRC, MAC-CE can be used to update the PL offset associated with the UL TCI state

  - FFS: Details on MAC CE

Agreement

For the asymmetric DL sTRP/UL mTRP deployment scenarios, separate DL/UL TCI state mode of Rel-17/18 unified TCI framework can be configured for both FR1 and FR2.

- Joint TCI state mode can be configured at least for FR1

### RAN1#116bis

Agreement

For a UE configured with two SRS CLPC adjustment states, support Alt2 for indicating one of the SRS CLPC adjustment states to SRS:

- Alt2: When the parameter srs-PowerControlAdjustmentStates is set to 'separateClosedLoop', closedLoopIndex-r17 for SRS in the TCI state indicates one of the SRS CLPC adjustment states

  - The candidate value of i0 and i1 in closedLoopIndex-r17 for SRS refers to the first and the second CLPC adjustment state separate from PUSCH, respectively

Ericsson raised concerns on Alt2 due to potential issues with beam management.

Agreement

For indicating TPC command for those two SRS CLPC adjustment states through DCI when the UE is configured with two SRS CLPC adjustment states, support Option3:

- Option 3: enhance the legacy DCI format 2_3 of higher layer parameter srs-TPC-PDCCH-Group = typeA and typeB;

Agreement

For enhancing DCI format 2_3 for indicating TPC command for two SRS CLPC adjustment states, support Alt2:

- Alt2: Introduce one 1-bit closed-loop-indicator field for each TPC command in DCI format 2_3

  - This 1-bit closed-loop-indicator indicates the first SRS CLPC adjustment state or the second SRS CLPC adjustment state.

Note: this 1-bit indicator is present for the CC where two SRS CLPC adjustment states are configured.

Agreement

For FR1, a joint TCI state can be associated with a PL offset.

- When a joint TCI state associated with a PL offset is applied for the PUSCH/PUCCH/SRS transmission, the UE shall calculate the Tx power of the PUSCH/PUCCH/SRS based on the DL PL RS and PL offset associated with this joint TCI state.

  - Reuse the legacy uplink power control formulation by replacing legacy PL with a PL which is derived from the DL PL RS and the PL offset.

  - FFS: The UE can update UL PL in a way that new UL PL = current UL PL + an update delta indicated by the NW.

Agreement

Support applying PL offset on PDCCH-order PRACH towards a UL TRP in FR1.

- Note: The DL reference timing determination for PDCCH-order PRACH transmission to an UL TRP is still based on the DL RS defined in current RAN4 specification

- Above is subject to a separate UE capability signaling

Agreement

Consider and down-select one from the following alts for indicating a PL offset for PDCCH-order PRACH transmission at least for FR1.

- Alt1: RRC configures multiple PL offset values in PRACH-Config and PDCCH-order DCI indicates one of them through one DCI field.

- Alt2: PDCCH order DCI indicates one PL offset value

- Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission

- Alt4: The PDCCH order DCI indicates one TCI state associated with a PL offset and the associated PL offset is applied on the PRACH transmission.

- Alt5: RRC configures one PL offset value for PRACH and the PDCCH order DCI indicates whether this PL offset value is applied on PRACH transmission or not.

Note: Other alternatives are not precluded

Agreement

For the association between PL offset and joint/UL TCI state, consider and down-select one from the following Alts:

- Alt1a: One PL offset value is configured in a joint or UL TCI state by RRC only

- Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s).

- Alt2a: A list of PL offset configurations is configured by RRC in BWP/CC and each PL offset configuration contains one PL offset value. One new RRC parameter is introduced in a joint or UL TCI state to indicate one of the configured PL offset configurations.

- Alt2b: A list of PL offset configurations is configured by RRC in BWP/CC and each PL offset configuration contains one PL offset value. One new RRC parameter is introduced in a joint or UL TCI state to indicate one of the configured PL offset configurations. A MAC CE can update the association between a joint or UL TCI state and PL offset configuration

- Alt3: A list of PL offset configurations is configured by RRC in BWP/CC and each PL offset configuration contains one PL offset value.  A MAC CE can activate/indicate one PL offset configuration for each activated joint or UL TCI state. In each joint or UL TCI state, the initial PL offset value is 0dB.

- Alt4: A list of PL offset values is provided in a joint or UL TCI state by RRC. Each PL offset value is applied to a corresponding measured PL range.

Other alternatives are not precluded.

Agreement

For the asymmetric DL sTRP/UL mTRP deployment scenarios, study whether and how to indicate TPC command for SRS CLPC adjustment states through DCI format 1_1 and/or 0_1 when the UE is configured two SRS CLPC adjustment states.

### RAN1#117

Agreement

For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, further study and down-select one from the Alt1 and Alt3 by RAN1#118 meeting:

- Alt1: RRC configures multiple PL offset values [in PRACH-Config] and PDCCH-order DCI indicates one of them through one DCI field

- Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission

- FFS: the details of DCI field design.

Agreement

In Rel-19, the value range of starting bit of block in DCI format 2-3 is extended from 1~31 to 1~X.

- FFS the value of X>31

FFS: Condition under which the above is applicable

Agreement

Introduce a new RRC parameter per BWP/CC to indicate that two separate SRS CLPC adjustment states are configured for SRS in a BWP/CC

Agreement

For the association between PL offset and joint/UL TCI state, support the following

- Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s).

Conclusion

There is no consensus on the following proposal:

Proposal 1.6: Support to update a UL PL for a joint/UL TCI state as follows:

- When this joint/UL TCI state is activated and it is not in the current active TCI state list, a UL PL is calculated as: UL PL = PL estimated from DL PL RS – the value of PL offset.

- When this joint/UL TCI state is activated and it is in the current active TCI state list, the UE updates the UL PL as: new UL PL = current UL PL + the updated delta indicated by the NW.

Agreement

For the asymmetric DL sTRP/UL mTRP scenarios, study and decide the value range and candidate values of PL offset value

Agreement

For the asymmetric DL sTRP/UL mTRP scenarios, study whether/how to consider PL offset in PHR calculation, including Type 1 PHR based on actual PUSCH transmission, Type 1 PHR based on reference PUSCH, Type 3 PHR based on actual SRS and Type 3 PHR based on reference SRS

Conclusion

For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework:

- When rel-17 unified TCI/ICBM is configured:

  - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied.

  - For FR2: {one DL TCI state + one UL TCI state} can be applied.

- When rel-18 unified TCI is configured:

  - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied.

  - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

### RAN1#118

Agreement

For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, support Alt3:

- Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission

  - FFS the detailed design of DCI format: e.g., how to indicate one of the indicated joint/UL TCI states or whether to apply the PL offset in the indicated TCI state or not.

Agreement

For the asymmetric DL sTRP/UL mTRP scenarios, the value range of PL offset includes at least [-10, 60] dB

- FFS: Extending the range to lower than -10dB

- Step size is 4dB

Agreement

For the asymmetric DL sTRP/UL mTRP scenarios, support to include PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH

Agreement

Study whether to support Type 3 PHR reporting in a serving cell/BWP where the UE is configured with two separate SRS CLPC adjustment states.

- Continue to study whether to support including PL offset in the calculation of Type 3 PHR.

Agreement

About the extended value range 1~X of starting bit of blocks in DCI format 2_3 in Rel-19, down-select one from the following Alts in RAN1#118bis:

- Alt1: X = 45 (to be captured in RAN2 spec)

  - This feature is a separate UE capability and is appliable to any rel-19 UE who supports this UE capability, regardless this UE supports two separate SRS CLPC adjustment states or not.

  - Note: X=45 can be used for operations in FR1 in shared spectrum or FR2-2 and X = 43 otherwise

- Alt2: X = 44 (to be captured in RAN2 spec)

  - This feature is only applicable to UE who is configured with two separate SRS CLPC adjustment states.

  - Note: X=44 can be used for operations in FR1 in shared spectrum for FR2-2 and X = 42 otherwise

- Above extended value range is applied to startingBitOfFormat2-3.

Agreement

- Study whether/how to apply PL offset for SRS resource set when the SRS resource set is not configured with TCI state

- Study whether/how to apply one of the two separate SRS CLPC adjustment states on the SRS resource set when the SRS resource set is not configured with TCI state

  - E.g., defining i0 as the default CLPC for SRS resource set in this case. E.g,, configure one of the separate SRS CLPC adjustment states to the SRS resource set.

### RAN1#118bis

R1-2408995	Summary #1 on Rel-19 asymmetric DL sTRP/UL mTRP	Moderator (OPPO)

From Monday session

Agreement

- The lower limit of the value range of PL offset is extended to -12 dB.

R1-2408996	Summary #2 on Rel-19 asymmetric DL sTRP/UL mTRP	Moderator (OPPO)

From Tuesday session

Agreement

Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios:

- Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature.

- One downlink reference timing is supported and applied to both TAGs.

  - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG

- One single n-TimingAdvanceoffset is configured and applied to both TAGs.

- Any of the TCI states can be associated with any one of the two TAGs.

- The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA

- The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA.

- Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA

  - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped.

- FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

Agreement

For indicating PL offset for PDCCH-order PRACH, introduce a new 1-bit DCI field in DCI format 1_0:

- This DCI field exists when the corresponding RRC parameter (which is a new RRC used to configure the presence of this 1-bit DCI field) is enabled and at least one TCI state is configured with PL offset.

- When one joint/UL TCI state is indicated in Rel-17 unified TCI,

  - the bit field index 0 of this field indicates that PL offset is not included in the PRACH transmission power calculation

  - the bit field index 1 of this field indicates that the PL offset associated with the indicated TCI state is included in the PRACH transmission power.

- FFS: Whether the bit field can be used to indicate other information

- FFS: When two joint/UL TCI states are indicated in Rel-18 unified TCI

R1-2408997	Summary #3 on Rel-19 asymmetric DL sTRP/UL mTRP	Moderator (OPPO)

From Wednesday session

Agreement

For indicating PL offset for PDCCH-order PRACH, when two joint/UL TCI states are indicated in Rel-18 unified TCI, down-select one from the following Alts for the 1-bit DCI field in DCI 1_0 which indicates the application of PL offset on PRACH transmission:

- Alt2:

  - the bit field index 0 of this field indicates that the PL offset associated in the first indicated joint/UL TCI state is included in the PRACH transmission power calculation

  - the bit field index 1 of this field indicates that the PL offset associated in the second indicated joint/UL TCI state is included in the PRACH transmission power calculation.

  - FFS: Whether a restriction that only one of the indicated joint/UL TCI states can be configured with PL offset is needed.

FFS: If other information can be indicated by this same 1-bit DCI field for the PDCCH-order PRACH transmission

R1-2409254	Summary #4 on Rel-19 asymmetric DL sTRP/UL mTRP	Moderator (OPPO)

From Thursday session

Agreement

About the extended value range 1~X of starting bit of blocks in DCI format 2_3 in Rel-19, support Alt1:

- Alt1: X = 45 (to be captured in RAN2 spec)

  - This feature is a separate UE capability and is appliable to any Rel-19 UE who supports this UE capability, regardless this UE supports two separate SRS CLPC adjustment states or not.

  - Note: X=45 can be used for operations in FR1 in shared spectrum or FR2-2 and X = 43 otherwise

Agreement

Support DCI format 1_1 to indicate TPC command for SRS CLPC adjustment state(s) separate from PUSCH:

- (Working Assumption) Introduce a 2-bit TPC command field to indicate TPC command for SRS associated with separate SRS CLPC adjustment state where:

  - The 2-bit TPC command field is present if UE reports supporting a dedicated UE capability, and a corresponding RRC parameter is configured (which is a new RRC to enable this).

- (Working Assumption) Introduce a 1-bit SRS close-loop indicator to indicate one of the two separate SRS CLPC adjustment states for the TPC command.

The 1-bit SRS close-loop indicator is present if UE reports supporting another dedicated UE capability and a corresponding RRC parameter is configured (which is a new RRC to enable this) and two separate SRS CLPC adjustment states are configured.