3GPP TSG-RAN WG2 Meeting #128                                                                  R2-2409687

Orlando, USA, 18th – 22nd November, 2024

Source: 		  ZTE Corporation, Sanechips

Title:                  Remaining issues of AIoT Paging

Agenda item:    8.2.3

Document for:  Discussion and Decision

- Introduction

In RAN#102, a study item on solutions for Ambient IoT (Internet of Things) in NR has been agreed and in RAN#103, the SID is further refined in [1].

In RAN2#125bis~RAN2#127bis meetings, RAN2 has achieved much progress on the aspects of protocol stack, functionalities, random access-like and paging-like  procedures for Ambient IoT. In this contribution, we will further discuss the remaining issues of paging-like procedure in AIoT air interface and give our proposals.

- Discussion

2.1 Delta paging and re-access of devices

In previous meetings, in A-IoT Paging and A-IoT Random Access topics, RAN2 has achieved the following agreements related to multiple AIoT paging messages delivery:

- Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices. (RAN2#127) - The A-IoT paging message can include information to avoid duplicate response from the device to a reader. This information should be short and simple. FFS how to indicate. Wait for further information and requirements from other WGs and make this decision in normative phase. (RAN2#127bis) - Support optional explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose. FFS for following D2R data. (RAN2#127bis)

According to the agreements, there is the case that a reader triggers multiple/subsequent AIoT paging messages that are associated with the same service request from CN/AF (Case-1). In a certain paging round, a device can determine whether it has accessed with successfully contention resolution via exchanging Msg1 and Msg2 with the reader. The device also can determine its Msg3 transmission has been successfully received. The device that has received ACK for Msg3 transmission can internally mark its status as “has been inventoried”, meanwhile, the device that determines the failure of Msg3 transmission according to the NACK feedback can internally mark its status as “has not been inventoried”. When subsequent paging messages are received, for the device in the status of “has been inventoried”, it can ignore the subsequent paging messages. Meanwhile, for the device in the status of “has not been inventoried”, it still needs to response to the subsequent paging messages.

Observation 1a: In a certain paging round, a device can determine whether it has accessed with successfully contention resolution via exchanging Msg1 and Msg2 with the reader. The device also can determine whether its Msg3 transmission has been successfully received based on the explicit R2D failure/success feedback indication for Msg3 (if provided).

It can be further noted that, for A-IoT Random Access topic, RAN2#127bis meeting has achieved the following agreements:

- For 2step and 3step CBRA, The A-IoT device performs re-access in another opportunity provided by the reader (i.e. retry the random access) at least in case of contention resolution failure. - R2D message is used to provide access occasion(s) which can be used for re-access purpose. One option to be captured in the TR is that this R2D message is AIoT paging message. FFS additional indications to differentiate if needed in paging message. FFS if other message can be used.

Taking into account all above agreements from both A-IoT Paging and A-IoT Random Access topics, we have the following observation:

Observation 1b: For a certain service request from CN/AF, after the initial paging, the subsequent AIoT paging messages triggered by the reader can be also seen as the R2D messages used to provide access occasion(s) with re-access purpose for the devices that have experienced contention resolution failures or Msg3 transmission failures.

Furthermore, for Case-1, there may be an assumption that same device IDs information will be included in the initial and all subsequent paging messages. However, since the device that with status of “has not been inventoried” (e.g., has sent Msg3 but received no ACK) anyway needs to respond the subsequent paging messages, we no longer see the need to provide same device IDs information in the subsequent paging messages. Therefore, the subsequent paging messages can be simplified to just a trigger message without any device IDs information. Any device with status of “has not been inventoried” needs to respond to such simplified paging messages.

There may be a possible rare case that, the reader may want to (re)inventory all the devices even in the subsequent paging message. In such case, including no device IDs information is intentional and means all devices (even the devices that have successfully responded in previous paging round) receiving this paging message are selected. In order to differentiate this case, a delta paging indication (e.g., simple 1 bit indication) can be introduced to the subsequent paging messages. For subsequent paging messages including no device ID information but with a delta paging indication, only the devices with status of “has not been inventoried” need to respond.

Although RAN2 has agreed a short and simple indication to avoid duplicate response from the device to a reader will be discussed later, e.g., in normative phase and based on further information and requirements from other WGs, we still think it’s useful to conclude that, for a certain service request from CN/AF, subsequent AIoT paging messages can be sent after the initial paging message and includes no device ID information but with a delta paging indication. Such subsequent AIoT paging messages without device ID information but with a delta paging indication can be used to provide access occasion(s) with re-access purpose for the devices that have experienced contention resolution failures or Msg3 transmission failures.

Proposal 1: For a certain service request from CN/AF, subsequent AIoT paging messages without device ID information but with a delta paging indication can be used to provide access occasion(s) with re-access purpose for the devices that have experienced contention resolution failures or Msg1/Msg3 transmission failures.

2.2 Necessity of session ID in paging

In previous RAN2 meetings, besides the Case-1, another case was also mentioned where multiple paging messages can be triggered by different service requests and are executed in parallel (Case-2). Some companies indicated that typical number of CN service requests in parallel is 4.

As mentioned above, for Case-1, if a device confirms its Msg3 transmission has been received, it does not need to response the subsequent paging messages. However, if another paging message triggered by different service request from CN/AF can also reach to this device at the same time, the device may become confused on whether it needs to respond or it may just incorrectly ignore the “new” paging message. So in order that the device can differentiate these two paging messages, it’s proposed in some previous contributions that reader can provide an ID (e.g., session ID) in the paging message to associate it with a certain CN service request. Shortly to say, if a device with status of “has been inventoried” receives another paging message and the session ID in this paging is different from the one indicated in a previous paging messages, the device would understand these two paging messages are associated with different requests from the CN and still needs to response to the latter paging message. But if the session ID is same as the one in previous paging messages, the device can skip the latter paging message.

However, with the following considerations, we see not much necessity for supporting Case-2 in AIoT air interface:

- Considering that the typical AIoT operation is to inventory a device group or all devices within a certain area, we understand that the CN does not need to trigger multiple service requests in succession for the same device group or the same area within a short time period. Even if this occurs, it may be in abnormal scenario and just be a rare case. A more reasonable way for CN would be to wait for the results of the previous inventory request before deciding to initiate another inventory request. So we assume Case-2 is rare case and the reader will rarely receive multiple different CN service requests targeting the same batch of devices in a short time period.

Observation 2a: Considering that the typical AIoT operation is to inventory a device group or all devices within a certain area, the CN does not need to trigger multiple service requests in succession for the same device group or the same area within a short time period.

- Even if the reader receives another service request from CN/AF before it finishes handling a previous one, it’s possible for the reader to handle this by its implementation, e.g., to hold the new CN service request until the previous request is completely processed. This can be seen as a serial processing method. There may be comments that serial processing on two CN requests might introduce additional delay for the devices that are targeted by the second request. However, if we consider from the resources perspective, in the serial processing case, the AIoT paging/RA procedures for each CN service request can be allocated more resources and potentially requires less time to finish. When multiple CN services are processed in parallel, the AIoT paging/RA procedures for each CN request may be allocated fewer resources, increasing the time required for completing the procedures for each CN request. Therefore, the total time for processing multiple CN service requests serially may not necessarily be significantly longer than the time required for processing multiple services in parallel.

Observation 2b: From reader perspective, if only one CN request is processed at a time, more resources can be allocated to the AIoT procedures of this CN request. Therefore, the total time for processing multiple CN service requests serially may not necessarily be significantly longer than the time required for processing multiple CN service requests in parallel.

We have also observed the following complexities for the devices for supporting Case-2:

- If session ID is introduced in paging message, the device needs to store at most [Maximum number of concurrent CN service requests supported −1] session IDs, then the device can determine whether the newly received session ID is different from all previously received session IDs. This imposes additional storage overhead on the device, and the longer the session ID, the greater the overhead.

- It needs to further consider the value range of session ID and the number of bits required in paging message. Some companies thought that the CN may also allocate a session ID for each CN service request, and it can be directly used in the paging message in AIoT air interface. However, considering that the CN may keep triggering services, the session ID assigned by the CN might continue to grow until it becomes very large, which would occupy too many bits if directly placed in the paging message. So this way is infeasible. If agreed to introduce, the session ID in the paging message should be (re)assigned by the reader, with consideration that only a limited number of different CN service requests need to be supported in AIoT air interface in parallel. But even so, considering it’s possible that new CN requests exceeding the maximum supported request number may temporarily arrive, if the length of session ID is set too short, there may occur session ID wrap-around issue, which may cause the device to incorrectly ignore the paging messages for new CN service request.

Observation 2c: If a session ID is to be introduced in paging to facilitate the device to identify a paging triggered by a different CN/AF service request, it will impose additional storage overhead on the device (multiple times of the total length of a session ID), and the longer the session ID, the greater the overhead. Moreover, if a session ID is too short, there may be session ID wrap-around issue.

Based on the above observations, we give the following proposal:

Proposal 2: There is no need to introduce session ID in paging message

2.3 Necessity of reader ID in paging

In previous meeting, there was some discussion on whether AIoT devices need to differentiate multiple paging messages triggered by different readers. A corresponding case has been mentioned that “one CN AIoT service request may be sent to multiple AIoT readers”. Some companies suggested to introduce session ID and reader ID in the paging message to address this case. If a device receives two paging message with the same session ID but different reader IDs, the device can choose to respond to only one of the paging messages and ignore the other.

Firstly, RAN4 has discussed the suitable number of simultaneously active BS readers. It was observed that activating only a single reader can significantly improve the baseline SINR. And it’ required that the number of simultaneously active readers shall ensure the baseline SINR meets the target value. So RAN4 has achieved the following agreements:

Agreement in RAN4#112: - Minimum distance between active readers: 60m as baseline, other values can be reported by other companies. - 2 readers are activated in one drop as baseline

According to RAN4 agreement, it can be seen that in a certain area, at most two readers can perform AIoT operations at the same time and they also need to keep a sufficiently large distance between them. Considering the very limited demodulation capability of the devices, even for devices located at the boundary between two readers, from the RAN2 perspective, it is very unlikely the device could simultaneously demodulate two paging messages. Therefore, it’s useless to let two BS readers to send the same paging message to the same device simultaneously.

Observation 3a: According to RAN4 agreement, at most two readers in a certain area can perform AIoT operations at the same time and they need to keep a sufficiently large distance between them. Considering the very limited demodulation capability of the devices, it is very unlikely the device could demodulate two paging messages at the same time. Therefore, it’s useless to let two BS readers to send the same paging message to the same device simultaneously.

Secondly, based on the understanding of the AIoT paging procedure, upon reception a CN service request, the reader will allocate AIoT resources according to the scale of the targeted devices. It can also trigger multiple paging rounds and adjust the scheduling parameters to control the access of the devices based on the conflicts situation. So even if it can be assumed that the device can receive paging messages from two BS readers, and if a reader (e.g., reader-B) has triggered inventory for the device but the device can deliberately choose not to respond it (device may choose to respond another reader, reader-A), it will lead to a waste of this reader's resources, and the multiple paging rounds would be performed in vain. Moreover, the reader-B will also waste some time waiting for the device to respond which may result in a delay in some other services.

Observation 3b: Having two BS readers simultaneously perform the same AIoT service for one device, but allowing the device to respond to only one of the readers, would result in resource wastage and service delay for the other reader.

Finally, we see no clear justification for CN to trigger multiple readers to perform a same service request at the same time as it will also cause additional signalling overhead in the interfaces between CN and reader. So we assume CN needs to avoid such case based on correct association among [CN node, reader, devices] which may be established in previous inventory procedure.

Based on above analysis, we give the following proposals:

Proposal 3a: RAN2 assume the case that a device receives multiple paging messages from different readers can be avoided and no need to address it.

However, in the following two scenarios, we see the necessity of introducing reader ID in paging message to address the potential issues:

- Scenario 1(reader ID is needed in the paging to help device identify a new reader): According to the previous discussion on multiple paging rounds, we assume the device which have internally marked its status as “has been inventoried” could skip a paging message with a delta paging indication. However, for a device that has previously performed an inventory in another reader (old reader), when it moves to a new reader, as there are no legacy SIB messages to help it identify the new cell, it might continue to maintain its status and incorrectly ignore a paging message received in the new reader. In order to avoid this issue, it’s suggested to introduce a short and simple reader ID in the paging message, the device needs to record the reader ID of the reader where it was successfully inventoried in last time. Then if the device later receives a new paging with different reader ID, the device can clear the status and respond to this paging message as being inventoried for the first time.

Proposal 3b: It’s suggested to introduce a reader ID in the paging message.

- Scenario 2(reader ID is needed in D2R message to help reader identify messages that do not aim to it): Also for the case that a device locates at the boundary between two readers, although we assume it’s very likely the device cannot simultaneously demodulate two paging messages from different readers due to the limited device capability, it still needs to consider the case that the response (just triggered by one paging from one reader, e.g., reader-A) sent by the device can be demodulated by both of the readers as they have higher demodulation capabilities. To address this case, it’s suggested that the device can include reader ID of the reader (e.g., reader-A) in the UL D2R messages and the reader-A is the one from which the device receives the paging messages. If the reader-B receives the Msg3, e.g., the UL D2R message containing device ID information, but identifies the reader ID included in Msg3 does not match its own reader ID, reader-B can choose to discard the received Msg3.

Proposal 3c: RAN2 discuss whether it’s needed to introduce a reader ID in the D2R messages, e.g., Msg3.

2.4 Necessity of having subsequent data indication in paging

In previous meetings, there was some discussion on whether AIoT service type (e.g., inventory only, command only etc.) need to be included in AIoT paging message. Per our understand, as different service type may have different requirements on the DL/UL resources, e.g., "read" command may need large uplink resources allocation while "write" command may need large downlink resources, this information is needed by the reader to facilitate reader to allocate AIoT resources more suitably. This can be further discussed in the sub-topic of assistant information from CN.

However, giving the device just an AIoT service type indication in the paging message might not be very useful. In below, we will discuss some other more generic/useful indications:

- For paging triggered by inventory, the device doesn’t need to receive further data, so it might be able to stop monitoring for a while during the current paging round after successfully completing Msg3 transmission. But for command only or inventory + command services, after the device completes Msg3 transmission, it still needs to wait to receive DL command data. Therefore, in order that the device can know that it needs to wait for further data, an indication about “having subsequent data” in the paging message can be considered.

Proposal 4a: In order to let device know that that it needs to wait for the subsequent command data after successfully completing Msg3 transmission (e.g., in command only or inventory + command cases), it’s suggested to introduce indication about “having subsequent data” in paging message.

- For command only service, we assume the reader generally receives the command data firstly before triggering paging. Therefore, after the device completes Msg3 transmission, it can assume that the DL data can be sent to it soon. Meanwhile, for inventory + command service case, we understand from RAN2 perspective, the CN may need to firstly obtain the inventory results before sending command data to the certain devices, Thus, it can be assumed that when the reader triggers paging, it has only received the inventory request, e.g., without command data. So after the reader receives the inventory responses, it needs to firstly send the inventory response to the CN and then can get the DL command data. It is easy to see that, in this inventory + command service case, the duration the device may need to wait for command data might be longer (e.g., longer than that in command only service case). Therefore, besides the indication about “having subsequent data”, another indication about “expected waiting time for subsequent data” can also be considered to introduce in paging message.

Proposal 4b: In order to let device know that it may need to wait for more time for the subsequent command data (e.g., in inventory + command cases), another indication about “expected waiting time for subsequent data” can also be considered.

2.5 Device ID related issues

- 2.2.1 Reference from SA2 study: Structured device ID and mask scheme

Generally, the device ID information needs to be provided within the DL paging message to facilitate the device to compare its stored device ID with the provided device ID and determine whether it needs to respond the AIoT service request in AIoT air interface. Please note, here device ID information is just a general saying.

SA2 has agreed that an ambient IoT Device Identifier can be assigned by an operator or by a third party. Some options for device ID structure design mentioned in SA2 study [4] are cited as below:

- EPC-like Device ID: In legacy RFID, Electronic Product Code (EPC) is used to identify the tags which is defined by GS1 in the EPC Tag Data Standard. EPC can be seen as a means for unique identification of physical objects independently of any technology for storing and transmitting such EPC information. So EPCs can also be used independently of existing RFID tags. As shown in the Table A-3 in [4], an EPC starts with a header value that indicates the EPC scheme followed by the bitwise representation of a particular encoding scheme. Some EPC Binary Encoding schemes are listed as examples in the following Table:

| EPC scheme | Header value | Coding scheme specific bits |
| --- | --- | --- |
| Serialised Global Trade Item Number (SGTIN) | 0011 0000 | SGTIN-96 as defined in clause 14.6.1.1 of [10] EPC Header; Filter; Partition; GS1 Company Prefix; Indicator / Item Reference; Serial. 8 bits; 3 bits; 3 bits; 20-40 bits; 24-4 bits; 38 bits. ………. |
| Serial Shipping Container Code (SSCC) | 0011 0001 | SSCC-96 as defined in clause 14.6.2.1 of [10] EPC Header; Filter; Partition; GS1 Company Prefix; Extension / Serial Reference; (Reserved). 8 bits; 3 bits; 3 bits; 20-40 bits; 38-18 bits; 24 bits. ………. |

- AIoT Device ID defined by 3GPP. Such device ID may be similar as 5G-SUPI (Subscription Permanent Identifier) type ID and at least contain country/network code and the subscriber number which is used to uniquely identify a user within the related network:

  - Home Network Identifier + Owner Identifier + Instance Identifier  (Solution#2)

  - Operator ID + Group ID + Instance ID  (Solution#10)

It can be seen that the mentioned options for AIoT device ID are generally kind of structured design. It typically consists of three parts: the first part is for identifying the country and/or network/operator, the second part is for identifying the company or the owner or the service group to which the device belongs, and the third part is for identifying each individual device and should be unique at least within the scope defined by the first two parts.

Such structured design for AIoT device ID will facilitate selecting all devices that belong to a specific network, company, or service category, or just simply choosing a subset of devices, by applying simple mask/filter rules.

Observation 4a: It can be seen from SA2 study that, in some AIoT deice ID designs, e.g., EPC-like or 5G-SUPI like, it typically consists of three parts: a part for identifying the country and/or network/operator, a part for identifying the company or the owner or the service group to which the device belongs, and a part for identifying each individual device uniquely at least within the scope defined by the first two parts. Such structured design for AIoT device ID will facilitate selecting a subset of devices by applying simple mask/filter rules.

- 2.2.2 How to indicate group ID or multiple IDs

Both multiple IDs and a group ID can be used to indicate multiple A-IoT devices for AIoT operation. To include a group ID in paging message can be seen as an explicit way to indicate multiple devices if these devices previously have been gathered into a same group and assigned with a group ID.

With reference to the Observation 4a, it’s also possible to implicitly indicate a group ID, e.g., by applying a “mask/filter” on structured device ID. For example, the “mask/filter” may have a certain binary pattern for a range of bits [m, n] on the second part of structured device ID, e.g., “company/owner/group” part. A device can perform bitwise AND operation between its device ID and the mask to check whether the bits [m, n] in its own ID are aligned with the binary pattern in the “mask/filter”, if yes, the device can consider being selected for the operation on the group which is represented by the mask pattern.

Observation 4b: For indicating group ID in service request message and further paging message, a group ID can be explicitly included or can be implicitly indicated by applying a “mask/filter” on the “company/owner/group” part of structured device ID.

Also with reference to observation 4a and observation 4b, one way to indicate multiple device IDs is also to apply a “mask/filter” on structured device ID, e.g., on the “instance ID” or “serial number” part. For example, if the lowest “n” bits of the device ID are masked (the mask rule may be setting the lowest “n” bits to 1 and all other bits to 0, and the device performs a bitwise AND operation between its own ID and the mask), then maximum 2n devices can be selected. When serial number part of such structured device IDs of the devices belonging to a same owner are continuous, this way would be signaling-efficient.

There is another way to explicitly indicate multiple IDs, e.g., to include a list of device IDs in the DL service request or paging message, where each entry corresponds to the ID of an individual device. This way is generally used to indicate multiple devices with discrete/discontinuous ID numbers. Just due to the discrete ID numbers, it’s not easy to indicate these devices with a way of “mask/filter”. Given that the device ID might be a long bit string, this way may be usually applied when a small number of devices need to be indicated.

Observation 4c: For indicating multiple IDs in service request message and further paging message, a list of device IDs can be explicitly included or can be implicitly indicated by applying a “mask/filter” on the “instance ID” part of structured device ID.

- 2.2.3 Further grouping devices and applying RAN mask rule

According to the use cases discussed in SA1, it’s possible for the reader to receive a service request from CN/AF which aims to a large number of devices. Also with reference to the discussion in above sections, kind of upper layer filter/mask scheme can be applied to structured device ID to achieve multiple IDs for AIoT operation, e.g., inventory. The upper layer filter/mask is applied generally according to a business requirement (e.g., to filter the devices belonging to a certain company, owner or business type) or it may not even be applied (e.g., upper level just want to do an inventory for all the devices). Please note, when the CN/AF triggers service request for AIoT operation, it generally doesn’t need to consider the resources or capacity etc. factors in AIoT air interface.

According to our analysis in previous meeting [R2-2407148], the AIoT inventory request from upper layer, even for a single business, may reach the scale of hundreds of thousands of devices for one time inventory. There may also be stringent time restriction for the entire inventory operation, e.g., hour level or even shorted. It’s easy to understand such service request may cause heavy load and possible heavy collision during random access procedure at the AIoT air interface.

Observation 4d: Although kind of upper layer filter/mask scheme may be supported, it’s still possible for BS reader to receive service requests, e.g., inventory for a very large number of devices which will further cause heavy load and possible heavy collision for the following random access procedure at the AIoT air interface.

Therefore, we think to apply addition mask/filter in the RAN level is needed with intention to reasonably split a CN/AF service request into several paging messages and each of the paging messages only aims at a smaller group containing fewer devices. By this way, it can be helpful to reduce the collision probability within each paging and improves the overall efficiency at AIoT air interface.

With reference to Observation 4c in section 2.2.2, if discrete device IDs are contained in the service request from CN/AF, the reader can additionally grouping them into several smaller groups. Or if some upper layer mask is provided to implicitly indicate multiple device IDs, the reader can apply additional “mask/filter”, e.g., on the “instance ID” part of structured device ID.

Proposal 5a: For a CN/AF service request with very large number of target devices, it is also necessary for the RAN to apply the additional mask/filter, e.g., on the “instance ID” part of structured device ID, with intention to reasonably split such service request into several air interface paging messages and each of them will address a smaller number of devices.

It’s easy to understand, to facilitate the reader to apply additional mask rules, it should at least know the structure of the device ID and the mask rules that have already been applied by upper layers.

Proposal 5b: To facilitate the reader to apply additional mask rules when necessary, the reader should at least know the structure of the device ID and the mask rules that have already been applied by upper layer.

- Conclusion

According to above discussion for paging procedure in air interface, the following proposals are given:

Proposal 1: For a certain service request from CN/AF, subsequent AIoT paging messages without device ID information but with a delta paging indication can be used to provide access occasion(s) with re-access purpose for the devices that have experienced contention resolution failures or Msg3 transmission failures.

Proposal 2: There is no need to introduce session ID in paging message

Proposal 3a: RAN2 assume the case that a device receives multiple paging messages from different readers can be avoided and no need to address it.

Proposal 3b: It’s suggested to introduce a reader ID in the paging message.

Proposal 3c: RAN2 discuss whether it’s needed to introduce a reader ID in the D2R messages, e.g., Msg3.

Proposal 4a: In order to let device know that that it needs to wait for the subsequent command data after successfully completing Msg3 transmission (e.g., in command only or inventory + command cases), it’s suggested to introduce indication about “having subsequent data” in paging message.

Proposal 4b: In order to let device know that it may need to wait for more time for the subsequent command data (e.g., in inventory + command cases), another indication about “expected waiting time for subsequent data” can also be considered.

Proposal 5a: For a CN/AF service request with very large number of target devices, it is also necessary for the RAN to apply the additional mask/filter, e.g., on the “instance ID” part of structured device ID, with intention to reasonably split such service request into several air interface paging messages and each of them will address a smaller number of devices.

Proposal 5b: To facilitate the reader to apply additional mask rules when necessary, the reader should at least know the structure of the device ID and the mask rules that have already been applied by upper layer.

- References

- RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, RAN#103, March, 2024

- TR 22.840 v19.0.0: "Study on Ambient power-enabled Internet of Things", 3GPP SA1, Dec. 2023

- TR 38.848 v18.0.0: "Study on Ambient IoT (Internet of Things) in RAN (Release 18)", RAN#101, Sep. 2023

- TR 23.700-13 v1.1.0 (2024-10), Study on Architecture support of Ambient power-enabled Internet of Things (Release 19), October. 2024