3GPP TSG-RAN WG2 Meeting #128			        R2-2409692

Orlando, United States, Nov 18th – 22nd, 2024

Agenda Item:	8.5.4 (Netw_Energy_NR-enh-Core)

Source: 	LG Electronics Inc.

Title:         	Discussion on common signal and channel adaptation

Document for: 	Discussion and Decision

Introduction

In this contribution, we discuss aspects related to the potential adaptations of the common signal/channel transmissions.

Discussion

- 2.1 Adaptation of PRACH in time domain

In RAN1#116bis meetings, it is agreed to define additional PRACH resource for NES-capable UEs, for PRACH adaptation in time domain. Further, in RAN1#118 meeting, it is agreed to use DCI for PRACH adaptation for NES-capable UE , e.g., to indicate activation/deactivation of NES PRACH resource.

[RAN1#116bis] Agreement For adaptation of PRACH in time-domain, support at least the following: - Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any) - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs - Configuration of additional PRACH resources is provided by semi-static signalling - FFS: details including whether there is overlap of additional PRACH resources and PRACH resources for legacy UEs - FFS: adaptation mechanism for additional PRACH resources - Note: No change to the existing PRACH configuration tables in 38.211 Agreement Support adaptation mechanisms of PRACH in time-domain for following: - UE in idle/inactive mode - UE in connected mode [RAN1#118] Agreement For the adaptation mechanism for additional PRACH resources (for CONNECTED mode UE and IDLE/INACTIVE mode UE), - At least DCI based adaptation is supported. No introduction of new DCI format. Agreement For DCI-based adaptation for additional PRACH resources, - Select from the following DCI format(s) to carry the adaptation indication. - DCI format 1_0 - DCI format 2_7 - DCI format 2_9 - FFS: existing (P-RNTI, SI-RNTI, CellDTRX-RNTI, PEI-RNTI, C-RNTI) or new RNTI used for detecting the DCI format.

Regarding configuration of additional PRACH resources, it should be provided via cell specific configuration (e.g., SIB1), given that additional PRACH resource can used by NES-capable UEs in RRC_IDLE/INACTIVE state. Since the legacy PRACH resource is provided via SIB1, it would be better to configure the additional PRACH resources for NES-capable UE by SIB1 as well.

Proposal 1. 	Additional PRACH resource for NES-capable UE is configured by SIB1.

In addition, according to the current specification, both 2-step RA and 4-step RA can be supported for legacy UE. In other words, if the additional PRACH resource for NES-capable UE is deactivated (e.g., by DCI) and legacy PRACH resource is configured for both 2-step RA and 4-step RA, NES-capable UE may perform 2-step RA based on the channel condition. Given that there is no critical reason to support PRACH adaptation mechanism only for 4-step RA resources, it would be beneficial to support additional PRACH resources for both 2-step RA and 4-step RA, in order to enable PRACH adaptation for both RA types.

Proposal 2. 	Support both 4-step RA and 2-step RA for PRACH adaptation in time domain.

- 2.2 Adaptation of paging occasions in the time domain

Regarding the adaptation of paging occasions, RAN2 made the following agreements so far:

RAN2#127bis:

1.	Select option-b as baseline for R19 NES paging enhancement.

2.	R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values)

3.	Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration.

4.	Legacy UE is not barred.

5.	Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

RAN2 selected option-b for paging occasion adaptation across the time domain

Option-b) Extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

It is FFS whether to extend the Ns value up to 8 or larger values. To configure larger Ns value, more bits should be reserved for configuration of firstPDCCH-MonitoringOccasionOfPO in SIB1. For this reason, some companies raised signalling overhead issue on option-b which requires more bits in SIB1. Since the available space in SIB1 is very limited, RAN2 agreed to aim at signalling overhead minimization in Ns extension.

The reason for introducing larger Ns is to compensate for the reduction in the paging capacity caused by the extended N value. The option-b obtains NES gain by increasing interval between PFs. As the interval between two PFs increases, the number of POs within a unit time decreases. To maintain the same number of POs with a unit time, Ns should increase in proportion to the interval between PFs, i.e. N. However, the paging adaptation aims to achieve the NES gain by reducing paging transmission when the paging load is low, so it is not necessary to fully guarantee the original number of POs. Regardless of how long interval between PFs is supported, the extended value of Ns should ensure an acceptable signalling overhead.

The required bits to configure NES specific firstPDCCH-MonitoringOccasionOfPO with different subcarrier spacing is shown in Table 1. (Table 1 assumes the paging cycle is 256sf and N is 1/256.)

Table 1. firstPDCCH-MonitoringOccasionOfPO bit with different SCS and Ns value

| SCS (Sub-Carrier Spacing) | Ns value |  |  |  |
| --- | --- | --- | --- | --- |
|  | 8 | 16 | 32 | 64 |
| 15kHz | 128 | 256 | 512 | 1024 |
| 30kHz | 136 | 272 | 544 | 1088 |
| 60kHz | 144 | 288 | 576 | 1152 |
| 120kHz | 152 | 304 | 608 | 1216 |
| 480kHz | 168 | 336 | 672 | 1344 |

Though it is FFS whether to support the paging adaptation with 480kHz, it requires 168 bits when Ns is 8. Considering the very limited available space of SIB1, it is already too huge, and it seems impossible to support Ns greater than 8. Therefore, we propose not to support Ns greater than 8.

Proposal 3	Extended Ns only up to 8.

It is also still FFS how much to extend N. N is the number of total paging frames in paging cycle and the maximum extendable value depends on the paging cycle. longer interval between PFs by configuring smaller N provide more NES gain, as it allows more time for gNB in sleep mode between each PF. For the maximum paging cycle rf256, if N is set to T/256, only one paging frame occurs within a paging cycle and gNB can sleep at most for 255frames. If Ns is extended only up to 8, a single PF within a paging cycle may not be sufficient to transmit all paging. However, if the paging load is very low, supporting the N=1 can provide the largest NES gain, so there is no reason to block support. If more POs are need, the network can configure bigger N.

Proposal 4	Extended N up to T/256.

Conclusion

Regarding adaptation of PRACH in time domain

Proposal 1.	Additional PRACH resource for NES-capable UE is configured by SIB1.

Proposal 2. 	Support both 4-step RA and 2-step RA for PRACH adaptation in time domain.

Regarding adaptation of paging occasions in the time domain

Proposal 3	Extended Ns only up to 8.

Proposal 4	Extended N up to T/256.