3GPP TSG-RAN WG2 Meeting #128						R2-2409695

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:	8.5.3

Source:	Samsung

Title:	On-demand SIB1

Document for:	Discussion and decision

# Introduction

RAN2 discussed procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode in RAN2#127bis and following agreements were made:

- We will inherit all agreements made during SI phase to WI phase.

- A NES cell can include neighbouring NES cell’s WUS configuration.

- NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

- Failure of OD-SIB1 acquisition

  - In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

  - The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred

  - The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

- Broadcast status for OD-SIB1

  - A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

  - If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- Cell barring

  - Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

  - NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

- WUS Configuration update between cell A and NES cell

  - RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

In this contribution we discuss further details of on-demand SIB1.

# Discussion

2.1. SIB1 request configuration validity

UE is camped on cell A. UE acquires SIB X transmitted by cell A. SIB X includes SIB1 request configuration for multiples cells (e.g. Cell B, Cell C and Cell D).

UE reselects to Cell B from Cell A. Cell B provides SIB1 on demand. UE applies SIB1 request configuration of Cell B from SIB X of Cell A to acquire SIB1 of Cell A.

Cell B does not provide SIB X OR Cell B provides SIB X, but this SIB X does not include SIB1 request configuration of Cell C.

Later UE reselects to Cell C from Cell B. Cell C provides SIB1 on demand. The issue is whether UE can apply SIB1 request configuration of Cell C from SIB X of Cell A or not? If yes, how does UE know whether SIB1 request configuration of Cell C from SIB X of Cell A is still valid?

SIB1 request configuration of Cell C is not associated with valueTag/area id and even if valueTag/area id can be signalled for SIB1 request configuration of Cell C, Cell C cannot indicate valueTag/area id in MIB. Timer based approach may also not work, as different UEs may acquire SIB1 request configuration at different times and start timer at different times.

The simplest approach would be that UE applies the SIB1 request configuration from the currently camped cell for SIB1 request.

Proposal 1: UE applies the SIB1 request configuration from the currently camped cell for SIB1 request.

- UE is camped on Cell A. For reselection from Cell A to Cell B, UE applies SIB1 request configuration of Cell B acquired from Cell A for SIB1 request.

- UE is camped on Cell A. For SIB1 request to Cell A, UE applies SIB1 request configuration of Cell A acquired from Cell A.

2.2. UL carrier for SIB1 request

In legacy SI request is supported for both SUL and NUL. Cell which supports on demand SIB1 can be configured with both SUL and NUL. So SIB1 request should be supported for both SUL and NUL. UE should select the UL carrier before transmitting SIB1 request. Since SIB1 request is based on random access and UL carrier selection is performed at the initiation of random access procedure in legacy, both NUL and SUL can easily be supported for SIB1 request. Not supporting SUL may have impact on random access procedure as exception (i.e. no UL carrier selection for the case random access procedure is initiated for SIB1 request) may need to be defined in MAC spec.

Proposal 2: SIB 1 request is supported for both SUL and NUL.

2.3 Msg1 repetitions for SIB1 request

Msg1 repetitions is supported for UL coverage in R18 for random access (including Msg1 based SI request). RACH based SIB1 request (similar to Msg1 based SI request) is agreed for on-demand SIB1. So, it is reasonable to support Msg1 repetitions for SIB1 request for better UL coverage.

Proposal 3: Msg1 repetitions is supported for SIB1 request.

2.4 Handling legacy UEs

RAN2 has agreed that legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset.

To further reduce burden on legacy UEs, network can include cells supporting on demand SIB1 to list of excluded cells. This helps legacy UEs to avoid PBCH/MIB decoding and measurements of cells supporting on demand SIB1.

Proposal 4: Network includes cells supporting on demand SIB1 to list of excluded cells for legacy UEs.

2.5 SIB1 request in RRC_CONNECTED

Case 1: RRC Connection re-establishment

If selected cell provides SIB1 on demand and SIB1 request is not supported in RRC_CONNECTED state, UE will bar the selected cell according to green highlighted text. This results in delayed connection re-establishment.

5.2.2.5  Essential system information missing

The UE shall:

1> if in RRC_IDLE or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running:

2> if the UE is unable to acquire the MIB:

3> consider the cell as barred in accordance with TS 38.304 [20];

3> perform barring as if intraFreqReselection, or intraFreqReselectionRedCap for RedCap UEs, or intraFreqReselection-eRedCap for eRedCap UEs, is set to allowed;

2> else if the UE is unable to acquire the SIB1:

3> consider the cell as barred in accordance with TS 38.304 [20];

One of the following can be considered:

- Option 1: Support SIB1 request in RRC_CONNECTED. For RRC connection re-establishment case, UE can send SIB1 request using msg1 based mechanism to the selected cell.

- Option 2: Leave it to UE implementation. UE may not select the NES cell during RRC connection re-establishment.

Case 2: RRC CONNECTED while T311 is not running

UE needs SIB1 to acquire other SIBs (pink highlighted conditions below).

5.2.2.3.1              Acquisition of MIB and SIB1

1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and pagingSearchSpace and has received an indication about change of system information; or

1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1, and, UE has not acquired SIB1 in current modification period; or

1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1, and, the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1, and, si-BroadcastStatus for the required SIB(s) or posSI-BroadcastStatus for the required posSIB(s) is set to notBroadcasting in acquired SIB1 in current modification period; or

1> if the UE is in RRC_CONNECTED while T311 is running:

2> if ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell (TS 38.213 [13]) and if SIB1 acquisition is required for the UE:

3> acquire the SIB1, which is scheduled as specified in TS 38.213 [13];

3> if the UE is unable to acquire the SIB1:

4> perform the actions as specified in clause 5.2.2.5;

3> else:

4> upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2.

2> else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell:

3> perform the actions as specified in clause 5.2.2.5.

If PCell provides SIB1 on demand and SIB1 request is not supported in RRC_CONNECTED state, UE will not acquire SIB1. No further action is taken by UE as per current spec. UE will not be able to acquire other SIBs.

One of the following can be considered:

- Option 1: UE sends SIB1 request

  - DedicatedSIBRequest or new RRC message can be used for SIB1 request. Alternately, UE can send SIB1 request as in RRC IDLE/INACTIVE. Whether UE can send SIB1 request in RRC_CONNECTED can be signalled by network in RRC message.

- Option 2: UE does not send SIB1 request. UE rely on network to provide the latest SIB1 via RRC message. However, this is not possible for pink highlighted case unless network sends SIB1 in dedicated manner to UE every modification period. We may consider that network sends change notification for si-BroadcastStatus changes if cell supports on demand SIB1. However, this may result in too much overhead.

Supporting SIB1 request in RRC_CONNECTED can be beneficial for both case 1 and case 2.

Proposal 5: RAN2 to consider supporting SIB1 request in RRC_CONNECTED.

# Conclusion

Proposal 1: UE applies the SIB1 request configuration from the currently camped cell for SIB1 request.

- UE is camped on Cell A. For reselection from Cell A to Cell B, UE applies SIB1 request configuration of Cell B acquired from Cell A for SIB1 request.

- UE is camped on Cell A. For SIB1 request to Cell A, UE applies SIB1 request configuration of Cell A acquired from Cell A.

Proposal 2: SIB 1 request is supported for both SUL and NUL.

Proposal 3: Msg1 repetitions is supported for SIB1 request.

Proposal 4: Network includes cells supporting on demand SIB1 to list of excluded cells for legacy UEs.

Proposal 5: RAN2 to consider supporting SIB1 request in RRC_CONNECTED.

# References