3GPP TSG-RAN WG2 Meeting #128		R2-2409704

Orlando, USA, 18th – 22nd Nov 2024

Source:	vivo

Title:          Discussion on LCM for UE-sided model for Beam Management

Agenda Item:	8.1.2.2

Document for:	Discussion and Decision

# Introduction

At RAN2#127bis meeting, RAN2 achieved the following agreements regarding LCM for UE-sided model for Beam management use case:

Agreements for BM

- UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.   We should aim to align the design on how the applicable functionality are signaled.   FFS on the applicability reporting content.

- FFS if inference configuration can be signalled in step3.

- UE can report to the network when an applicable AI functionality becomes non-applicable.  FFS how this is signaled (e.g. explicitly/implicitly).  Consider different scenarios, whether it is regarding an active functionality)

- Data collection initiation and configuration for data collection is under network control.  FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server)

- For the purpose of discussion of AI/ML BM LCM operations, existing procedures and terminologies from the CSI Framework should be used, including those defined for aperiodic, semipersistent on PUCCH, semipersistent on PUSCH, and periodic reporting configurations (as/if defined in RAN1 pending response LS from RAN1).

- For now, RAN2 will not define terminology specific to the activation or deactivation for AI/ML models.  Can come back to this discussion later.

In this contribution, we will further discuss the functionality-based LCM for UE-sided model for Beam Management use case, focusing on issues not dependent on RAN1 reply.

# Discussion

## 2.1 Applicability reporting

The applicability reporting content is FFS. One issue is whether to support the reporting of neighbouring cell’s applicability. If supporting the reporting of neighbouring cell’s applicability, target gNB can get the applicable functionalities for target cell in handover preparation message and configure/activate the functionality in the handover command message. The signaling procedure may be as below:

Figure 1: the reporting of neighbouring cell’s applicability

One question is when source gNB performs steps 2/3 in Figure 1 to obtain the applicability reporting for target cell. If it is after handover decision, the handover failure rate will increase since it will extend the duration of handover preparation, which is undesirable.

- If reporting of neighbouring cell’s applicability is supported and is reported after handover decision, the handover failure rate will increase.

If source gNB obtains the applicability reporting for target cell before handover decision, source gNB (i.e., serving gNB) may not know in advance which neighbouring cell will be target cell. Therefore, applicability status associated with all the potential target cells needs to be reported. It is likely that there are multiple potential target cells around source cell. It will greatly increase the signaling overhead for applicability reporting. Moreover, the UE/gNB needs to maintain the updated applicability status for all the cells. It means that if the applicability status of any functionality of any cell changes (applicable -> non-applicable, or non-applicable -> applicable), the UE needs to trigger the UAI message, which greatly increases the frequency of reporting.

- If reporting of neighbouring cell’s applicability is supported and is reported before handover decision, the signaling overhead will increase significantly because applicability status associated with all the potential target cells needs to be reported and the applicability status change of any functionality of any cell will trigger the UAI message.

Besides, serving gNB may need to obtain the NW-sided additional conditions of neighbouring cells, e.g., associated IDs (FFS whether more information are needed depending on RAN1’s input), from neighbouring gNBs, which is in the scope of RAN3.

- If reporting of neighbouring cell’s applicability is supported, extra inter-node interaction is needed, which will require RAN3 involvement.

However, even if the reporting of neighbouring cell’s applicability is not supported, target gNB can still get the applicable functionalities during handover. Specifically, target gNB can includes the otherConfig IE in handover command message. Since RAN2 has agreed that RRCReconfigurationComplete message can be used to report applicable functionality, the UE can send the applicability reporting to target cell in the RRCReconfigurationComplete message (i.e., handover complete message) once the UE accesses to target cell. Then target gNB configures/activates the functionality.

- The UE can send the applicability reporting to target cell in the RRCReconfigurationComplete message (i.e., handover complete message) once the UE accesses to target cell if target gNB includes the otherConfig IE in the handover command.

It is not so critical to configure/activate the functionality in the handover command message at least for beam management use case. Taking the complexity into account, we think that it is unnecessary to support neighbouring cell’s applicability in applicability reporting. It is simple and has less specification impacts that the UE sends the applicability reporting after the UE accesses to target cell.

- For beam management, neighbouring cell’s applicability is not included in applicability reporting.

Another issue is the signaling design for non-applicable functionality reporting, i.e., when an applicable AI functionality becomes non-applicable. There may be two potential signaling designs:

- Alt1: For initial reporting, the UE reports the list of applicable functionalities. For subsequent reporting, the UE reports the change information (i.e., delta signaling using ToAddList and ToReleaseList structure). The ASN.1 example is for reference:

UEAssistanceInformation ::=         SEQUENCE {

...

applicabilityReporting ApplicabilityReporting OPTIONAL,

...

}

ApplicabilityReporting ::=     SEQUENCE {

applicableFuncToAddList               ApplicableFuncToAddList             OPTIONAL,

applicableFuncToReleaseList           ApplicableFuncToReleaseList         OPTIONAL

}

ApplicableFuncToAddList ::= SEQUENCE (SIZE (1..maxApplicableFunc)) OF ApplicableFunctionality

ApplicableFuncToReleaseList ::= SEQUENCE (SIZE (1..maxApplicableFunc)) OF FunctionalityIndex

- Alt2: The UE always reports the (updated) list of applicable functionalities regardless of initial reporting, or subsequent reporting (i.e., full-configuration-like signaling). If there is no applicable functionality, the list can be set to zero element. The ASN.1 example is for reference:

UEAssistanceInformation ::=         SEQUENCE {

...

applicabilityReporting ApplicableFuncList OPTIONAL,

...

}

ApplicableFuncList ::= SEQUENCE (SIZE (0..maxApplicableFunc)) OF ApplicableFunctionality

With Alt1, functionality ID or functionality index needs to be introduced to identify the functionalities which need to be released. Compared with Alt2, the IE structure is a little more complex. The core difference of Alt1 and Alt2 is that Alt1 is delta signaling design and Alt2 is full-configuration-like signaling design. If the reporting is frequent, e.g., applicability status frequently changes, Alt1 can save the signaling overhead and be better.

- Regarding the signaling design for applicability reporting, RAN2 considers the following two alternatives:

- Alt1: For initial reporting, the UE reports the list of applicable functionalities. For subsequent reporting, the UE reports the change information (i.e., delta signaling using ToAddList and ToReleaseList structure).

- Alt2: The UE always reports the (updated) list of applicable functionalities regardless of initial reporting, or subsequent reporting (i.e., full-configuration-like signaling). If there is no applicable functionality, the list can be set to zero element.

## 2.2 Signalling to configure the UE for training

For the training of the UE-sided model, the UE needs to obtain the training configuration (e.g., associated ID(s), CSI-RS configuration for taining) of the serving cell. For beam management use case, the UE is in connected state when collecting the training data. Therefore, there may be two possible approaches to obtain the training configuration of the serving cell:

- Approach 1: via the system information of the serving cell

Figure 2: via system information

- Approach 2: via the RRC dedicated signaling, e.g., after requested by the UE

Figure 3: via RRC dedicated signaling

For positioning feature, the network can broadcast the positioning assistance information via the posSIBs, or the network can send the related information via dedicated signaling. Similar to positioning feature, we think that both of the above two approaches should be supported and it is left to network implementation to support which one or both.

With Approach 1, it is up to NW implementation whether to broadcast the training configuration.

With Approach 2, it is FFS how the NW determines whether data collection should be initiated. We think that at least UE direct request can be supported to trigger the NW sending the training configuration. For other cases, it can be left to NW implementation or there is no specification impact (e.g., UE server request). Therefore, RAN2 does not need discuss them.

- For the signaling to configure the UE for training, support both two approaches:

- NW broadcasts the UE collection configuration;

- NW sends the collection configuration to the UE via dedicated signaling after receiving UE’s direct request.

It is left to network implementation to support which one or both.

# Conclusion

In the contribution, we have the following observations and proposals:

Applicability reporting

- If reporting of neighbouring cell’s applicability is supported and is reported after handover decision, the handover failure rate will increase.

- If reporting of neighbouring cell’s applicability is supported and is reported before handover decision, the signaling overhead will increase significantly because applicability status associated with all the potential target cells needs to be reported and the applicability status change of any functionality of any cell will trigger the UAI message.

- If reporting of neighbouring cell’s applicability is supported, extra inter-node interaction is needed, which will require RAN3 involvement.

- The UE can send the applicability reporting to target cell in the RRCReconfigurationComplete message (i.e., handover complete message) once the UE accesses to target cell if target gNB includes the otherConfig IE in the handover command.

- For beam management, neighbouring cell’s applicability is not included in applicability reporting.

- Regarding the signaling design for applicability reporting, RAN2 considers the following two alternatives:

- Alt1: For initial reporting, the UE reports the list of applicable functionalities. For subsequent reporting, the UE reports the change information (i.e., delta signaling using ToAddList and ToReleaseList structure).

- Alt2: The UE always reports the (updated) list of applicable functionalities regardless of initial reporting, or subsequent reporting (i.e., full-configuration-like signaling). If there is no applicable functionality, the list can be set to zero element.

Signalling to configure the UE for training

- For the signaling to configure the UE for training, support both two approaches:

- NW broadcasts the UE collection configuration;

- NW sends the collection configuration to the UE via dedicated signaling after receiving UE’s direct request.

It is left to network implementation to support which one or both.

# Reference

- RP-240774 Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface