3GPP TSG-RAN WG2 Meeting #128      	       	R2-2409706

Orlando, USA, 18th- 22nd Nov 2024

Source:	vivo

Title:	Discussion on NW side data collection

Agenda Item:	8.1.3

Document for:	Discussion and Decision

# Introduction

At the RAN2#127bis meeting, on NW side data collection, RAN2 made the following agreements[1]:

Agreements on NW side data collection

- Periodic logging is supported for training data collection procedure in R19

- Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

- Periodic reporting of logged data is not supported.

- On-demand reporting of the logged measurements will be specified

- UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.   FFS of details of the message

- The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network.

- Low priority SRB will be used.    FFS new SRB or use of SRB4

- For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress.

In this contribution, we will further discuss some remaining issues on NW-side data collection for model training for beam management.

# Discussion

## Event-triggered logging

RAN2 has agreed that event-triggered data logging will be supported. At least radio condition-based event triggered logging will be supported. RAN2 has also agreed that, for data collection for both NW-sided/UE sided BM model training, L1-RSRPs need to be collected by UE. Based on network model deployment and model performance, operator may consider model re-training for some areas with poor model performance, e.g., cell edge with low RSRP or cell center with high RSRP. That is, collecting data for both good and bad radio conditions is beneficial to improve the model performance at specific radio status in a high-efficiency way.

Therefore, we propose:

- Radio channel condition such as RSRP threshold, (e.g., RSRP <= Thresh1 or RSRP >= Thresh2) can be considered as event-triggered condition for data logging for beam management.

On whether additional radio conditions would be Event-triggered for data logging, RAN2 should wait for RAN1 progress on other data to be collected.

In addition, RAN1 discussed and reached the following WA regarding associated ID.

Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

The above WA is related to the generalization of the model for beam management. Specifically, when the associated ID of a specific cell is the same during the training and inference phases, the UE assumes the NW-side additional conditions for UE-sided model are fulfilled.

Correspondingly, for NW-sided model, only the data collected by the UE under specific cell(s) with the specific associated ID(s) can be used for model training.

- Introduce an area (e.g., cell ID) configuration to restrict the area in which the UE performs training data logging. And the area may associate with associated ID(s).

## Availability indication of logged data

At the last RAN2 meeting, it was agreed that the UE can indicate the availability of logged data to the network to assist network to trigger UEInformationRequest message. The trigger/definition of availability indication is FFS. From our understanding, at least the following cases should be considered:

- When logging data, based on NW configuration, UE AS buffer may be full, and UE can indicate it to network via UAI. In this case, UE does not need to indicate how much data it has in its buffer; the indication can just be a single bit to NW. Based on this indication the network then can request to report the logged data.

- When the serving cell does not configure or enable UE to submit UAI, UE may not be able to report this availability indication to the network. To address this issue, similar to other reports in MDT (e.g., logged MDT, RLF), the availability indication can be introduced in the UE-MeasurementsAvailable within the complete message. During the SI phase, it was agreed that the data collection for model training should focus on the RRC_CONNECTED state (for both data generation and reporting). Furthermore, to avoid further discussion on the RRC status transition, we think the UE should release the training data collection configuration and logged training data upon transitions from RRC_CONNECTED state to other states.

UE-MeasurementsAvailable information element -- ASN1START -- TAG-UE-MeasurementsAvailable-START UE-MeasurementsAvailable-r16 ::= SEQUENCE { logMeasAvailable-r16 ENUMERATED {true} OPTIONAL, logMeasAvailableBT-r16 ENUMERATED {true} OPTIONAL, logMeasAvailableWLAN-r16 ENUMERATED {true} OPTIONAL, connEstFailInfoAvailable-r16 ENUMERATED {true} OPTIONAL, rlf-InfoAvailable-r16 ENUMERATED {true} OPTIONAL, ..., [[ successHO-InfoAvailable-r17 ENUMERATED {true} OPTIONAL, sigLogMeasConfigAvailable-r17 BOOLEAN OPTIONAL ]], [[ successPSCell-InfoAvailable-r18 ENUMERATED {true} OPTIONAL ]] } -- TAG-UE-MeasurementsAvailable-STOP -- ASN1STOP

- After the NW trigger UEInformationRequest message, the UE will feedback the logged data via UEInformationResponse message. However, the logged data may exceed the maximum size that one single RRC message can carry. Similar to logged MDT, the UEInformationResponse can include another availability indication to assist network to trigger another UEInformationRequest message.

Based on the above analysis, we propose:

- A single bit indication would be sufficient for UE to indicate to NW that UE has available logged data. No additional information is needed.

- The availability indication of logged data can be sent to network via the following messages:

- UEAssistanceInformation, if configured to provide availability indication of logged data

- RRCReconfigurationComplete

- UEInformationResponse

- UE should release the training data collection configuration and logged training data upon transitions from RRC_CONNECTED to RRC_INACTIVE/IDLE.

## SRB for logged data reporting

RAN2 has agreed to use SRB to report the logged data to network. But RAN2 did not conclude on the SRB type, i.e., whether to introduce a new SRB or reuse SRB4. For the motivation to introduce a new SRB, some argue to have a low priority SRB. We do not see the necessity to introduce a new SRB given that SRB4 is already a low priority SRB for QoE reporting. Thus, it’s straightforward to reuse SRB4 for logged training data reporting.

- SRB4  is reused for logged training data reporting.

# Conclusion

In this contribution, some remaining issues on NW-side data collection for model training for beam management and we have the following proposals:

Event-triggered data logging

- Radio channel condition such as RSRP threshold, (e.g., RSRP <= Thresh1 or RSRP >= Thresh2) can be considered as event-triggered condition for data logging for beam management.

- Introduce an area (e.g., cell ID) configuration to restrict the area in which the UE performs training data logging. And the area may associate with associated ID(s).

Availability of logged data

- A single bit indication would be sufficient for UE to indicate to NW that UE has available logged data. No additional information is needed.

- The availability indication of logged data can be sent to network via the following messages:

- UEAssistanceInformation, if configured to provide availability indication of logged data

- RRCReconfigurationComplete

- UEInformationResponse

- UE should release the training data collection configuration and logged training data upon transitions from RRC_CONNECTED to RRC_INACTIVE/IDLE.

SRB for data reporting

- SRB4  is reused for logged training data reporting.

- Reference

- RAN2#127bis Chairman notes