3GPP TSG-RAN WG2 Meeting#128	R2-2409708

Orlando, USA, 18th – 22nd Nov 2024

Source:	vivo

Title:	Discussion on A-IoT Paging

Agenda Item:	8.2.3

Document for:	Discussion and Decision

- Introduction

RAN2#127bis meeting [1] has come to several agreements have been reached on the functionality of paging in the context of Ambient IoT:

Agreements

- The A-IoT paging message can include information to avoid duplicate response from the device to a reader.  This information should be short and simple.  FFS how to indicate.  Wait for further information and requirements from other WGs and make this decision in normative phase.

- Based on this information, the device determines whether to skip sending the response to paging.

- From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs.    RAN2 will determine the need for this after considering aspects related to RAN2 multiplexing and other WGs.

- If multiple device IDs in single paging is supported, if A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA.

In this contribution, we further discuss some remaining issues on A-IoT paging left for SI phase.

- Discussion

## Content in paging message

### 2.1.1	Information to differentiate service types

In the last meeting, the issue of whether service type is needed in paging message was raised. We see little benefit from inclusion of paging cause to indicate the service type in the A-IoT paging message. In NR, paging cause is introduced to assist a device to determine how to handle a received paging message, e.g. respond the paging message immediately or ignore the paging message. For example, for the dual-USIM UE, if a USIM is paged with a low priority cause value while the other USIM has a high priority service ongoing, the UE may ignore the paging triggered by low priority service, assuming that the two USIMs cannot work simultaneously. On the contrary, due to the low capability of A-IoT device, paging is what it must respond to once it is paged. Besides, since paging message is used to address all types of the A-IoT service, to finish any service, A-IoT device should respond to paging message with no differentiation in the service type. Thus, there is no need for a device to be informed about the cause why it is being paged for.

- NR paging cause helps UE to decide on service priority and whether to respond; while A-IoT device needs to respond to A-IoT paging messages with the same priority regardless of A-IoT service type.

- No need to include an AS paging cause to indicate A-IoT service type in the A-IoT paging message, i.e., device identifies the service directly from the upper layer container in the A-IoT paging message.

### 2.1.2	Information to differentiate readers for the same service

In parallel, RAN3 is discussing the proximity issue for A-IoT device and has come with the following conclusion [2].

Use cases for locating an AIoT device: - Find an appropriate “reader” close to the A-IoT device; - Find where the A-IoT device is. Support locating the A-IoT device at “reader” granularity. AIoT device location information may be used for the following purposes: (a) improving the AIoT operation itself, e.g. by sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly. (b) providing location information to the consumer of the AIoT service. Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes. Topology 1 For support of AIoT device location, the AIoT RAN node may report an identifier of the reader in the Inventory Report message to the AIoT CN and the AIoT CN knows the location of the identified reader. Topology 2 For support of AIoT device location, information is reported to the AIoT CN in the Inventory Report to enable the AIoT CN to determine the location of the AIoT_enabled UE Reader to which the AIoT device responded. The information enabling the AIoT CN to determine the location of the AIoT_enabled UE Reader to which the AIoT device responded to the inventory may be one or more of the following: (a) the AIoT_enabled UE Reader knows its location (e.g. if AIoT_enabled UE has GNSS info) and this is reported to the AIoT CN in the Inventory Report. (b) an identifier of the AIoT_enabled UE Reader is reported to the AIoT CN in the Inventory Report and the AIoT CN has determined the location of the identified UE using legacy LCS procedure(s). (c) an identifier of the AIoT_enabled UE Reader is reported to the AIoT CN in the Inventory Report and the AIoT CN has determined the location of the identified UE via User location information reported by the AIoT_enabled gNB (e.g. in almost all UL NGAP messages).

- RAN3 has agreed:

- the scenario that CN may send a Command to multiple readers does exist;

- an identifier of A-IoT reader is employed for CN to determine the location of the reader to which the A-IoT device responded.

However, as agreed in the last meeting, information to avoid duplicate response from the device to a reader can be included in the A-IoT paging message. As for the multi-reader scenario, device may receive paging messages with the same information from different readers. In this sense, the device may only respond once for the sake of avoiding repeated responses to the same service and resource consumption in A-IoT interface. However, if the service is of the proximity purpose, the device is expected to respond to different readers for the accurate positioning of the device. That is, if device is inventoried under multiple readers, the position of the device is around the junction of these readers.

One way to solve the issue is to directly include the reader ID in paging message in order that the device can identify the paging messages with the same purpose are from the different readers. Then device may perform access to different readers accordingly. However, the device has no insight about location service even with revealed reader ID. It is the reader that can be informed of location purpose from CN. The reader has the obligement to inform the device to respond to the paging for this certain service without revealing the service type directly. In this understanding, another way is that the reader includes an indication in a paging message about whether the device should always respond to the paging message regardless of whether access successes or not. Besides, it is easier to count on NW implementation by assigning different session IDs to different readers but for a same proximity service. In this way, device can respond to every paging message associated to the same locating service implicitly ensured by CN.

- RAN2 to support the scenario that one A-IoT device can be required to respond to A-IoT paging messages associated with the same service from different readers.

- RAN2 to discuss the following ways to fulfil the need of responses to multiple readers:

- Option 1: including reader ID in A-IoT paging message;

- Option 2: including an indication in A-IoT paging message, to indicate that UE should respond to the current A-IoT paging message regardless of whether access successes;

- Option 3: CN implementation to group the responses from different readers for the same service with no further enhancement in A-IoT paging message.

## Device behavior upon receiving paging messages

In consideration to information to avoid duplicate response in a paging message, it is only agreed that device can determine whether to skip sending the response to the paging message. We would like to further talk about the detailed criterion on how the device makes the determination.

Based on the progress in Random access, the re-access can be triggered for contention resolution failure as a baseline scheme, and an optional failure indication about MSG3/ the following D2R data transmission. For CFRA case, the failure indication is also applied for a failed MSG2 transmission to let device aware of the re-access necessity. In this understanding, from the device perspective, the respond to paging is determined by whether contention is resolved or whether a failure indication is received. If one of the conditions are not satisfied, the device should respond to the paging message in even showing the same information as the last one it has replied to.

- A-IoT device should respond to an A-IoT paging message under the condition that:

- the paging message is the first received paging message; or

- the message includes different information (to avoid duplicate responses) from the last paging message the device responds; or

- the message includes the same information (to avoid duplicate responses) as the last one the device responds, and the last response fails.

## Split of paging message

Since RAN2 has agreed that it is a valid case that multiple device IDs are carried in one A-IoT paging message, where the exact format of paged device ID should be up to SA2 design. In this sense, the identification of device(s) can take up a huge number of bits in paging message. Except for the device identification, paging also needs to carry the AS transmission information, such as the agreed information to avoid device’s repeated accesses and scheduling parameters for random access. And for the part to trigger random access, it can be sent from reader usually. If the above information is contained in one single message, it may cause a lot of burden for transmission on A-IoT interface. Besides, segmentation in R2D direction is not agreed yet, which makes it more necessary to reduce the message size in terms of paging message.

- A-IoT Paging message is used as Msg0 in random access procedure, and sent for multiple times.

- Based on the agreed content of A-IoT paging message, the message size of a single A-IoT paging message can be huge.

Seeing from the above analysis, the transmitting frequency and signalling overhead leads to the idea that paging can be realized by not only a single message. As mentioned, the agreed content can be partitioned naturally into information from CN (e.g., device identification from upper layer) and AS parameters related to random access. We consider it is useful to split the agreed contents based on their functions into two individual R2D messages to realize the original functionality of paging. In turn, it eases the burden for reception of large message over A-IoT interface from device point of view.

- RAN2 to discuss whether paging can be realized by two separate messages based on the functionality of contents, i.e., information from CN (e.g., device identification) and RA-related parameters. Whether to combine or split them can be up to reader implementation.

## Impact from device unavailability on paging

RAN1 parallel study [3] over A-IoT device unavailability for energy harvesting shows no RAN2 impact at least for the current stage.

The following directions, not for down-selection, are studied regarding the potential impact of device unavailability due to energy harvesting: Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable. Direction 2: Reader can provide information to a device based on which the device may become available/unavailable.

From our point of view, the minimum impact from device unavailability in Direction 2 can be relieved by implementation. For example, the charging pattern of devices within the range of reader is known by itself according to some certain periodicity, while such periodicity can be controlled according to the period of paging round based on reader implementation. In this understanding, A-IoT device unavailability has little impact on paging.

- There is no impact from device unavailability to paging at least in SI phase from RAN2 perspective, which be revisited during WI based on RAN1 further input, if any.

- Conclusion

On a basis of above analysis, we make the following proposals regarding AIoT paging:

Content in paging message

- Information to differentiate service types

- NR paging cause helps UE to decide on service priority and whether to respond; while A-IoT device needs to respond to A-IoT paging messages with the same priority regardless of A-IoT service type.

- No need to include an AS paging cause to indicate A-IoT service type in the A-IoT paging message, i.e., device identifies the service directly from the upper layer container in the A-IoT paging message.

- Information to differentiate readers for the same service

- RAN3 has agreed:

- the scenario that CN may send a Command to multiple readers does exist;

- an identifier of A-IoT reader is employed for CN to determine the location of the reader to which the A-IoT device responded.

- RAN2 to support the scenario that one A-IoT device can be required to respond to A-IoT paging messages associated with the same service from different readers.

- RAN2 to discuss the following ways to fulfil the need of responses to multiple readers:

- Option 1: including reader ID in A-IoT paging message;

- Option 2: including an indication in A-IoT paging message, to indicate that UE should respond to the current A-IoT paging message regardless of whether access successes;

- Option 3: CN implementation to group the responses from different readers for the same service with no further enhancement in A-IoT paging message.

Device behavior upon receiving multiple paging messages

- A-IoT device should respond to an A-IoT paging message under the condition that:

- the paging message is the first received paging message; or

- the message includes different information (to avoid duplicate responses) from the last paging message the device responds; or

- the message includes the same information (to avoid duplicate responses) as the last one the device responds, and the last response fails.

Split of paging message

- A-IoT Paging message is used as Msg0 in random access procedure, and sent for multiple times.

- Based on the agreed content of A-IoT paging message, the message size of a single A-IoT paging message can be huge.

- RAN2 to discuss whether paging can be realized by two individual messages based on the functionality of contents, i.e., information from CN (e.g., device identification) and RA-related parameters. Whether to combine or split them is up to reader implementation.

Impact from device unavailability on paging

- There is no impact from device unavailability to paging at least in SI phase from RAN2 perspective, which be revisited during WI based on RAN1 further input, if any.

- Reference

- Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China

- R3-245827	[TP for TR 38.769] TP for Locating Ambient IoT Devices

- TR 38.769	Study on solutions for ambient IoT (Internet of Things)