3GPP TSG RAN WG2 #128                                                                                   R2-2409717

Orlando, USA, Nov 18th – Nov 22nd, 2024

Source:	CATT

Title:	Consideration on LCM for Positioning use case

Agenda Item:	8.1.2.3

Document for:	Discussion and Decision

- Introduction

In last RAN2 meeting [1], some agreements have been achieved for LCM for Positioning use case:

Agreements: 1: The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. Step 5: The LMF requests the inferred location information using the LPP request location information message. Step 6: UE reports the inferred location using LPP provide location information message. 2: Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress). 3: Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress). 4: FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively). FFS the signalling details. 5: RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

Since aspects related to data collection should be covered in other section, in this contribution, we continue to discuss the LCM procedure related issues on Positioning use case.

- General LCM procedure

In last RAN2 meeting, it is agreed that UE reports the applicable functionality to the LMF by the LPP provide capabilities message. But it is FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively). FFS the signalling details.

In the LCM for BM case, we think step4 can be sent proactively using UAI message if e.g. an applicable functionality becomes non-applicable. For PoS case, change of applicable functionality is also possible. Since in TS37.355, the unsolicited capabilities can be sent from Target to the Server by unsolicited ProvideCapabilities message. This message can be easily reused to transmit the changed applicable functionality for AI/ML usage. But the signaling details should wait for RAN1’s progress.

Proposal 1: Support unsolicited LPP provide capabilities message for AI/ML based applicable functionality change. Signaling details should wait for RAN1’s progress.

- Positioning method

In the last RAN2 meeting, it is agreed RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

In RAN1 agreements, many parameters in the existing positioning methods are reused, such as timing information of DL RSTD or UE Rx-Tx time difference, LOS/NLOS soft/hard indicator, .etc. In TS38.305, the existing positioning methods are listed as below. The 3 highlighted lines are the NR UE side positioning which parameters are mentioned by RAN1 agreements:

Table 4.3.1-1: Supported versions of UE positioning methods

| Method | UE-based | UE-assisted, LMF-based | NG-RAN node assisted | SUPL Note 8 |
| --- | --- | --- | --- | --- |
| A-GNSS | Yes | Yes | No | Yes |
| OTDOA Note1, Note 2 | No | Yes | No | Yes |
| E-CID Note 4, Note 7 | No | Yes | Yes | Yes for E-UTRA |
| Sensor | Yes | Yes | No | No |
| WLAN | Yes | Yes | No | Yes |
| BluetoothNote 9 | Yes | Yes | No | No |
| TBS Note 5 | Yes | Yes | No | Yes (MBS) |
| DL-TDOA | Yes | Yes | No | Yes |
| DL-AoD | Yes | Yes | No | Yes |
| Multi-RTT | No | Yes | Yes | Yes |
| NR E-CID | No | Yes | Yes | Yes (DL NR E-CID) |
| UL-TDOA | No | No | Yes | Yes |
| UL-AoA | No | No | Yes | Yes |
| NOTE 1: This includes TBS positioning based on PRS signals. NOTE 2: In this version of the specification only OTDOA based on LTE signals is supported. NOTE 3: Void NOTE 4: This includes Cell-ID for NR method when UE is served by gNB. NOTE 5: In this version of the specification only for TBS positioning based on MBS signals. NOTE 6: Void NOTE 7: Enhanced Cell ID based on LTE signals. NOTE 8: This shows whether the positioning method is supported by SUPL ULP [16]. NOTE 9: UE-based positioning mode is supported only for Bluetooth AoD positioning method. |  |  |  |  |

Therefore, we think it is unnecessary to introduce new positioning method for AI/ML positioning. The AI/ML positioning related UE-side model cases could be considered as enhancements base on the existing positioning methods, e.g. DL-TDOA, DL-AoD, Multi-RTT. And some AI/ML specific parameters could be added into these methods which should depend on RAN1’s discussion.

Proposal 2: Consider AI/ML positioning of UE-side model as enhancements based on the existing positioning methods, e.g. DL-TDOA, DL-AoD, Multi-RTT.

- Applicability determination and Functionality management

The functionality management for PoS case has not been discussed in the last RAN2 meeting. In RAN1 R19 WI stage, the word of “functionality” is almost not used, and RAN1 has a lot of discussion on the LS from RAN2 [2]. Based on the LS, RAN1 agreement lists 3 options for applicability for inference for UE-side model, and concluded that the meaning and the granularity of “functionality“ for Applicable functionalities, Activated functionalities and Supported functionalities defined by RAN2 may or may not be the same with RAN1. The agreements have been achieved in RAN1 as below:

Agreement Incorporating below text into the general part of the LS In RAN1’s discussion of RAN 2 terminologies on beam management, - The concept/terminology “functionality“ of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-specific FGs - The concept/terminology “ functionality“ of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters or information/parameters indicated by UE - The Activated functionalities may be enabled based on CSI framework. Therefore, the meaning and the granularity of “functionality“ for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same, depends on certain option in RAN1, and the discussion is still ongoing

Observation 1: for BM case, the meaning and the granularity of “functionality“ for Applicable functionalities, Activated functionalities and Supported functionalities defined by RAN2 may or may not be the same with RAN1, and 3 options about applicability are summarized by RAN1 for ongoing discussion.

Furthermore, there is no any RAN1 agreement or even discussion about “NW-side additional condition” for PoS case. We don’t think the Applicability and Functionality management could be discussed on RAN2 without RAN1’s progress on such issue.

Observation 2: There is no RAN1 agreement and there is not even any discussion about “NW-side additional condition” for PoS case.

Proposal 3: For PoS case, RAN2 to postpone the discussion about Applicability and Functionality management and wait for RAN1’s progress.

- PoS Case 1

It is approved to focus on the first priority issues (case1/3a/3b) until end of 2024 in RAN#105 meeting [3]. And inside 3 first priority cases, RAN3 has already discussed on case 3a and 3b and some agreements have been achieved in last RAN3 meeting. Therefore, RAN2 should first focus on the specification impact on PoS case 1.

  - Inference related

For case 1, the UE-side model is deployed at UE side. There is no any agreement about the inference input/output of PoS case 1 in RAN1 discussion until RAN1#118 meeting.

The basic assumption is that UE-side model is trained by UE side (including by OTT server), and thus the type or format of the measurement corresponding to AI/ML model input is naturally known at UE side. If the AI/ML model is trained at other side such as NW side, assistance information on input type or format may be needed. But model transfer/delivery itself is still under without clear standardization support. Therefore, when UE uses an UE-side AI/ML model for inference, the UE can generate the measurement for AI/ML model inference using the same type or format of the measurement as training phase, by e.g. UE implementation or assistance information.

Proposal 4: For PoS case 1, UE generates the measurement for AI/ML model inference using the same type or format of the measurement as training phase.

For case 1, the model output is the location of UE. Therefore the legacy LPP message of ProvideLocationInformation can be directly reused. We do not foresee specification impact based on the model output on its own, but we think the LMF needs to understand whether the UE’s location is based on prediction or based on measurement. So an explicit indicator or a new field of predicted UE location may need to be introduced.

Proposal 5: For PoS case 1, RAN2 to discuss whether an explicit indicator or a new field of predicted UE location is needed to assist the LMF to identify the predicted/measured result.

For the reference signaling, RAN1 has the agreement for training data collection:

Agreement For training data collection of Case 1 and 2a, in terms of DL PRS configuration for collecting training data, RAN1 study the following options on assistance data, using legacy mechanisms as a starting point: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

The RS should be the same for training and inference for consistency. Since current LPP could already support the Options above, no RS request or transmission related specification impact is foreseen.

Observation 3: For PoS case 1, no RS request or transmission related specification impact is foreseen for model inference.

  - Monitoring related

For case 1, RAN1 has listed multiple options with regard to how to generate information on ground truth label:

- Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF.

Until RAN1#118 meeting, it is only concluded that Option A-4 could be realized by implementation and no further discussion is needed on Option A-4. All the other options are not discussed. Therefore, no discussion is needed on case 1 performance monitoring in RAN2 until triggered by RAN1 input.

Observation 4: All the specification impact related options regarding to the ground truth label generation for case 1 performance monitoring have not been discussed in RAN1 yet.

Proposal 6: For PoS case 1, no discussion is needed for performance monitoring in RAN2 until triggered by RAN1 input.

- Conclusion

In this contribution, we provide our views on the LCM related issues of for Positioning use case, and the observation and proposals are summarized as follows:

General LCM procedure

Proposal 1: Support unsolicited LPP provide capabilities message for AI/ML based applicable functionality change. Signaling details should wait for RAN1’s progress.

Positioning method

Proposal 2: Consider AI/ML positioning of UE-side model as enhancements based on the existing positioning methods, e.g. DL-TDOA, DL-AoD, Multi-RTT.

Applicability determination and Functionality management

Observation 1: for BM case, the meaning and the granularity of “functionality“ for Applicable functionalities, Activated functionalities and Supported functionalities defined by RAN2 may or may not be the same with RAN1, and 3 options about applicability are summarized by RAN1 for ongoing discussion.

Observation 2: There is no RAN1 agreement and there is not even any discussion about “NW-side additional condition” for PoS case.

Proposal 3: For PoS case, RAN2 to postpone the discussion about Applicability and Functionality management and wait for RAN1’s progress.

PoS Case 1

Proposal 4: For PoS case 1, UE generates the measurement for AI/ML model inference using the same type or format of the measurement as training phase.

Proposal 5: For PoS case 1, RAN2 to discuss whether an explicit indicator or a new field of predicted UE location is needed to assist the LMF to identify the predicted/measured result.

Observation 3: For PoS case 1, no RS request or transmission related specification impact is foreseen for model inference.

Observation 4: All the specification impact related options regarding to the ground truth label generation for case 1 performance monitoring have not been discussed in RAN1 yet.

Proposal 6: For PoS case 1, no discussion is needed for performance monitoring in RAN2 until triggered by RAN1 input.

- References

- RAN2_127b_Meeting_Report

- R1-2407604/R2-2407848 LS on applicable functionality reporting for beam management UE-sided model RAN2

- RP-242387	Moderator's summary on offline discussion about non-RAN2 aspects on REL-19 WI NR_AIML_Air	RAN1 vice-chair (CMCC)