3GPP TSG-RAN2 Meeting #128		R2-2409723

Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

Title:	Discussion on Introduction of NavIC L1 SPS to A-GNSS positioning

Source:	Reliance Jio, CEWiT

Agenda Item:	8.15

Document for:	Discussion and Decision

# Introduction

The Navigation with Indian Constellation (NavIC)/Indian Regional Navigation Satellite System (IRNSS) is an autonomous navigation system developed by the Indian Space Research Organisation (ISRO).

It consists of three Geostationary Earth Orbit (GEO) and four Geo Synchronous Orbit (GSO) satellites. The first generation NavIC satellites broadcast the signal in L5 band (1164.45–1188.45 MHz) and S band (2483.5–2500 MHz) with a carrier frequency of 1176.45 MHz (F1) and 2492.08 MHz (F2) respectively.

ISRO has now introduced second generation of NavIC satellite services. As compared to the first-generation satellite series, the second-generation satellite series includes Standard Position Services (SPS) signal in L1 navigation band (1563.42 – 1587.42 MHz) and incorporates an improved atomic lock. The inclusion of L1 navigation band improves the positioning, navigation, and timing services for civilian users, and facilitates better interoperability with other GNSS services.

The NavIC Signal-in-Space ICD (ISRO-IRNSS-ICD-SPS-L1-1.0) [1] for SPS in L1 frequency was published in August 2023 to the public providing the essential information on the NavIC signal-in-space, to facilitate the development of commercial navigation-based applications based on the NavIC L1 signal.

The motivation and scope for introduction of GNSS assistance for NavIC L1 signals in NR & LTE by extending the existing LPP framework [2] was discussed in RAN#105 TSG meeting. As an outcome of this discussion a new RAN2 led Release19 work item LCS_NAVIC_L1_SPS_NR_LTE was approved [3].

# RAN2 specification impact of NavIC L1 A-GNSS introduction

The introduction of NavIC L1 A-GNSS by extending the existing LPP framework has impacts on the stage 2 and stage 3 specifications respectively as captured in the table below.

| WG | TS No. | Description of change |
| --- | --- | --- |
| RAN2 | 38.305 36.305 | General description and ICD reference about the support for NavIC L1 SPS will be added in the stage 2 specification for NR & LTE positioning |
|  | 37.355 | Necessary NavIC L1 SPS assistance data information will be introduced in the stage 3 specification based on the existing GNSS framework. Major part of proposed work has normative impact to this specification. |

Following sections analyse the details of the above-mentioned impacts of NavIC L1 A-GNSS introduction to NR & LTE stage 2 , stage 3 specifications.

## Impact analysis on NR & LTE stage 2 specifications

The impact on stage 2 NR specification TS 38.305 and stage 2 LTE specification TS 36.305 is identical as summarised below:

- A new reference entry corresponding to NavIC Signal in space ICD for standard positioning service in L1 frequency to be introduced in Section 2 of the specifications.

- The reference to NavIC L1 SPS ICD [1] to be added to General description of GNSS section 8.1.1 and sub sections for the Ionospheric models, and GNSS-GNSS Time Offsets respectively.

## Impact analysis on stage 3 LPP specification

It is noted that NR & LTE share a common stage 3 LPP specification [4].

The major updates to Stage 3 specifications can be summarised as below:

- A new reference entry corresponding to NavIC Signal in space ICD [1] for standard positioning service in L1 frequency to be introduced in Section 2 of the specifications.

- Reference for the NavIC L1 ICD [1] is added to relevant text and description fields under section 6.5.2 A-GNSS Positioning.

- Updates to GNSS Assistance Data Elements, Section 6.5.2.2

- Ionospheric model

  - Updates to Klobuchar model

- The existing KlobucharModelParameter defnition supports word lengths of 8 bits two-complement representation for the parameter fields alfa2, alfa3, beta2, & beta3 respectively.

- The NavIC L1 ICD [1] specifies a higher word length definitions for these fields, hence repective extensions need to be defined for each of them as per as per ICD [1] Table 18: Message Type 10.

  - Introduction of new neQuick2 model

- The existing NeQuickModelParameter IE corresponds to NeQuick model for Galileo GNSS, which supports common NeQuick parameters for the 5 different sub-regions across the globe. Also the boundaries of these regions remain constant.

- The NeQuick model defined under NavIC L1 ICD[1] supports 3 sub-regions which cover Indian subcontinent. The neQuick parameters fo these 3 sub-regions are not common contrary to the assumtion in current LPP NeQuickModelParameter IE definition. Also the coverage of these sub-regions is dynamic based upon the parameters MODIPmax, MODIPmin, mLonmax, mLonmin parameters as defined in NavIC L1 ICD [1] Table 17: Message Type 8.

Hence, a new NeQuickModel2Parameter IE is proposed to be defined corresponding to NavIC L1 NeQuick model.

- Navigation Model

  - Introduction of new NavIC L1 clock model

- The NavIC L1 SPS clock model (as per ICD [1] Table 13) has higher precision word lengths defined for the following parameters which are common with the NavIC L5 clock model NavIC-ClockModel: navicL1-Toec,navicL1-af2, navicL1-af1, navicL1-af0,navicL1-Tgd.

- Following additonal parameters have been defined for NavIC L1 clock: navicL1-iscL1PorS, navicL1-iscL1D, navicL1-RSF.

- Hence, a new NavIC clock model IE NavIC-ClockModel2 is proposed to be defined in LPP.

  - Introduction of new NavIC L1 orbit model (as per ICD [1] Table 13)

- The NavIC L1 ICD [1] defines a new orbit model which is an evolved model compared to the one used by NavIC L5 (defined as NavModel-NavIC-KeplerianSet in LPP)

- Hence, it is proposed to define a new IE NavModel-NavIC-KeplerianSet2 to support NavIC L1 GNSS assistance.

- Almanac

  - Introduction of new NavIC L1 Almanac IE

- The NavIC L1 SPS Almanac model (as per ICD [1] Table 27: Almanac Parameters Definitions) has higher precision word lengths defined for the following parameters which are common with the NavIC L5 clock model AlmanacNavIC-AlmanacSet: AlmE, AlmOMEGADOT, & Almaf0.

- Following additonal parameters have been defined for NavIC L1 Almanac: inclination i0.

- Hence, a new NavIC Almanac model IE AlmanacNavIC-AlmanacSet2 is proposed to be defined in LPP.

- NavIC-GridModelParameter

  - Introduction of ITOW & TOI as optional fields applicable for NavIC L1 only

- The NavIC L1 SPS messages use a new timing convention (using ITOW & TOI parameters) compared to the one used in NavIC L5 SPS messages, including  for GridModelParameters msg.

- The Interval Time of Week (ITOW), identifies the start time of the 2 hour interval since the beginning of the NavIC week. Time of Interval (TOI) is the count of the number of 18 sec message intervals in each 2 hour ITOW. Together they provide a reference time corresponding to the reception of GridModelParameters at LMF.

- The NavIC L1 receicver can use this time stamp to determine the validity of the shared GridParameterValues.

- However, this reference is not readily interchangeable with the NavIC L5 timing reference Time of Week Count (TOWC), which indicates the number of 12 second counts at which the next subframe will begin.

- Hence, introduction of two fields ITOW, & TOI is proposed to IE NavIC-GridModelParameter

- Respective updates to GNSS Assistance Data Request Elements, LPP [4] Section 6.5.2.4

- GNSS Capability Information Elements, LPP[4] Section 6.5.2.10

- NavIC L1 updates to Common GNSS Information Elements, LPP[4] Section 6.5.2.13

The detailed Stage 3 specification [4] impacts have been documented further in following sub-sections.

## GNSS Assistance Data (Section 6.5.2.1)

| Data Elements and field | Impact description |
| --- | --- |
| A-GNSS-ProvideAssistanceData | No impact |
| GNSS-CommonAssistData | No impact |
| GNSS-GenericAssistData | No impact |
| GNSS-PeriodicAssistData | No impact |

## Data Elements and field (Section 6.5.2.2)

| Data Elements and field | Impact description |
| --- | --- |
| GNSS-ReferenceTime | No impact |
| GNSS-SystemTime | No impact |
| GPS-TOW-Assist | No impact. |
| NetworkTime | No impact |
| GNSS-ReferenceLocation | No impact |
| GNSS-IonosphericModel | Introduce new model parameter neQuickModel2 for NavIC L1 |
| KlobucharModelParameter | - Extended bit parameters to be defined to match NavIC L1 word length for following parameters: alfa2, alfa3, beta2, beta3 |
| KlobucharModel2Parameter | No impact |
| NeQuickModelParameter | No impact |
| NeQuickModel2Parameter | New NeQuickModel2Parameter IE to be introduced to represent NavIC L1 neQuick model |
| GNSS-EarthOrientationParameters | No impact |
| GNSS-RTK-ReferenceStationInfo | No impact |
| GNSS-RTK-CommonObservationInfo | No impact. |
| GNSS-RTK-AuxiliaryStationData | No impact |
| GNSS-TimeModelList | No impact |
| GNSS-DifferentialCorrections | No impact. |
| GNSS-NavigationModel > NavModelSatelliteList >>GNSS- NavModelSatelliteElement >>> GNSS-ClockModel >>> GNSS-orbitModel >>> svHealth bit string >>> iod bit string | - navic-ClockModel2-r19 to be added to GNSS-ClockModel - navic-KeplerianSet2-r19 to be added to GNSSorbitModel - To be updated with NavIC L1 svHealth bit - To be updated with NavIC L1 iod value |

| Data Elements and field | Impact description |
| --- | --- |
| StandardClockModelList | No impact |
| NAV-ClockModel | No impact |
| CNAV-ClockModel | No impact |
| GLONASS-ClockModel | No impact |
| SBAS-ClockModel | No impact |
| BDS-ClockModel | No impact |
| BDS-ClockModel2 | No impact |
| NavIC-ClockModel | Add description to indicate that it refers to NavIC L5 clock model |
| NavIC-ClockModel2 | New NavIC-ClockModel2 IE to be introduced to represent NavIC L1 clock model |
| NavModelKeplerianSet | No impact |
| NavModelNAV-KeplerianSet | No impact |
| NavModelCNAV-KeplerianSet | No impact |
| NavModel-GLONASS-ECEF | No impact |
| NavModel-SBAS-ECEF | No impact |
| NavModel-BDS-KeplerianSet | No impact |
| NavModel-BDS-KeplerianSet2 | No impact |
| NavModel-NavIC-KeplerianSet | No impact |
| NavModel-NavIC-KeplerianSet2 | New NavModel-NavIC-KeplerianSet2 IE to be introduced to represent NavIC L1 orbit model |
| GNSS-RealTimeIntegrity | No impact |
| GNSS-DataBitAssistance > GNSS-DataBitsSatList >> GNSS-DataBitsSatElement >>> GNSS-DataBitsSgnList >>>> GNSS-DataBitsSgnElement >>>>> gnss-DataBits | gnss-DataBits of NavIC L1 updated in ‘GNSS-DataBitAssistance field descriptions’ table. |
| GNSS-AcquisitionAssistance | No impact |
| GNSS-Almanac > AlmanacList >>AlmanacElement | weekNumber and weekNumber-ext-r16 can be reused for NavIC L1. keplerianNavIC-Almanac2-r19 to be added to AlmanacElement The reference to NavIC L1 ICD file shall be added to the description. |
| AlmanacKeplerianSet | No impact |
| AlmanacNAV-KeplerianSet | No impact |
| AlmanacReducedKeplerianSet | No impact |
| AlmanacMidiAlmanacSet | No impact |
| AlmanacGLONASS-AlmanacSet | No impact |
| AlmanacECEF-SBAS-AlmanacSet | No impact |
| AlmanacBDS-AlmanacSet | No impact |
| AlmanacNavIC-AlmanacSet | No impact (Potential correction by introducing navicL5-i0-r19) |
| AlmanacNavIC-AlmanacSet2 | New AlmanacNavIC-AlmanacSet2 IE to be introduced to represent NavIC L1 Almanac model |
| GNSS-UTC-Model | No impact |
| UTC-ModelSet1 | No impact |
| UTC-ModelSet2 | - UTC-ModelSet2 can be reused for NavIC L1. - The reference to NavIC L1 ICD file to be added in description. - Description of utcWNlsf-ext to be updated |
| UTC-ModelSet3 | No impact |
| UTC-ModelSet4 | No impact |
| UTC-ModelSet5 | No impact |
| GNSS-AuxiliaryInformation | No impact |
| BDS-DifferentialCorrections | No impact |
| BDS-GridModelParameter | No impact |
| GNSS-RTK-Observations | No impact |
| GLO-RTK-BiasInformation | No impact |
| GNSS-RTK-MAC-CorrectionDifferences | No impact |
| GNSS-RTK-Residuals | No impact |
| GNSS-RTK-FKP-Gradients | No impact |
| GNSS-SSR-OrbitCorrections | No impact |
| GNSS-SSR-ClockCorrections | No impact |
| GNSS-SSR-CodeBias | No impact |
| GNSS-SSR-URA | No impact |
| GNSS-SSR-PhaseBias | No impact |
| GNSS-SSR-STEC-Correctio | No impact |
| GNSS-SSR-GriddedCorrection | No impact |
| NavIC-DifferentialCorrections | Update description to clarify that NavIC-DifferentialCorrections are applicable to NavIC L5 |
| NavIC-GridModelParameter | Introduce NavIC L1 ITOW & TOI as optional fields applicable for NavIC L1 only. The description to be updated appropriately |

## GNSS Assistance Data Request (Section 6.5.2.3)

| Data Elements and field | Impact description |
| --- | --- |
| A-GNSS-RequestAssistanceData | No impact |
| GNSS-CommonAssistDataReq | No impact |
| GNSS-GenericAssistDataReq | No impact |
| GNSS-PeriodicAssistDataReq | No impact |

## GNSS Assistance Data Request Elements (Section 6.5.2.4)

| Data Elements and field | Impact description |
| --- | --- |
| GNSS-ReferenceTimeReq | No impact |
| GNSS-ReferenceLocationReq | No impact |
| GNSS-IonosphericModelReq | Introduce new request parameter neQuickModel2Req-r19 |
| GNSS-EarthOrientationParametersReq | No impact |
| GNSS-RTK-ReferenceStationInfoReq | No impact |
| GNSS-RTK-AuxiliaryStationDataReq | No impact |
| GNSS-SSR-CorrectionPointsReq | No impact |
| GNSS-TimeModelListReq | No impact |
| GNSS-DifferentialCorrectionsReq | No impact |
| GNSS-NavigationModelReq > StoredNavListInfo >> SatListRelatedDataList >>> SatListRelatedDataElement >>>>> clockModelID >>>>> orbitModelID >ReqNavListInfo >> clockModelID-PrefList >> orbitModelID-PrefList | - Update SatListRelatedDataElement with clockModelID & orbitModelID extensions to support clockModel & orbitModel IDs beyond #8 - Update ReqNavListInfo with clockModelID-PrefList & orbitModelID-PrefList extension to support clockModel & orbitModel IDs beyond #8 |
| GNSS-RealTimeIntegrityReq | No impact |
| GNSS-DataBitAssistanceReq | No impact |
| GNSS-AcquisitionAssistanceReq | No impact |
| GNSS-AlmanacReq > modelID | Introduce modelID-Ext to be introduced for supporting Almanac model beyond #8 |
| GNSS-UTC-ModelReq | No impact |
| GNSS-AuxiliaryInformationReq | No impact |
| BDS-DifferentialCorrectionsReq | No impact |
| BDS-GridModelReq | No impact |
| GNSS-RTK-ObservationsReq | No impact |
| GLO-RTK-BiasInformationReq | No impact |
| GNSS-RTK-MAC-CorrectionDifferencesReq | No impact |
| GNSS-RTK-ResidualsReq | No impact |
| GNSS-RTK-FKP-GradientsReq | No impact |
| GNSS-SSR-OrbitCorrectionsReq | No impact |
| GNSS-SSR-ClockCorrectionsReq | No impact |
| GNSS-SSR-CodeBiasReq | No impact |
| GNSS-SSR-URA-Req | No impact |
| GNSS-SSR-PhaseBiasReq | No impact |
| GNSS-SSR-STEC-CorrectionReq | No impact |
| GNSS-SSR-GriddedCorrectionReq | No impact |
| NavIC-DifferentialCorrectionsReq | No impact |
| NavIC-GridModelReq | No impact |

## GNSS Location Information (Section 6.5.2.5)

| Data Elements and field | Impact description |
| --- | --- |
| A-GNSS-ProvideLocationInformation | No impact |

## GNSS Location Information Elements (Section 6.5.2.6)

| Data Elements and field | Impact description |
| --- | --- |
| GNSS-SignalMeasurementInformation | No impact |
| MeasurementReferenceTime | No impact |
| GNSS-MeasurementList | No impact |
| GNSS-LocationInformation | No impact |

## GNSS Location Information Request (Section 6.5.2.7)

| Data Elements and field | Impact description |
| --- | --- |
| A-GNSS-RequestLocationInformation | No impact |

## GNSS Location Information Request Elements (Section 6.5.2.8)

| Data Elements and field | Impact description |
| --- | --- |
| GNSS-PositioningInstructions | No impact |

## GNSS Capability Information (Section 6.5.2.9)

| Data Elements and field | Impact description |
| --- | --- |
| A-GNSS-ProvideCapabilities | No impact |

## GNSS Capability Information Elements (Section 6.5.2.10)

| Data Elements and field | Impact description |
| --- | --- |
| GNSS-CommonAssistanceDataSupport | No impact |
| GNSS-ReferenceTimeSupport | No impact |
| GNSS-ReferenceLocationSupport | No impact |
| GNSS-IonosphericModelSupport | Introduce new iono model #3: neQuick2-r19 parameter |
| GNSS-EarthOrientationParametersSupport | No impact |
| GNSS-RTK-ReferenceStationInfoSupport | No impact |
| GNSS-RTK-AuxiliaryStationDataSupport | No impact |
| GNSS-GenericAssistanceDataSupport | No impact |
| GNSS-TimeModelListSupport | No impact |
| GNSS-DifferentialCorrectionSupport | No impact |
| GNSS-NavigationModelSupport > clockModel > orbitModel | - Introduce new clockModelExt to support clock models beyond #8 and define model #9 bit for NavIC L1 clock model - Introduce new orbitModelExt to support clock models beyond #8 and define model #9 bit for NavIC L1 orbit model - Update the description fields |
| GNSS-RealTimeIntegritySupport | No impact |
| GNSS-DataBitAssistanceSupport | No impact |
| GNSS-AcquisitionAssistanceSupport | No impact |
| GNSS-AlmanacSupport | - Introduce new almanacModelExt to support Almanac model IDs beyond #8 - Update field description - (Include R2-2408215 updates) |
| GNSS-UTC-ModelSupport | - Update field description for NavIC L1 - (Include R2-2408215 updates) |
| GNSS-AuxiliaryInformationSupport | No impact |
| BDS-DifferentialCorrectionsSupport | No impact |
| BDS-GridModelSupport | No impact |
| GNSS-RTK-ObservationsSupport | No impact |
| GLO-RTK-BiasInformationSupport | No impact |
| GNSS-RTK-MAC-CorrectionDifferencesSupport | No impact |
| GNSS-RTK-ResidualsSupport | No impact |
| GNSS-RTK-FKP-GradientsSupport | No impact |
| GNSS-SSR-OrbitCorrectionsSupport | No impact |
| GNSS-SSR-ClockCorrectionsSupport | No impact |
| GNSS-SSR-CodeBiasSupport | No impact |
| GNSS-SSR-URA-Support | No impact |
| GNSS-SSR-PhaseBiasSupport | No impact |
| GNSS-SSR-STEC-CorrectionSupport | No impact |
| GNSS-SSR-GriddedCorrectionSupport | No impact |
| NavIC-DifferentialCorrectionsSupport | No impact |
| NavIC-GridModelSupport | No impact |

## GNSS Capability Information Request (Section 6.5.2.11)

| Data Elements and field | Impact description |
| --- | --- |
| A-GNSS-RequestCapabilities | No impact |

## GNSS Error Elements (Section 6.5.2.12)

| Data Elements and field | Impact description |
| --- | --- |
| A-GNSS-Error | No impact |
| GNSS-LocationServerErrorCauses | No impact |
| GNSS-TargetDeviceErrorCauses | No impact |

## Common GNSS Information Elements (Section 6.5.2.13)

| Data Elements and field | Impact description |
| --- | --- |
| GNSS-FrequencyID | The description of NavIC L1 shall be added |
| GNSS-ID | No impact |
| GNSS-ID-Bitmap | No impact |
| GNSS-Link-CombinationsList | No impact |
| GNSS-NavListInfo | No impact |
| GNSS-NetworkID | No impact |
| GNSS-PeriodicControlParam | No impact |
| GNSS-ReferenceStationID | No impact |
| GNSS-SignalID | Add one bit to indicate the NavIC L1 signal. |
| GNSS-SignalIDs | Add one bit to indicate the NavIC L1 signal. |
| GNSS-SubNetworkID | No impact |
| SBAS-ID | No impact |
| SBAS-IDs | No impact |
| SV-ID | No impact |

# Conclusion

This contribution summarizes the impacts of NavIC L1 A-GNSS introduction to NR & LTE stage 2 and stage 3 specifications respectively.

# Observation:

Based upon the specification impact analysis in preceding sections it is observed that the existing A-GNSS framework in LPP can be re-used for introduction of A-GNSS support for NavIC L1 signal.

# Proposal:

The normative work on NavIC L1 A-GNSS support introduction to NR and LTE to be based upon the analysis of Stage 2 and Stage 3 specification impacts as captured in previous sections.

# References

- ISRO-IRNSS-ICD-SPS-L1-1.0, “NavIC Signal in space ICD for standard positioning service in L1 frequency, V1.0”, August 2023

- RP-241806, “Motivation-NavIC-L1-SPS-AGNSS-in-Rel19”, RAN#105 TSG meeting

- RP-242414, “WID NavIC L1 SPS AGNSS in 5G NR & LTE“, RAN#105 TSG meeting

- TS 37.355, LTE Positioning Protocol (LPP), Release 18