3GPP TSG-RAN WG2 #128	R2-2409731

Orlando, USA, November 2024

Agenda Item:	8.13.4

Source:	OPPO

Title:	Service continuity of multi-hop U2N relay

Document for:	Discussion, Agreement

Introduction

The objective of multi-hop U2N relay is as follows, this paper will discuss the service continuity related issues of multi-hop U2N relay.

Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

Discussion

Case A: multi-hop indirect to direct path switching

For the switching from indirect path to direct path, the following procedure is used

Figure 16.12.6.1-1: Procedure for L2 U2N Remote UE intra-gNB switching from indirect to direct path 1. The Uu measurement configuration and measurement report signalling procedures are performed to evaluate both relay link measurement and Uu link measurement. The measurement results from L2 U2N Remote UE are reported when configured measurement reporting criteria are met. The sidelink relay measurement report shall include at least L2 U2N Relay UE's source L2 ID, serving cell ID (i.e., NCGI/NCI), and sidelink measurement quantity result. The sidelink measurement quantity can be SL-RSRP of the serving L2 U2N Relay UE, and if SL-RSRP is not available, SD-RSRP is used. 2. The gNB decides to switch the L2 U2N Remote UE onto direct Uu path. 3. The gNB sends the RRCReconfiguration message to the L2 U2N Remote UE. The L2 U2N Remote UE stops User Plane and Control Plane transmission via the L2 U2N Relay UE after reception of the RRCReconfiguration message with the path switch configuration. 4. The L2 U2N Remote UE synchronizes with the gNB and performs Random Access. 5. The UE (i.e., L2 U2N Remote UE in previous steps) sends the RRCReconfigurationComplete message to the gNB via the direct path, using the configuration provided in the RRCReconfiguration message. From this step, the UE (i.e., L2 U2N Remote UE in previous steps) uses the RRC connection via the direct path to the gNB. 6. The gNB sends the RRCReconfiguration message to the L2 U2N Relay UE to reconfigure the connection between the L2 U2N Relay UE and the gNB. The RRCReconfiguration message to the L2 U2N Relay UE can be sent any time after step 3 based on gNB implementation (e.g., to release Uu Relay RLC Channel and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration related to the L2 U2N Remote UE). 7. Either L2 U2N Relay UE or L2 U2N Remote UE's AS layer indicates upper layers to release PC5 unicast link after receiving the RRCReconfiguration message from the gNB. The timing to execute link release is up to UE implementation. 8. The data path is switched from indirect path to direct path between the UE (i.e., previous L2 U2N Remote UE) and the gNB. The PDCP re-establishment or PDCP data recovery in uplink is performed by the UE (i.e., previous L2 U2N Remote UE) for lossless delivery during path switch if gNB configures it. NOTE 1: Step 8 can be executed any time after step 4. Step 8 is independent of step 6 and step 7.

The above procedure is generally applicable to indirect path to direct path switching no matter in single hop or multi-hop case.

RAN2 to confirm that the intra-gNB switching from indirect to direct path procedure defined in TS 38.300 for single hop case can be used as the baseline for multi-hop indirect to direct path switching.

For measurement of Uu link and relay link in indirect path to direct path switching, the following events are defined:

- Event X1 (Serving L2 U2N Relay UE becomes worse than threshold1 and NR Cell becomes better than threshold2)

- Event X2 (Serving L2 U2N Relay UE becomes worse than threshold)

For event X1/X2, the difference between single-hop and multi-hop U2N relay lies in defining the serving Relay UE, as multiple relays exist on the serving relay link. Since the measurement event is used for link quality monitoring, and the remote UE only has a direct link with the first relay UE, in the case of multi-hop U2N, the serving U2N Relay in event X1/X2 should be the first relay UE. With this understanding, event X1/X2 can be reused for multi-hop indirect path to direct path switching.

For event X1 and X2, the serving U2N Relay is the first relay in the multi-hop U2N Relay case.

RAN2 to discuss event X1 /X2 can be reused in multi-hop indirect to direct path switching with the understanding that the “first relay UE” in multi-hop relay link is “serving L2 U2N Relay UE” to be reported.

Case B: multi-hop indirect to single-hop indirect path switching

For the switching from indirect path to indirect path, the following procedure is used:

Figure 16.12.6.3-1: Procedure for L2 U2N Remote UE intra-gNB switching from indirect to indirect path via a target L2 U2N Relay UE in RRC_CONNECTED 1. The L2 U2N Remote UE reports one or multiple candidate L2 U2N Relay UE(s) and sidelink measurement between the L2 U2N Remote UE and the source L2 U2N Relay UE to the source gNB, after it measures/discovers the candidate L2 U2N Relay UE(s): - The L2 U2N Remote UE filters the appropriate L2 U2N Relay UE(s) according to relay selection criteria before reporting. The L2 U2N Remote UE shall report only the L2 U2N Relay UE candidate(s) that fulfil the higher layer criteria; - The reporting includes at least a L2 U2N Relay UE ID, a L2 U2N Relay UE's serving cell ID, and a sidelink measurement quantity information. SD-RSRP is used as sidelink measurement quantity. 2. The gNB decides to switch the L2 U2N Remote UE to a target L2 U2N Relay UE under the same gNB. 3. The gNB sends an RRCReconfiguration message to the target L2 U2N Relay UE, which includes at least the L2 U2N Remote UE's local ID and L2 ID, Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration. 4. The gNB sends the RRCReconfiguration message to the L2 U2N Remote UE. The RRCReconfiguration message includes at least the target L2 U2N Relay UE ID, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic, and the associated end-to-end radio bearer(s). The L2 U2N Remote UE stops UP and CP transmission over the indirect path via the source L2 U2N Relay UE after the reception of the RRCReconfiguration message from the gNB. 5. The L2 U2N Remote UE establishes PC5-RRC connection with the target L2 U2N Relay UE. 6. The L2 U2N Remote UE sends RRCReconfigurationComplete message to the gNB via the target L2 U2N Relay UE. 7. The gNB sends the RRCReconfiguration message to the source L2 U2N Relay UE to reconfigure the connection between the source L2 U2N Relay UE and the gNB. The RRCReconfiguration message to the source L2 U2N Relay UE can be sent any time after step 4 based on gNB implementation (e.g., to release Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration related to the L2 U2N Remote UE). 8. Either source L2 U2N Relay UE's AS layer or L2 U2N Remote UE's AS layer indicates upper layers to release PC5 unicast link after receiving the RRCReconfiguration message from the gNB. The timing to execute link release is up to UE implementation after step 4 or step7. 9. The data path is switched from the source L2 U2N Relay UE to the target L2 U2N Relay UE between the L2 U2N Remote UE and the gNB. This step can be any time after step 6.

Like Case A, since the target side is single hop U2N Relay, the above indirect to indirect path switch procedure can be reused as the baseline for multi-hop indirect path to single hop indirect path switching.

RAN2 to confirm the intra-gNB switching from indirect path to indirect path procedure defined in TS 38.300 in single hop case can be used as baseline for multi-hop indirect to single hop indirect path switching procedure.

For the measurement event defined for indirect path to indirect path switching, the following events can be used:

- Event Y2 (Candidate L2 U2N Relay UE becomes better than threshold)

- Event Z1 (Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2)

For the evaluation of Candidate L2 U2N Relay, since the target path is a single-hop indirect path, there is no difference compared to the current mechanism. For the evaluation of Serving L2 U2N Relay, similar to Case A, the serving L2 U2N Relay should be the first relay in the multi-hop relay link.

For event Z1, the Serving U2N Relay is the first relay in the multi-hop U2N Relay case.

RAN2 to discuss the following measurement event can be reused in multi-hop indirect to direct path switching:

- Event Y2

- Event Z1 with the understanding that the “first relay UE” in multi-hop relay link is “serving L2 U2N Relay UE” to be reported.

Case C/D: path switch to multi-hop indirect path

For the path switch to multi-hop indirect path, it is restricted in the WID that all relays at the target multi-hop relay link should be in RRC connected state to the same cell.

Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell.

With this restriction, we can further analyse whether there are additional impacts on the current path-switching framework.

For the path switching preparation procedure, Y1/Y2/Z1 are defined for the path switching to indirect path:

- Event Y1 (PCell becomes worse than threshold1 and candidate L2 U2N Relay UE becomes better than threshold2)

- Event Y2 (Candidate L2 U2N Relay UE becomes better than threshold)

- Event Z1 (Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2)

Like the above analysis to Case A and B, with the understanding that the “first relay” in multi-hop relay is the serving/candidate relay to be reported, Y1/Y2/Z1 can be reused.

With the understanding that the “first relay” in multi-hop relay is the serving or candidate relay to be reported, Y1/Y2/Z1 can be reused for the path switching to multi-hop indirect path.

RAN2 to discuss event Y1/Y2 can be reused in direct path to multi-hop indirect path switching with the understanding that the “first relay” in multi-hop relay is the “candidate” relay to be reported.

RAN2 to discuss event Y2/Z1 can be reused in single-hop indirect path to multi-hop indirect path switching with the understanding that the “first relay” in multi-hop relay is the “serving” or “candidate” relay to be reported.

For the measurement report, in the current path switching to single-hop indirect path procedure, the L2 U2N Remote UE reports one or multiple candidate L2 U2N Relay UE(s) (L2 ID, Cell ID, SD-RSRP) that fulfil both AS and upper layer criteria. The principle that remote UE reports candidate L2 U2N Relay UE(s) that fulfil both AS and upper layer criteria should be reused. And the candidate L2 U2N Relay UE(s) in multi-hop relay case should be the first relay, since only the RSRP towards the first relay can be measured by the remote UE. It is also sufficient to report L2 ID, Cell ID, and the RSRP of the first relay since all the relay UEs at the multi-hop relay link are in RRC connected state to the same cell in the same gNB, network can identify the target link via the same information as legacy.

It is sufficient to report L2 ID, Cell ID, and the RSRP of the first relay (as candidate L2 U2N Relay) since all the relay UEs at the multi-hop relay link are in RRC connected state to the same cell in the same gNB.

RAN2 to confirm that the measurement reporting from remote UE which includes L2 ID, serving cell ID, and SD-RSRP of the candidate first relay, as per legacy, is sufficient for the path switching to multi-hop indirect path.

Then for the path switching execution, gNB can decide the target path based on the information from the remote UE and do the reconfiguration for the relay UEs at the target path and the remote UE as in legacy.

For the path switching execution to multi-hop indirect path, gNB can decide the target path based on the information from the remote UE and do the reconfiguration for the relay UEs at the target path and the remote UE as in legacy.

Conclusion

We have the following observations:

Observation 1	For event X1 and X2, the serving U2N Relay is the first relay in the multi-hop U2N Relay case.

Observation 2	For event Z1, the Serving U2N Relay is the first relay in the multi-hop U2N Relay case.

Observation 3	With the understanding that the “first relay” in multi-hop relay is the serving or candidate relay to be reported, Y1/Y2/Z1 can be reused for the path switching to multi-hop indirect path.

Observation 4	It is sufficient to report L2 ID, Cell ID, and the RSRP of the first relay (as candidate L2 U2N Relay) since all the relay UEs at the multi-hop relay link are in RRC connected state to the same cell in the same gNB.

Observation 5	For the path switching execution to multi-hop indirect path, gNB can decide the target path based on the information from the remote UE and do the reconfiguration for the relay UEs at the target path and the remote UE as in legacy.

We have the following proposals:

Proposal 1	RAN2 to confirm that the intra-gNB switching from indirect to direct path procedure defined in TS 38.300 for single hop case can be used as the baseline for multi-hop indirect to direct path switching.

Proposal 2	RAN2 to discuss event X1 /X2 can be reused in multi-hop indirect to direct path switching with the understanding that the “first relay UE” in multi-hop relay link is “serving L2 U2N Relay UE” to be reported.

Proposal 3	RAN2 to confirm the intra-gNB switching from indirect path to indirect path procedure defined in TS 38.300 in single hop case can be used as baseline for multi-hop indirect to single hop indirect path switching procedure.

Proposal 4	RAN2 to discuss the following measurement event can be reused in multi-hop indirect to direct path switching:

-	Event Y2

-	Event Z1 with the understanding that the “first relay UE” in multi-hop relay link is “serving L2 U2N Relay UE” to be reported.

Proposal 5	RAN2 to discuss event Y1/Y2 can be reused in direct path to multi-hop indirect path switching with the understanding that the “first relay” in multi-hop relay is the “candidate” relay to be reported.

Proposal 6	RAN2 to discuss event Y2/Z1 can be reused in single-hop indirect path to multi-hop indirect path switching with the understanding that the “first relay” in multi-hop relay is the “serving” or “candidate” relay to be reported.

Proposal 7	RAN2 to confirm that the measurement reporting from remote UE which includes L2 ID, serving cell ID, and SD-RSRP of the candidate first relay, as per legacy, is sufficient for the path switching to multi-hop indirect path.