3GPP TSG-RAN WG2 Meeting #128	R2-2409732

Orlando, USA, November 2024

Agenda Item:	8.13.3

Source:	OPPO

Title:	Control plane procedures of multi-hop U2N relay

Document for:	Discussion, Agreement

Introduction

The objective of multi-hop U2N relay is as follows, this paper will discuss the control plane procedures of multi-hop U2N relay.

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3]

Discussion

Comparison of Approach-1 and Approach-2

For the modeling of multi-hop U2N Relay, Approach 1 and Approach 2 are proposed and discussed in the offline discussion

Proposal 1 – In one approach (“approach 1”) of U2N relays, each of the Intermediate Relay UEs must be in RRC_CONNECTED when the U2N remote UE is in RRC_CONNECTED. Proposal 3 –In one approach (“approach 2”) of U2N relays, Intermediate Relay UEs (other than the Last Relay UE) can be in RRC_IDLE/RRC_INACTIVE when the U2N remote UE is in RRC_CONNECTED.

Firstly, regardless of whether Approach 1 or Approach 2 is used, the scenario where all multi-hop relay UEs are in RRC connected state within the same cell to serve the CONNECTED remote UE’s data should be supported as indicated in the WID for service continuity.

The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell.

The scenario where all multi-hop U2N Relay UEs are in an RRC CONNECTED state within the same cell to serve the CONNECTED remote UE’s data should be supported as indicated in the WID.

RAN2 confirms that the scenario where all the multi-hop U2N Relay UEs are in RRC_CONNECTED state within the same cell as the U2N remote UE is supported.

Beyond the above basic scenario, the question is whether to additionally support the following aspects:

- The intermediate relay UEs can connect or camp to different cells than the last relay UE.

- The intermediate relay UEs can be in RRC IDLE/INACTIVE/out-of-coverage to serve the CONNECTED remote UE’s data.

For the first aspect, if the intermediate relay UE camps/connects to a cell that supports U2N Relay operation, the intermediate relay UE can act as the last relay directly. There is no reason to pursue the case where a UE with a direct Uu connection to cell1 which supports U2N Relay service, acts as an intermediate relay UE via a last relay in cell2.

For a UE that connects to a cell supporting U2N Relay operation, it should act as the last relay instead of an intermediate relay.

If a UE camps/connects to a cell that does not support U2N relay, it cannot serve as an intermediate relay UE as well. Allowing a UE to perform relay operations in a relay-incapable cell goes against the legacy principle, which states that relay services can only be performed if supported by the cell. The possibility of a UE performing relay operations based on pre-configuration has been discussed in both U2N and U2U cases, and RAN2 has concluded that it is not supported.

Agreement in R17 for U2N Relay:

Proposal 10: L2 U2N relay UE should be always connected to a SL Relay Capable gNB for relay operation including transmission of discovery message

Agreement in R18 for U2U Relay:

Add an indication in SIB12 for L2 U2U operation.

Allowing UE connected to a U2N Relay incapable cell to act as intermediate relay goes against the principle since R17, which states that the relay operation should only be performed if supported by the connected cell.

Besides using pre-configuration, there are some proposals to rely on the last relay or the cell of the last relay to provide the multi-hop relay configuration for the intermediate relay UE camps in relay incapable cell. This kind of mechanism increases the complexity of coordination among different cells. Given that supporting intermediate relay UEs connecting or camping to different cells than the last relay UE does not bring any benefit but adds complexity, it is proposed not to pursue this scenario.

RAN2 not pursue the scenario for L2 multi-hop U2N Relay where intermediate relay UE and last relay UE camp on different cells.

For the second aspect, the main motivation is to save the signalling/latency due to RRC setup procedure of the intermediate relay UEs. However, the actual benefits are not clear due to the complexity involved in relying on PC5 to allocate local IDs, split QoS, and derive RLC configurations:

- For CP aspect, the RRC setup of the remote UE must occur after local ID allocation, which requires coordination and signaling exchanges over the PC5/Uu link between different nodes.

- For UP aspect, the first DRB transmission must occur after QoS split coordination and bearer configuration information exchange. For example, similar to the U2U Relay, the remote UE needs to indicate the flow-to-bearer mapping to each intermediate relay after acquiring it from the network.

Allow intermediate relay UEs in IDLE/INACTIVE/out-of-coverage will lead to extra signalling and latency due to the complex procedure of relying on PC5 for local ID allocation/QoS Splitting and bearer configuration.

Besides, the service continuity cannot be supported since the intermediate relay UE is out of the network control and there is no support of U2U relay service continuity.

Service continuity cannot be supported in approach 2 based on U2U Relay structure.

At last, the complexity of CP design of approach 2 cannot be avoided. The following table summarizes the control plane handling for remote UE’s data transmission via multi-hop relaying for Approach 1 and Approach 2.

Table 1. control plane handling for remote UE’s data transmission

| Issues | Approach 1 | Approach 2 |
| --- | --- | --- |
| Connection establishment/control | Same mechanism as in R17 | Issue-1: How to trigger the IDLE/INACTIVE last Relay UE to CONNECTED. Issue-2: How to forward the SRB0 of the remote UE. |
| QoS split | Controlled by network as in R17 | Issue-1: Whether QoS split is controlled by network (which network) or split based on Relay UE (which relay) implementation or in a combined manner; Issue-2: How to coordinate the split QoS between IDLE/INACTIVE Relay and network. |
| UE ID allocation | Controlled by network as in R17 | Issue-1: Whether network (which network) or relay UE (which relay) to do the UE ID allocation; Issue-2: How to avoid UE ID collision. |
| BEARER ID configuration | Controlled by network as in R17 | Issue-1: Whether network or relay UE to configure the BEARER ID; Issue-2: Whether the BEARER ID can be different on different hops |
| Egress RLC channel configuration | Controlled by network as in R17 | Issue-1: How for the IDLE/INACTIVE multi-hop Relay UE to derive the RLC configuration for the Uu bearer; |

As shown in the above table, supporting Approach 2 will complicate control plane handling and leave many issues unresolved.

Allowing non-connected intermediate Relay increases the complexity of the control plane design for multi-hop U2N Relay.

Considering the unjustified benefits, unresolved issues, further complexity, and the limited TU allocated for R19 multi-hop U2N Relay, it is suggested to begin with the simple and baseline scenario, i.e., to support Approach-1.

For L2 multi-hop U2N relay, R2 adopt approach-1, i.e., each of the Intermediate Relay UEs must be in RRC_CONNECTED when the U2N remote UE is in RRC_CONNECTED.

If all the L2 multi-hop U2N relay UEs are in RRC connected state to serve the CONNECTED remote UE’s data, as analyzed in above Table 1, the QoS split and SRAP configurations which include UE ID, BEARER ID, and RLC channel configuration can be handled by the network as in the single-hop case.

When all the L2 multi-hop U2N relay UEs are in RRC connected state to serve the CONNECTED remote UE’s data, the QoS split and SRAP configurations which include UE ID, BEARER ID and RLC channel configuration can be handled by the network as in R17 single hop U2N Relay.

For L2 multi-hop U2N relay, R2 discuss whether to rely on gNB to split the E2E QoS to per-hop QoS as in R17 single-hop U2N Relay.

For L2 multi-hop U2N relay, R2 discuss whether to rely on gNB to do the SRAP configuration (e.g., UE ID, BEARER ID and egress RLC channel configurations).

SI/Paging

In R17 single hop U2N Relay, Paging and SIB forwarding is achieved by the following procedure.

Figure 5.8.9.8.1-1: Remote UE information This procedure is used by the L2 U2N Remote UE in RRC_IDLE/RRC_INACTIVE to inform about the required SIB(s) /posSIB(s), provide Paging related information to the connected L2 U2N Relay UE, request the SFN-DFN offset from the connected L2 U2N Relay UE, and trigger L2 U2N Relay UE in RRC_IDLE/RRC_INACTIVE to enter RRC_CONNECTED during indirect path addition/change in MP operation. This procedure is also used by the L2 U2U Remote UE to send end-to-end PC5 connection release/failure related information to L2 U2U Relay UE Figure 5.8.9.9.1-1: Uu message transfer in sidelink The purpose of this procedure is to transfer Paging message and System Information from the L2 U2N Relay UE to the L2 U2N Remote UE in RRC_IDLE/RRC_INACTIVE.

In multi-hop U2N relay, the process for acquiring SIB(s) at the remote UE is similar to the R17 single hop case. The remote UE informs the required SIB(s) to the first relay UE, which then transfers the SI to the remote UE.

For SIB/posSIB acquisition at the IDLE/INACTIVE Remote UE, as in R17, the remote UE informs the required SIB(s)/posSIB(s) to the first relay UE via RemoteUEinformationSidelink message, and the first relay then transfers the valid SIB(s)/posSIB(s) requested by the remote UE via UuMessageTransferSidelink.

For the SIB(s) acquisition at the intermediate relay, assuming that an IDLE/INACTIVE intermediate relay UE cannot acquire SIB(s) directly from the network (as it should act as the last relay), the process is similar to that of the remote UE. The intermediate relay UE relies on its serving relay (another intermediate relay UE or the last relay UE) to transfer the required SIB(s)/posSIB(s). These required SIB(s)/posSIB(s) include both itswon requirements and those of its child UEs.

For SIB/posSIB acquisition at the IDLE/INACTIVE intermediate relay UE, as in R17, the intermediate relay UE informs the required SIB(s)/posSIB(s) (including the requirements from itself and its child UEs) to the serving relay UE (another intermediate relay UE or the last relay) via RemoteUEinformationSidelink message, and the serving relay UE then transfers the valid SIB(s)/posSIB(s) requested by the intermediate UE via UuMessageTransferSidelink.

For Paging message forwarding, similar to the above analysis of SIB forwarding, the mechanism for the IDLE/INACTIVE remote UE is the same as in R17. The remote UE provides its paging related information to the first relay UE and relies on the first relay UE to help acquire the Paging message. For the IDLE/INACTIVE intermediate relay UE, since it cannot receive the paging message directly from the network, it also needs to acquire its own paging and the paging requested by its child UEs via the serving relay UE.

For Paging acquisition at the IDLE/INACTIVE Remote UE, as in R17, the Remote UE provides Paging related information to the first relay UE via RemoteUEinformationSidelink message, and the first relay then transfers the related paging message to the remote UE via UuMessageTransferSidelink.

For Paging acquisition at the IDLE/INACTIVE intermediate relay UE, as in R17, the intermediate relay UE provides Paging related information (including its own information and the information of the child UE(s)) to the serving relay UE (another intermediate relay UE or the last relay UE) via RemoteUEinformationSidelink message. The serving relay then transfers the related paging messages to the intermediate relay UE via UuMessageTransferSidelink.

For intermediate relay UE in RRC CONNECTED state, it cannot be configured with CORESET, it can perform paging forwarding for its child UEs via dedicated RRC message from the network as in R17.

For CONNECTED intermediate Relay UE, as in R17, upon reception of paging request, it reports the received paging related information from child UEs to the network via SUI message.

Notification message

For notification message, it is used by a U2N/U2U Relay UE to send notification to the connected U2N/U2U Remote UE on the other side link status.

##### 5.8.9.10.2 Initiation The Relay UE may initiate the procedure when one of the following conditions is met: 1> if the UE is acting as U2N Relay UE: 2> upon Uu RLF as specified in 5.3.10; 2> upon reception of an RRCReconfiguration including the reconfigurationWithSync; 2> upon cell reselection; 2> upon L2 U2N Relay UE's RRC connection failure including RRC connection reject as specified in 5.3.3.5 and 5.3.13.10, and T300 expiry as specified in 5.3.3.7, and RRC resume failure as specified in 5.3.13.5; 1> if the UE is acting as L2 U2U Relay UE: 2> upon detection of PC5 RLF for the hop between the L2 U2U Relay UE and L2 U2U Remote UE as specified in 5.8.9.3; 2> upon PC5-RRC connection release for the per-hop link between the L2 U2U Relay UE and L2 U2U Remote UE as specified in 5.8.9.5

In multi-hop U2N Relay, the intermediate relay UE, as illustrated in the following figure for a 2-hop relay case, is involved in multiple links towards the network: the link between the intermediate relay UE and the network, the link between the intermediate relay UE and the last relay UE, and the link between the last relay UE and the network.

Figure 1. The topology of 2-hop U2N Relay

As shown in the above figure, the intermediate relay can directly detect the status of link1 and link3, while the status of link2 is notified by the last relay UE. The question that arises is how the intermediate relay UE should indicate and handle the status of each link in different scenarios.

Firstly, for link1 in Figure 1, the PC5 link between the intermediate relay UE and its serving relay (which could be another intermediate relay UE or the last relay), PC5 RLF or PC5 release may occur. These events can impact the child UE and should therefore be indicated to the child UE.

RAN2 to discuss intermediate Relay UE sends NotificationMessageSidelink to its connected child UE upon PC5 RLF or PC5 link release with its parent relay UE.

Next, for link3 in Figure 1, which represents the E2E Uu link between the intermediate relay UE and the network, events such as Uu RLF, handover, cell reselection, and RRC connection failure may occur. Similar to the R17 U2N Relay, the intermediate relay UE should send a notification message to its child UE upon the occurrence of Uu RLF, handover, cell reselection, or RRC connection failure..

As in R17, the intermediate Relay UE sends NotificationMessageSidelink to its connected child UE upon the following conditions:

- upon Uu RLF;

- upon reception of an RRCReconfiguration including the reconfigurationWithSync;

- upon cell reselection;

- upon L2 U2N Relay UE's RRC connection failure including RRC connection reject, and T300 expiry, and RRC resume failure

Finally, for the links that are not directly detected by the intermediate relay UE, such as the link between the last relay UE and the network (link2 in Figure 1) or the link between two other intermediate relay UEs, the link status is notified to the intermediate relay UE by its serving relay UE. In this case, the intermediate relay needs to:

- Act upon the received notification message. Based on the R17 mechanism, upon receiving a notification message, a UE triggers RRC re-establishment if in RRC CONNECTED state or triggers PC5 release/considers cell change if in RRC IDLE/INACTIVE state.

- Notify the status to its child UE via a notification message.

As an intermediate relay UE, upon notification message reception, it needs to act to the received notification message similar to R17 Remote UE and notify the status to its connected child UE.

For the action upon notification message reception at the intermediate relay, it is suggested to follow the R17 design for L2 U2N Remote UE.

##### 5.8.9.10.4 Actions related to reception of NotificationMessageSidelink message Upon receiving the NotificationMessageSidelink, the Remote UE shall: 1> if the UE is acting as U2N Remote UE: 2> if the indicationType is included: 3> if the UE is L2 U2N Remote UE in RRC_CONNECTED: 4> if MP is configured and MCG transmission (i.e. direct path) is not suspended; 5> if the indicationType is relayUE-HO; 6> suspend indirect path transmission; 5> else: 6> initiate the indirect path failure information procedure as specified in 5.7.3c to report indirect path failure; 4> else if T301 is not running, initiate the RRC connection re-establishment procedure as specified in 5.3.7; 3> else (the UE is L3 U2N Remote UE, or L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE): 4> if the PC5-RRC connection with the U2N Relay UE is determined to be released: 5> indicate upper layers to trigger PC5 unicast link release; 4> else (i.e., maintain the PC5 RRC connection): 5> if the UE is L2 U2N Remote UE and the indicationType is relayUE-HO or relayUE-CellReselection: 6> consider cell re-selection occurs;

As in R17, upon reception of NotificationMessageSidelink, the CONNECTED intermediate Relay UE triggers RRC Re-establishment, IDLE/INACTIVE intermediate Relay UE releases the PC5 unicast link with its parent relay UE or considers cell reselection happens.

The key question regarding how to notify the status to its child UE is whether the intermediate relay UE should send a notification message to its child UE immediately upon receiving a notification message from its serving relay UE, or after it has processed the received notification message.

If the intermediate relay UE forwards the notification to the child UE directly upon receiving it from its parent, the notification message needs to be enhanced to include the link status of multiple hops or different links generated by each relay UE. Additionally, after the intermediate relay UE processes the notification message from its serving relay, it may need to send another notification message to the child UE due to possible PC5 link release, cell change, or RRC re-establishment.

If the intermediate relay UE forwards the received notification message directly to the connected child UE, the notification message needs to be enhanced to indicate the hop/relay information for each notification. Additionally, another notification message may be needed after the intermediate relay UE processes the previous notification.

If the intermediate relay UE processes the received notification message before transmitting it to its connected UE, it will handle it similarly to the R17 remote UE. Upon receiving a notification message, the intermediate relay UE will first trigger RRC re-establishment if in RRC CONNECTED state, or maintain/release the PC5 connection with the parent relay UE if in RRC IDLE/INACTIVE state. After performing RRC re-establishment, cell change, or PC5 release with its parent relay UE, it will generate a notification message based on the latest status, such as cell change or PC5 release, and notify the child UE accordingly

If the intermediate relay UE handles the received notification message before transmitting notification message to its connected child UE, it first performs RRC Reestablisment, cell change or PC5 release. Then, it generates a notification message to indicate the latest status via existing cause value (e.g., cell reselection or PC5 RLF) to the connected child UE.

RAN2 to discuss the operation of intermediate Relay UE upon receiving NotificationMessageSidelink from serving relay:

- Option-1: Forwards the received notification message directly to the connected child UE, with enhancements to NotificationMessageSidelink to indicate the hop/relay information.

- Option-2: Handles the received notification message first (e.g., trigger RRC Reestablishment, cell change or PC5 release) and then generates NotificationMessageSidelink to its connected child UE based on the latest status.

Conclusion

We have the following observations:

Observation 1	The scenario where all multi-hop U2N Relay UEs are in an RRC CONNECTED state within the same cell to serve the CONNECTED remote UE’s data should be supported as indicated in the WID.

Observation 2	For a UE that connects to a cell supporting U2N Relay operation, it should act as the last relay instead of an intermediate relay.

Observation 3	Allowing UE connected to a U2N Relay incapable cell to act as intermediate relay goes against the principle since R17, which states that the relay operation should only be performed if supported by the connected cell.

Observation 4	Allow intermediate relay UEs in IDLE/INACTIVE/out-of-coverage will lead to extra signalling and latency due to the complex procedure of relying on PC5 for local ID allocation/QoS Splitting and bearer configuration.

Observation 6	Allowing non-connected intermediate Relay increases the complexity of the control plane design for multi-hop U2N Relay.

Observation 7	When all the L2 multi-hop U2N relay UEs are in RRC connected state to serve the CONNECTED remote UE’s data, the QoS split and SRAP configurations which include UE ID, BEARER ID and RLC channel configuration can be handled by the network as in R17 single hop U2N Relay.

Observation 8	As an intermediate relay UE, upon notification message reception, it needs to act to the received notification message similar to R17 Remote UE and notify the status to its connected child UE.

Observation 9	If the intermediate relay UE forwards the received notification message directly to the connected child UE, the notification message needs to be enhanced to indicate the hop/relay information for each notification. Additionally, another notification message may be needed after the intermediate relay UE processes the previous notification.

Observation 10	If the intermediate relay UE handles the received notification message before transmitting notification message to its connected child UE, it first performs RRC Reestablisment, cell change or PC5 release. Then, it generates a notification message to indicate the latest status via existing cause value (e.g., cell reselection or PC5 RLF) to the connected child UE.

We have the following proposals:

Proposal 1	RAN2 confirms that the scenario where all the multi-hop U2N Relay UEs are in RRC_CONNECTED state within the same cell as the U2N remote UE is supported.

Proposal 2	RAN2 not pursue the scenario for L2 multi-hop U2N Relay where intermediate relay UE and last relay UE camp on different cells.

Proposal 3	For L2 multi-hop U2N relay, R2 adopt approach-1, i.e., each of the Intermediate Relay UEs must be in RRC_CONNECTED when the U2N remote UE is in RRC_CONNECTED.

Proposal 4	For L2 multi-hop U2N relay, R2 discuss whether to rely on gNB to split the E2E QoS to per-hop QoS as in R17 single-hop U2N Relay.

Proposal 5	For L2 multi-hop U2N relay, R2 discuss whether to rely on gNB to do the SRAP configuration (e.g., UE ID, BEARER ID and egress RLC channel configurations).

Proposal 6	For SIB/posSIB acquisition at the IDLE/INACTIVE Remote UE, as in R17, the remote UE informs the required SIB(s)/posSIB(s) to the first relay UE via RemoteUEinformationSidelink message, and the first relay then transfers the valid SIB(s)/posSIB(s) requested by the remote UE via UuMessageTransferSidelink.

Proposal 7	For SIB/posSIB acquisition at the IDLE/INACTIVE intermediate relay UE, as in R17, the intermediate relay UE informs the required SIB(s)/posSIB(s) (including the requirements from itself and its child UEs) to the serving relay UE (another intermediate relay UE or the last relay) via RemoteUEinformationSidelink message, and the serving relay UE then transfers the valid SIB(s)/posSIB(s) requested by the intermediate UE via UuMessageTransferSidelink.

Proposal 8	For Paging acquisition at the IDLE/INACTIVE Remote UE, as in R17, the Remote UE provides Paging related information to the first relay UE via RemoteUEinformationSidelink message, and the first relay then transfers the related paging message to the remote UE via UuMessageTransferSidelink.

Proposal 9	For Paging acquisition at the IDLE/INACTIVE intermediate relay UE, as in R17, the intermediate relay UE provides Paging related information (including its own information and the information of the child UE(s)) to the serving relay UE (another intermediate relay UE or the last relay UE) via RemoteUEinformationSidelink message. The serving relay then transfers the related paging messages to the intermediate relay UE via UuMessageTransferSidelink.

Proposal 10	For CONNECTED intermediate Relay UE, as in R17, upon reception of paging request, it reports the received paging related information from child UEs to the network via SUI message.

Proposal 11	RAN2 to discuss intermediate Relay UE sends NotificationMessageSidelink to its connected child UE upon PC5 RLF or PC5 link release with its parent relay UE.

Proposal 12	As in R17, the intermediate Relay UE sends NotificationMessageSidelink to its connected child UE upon the following conditions:

-	upon Uu RLF;

-	upon reception of an RRCReconfiguration including the reconfigurationWithSync;

-	upon cell reselection;

-	upon L2 U2N Relay UE's RRC connection failure including RRC connection reject, and T300 expiry, and RRC resume failure

Proposal 13	As in R17, upon reception of NotificationMessageSidelink, the CONNECTED intermediate Relay UE triggers RRC Re-establishment, IDLE/INACTIVE intermediate Relay UE releases the PC5 unicast link with its parent relay UE or considers cell reselection happens.

Proposal 14	RAN2 to discuss the operation of intermediate Relay UE upon receiving NotificationMessageSidelink from serving relay:

Option-1: Forwards the received notification message directly to the connected child UE, with enhancements to NotificationMessageSidelink to indicate the hop/relay information.

Option-2: Handles the received notification message first (e.g., trigger RRC Reestablishment, cell change or PC5 release) and then generates NotificationMessageSidelink to its connected child UE based on the latest status.