3GPP TSG-RAN WG2 Meeting #127bis	R2-2409737

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.1.2.2

Source:	LG Electronics Inc.

Title:	NW side Data Collection

Document for:	Discussion and Decision

1. Introduction

During the RAN2 #127bis meeting, RAN2 reached the following agreements.

- Periodic logging is supported for training data collection procedure in R19 - Event-triggered data logging will be supported. At least radio condition based event triggered logging will be supported. FFS the details of radio condition based event. FFS if other events are supported. - Periodic reporting of logged data is not supported. - On-demand reporting of the logged measurements will be specified - UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection. FFS of details of the message - The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest. FFS trigger/definition of availability indication. and FFS how data availability indication is sent to the network. - Low priority SRB will be used. FFS new SRB or use of SRB4 - For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE. FFS if other data needs to be collected based on RAN1 progress.

This contribution aims to specify the configuration and procedure involved in data collection for BM use case.

2. Discussion

Logging architecture

RAN2 has agreed to introduce logging mechanism for data collection. When logging is necessary, the UE transfers data from the L1 (Physical layer) to the L3 (RRC layer), and it reports the logged data via L3 reporting. However, if logging is not required, data collection and reporting through RRC may lead to unnecessary internal operation within the UE to exchange data between layers.

For beam management use case, the data contents, such as L1-RSRP and/or beam index, may not differ depending on the purpose of data collection (e.g., inference, monitoring, or training). Therefore, whether the data is reported via L1 or L3 can be determined by whether the UE logs the data, not by the specific purpose of collection. Consequently, it is more efficient to consider L3 reporting (e.g., MDT reporting) only for logging-based reporting, i.e., when logging is not necessary, data is transmitted via L1 reporting (e.g., CSI reporting)

<Figure 1. Example of Logging architecture>

Proposal 1: L3 reporting (e.g., MDT reporting) is only for logging-based reporting, i.e., when logging is not necessary, training data is transmitted via L1 reporting (e.g., CSI reporting)

Configuration

In order to enable logging within the RRC, a dedicated CSI configuration for RRC logging can be newly established. This involves configuring the resource configuration for periodic, aperiodic, and semi-persistent CSI-RS, separate from the logging and reporting settings for each resource/resource set. However, CSI reporting may still be required for inference and monitoring operations where logging is unnecessary for the same resource/resource set, in which case the conventional CSI reporting via the L1 layer will continue to be used. Establishing a separate resource configuration solely for logging, identical to the one used for CSI reporting, would result in unnecessary duplication. Therefore, it is preferable that resource configuration for logging refer to the resource configuration of conventional CSI framework.

Observation 1. CSI reporting may still be required for inference and monitoring operations where logging is unnecessary, in which case the conventional CSI reporting via the L1 layer will continue to be used. Establishing a separate resource configuration solely for logging, identical to the one used for CSI reporting, would result in unnecessary duplication.

Proposal 2. Resource configuration for logging refers to the resource configuration of conventional CSI framework.

To set L3 reporting configuration for L1 measurements, the configuration can be affected from two parts. One part is for L1 measurement-related configuration (e.g., conventional CSI framework), while the other part is for L3 logging-related configuration.

<Figure 2. Configuration structure>

The L1 measurement-related configuration can be further divided into three sub-parts: Triggering State, Report and Resource Configuration, and Physical Layer Configuration.

- Physical Layer configuration: Contain references to NZP CSI-RS resources, CSI-IM resources, and SSB resources in a CSI-RS resource set

- Triggering State: Contain trigger states for dynamically selecting one or more aperiodic and semi-persistent reporting configuration and/or triggering one or more aperiodic CSI-RS resource sets

- Report and Resource Configuration: Contain reporting configuration for a periodic/semi-persistent/aperiodic report and resource configuration including a group of one or more NZP-CSI-RS-ResourceSet, CSI-IM-ResourceSet and/or CSI-SSB-ResourceSet

The L3 Logging configuration can be further divided into two sub-parts: Logging object and Logging-related configuration

- Logging Object: Logging object configuration can include resource configuration for obtaining L1 measurement results

- Logging-related configuration:  Logging configuration can include logging interval to indicate the periodicity for logging.

According to Observation 1, Logging Object can be linked to one of configurations in CSI reporting settings. There may five alternatives as below:

- Alternative 1. Link to reference signal, such as certain CSI-RS(s)/SSB(s) in L1 measurement-related configuration as shown in (1) of Figure 2.

- Alternative 2. Link to resource set ID associated with nzp-CSI-RS/csi-IM/csi-SSB resource(s) in L1 measurement-related configuration as shown in (2) of Figure 2.

- Alternative 3. Link to resource config ID associated with at least one resource set ID in L1 measurement-related configuration as shown in (3) of Figure 2.

- Alternative 4. Link to report config ID associated with resource config ID(s) in L1 measurement-related configuration as shown in (4) of Figure 2.

- Alternative 5. Link to entry index value of triggering state list associated with report config ID in L1 measurement-related configuration as shown in (5) of Figure 2.

From the perspective that the logging object is ultimately the reporting object, Alternative 4 makes sense to indicate logging object. Alternative 4 can also reuse the legacy DCI/MAC CE for CSI reporting to trigger logging. On the other hand, for Alternative 1-3, a separate DCI/MAC CE/RRC signalling is needed to indicate when to start/stop logging, especially for aperiodic/semi-persistent CSI-RS. Therefore, RAN2 could consider designing the signaling based on Alternative 4 to enable simpler UE operations. However, an LS should be sent to RAN1 to consult on whether the RS designated for logging can be specified through the CSI report config(CSI-ReportConfig).

Proposal 3. Send an LS to RAN1 to consult on whether the RS for logging object can be specified through the CSI report config

Logging mechanism

RAN2 should decide when to start and how to perform logging.

Initiation of logging

In legacy CSI reporting, periodic CSI reporting is triggered if the UE receives CSI configuration. Likewise, the logging of periodic CSI-RS can be triggered if the UE receives the logging configuration associated with the periodic CSI-RS.

Proposal 4. For periodic CSI-RSs, logging is initiated if logging configuration associated with the periodic CSI-RSs is received.

Conversely, according to TS 38.214, semi-persistent CSI-RS and aperiodic CSI-RS do not support periodic CSI reporting and therefore require additional DCI/MAC CE for reporting. Note that the logging object is essentially a reporting object. A straightforward approach to activating logging would be to reuse the existing DCI/MAC CE for CSI reporting, thus leveraging existing signalling. When the UE receives a DCI/MAC CE for CSI reporting, it can decide to either log or report the L1 measurement results based on its logging requirements. However, further discussion in RAN2 is necessary to fully evaluate this approach.

| CSI-RS Configuration | Periodic CSI Reporting | Semi-Persistent CSI Reporting | Aperiodic CSI Reporting |
| --- | --- | --- | --- |
| Periodic CSI-RS | No dynamic triggering/activation | For reporting on PUCCH, the UE receives an activation command, as described in clause 6.1.3.16 of [10, TS 38.321]; for reporting on PUSCH, the UE receives triggering on DCI | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |
| Semi-Persistent CSI-RS | Not Supported | For reporting on PUCCH, the UE receives an activation command, as described in clause 6.1.3.16 of [10, TS 38.321]; for reporting on PUSCH, the UE receives triggering on DCI | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |
| Aperiodic CSI-RS | Not Supported | Not Supported | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |

<TS 38.214. Table 5.2.1.4-1: Triggering/Activation of CSI Reporting for the possible CSI-RS Configurations. >

Proposal 5. RAN2 needs to discuss how/when logging for aperiodic/semi-persistent CSI-RS can be activated, e.g., whether to activate logging upon receiving CSI reporting DCI/MAC CE as legacy CSI reporting mechanism

logging operation

In the last meeting, RAN2 agreed to introduce periodic logging and event-triggered data logging. In legacy MDT operations, if the report type is periodic, the UE logs data at the specified logging interval. For event-triggered reporting, the UE performs logging at the logging interval only when the event conditions are met. In both cases, logging occurs according to the logging interval.

Similarly, for periodic/semi-persistent CSI-RS logging, the UE can log data at each logging interval, and the minimum logging interval can be the transmission periodicity of the CSI-RS.

<logging with periodic reporting w/ logging interval>

<logging with semi-persistent reporting w/ logging interval>

On the other hand, for aperiodic CSI-RS logging, a logging interval is not suitable, as the UE logs data only when it receives DCI/MAC CE. This makes it difficult for the network to pre-configure a logging interval, as the occurrence of DCI/MAC CE triggers is unpredictable. Therefore, the logging interval is not appropriate to aperiodic CSI-RS. Therefore, for aperiodic CSI-RS, logging can be triggered by DCI/MAC CE for logging.

<logging with aperiodic reporting>

Proposal 6. Logging interval is applied to periodic/semi-persistent CSI-RS, where logging interval is configured by network. Logging interval is not applied to aperiodic CSI-RS

Proposal 7. For aperiodic CSI-RS, UE performs logging upon each instance of receiving DCI/MAC CE for logging

Additional case for consideration

For logging interval-based operation, there may be instances where more than one L1(CSI-RS) measurement result is delivered within a single interval. In such cases, logging the most recent L1 measurement result at the logging interval would ensure that L3 always provides the latest data available, minimizing the impact of multiple results within a single interval. This approach keeps reporting consistent and prevents less relevant data from logging. There may also be cases where no L1 measurement result is delivered within a single interval. In this case, since there is no issue if the UE skips the logging operation, the logging can simply be skipped.

Proposal 8: For logging interval-based operations:

- If multiple L1 measurement results are delivered within a single interval, the UE logs only the most recent result to ensure the latest data is logged

- If no L1 measurement result is delivered, the UE skips the logging operation for that interval

Reporting Mechanism

In the last RAN2 meeting, triggering/definition of availability indication and how data availability indication is sent to the network were left as FFS.

RAN2 #127bis The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest. FFS trigger/definition of availability indication. and FFS how data availability indication is sent to the network.

When the UE can no longer perform logging (e.g., when it reaches its buffer limitation or enters a specific power state), the UE can indicate that logged data is available, allowing the network to retrieve the data. In these cases, there is no need for additional signaling to specify the reason for the logging failure. This is because it is difficult to specify all potential internal issues of the UE. Also, even if the network is aware of the reason, no further action or change in network behavior is expected based on this information. The UE can inform the network of this status through UAI without needing further details. The UE’s behavior during handover (HO) or RRC connection will be discussed in the following section.

Proposal 9 UE indicates that logged data is available when UE can no longer perform logging (e.g., when it reaches its buffer limitation or enters a specific power state)

Proposal 10. Available indication is transmitted via UAI. No need additional reason for triggering of availability indication

It was also left as FFS whether new SRB is introduced or SRB4 can be used for reporting of logged data.

RAN2 #127bis Low priority SRB will be used. FFS new SRB or use of SRB4

The UE's logged data can only be transmitted when requested by the network. In other words, even if both QoE and logging operations are configured, the network can prioritize either logging data or QoE data based on its implementation. Therefore, there is no issue with using the existing SRB4 for transmitting the logged data. As such, SRB4 can be used for the transmission of logged data.

Proposal 11. SRB4 is used for the transmission of logged data

UE behavior during HO or RRC state transitions

For gNB-centric training, there may be no guarantee that the UE can transmit the logged data to the gNB that has configured the logging. For example, transmission may fail due to radio quality issues in the serving cell before handover. Therefore, the UE should retain the logged data when performing handover, and the transmission of logged data from the source gNB to the target gNB may be necessary.

Proposal 12. UE retains logged data during HO

Proposal 13. UE indicates that logged data is available during HO (i.e., in RRCReconfigurationComplete message)

When transitioning to RRC Idle/Inactive, the network may not always be able to retrieve the logged data. For example, network resource constraints (e.g., network load issues) may prevent the retrieval of the logged data. Therefore, the UE should retain the logged data during the transition to RRC idle/inactive state and notify the network when re-establishing the connection that logged data is available.

Proposal 14.  UE retains logged data when transitioning to RRC idle/inactive

Proposal 15. UE indicates that logged data is available and when transitioning to RRC connected state (i.e., in RRCSetupComplete /RRCResumeComplete message)

However, if the UE receives a new logging configuration or determines that the valid time has expired, it should consider the logged data as no longer valid and clear it for the next logging operation. The conditions for releasing logged data can follow the existing logged MDT operation.

Proposal 16. UE releases logged data under the following conditions (same as Logged MDT):

- upon receiving new configuration

- upon power off

- upon deregistration

- upon a certain time later, FFS length of timer (like 48-hour timer in logged MDT)

- upon successful delivery of the UEInformationResponse message (w/ logged data)

3. Conclusion

Proposal 1: L3 reporting (e.g., MDT reporting) is only for logging-based reporting, i.e., when logging is not necessary, training data is transmitted via L1 reporting (e.g., CSI reporting)

Observation 1. CSI reporting may still be required for inference and monitoring operations where logging is unnecessary, in which case the conventional CSI reporting via the L1 layer will continue to be used. Establishing a separate resource configuration solely for logging, identical to the one used for CSI reporting, would result in unnecessary duplication.

Proposal 2. Resource configuration for logging refers to the resource configuration of conventional CSI framework.

Proposal 3. Send an LS to RAN1 to consult on whether the RS for logging object can be specified through the CSI report config

Proposal 4. For periodic CSI-RSs, logging is initiated if logging configuration associated with the periodic CSI-RSs is received.

Proposal 5. RAN2 needs to discuss how/when logging for aperiodic/semi-persistent CSI-RS can be activated, e.g., whether to activate logging upon receiving CSI reporting DCI/MAC CE as legacy CSI reporting mechanism

Proposal 6. Logging interval is applied to periodic/semi-persistent CSI-RS, where logging interval is configured by network. Logging interval is not applied to aperiodic CSI-RS

Proposal 7. For aperiodic CSI-RS, UE performs logging upon each instance of receiving DCI/MAC CE for logging

Proposal 8: For logging interval-based operations:

- If multiple L1 measurement results are delivered within a single interval, the UE logs only the most recent result to ensure the latest data is logged

- If no L1 measurement result is delivered, the UE skips the logging operation for that interval

Proposal 9. UE indicates UE indicates that logged data is available when UE can no longer perform logging (e.g., when it reaches its buffer limitation or enters a specific power state)

Proposal 10. Available indication is transmitted via UAI. No need additional reason for triggering of availability indication

Proposal 11. SRB4 is used for the transmission of logged data

Proposal 12. UE retains logged data during HO

Proposal 13. UE indicates that logged data is available during HO (i.e., in RRCReconfigurationComplete message)

Proposal 14.  UE retains logged data when transitioning to RRC idle/inactive

Proposal 15. UE indicates that logged data is available and when transitioning to RRC connected state (i.e., in RRCSetupComplete /RRCResumeComplete message)

Proposal 16. UE releases logged data under the following conditions (same as Logged MDT):

- upon receiving new configuration

- upon power off

- upon deregistration

- upon a certain time later, FFS length of timer (like 48-hour timer in logged MDT)

- upon successful delivery of the UEInformationResponse message (w/ logged data)