3GPP TSG-RAN WG2 Meeting #128	R2-2409742

Orlando, USA, Nov. 18th – 22nd , 2024

Agenda Item:   8.2.5

Source:        Xiaomi

Title:           Considerations on TP2 related aspects between BS and UE reader

Document for:  Discussion and Decision

- Introduction

During the last meeting, RAN2 achieved the following agreements about the TP2 related aspects between BS and UE reader.[1]

Agreements

- Dedicated resource configuration is only given to the UE reader via dedicated signalling. Mechanisms for shared resource pool amongst readers are not considered in this release.

- The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control. FFS how valid resource is determined – we will consider temporary out of connection scenarios. FFS on resource validity across multiple cells.

- The AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc).

In this contribution, we provide our further considerations on the TP2 related aspects between BS and UE reader.

- Discussion

  - Resource allocation for the AIoT air interface

In last meeting, we achieved the following agreement on the resource allocation for the AIoT air interface:

=> Dedicated resource configuration is only given to the UE reader via dedicated signalling. Mechanisms for shared resource pool amongst readers are not considered in this release.

However, there is no consensus on the further details of the resource allocation for R2D and D2R, and the proposed options are quoted below. [2]

- Option1: Upon receiving the dedicated resource configuration from gNB, UE reader directly uses the configured dedicated resources for the AIoT air interface.

- Option2: Upon receiving the dedicated resource configuration from gNB, UE reader selects the resource from the configured dedicated resources for A-IoT air interface.

For option1, we think it is difficult for the gNB to perform the resource allocation directly. In more details, if the resource allocation is done in a dynamic way, the UE reader needs to indicate the AIoT operation related information before sending each R2D command over the A-IoT interface This will result in significant signaling overhead on the Uu interface and introduce additional latency due to the resource allocation process. If the resource allocation is done in a semi-static way, e.g., similar to CG in the NR, as configuration only supports periodic resource allocation with fixed time/frequency domain resources, it is difficult to efficiently allocate resources to accommodate the variable data transmission sizes in each AIoT operation for the UE reader, and it also limits the UE reader's behaviour to the available periodic resources.

From RAN2 perspective, compared to option 1, we prefer to adopt option 2, as it reduces the resource allocation signaling between the network and the UE reader, and also provides more flexibility for the UE reader. In more detail, once a UE is selected as a reader by the network, the network can provide one or more resource pools to the UE reader. Then, when the UE reader performs R2D transmission, it can select resources from the configured resource pool for the R2D transmission, and also for the D2R transmission if any. Regarding the configured resource pool, it can be shared among multiple UEs or dedicated to a single UE. If the resource pool is shared among multiple UEs, whether or how to avoid the resource collision among these UEs needs to be further discussed.

Since the resource allocation relies on the RAN1 discussion, which has already been discussed and captured in the TR for further discussion in the WI phase, we can continue this discussion in the later WI phase and coordinate with RAN1.

Observation 1: Dynamic resource allocation via the gNB directly will introduce significant signalling overhead and latency.

Observation 2: Configured grant-based resource allocation via the gNB directly only supports periodic and fixed time/frequency domain resources, which is inefficient for the variable sizes of AIoT transmissions and may restrict the behaviour of UE reader.

Proposal 1: From the RAN2 perspective, the UE reader selects the resource from the configured dedicated resources for A-IoT air interface. Can be further discussed in the WI phase and coordinated with RAN1.

  - Validity of AIoT radio resource

In the last meeting, we achieved the following agreement, and agreed that UE can perform the AIoT procedure as long as there is valid AIoT resource configuration.

=> The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control. FFS how valid resource is determined – we will consider temporary out of connection scenarios. FFS on resource validity across multiple cells.

But if the UE is out of connection, how the UE determines whether there is a valid resource is still FFS. During the discussion, it was proposed that AIoT resources could be configured with a valid duration, and the UE would consider the resource valid during that period. However, we think it is unfeasible for the out-of-connection case because it is difficult for the network to be aware of the exact moment the UE becomes out of connection, making it very challenging for the network to reserve and manage resources configured for the AIoT procedure.

Observation 3: It is unfeasible for the network to configure the validity duration for the AIoT resource if the UE reader is out of network control.

Proposal 2: Not to support the valid duration for AIoT resources used in the out-of-connection case.

As for the validity of resources across multiple cells, from the perspective of the UE reader, it benefits the continuity of AIoT service across cells. When the UE moves to a new cell, the UE reader can continue to use the current AIoT configuration without needing to perform configuration acquisition. This ensures service continuity and reduces the possibility of an RRC state transition if the UE can perform the AIoT procedure while out of connection. However, the validity across multiple cells depends on the network's coordination of the AIoT resource. From the network's perspective, coordinating resources across multiple cells is neither flexible nor efficient for resource scheduling.

As this can be considered as an enhancement for the UE reader, we can focus on the resource validity within the serving cell that provides the AIoT resource configuration in this release, which can reduce the complexity and be further enhanced in future releases. If there is strong support for resource validity across multiple cells, we prefer to focus on intra-gNB cells over the inter-gNB cell scenario, which have fewer specification impacts and can reduce the complexity of network resource coordination.

Observation 4: Resource validity across multiple cells requires for the network coordination.

Proposal 3: As a baseline, the AIoT resources used in the out-of-connection case are only valid within the current serving cell.

Proposal 4: If the resource validity can be extended to multiple cells, the intra-gNB cells can be considered.

Based on the analysis above, we think a straightforward approach would be for the network to indicate the AIoT resource configuration used for out-of-connection scenarios, if it wishes to support the UE reader in doing so. The AIoT resource configuration can be the same as or separate from that for the connection state. This depends on the network's decision, and we have no strong opinion on it. From the perspective of the UE reader, if it is configured with AIoT resources for out-of-connection scenarios within its current serving cell, it assumes these resources are valid and performs the AIoT procedure accordingly until the network reconfigures them.

Proposal 5: If the UE reader is configured with AIoT resources for out-of-connection scenarios within its serving cell, it assumes these resources are valid and performs the AIoT procedure accordingly until the network reconfigures them.

  - Device data aggregated on the UE reader side

The following is indicated as interim conclusion in the SA2 TR to express that the aggregation needs to be discussed in the RAN WG and to have the coordination cross the WGs. [3]

Editor’s Note:	For RRC based solution of topology 2, whether the aggregation can be performed by the RAN is FFS and coordination with RAN WGs is needed.

Editor’s Note:	How the aggregation can be done is FFS.

From RAN2 perspective, we agreed to support paging multiple devices for response in one A-IoT procedure. For the procedure involving multiple devices, upon receiving the response from the device, there are two options for the reader to forward it to the network:

- Option 1: Forward the device response one by one immediately upon receiving it. Take inventory as an example, as shown below:

Fig.1 Non-aggregated data forwarding on reader side

- Option 2: Aggregate multiple responses and forward them together in one message. Take inventory as an example, as shown below:

Fig.2 Aggregated data forwarding on reader side

For the options above, we can note that option1 could result in a signalling storm between the reader and the A-IoT CN node, while option2 would have a significantly smaller signalling overhead at the expense of response latency.

Actually, RAN3 has discussed this issue for TP1 in the previous meeting and achieved the following agreement. [4]

In topology1, it is FFS whether in response to a single Inventory Request, AIoT RAS provides only a single Inventory Report message or there can be also multiple Inventory Report messages provided.

However, whether to perform the aggregation has different impacts on different interfaces, and needs to be discussed separately in RAN3 and RAN2. Additionally, the signaling storm caused by the sequential data forwarding in TP2 has a much more serious impact compared to TP1, because we usually care more about the radio resource consumption in the Uu interface, rather than the network interface.

From this view, Option 2 is a more suitable and efficient choice for TP2. Furtherly, considering the increased response latency, Option 2 can be applied to the A-IoT procedure, which is not sensitive to latency, such as the inventory use case. As for the command use case, we can analyze it further or leave it to the UE implementation.

This issue needs to be discussed anyway to clarify the UE reader behavior on the data forwarding for the AIoT device, but since it depends somewhat on the protocol stack structure and requires coordination with other groups, we can discuss it in the WI phase.

Observation 5: In the TP2, UE reader forwarding the D2R data transmissions one by one may result in the signalling storm in the Uu interface.

Proposal 6: In TP2, RAN2 to discuss whether the UE reader can aggregate the received D2R data transmissions over Uu interface. Can be discussed in the WI phase to coordinate with other WGs.

- Option1: UE reader forwarding the received D2R data transmission one by one.

- Option2: UE reader aggregating the received D2R data transmission over Uu interface and forwarding to BS together, at least for the inventory use case.

  - Service continuity during the UE reader mobility

According to the SID, the AIoT device does not support mobility. However, in TP2, the UE serving as the intermediate node can be a normal UE with the AIoT functionalities so that it needs to support the legacy mobility as in the NR system, including the cell selection/reselection and the handover.

However, during the mobility of the UE reader, it may affect the AIoT procedure on the AIoT air interface. For example, if the UE moves to the edge of the BS coverage or to a BS that does not support the AIoT procedure, the AIoT procedure may be interrupted, which could cause data loss.

As such, it is necessary to consider how to ensure service continuity and reduce data loss during the mobility of the UE reader. For instance, the UE should prioritize moving to a cell that supports at least the ongoing AIoT procedure during cell selection, reselection, or handover procedures. Moreover, if the UE is moving to the edge of a BS's coverage area or to a BS that does not support the AIoT procedure, the UE reader can transmit the collected device data to the network as soon as possible to minimize data loss.

Observation 6: The UE reader can be a normal UE with AIoT functionalities and needs to support the mobility including the cell selection/reselection and handover.

Observation 7: Service interruption or data loss may occur if the UE reader moves to the edge of a BS or to a BS that does not support A-IoT functionalities.

Proposal 7: RAN2 considers to support the Ambient IoT service continuity during the UE reader mobility.

  - UE BSR for the device data forwarding

In the existing mechanism, there is pre-emptive Buffer Status Report (BSR) mechanism which is introduced for the IAB-node to reduce the UL scheduling latency through signalling of a Pre-emptive BSR to its parent node.

Similarly, in the A-IoT system, the UE reader could also consider a BSR mechanism similar to the pre-emptive BSR. In detail, the UE reader may be able to predict the volume of data it will receive from devices. This prediction could be based on factors such as the estimated number of device responses during an inventory procedure or the expected size of the data to be read from the device. With this predicted data volume, the UE reader could initiate a pre-emptive BSR to reduce data transmission delays. This is particularly beneficial for delay-sensitive processes, such as command use case.

Observation 8: UE reader can predict the data volume from the devices based on the estimated number of device and/or the expected data size per device, which can be used to initiate the pre-emptive BSR for the latency reduction.

Proposal 8: The pre-emptive BSR can be supported by the UE reader for the data transmission latency reduction.

  - Impacts on the UE power control

In the existing NR system, UE power control only considers the power consumption on the Uu signalling interface. However, in the Ambient IoT system, if a UE serves as an intermediate node or a CW node, there is additional power consumption on the R2D and/or CW transmissions, in addition to the existing NR Uu signaling transmission. Therefore, to achieve accurate power control for a UE acting as an intermediate node or CW node in the Ambient IoT system, the network should also account for this additional power consumption during R2D and/or CW transmissions. Otherwise, it may lead to inaccurate power control for the UE reader.

To achieve accurate power control on the UE serving as intermediate node or CW node in the Ambient IoT system, following options can be considered:

- Option 1: The UE determines the power consumption on R2D and/or CW transmissions and indicates it to the network for power control, for example, by using a new PHR type.

- Option 2: The network performs a hard split of the power consumption on the R2D and/or CW transmissions, and NR Uu interface transmissions, and indicates this split to the UE.

- Option 3: The calculation of the PCmax takes into account the power consumption on the R2D and/or CW transmissions, which can be further discussed and determined in RAN4.

Observation 9: In NR system, only the UE power consumption on the Uu signaling transmission is considered for UE power control.

Proposal 9: In the AIoT system, if the UE serves as an intermediate node or CW node, the power consumption on the R2D and/or CW transmissions should also be considered for the UE power control.

  - Active time control of PDRCH reception

In the current spec, if the UE is configured with DRX, it does not need to continuously monitor PDCCH for power saving. For a UE serving as an intermediate node in the A-IoT system, it also needs to monitor the PDRCH in the A-IoT interface as well as the PDCCH, and the impacts of the UE DRX on the PDRCH need to be considered.

If there are separate reception module for the PDCCH and PDRCH, the control of the PDCCH and PDRCH monitoring is separate, there is no need for the coordination of the both. However, the additional reception module for PDRCH monitoring on the UE side will increase costs to some extent.

If there is a shared reception module for PDCCH and PDRCH, the DRX configured for the UE applies to both PDCCH and PDRCH monitoring. During the inactivity period for PDCCH monitoring, the UE will also not monitor the PDRCH. To avoid missing PDRCH, the UE reader should remain active for PDRCH reception during A-IoT operation, and the impacts on the UE DRX need to be considered, such as not to configure UE reader with the DRX to always enable PDRCH monitoring, or to configure the UE DRX on-duration time with PDRCH reception taken into account, etc.

Observation 10: In case the reception module for PDCCH and PDRCH is shared, the active time of the UE DRX will also impact on the PDRCH reception.

Proposal 10: If the PDCCH and PDRCH share the same reception module on the UE reader side, the UE DRX configuration should take into account the PDRCH monitoring time to avoid missing the PDRCH.

  - Co-existence of the A-IoT function and CA

Carrier aggregation (CA) is used to increase data rates and improve network capacity by utilizing multiple frequency bands simultaneously. However, due to the limited capabilities of A-IoT devices, such as the small size of data packets they handle, there is no need for UE reader to support CA, as it would only add unnecessary complexity to both the UE and the network. Therefore, we propose that a UE serving as an intermediate node in the A-IoT system should not be required to operate in CA mode. Even if the UE can be configured with CA, to reduce the complexity and the impact on the spec, we propose that only the radio resource of the PCell can be configured to UE for the A-IoT operation towards the device.

Proposal 11: For the co-existence of the A-IoT function and CA, the following options can be considered.

- Option1: Not to consider the UE reader operating with CA.

- Option2: For a UE reader operating with CA, only the radio resource of the PCell can be configured for the A-IoT operation in the A-IoT interface.

- Conclusions

According to the analysis given above, we have the following observations and proposals:

Resource allocation for the AIoT air interface

Observation 1: Dynamic resource allocation via the gNB directly will introduce significant signalling overhead and latency.

Observation 2: Configured grant-based resource allocation via the gNB directly only supports periodic and fixed time/frequency domain resources, which is inefficient for the variable sizes of AIoT transmissions and may restrict the behaviour of UE reader.

Proposal 1: From the RAN2 perspective, the UE reader selects the resource from the configured dedicated resources for A-IoT air interface. Can be further discussed in the WI phase and coordinated with RAN1.

Validity of AIoT radio resource

Observation 3: It is unfeasible for the network to configure the validity duration for the AIoT resource if the UE reader is out of network control.

Observation 4: Resource validity across multiple cells requires for the network coordination.

Proposal 2: Not to support the valid duration for AIoT resources used in the out-of-connection case.

Proposal 3: As a baseline, the AIoT resources used in the out-of-connection case are only valid within the current serving cell.

Proposal 4: If the resource validity can be extended to multiple cells, the intra-gNB cells can be considered.

Proposal 5: If the UE reader is configured with AIoT resources for out-of-connection scenarios within its serving cell, it assumes these resources are valid and performs the AIoT procedure accordingly until the network reconfigures them.

Device data aggregated on the UE reader side

Observation 5: In the TP2, UE reader forwarding the D2R data transmissions one by one may result in the signalling storm in the Uu interface.

Proposal 6: In TP2, RAN2 to discuss whether the UE reader can aggregate the received D2R data transmissions over Uu interface. Can be discussed in the WI phase to coordinate with other WGs.

- Option1: UE reader forwarding the received D2R data transmission one by one.

- Option2: UE reader aggregating the received D2R data transmission over Uu interface and forwarding to BS together, at least for the inventory use case.

Service continuity during the UE reader mobility

Observation 6: The UE reader can be a normal UE with AIoT functionalities and needs to support the mobility including the cell selection/reselection and handover.

Observation 7: Service interruption or data loss may occur if the UE reader moves to the edge of a BS or to a BS that does not support A-IoT functionalities.

Proposal 7: RAN2 considers to support the Ambient IoT service continuity during the UE reader mobility.

UE BSR for the device data forwarding

Observation 8: UE reader can predict the data volume from the devices based on the estimated number of device and/or the expected data size per device, which can be used to initiate the pre-emptive BSR for the latency reduction.

Proposal 8: The pre-emptive BSR can be supported by the UE reader for the data transmission latency reduction.

Impacts on the UE power control

Observation 9: In NR system, only the UE power consumption on the Uu signaling transmission is considered for UE power control.

Proposal 9: In the AIoT system, if the UE serves as an intermediate node or CW node, the power consumption on the R2D and/or CW transmissions should also be considered for the UE power control.

Active time control of PDRCH reception

Observation 10: In case the reception module for PDCCH and PDRCH is shared, the active time of the UE DRX will also impact on the PDRCH reception.

Proposal 10: If the PDCCH and PDRCH share the same reception module on the UE reader side, the UE DRX configuration should take into account the PDRCH monitoring time to avoid missing the PDRCH.

Co-existence of the AIOT function and CA

Proposal 11: For the co-existence of the A-IoT function and CA, the following options can be considered.

- Option1: Not to consider the UE reader operating with CA.

- Option2: For a UE reader operating with CA, only the radio resource of the PCell can be configured for the A-IoT operation in the A-IoT interface.

- References

- R2_127_ChairNotes_24-08-23_13-30

- R2-2406610 Discussion on A-IoT Topology 2, Samsung, Rel-19

- 3GPP TR 23.700-13 V1.01.0 Study on Architecture support of Ambient power-enabled Internet of Things (Release 19)

- RAN3 #124 Meeting Report Fukuoka, Japan, 20/05/2024 - 24/05/2024