3GPP TSG-RAN WG2 Meeting #128		R2-2409763

Orlando, USA, 18th – 22nd Nov. 2024

Source:	vivo

Title:	Discussion on LP-WUS/WUR in RRC_CONNECTED

Agenda Item:	8.4.4

Document for:	Discussion and Decision

- Introduction

In RAN2#127 and RAN2#127bis meetings [1][2], some progress on LP-WUS Option 1-2 has been made in RAN2, as below.

For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1. For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2. In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring. When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected.

Meanwhile, RAN1 has agreed with some detailed design for option 1-2 in RAN1#118 meeting [3] and confirmed the working assumption to support both Option 1-1 and Option 1-2 in RAN1#118bis meeting [4].

Agreement For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. - LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - UE is configured with legacy C-DRX configurations as Rel-18. - UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration). - FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration. - LP-WUS triggers the start of a timer during which UE monitors PDCCH. - FFS the timer is existing timer and/or new timer. - UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected. - drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL. - No impact on RRM/RLM/BFD measurement requirements is assumed. - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1). - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up. - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not. - UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS. Confirm following working assumption Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions.

In this contribution, we will further discuss the RAN2 impacts on RRC_CONNECTED procedures with LP-WUS/WUR from RAN2 perspective.

- Discussion

  - RAN2 impacts on Option 1-2

LP-WUS monitoring and PDCCH monitoring behaviour

In RAN2#127bis meeting, RAN2 discussed the LP-WUS monitoring behaviour. It was easily agreed that LP-WUS monitoring is performed outside legacy C-DRX Active Time. However, companies have different understandings on whether the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2 as below. This will impact whether the LP-WUS monitoring could be overlapped with the duration indicated by legacy drx-onDurationTimer.

For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2.

Based on the online/offline discussion, the reason for this FFS is some companies think the legacy drx-onDurationTimer should be running for periodic CSI/L1-RSRP report and SP CSI report as well as the RRM/RLM/BFD measurement.

On one hand, some companies think the drx-onDurationTimer won’t be started for this case considering the RAN1 agreement that “UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS”, thus the duration indicated by legacy drx-onDurationTimer is also included in the period outside legacy C-DRX Active Time and the current conclusion on LP-WUS monitoring has covered this duration.

Observation 1: If the drx-onDurationTimer is not started, i.e. the duration is not the C-DRX Active time, the current conclusion on LP-WUS monitoring has covered the duration indicated by drx-onDurationTimer.

On the other hand, some companies argue that it will impact the CSI/L1-RSRP report and RRM/RLM/BFD measurement if drx-onDurationTimer is not started. Thus, they think the legacy drx-onDurationTimer should be started for CSI/L1-RSRP report and RRM/RLM/BFD measurement, but no PDCCH is monitored during this timer is running. However, the same thing happens for DCP which is introduced in Rel-16. When DCP indication associated with the current DRX cycle indicated not to start drx-onDurationTimer or when no DCP indication is received, drx-onDurationTimer will not be started, while UE could still report CSI/L1-RSRP and perform RRM/RLM/BFD measurement. Thus, we think the same UE behaviour, e.g. on CSI/L1-RSRP report, RRM/RLM/BFD measurement when drx-onDurationTimer is not started as shown in Part 1 of Fig.1 for Rel-16 DCP could be reused, and if this duration overlaps with other C-DRX Active Time, e.g. the LP-WUS triggered active time as shown in Part 2 of Fig.1, the UE behaviour within the overlapped part should be same as the UE behaviour in the related C-DRX Active Time.

Figure 1: Time duration while introducing Option 1-2 LP-WUS

Observation 2: If the drx-onDurationTimer is not started, UE behaviour within the duration indicated by drx-onDurationTimer including CSI/L1-RSRP report and RRM/RLM/BFD measurement for Rel-16 DCP could be reused for LP-WUS.

In Rel-16 DCP design, if the drx-onDurationTimer is not started, the PDCCH monitoring won’t be performed within drx-onDurationTimer [5], which is also aligned with the RAN1 agreement on PDCCH monitoring for LP-WUS Option 1-2. Besides, current specification has captured the UE behaviour on CSI/L1 RSRP report and RRM/RLM/BFD measurement within the duration indicated by drx-onDurationTimer when drx-onDurationTimer is not started, which could be reused directly for LP-WUS. Thus, it is quite straightforward that the legacy drx-onDurationTimer is not started if the new timer is configured in Option 1-2.

Proposal 1: For Option 1-2 (i.e., new timer is configured), the legacy drx-onDurationTimer is not started, i.e., the duration indicated by drx-onDurationTimer is not the C-DRX Active time.

CSI report behaviour related to LP-WUS Option 1-2

In RAN2#127bis meeting, we agreed for Option 1-2, a new timer triggered by LP-WUS is introduced, and UE is in C-DRX Active Time when this new timer is running:

In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring.

Thus, the UE behaviour not only on the PDCCH monitoring but also on the CSI/L1-RSRP report within this time duration triggered by LP-WUS (i.e., Part -2 in Fig.1) should be the same as the legacy C-DRX Active Time. There is no motivation to differentiate the CSI/L1-RSRP reporting behaviour as legacy C-DRX Active Time for LP-WUS triggered C-DRX Active Time. Specifically, UE reports the periodic CSI on PUCCH, Semi-periodic CSI report on PUCCH and Semi-periodic CSI report on PUSCH during this timer is running triggered by LP-WUS.

Proposal 2: For Option 1-2, during the timer triggered by LP-WUS is running, periodic CSI/L1-RSRP report and SP CSI report behaviour are the same as legacy drx-onDurationTimer duration without DCP configuration, i.e. always report during this timer triggered by LP-WUS.

RAN1 has the following agreement in RAN1#118 meeting [3], regarding the periodic CSI/L1-RSRP reporting:

- For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1). - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up. - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not.

As we have analyzed above, the same issues happened in Rel-16 DCP and we concluded that UE determines whether to report the periodic CSI/L1-RSRP based on the network configuration for DCP, i.e., UE determines whether to report the periodic CSI that is L1-RSRP on PUCCH based on the ps-TransmitPeriodicL1RSRP parameter and whether to report the periodic CSI that is not L1-RSRP on PUCCH based on the ps-TransmitOtherPeriodicCSI parameter. We think the same principle could be considered here for legacy drx-onDurationTimer in Option 1-2. Thus, we propose:

Proposal 3: For Option 1-2, within legacy drx-onDurationTimer duration and the MAC entity is not in Active Time, it is up to network configuration to determine whether to report the periodic CSI/L1-RSRP, i.e. same as in Rel-16 DCP.

Coexistence with PDCCH skipping

Besides, since UE is in C-DRX Active Time when the new timer triggered by LP-WUS is running, similar to Rel-16 DCP which also triggers UE enters C-DRX Active Time, PDCCH skipping could also work together with the LP-WUS. In this case, PDCCH skipping indication is used to stop the PDCCH monitoring as legacy.

When UE receives the PDCCH skipping indication, UE will not perform PDCCH monitoring. But the PDCCH skipping indication will not impact the C-DRX Active Timer switch. In this case, it is possible that UE is still within the C-DRX Active Time (but just has stopped the PDCCH monitoring), in this way, UE won’t switch back to the LP-WUS monitoring since we only agreed UE goes back to LP-WUS monitoring when UE is not in C-DRX active time as below.

In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring.

However, the network may want to trigger UE to perform the data transmission again later after it indicates UE to skip the PDCCH monitoring within this C-DRX Active Time in some cases, e.g. the drx-InactivityTimer is set to a long value, and considering the LP-WUS monitoring only consume little power, we think UE could also go bake to LP-WUS monitoring to reduce the data transmission latency in this case.

Proposal 4: For Option 1-2, when LP-WUS is used together with PDCCH skipping, the UE stops the PDCCH monitoring upon receiving the PDCCH skipping indication, and starts the LP-WUS monitoring.

Impacts on other legacy C-DRX Active Time

RAN1 agreed when Option 1-2 is configured, PDCCH monitoring behaviour related to other legacy DRX timers won’t be affected. And RAN2 also made the related agreement as follows:

When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected.

According to the agreement, we think when UE is in C-DRX Active Time triggered by LP-WUS, the detailed PDCCH monitoring behaviour related to other legacy DRX timers is as follows:

- For drx-InactivityTimer: if the PDCCH indicates a new transmission (DL, UL or SL) on a Serving Cell, the UE will start the C-DRX drx-InactivityTimer timer.

- For HARQ RTT timer and retransmission timer: if a MAC PDU is received in a configured downlink assignment/configured uplink grant or if a PDCCH indicates a DL/UL transmission, the UE will start the HARQ RTT timer and stop the retransmission timer for the corresponding HARQ process; besides, the UE starts the corresponding retransmission timer after the HARQ RTT timer expires.

However, according to current C-DRX pattern, not only the above DRX timers but also the ra-ContentionResolutionTimer, msgB-ResponseWindow may occur within the C-DRX Active Time, we think the PDCCH monitoring behaviour should also not be affected when LP-WUS Option 1-2 is used.

Proposal 5: When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to ra-ContentionResolutionTimer and msgB-ResponseWindow are not affected if LP-WUS Option 1-2 is used.

  - RAN2 views on LP-WUS Options

Coexistence between LP-WUS Option 1-1 and Option 1-2

In RAN1#118bis meeting, RAN1 has agreed to support both Option 1-1 and Option 1-2 with them working alone as below:

For Option 1-1 and Option 1-2, RAN1 has confirmed to support them working alone, since we have studied Option 1-1 and Option 1-2 througly, there is no motivation not to support Option 1-1 or Option 1-2 in RAN2 perspective. From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions

However, it is still FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE. In our view, it is not necessary since the effects achieved by using Option 1-1 can be accomplished through a proper configuration of Option 1-2.

For example, if there is DL transmission before or within the drx-ondurationTimer duration, for Option 1-1, the network will send LP-WUS to trigger the UE to start the drx-ondurationTimer and enter the C-DRX Active Time for PDCCH monitoring; and for Option 1-2, the network could also send the LP-WUS which is also located before the drx-ondurationTimer to trigger a new timer for PDCCH monitoring. And it could be observed that both Options alone could work well, i.e., without data transmission latency. Besides, the Option 1-2 could work in more time duration except the duration before the drx-ondurationTimer than Option 1-1.

On the other hand, if the LP-WUS Option 1-1 is configured together with the Option 1-2 for the same UE, it would bring more complexity from joint configuration and joint operation perspective, e.g., network may need to avoid the WUS MO for option 1-1 colliding with WUS MO for option 1-2 or proper UE behavior needs to be defined.

To avoid the complexity for network configuration and UE implementation, and to save the standardization effort, we prefer not to support the coexistence between Option 1-1 and Option 1-2.

Figure 2: Coexistence between LP-WUS Option 1-1 and Option 1-2

Proposal 6: Don’t support Option 1-1 and Option 1-2 simultaneously configured for the same UE.

Whether support Option 1-3

For Option 1-3, RAN1 didn’t have any conclusion on it. Since Option 1-3 is transparent to MAC, which only impacts the physical layers design, we think it is better to wait for RAN1 progress, if any. RAN2 should focus on Option 1-1 and Option 1-2 and won’t discuss Option 1-3 unless RAN1 triggers.

Proposal 7: RAN2 won’t discuss Option 1-3 unless RAN1 triggers.

  - LP-WUS operation for UE configured with NR-DC

RAN1 discussed the LP-WUS configured in CA and DC case and had the following agreements:

Agreement in RAN1#118 meeting: LP-WUS is at least supported for the case where a UE is configured with CA in RRC CONNECTED mode - FFS: DC Agreement in RAN1#118bis meeting: LP-WUS is supported when UE is configured with NR-DC in RRC CONNECTED mode - Above is supported for the case PDCCH monitoring is triggered by LP-WUS in the same cell group - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable

It could be observed that RAN1 agreed LP-WUS is supported when UE is configured with CA and NR-DC. For NR-DC case, the LP-WUS is configured separately in MCG and SCG, where the LP-WUS related to MCG could only trigger the PDCCH monitoring of the serving cells in MCG and the LP-WUS related to SCG could only trigger the PDCCH monitoring of the serving cells in SCG.

And for which serving cell(s) of the corresponding cell group the PDCCH monitoring triggered by a LP-WUS is applicable, we think the same mechanism as Rel-16 DCP could be used, i.e., all activated serving cells in the cell group are applicable. Besides, since for both Option 1-1 and Option 1-2, the LP-WUS could trigger the UE enter the C-DRX Active Time, and when dual DRX is not configured, the PDCCH monitoring in all activated serving cell could be performed in the C-DRX active time. Thus, there is no need to restrict the LP-WUS only applicable to specific serving cell(s) within the cell group.

Proposal 8: For both option 1-1 and option 1-2 in NR-DC case, LP-WUS is configured separately in MCG and SCG, and LP-WUS could trigger the PDCCH monitoring of all activated serving cells within the same cell group.

  - Enable and disable for LP-WUS monitoring

RAN1 has also agreed that gNB RRC signalling should be used to enable/disable the operation of PDCCH monitoring triggered by LP-WUS, but it is still FFS whether to support UE assistance for the enable/disable functionality.

For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance.

We think the enable/disable operation could rely on the configuration of LP-WUS as usual in RAN2. Regarding the additional UE assistance, according to the WID in [6], current RRM/RLM/BFD/CSI/BM measurement and corresponding report are performed by MR as legacy. In this way, network has enough information to enable/disable the LP-WUS functionality. Besides, the enable/disable operation is a long-term configuration. There is no need to dynamically change the configuration. Therefore, we think additional assistance information from UE to gNB for enable/disable operation is not necessary.

Proposal 9: In RRC_CONNECTED mode, PDCCH monitoring triggered by LP-WUS is enabled/disabled by the corresponding configuration of LP-WUS. No additional assistance information from UE is needed.

Regarding the RRC message to carry the LP-WUS configuration, RAN2 have agreed the dedicated RRC message is used.

The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

In our understanding, it is straightforward to use the RRCResume and RRCReconfiguration messages as usual.

Proposal 10: LP-WUS configuration is included in RRCResume/RRCReconfiguration message.

  - Activation and deactivation procedure for LP-WUS monitoring

Regarding the activation and deactivation for LP-WUS monitoring, RAN1 has the following options:

- In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

From RAN2 perspective, the meaning of enabling LP-WUS monitoring is to activate the LP-WUS functionality while UE couldn’t use LP-WUS if the LP-WUS functionality has some activation mechanisms. In this case, UE could use the LP-WUS functionality after receiving the LP-WUS activation signalling, and UE stops using the LP-WUS functionality upon receiving the LP-WUS deactivation signalling.

Considering the LP-WUS configuration overhead is comparable to other dedicated configurations, e.g. L3 measurement configuration, besides the LP-WUS monitoring is not an urgent behaviour, there is no motivation to introduce additional activation/deactivation procedure for LP-WUS monitoring. UE could use the LP-WUS when the LP-WUS monitoring is enabled, i.e., when UE receives the RRC dedicated signalling with LP-WUS configuration. And if the network doesn’t expect the UE to use the LP-WUS, it could just send the RRC signalling to disable the LP-WUS monitoring for the UE.

Proposal 11: RAN2 assumes UE could monitor LP-WUS upon LP-WUS is enabled, no additional indication/condition are needed for activation/deactivation of LP-WUS functionality. The final decision is up to RAN1.

- Conclusion

In this contribution, we discuss the LP-WUS triggers PDCCH monitoring in RRC_CONNECTED. Based on the discussion, we have the following observations and proposals.

Observations:

Observation 1: If the drx-onDurationTimer is not started, i.e. the duration is not the C-DRX Active time, the current conclusion on LP-WUS monitoring has covered the duration indicated by drx-onDurationTimer.

Observation 2: If the drx-onDurationTimer is not started, UE behaviour within the duration indicated by drx-onDurationTimer including CSI/L1-RSRP report and RRM/RLM/BFD measurement for Rel-16 DCP could be reused for LP-WUS.

Proposals:

LP-WUS monitoring and PDCCH monitoring behaviour on Option 1-2

Proposal 1: For Option 1-2 (i.e., new timer is configured), the legacy drx-onDurationTimer is not started, i.e., the duration indicated by drx-onDurationTimer is not the C-DRX Active time.

CSI report behaviour related to LP-WUS Option 1-2

Proposal 2: For Option 1-2, during the timer triggered by LP-WUS is running, periodic CSI/L1-RSRP report and SP CSI report behaviour are the same as legacy drx-onDurationTimer duration without DCP configuration, i.e. always report during this timer triggered by LP-WUS.

Proposal 3: For Option 1-2, within legacy drx-onDurationTimer duration and the MAC entity is not in Active Time, it is up to network configuration to determine whether to report the periodic CSI/L1-RSRP, i.e. same as in Rel-16 DCP.

Coexistence with PDCCH skipping

Proposal 4: For Option 1-2, when LP-WUS is used together with PDCCH skipping, the UE stops the PDCCH monitoring upon receiving the PDCCH skipping indication, and starts the LP-WUS monitoring.

Impacts on other legacy C-DRX Active Time

Proposal 5: When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to ra-ContentionResolutionTimer and msgB-ResponseWindow are not affected if LP-WUS Option 1-2 is used.

RAN2 views on LP-WUS Options

Proposal 6: Don’t support Option 1-1 and Option 1-2 simultaneously configured for the same UE.

Proposal 7: RAN2 won’t discuss Option 1-3 unless RAN1 triggers.

LP-WUS operation for UE configured with NR-DC

Proposal 8: For both option 1-1 and option 1-2 in NR-DC case, LP-WUS is configured separately in MCG and SCG, and LP-WUS could trigger the PDCCH monitoring of all activated serving cells within the same cell group.

Enable and disable for LP-WUS monitoring

Proposal 9: In RRC_CONNECTED mode, PDCCH monitoring triggered by LP-WUS is enabled/disabled by the corresponding configuration of LP-WUS. No additional assistance information from UE is needed.

Proposal 10: LP-WUS configuration is included in RRCResume/RRCReconfiguration message.

Activation and deactivation procedure for LP-WUS monitoring

Proposal 11: RAN2 assumes UE could monitor LP-WUS upon LP-WUS is enabled, no additional indication/condition are needed for activation/deactivation of LP-WUS functionality. The final decision is up to RAN1.

- References

- RAN2#127_ChairNotes.

- RAN2#127bis_ChairNotes.

- 3GPP RAN1#118 meeting, Chairman notes

- 3GPP RAN1#118bis meeting, Chairman notes

- 3GPP TS 38.321: "NR; Medium Access Control (MAC); Protocol specification".

- RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO