3GPP TSG-RAN WG2 Meeting #128							   R2-2409784

Orlando, USA, Nov, 2024

Source: 			ZTE Corporation, Sanechips

Title: 	Unified random-access procedure for A-IoT

Agenda item:	8.2.4

Document for: Discussion and Decision

# Introduction

A-IoT study item considered the following types of RACH:

- CFRA, CBRA, 2-step and 3-step RA procedures.

The design of the device needs to be kept simple and we need to ensure that from the device perspective, complexity in supporting all these RACH types is not increased. With this in mind, RAN2 made the following agreement:

- RAN2 will aim to define one common design for all RA procedures (if technically possible)

This contribution proposes a unified RACH framework for all the RACH options for A-IoT.

# Discussion

For A-IoT, the design of random-access procedure should mainly target a simple overall design. Further, it is important that simplicity of the procedure is mainly from the device perspective to minimize the device costs. From this perspective, it is preferable to have a unified overall device procedure for various flavors of random-access procedure currently under discussion in RAN2 (e.g. CBRA, CFRA, 2-step, 3-step RACH etc).

The overall RACH procedure consists of the following:

- Procedure for transmission of the first UL message (including the details of the triggering message in DL)

- Procedure for CFRA/CBRA

- Procedure for contention resolution

In the following sections, we look at minimizing the differences at the device side so that the overall procedure is the same regardless of the above flavors of RACH.

# Msg1 scheduling

One of the main differences between NR and A-IoT is that the A-IoT random access procedure is always triggered by an initial DL trigger message (paging-like message). This initial DL message will contain at least the following:

- an indication of which A-IoT devices should respond

- time/frequency and other resources on which the devices should respond

Thus, unlike NR RACH preamble, it can be seen that the first UL message in A-IoT can be viewed as a scheduled message as far as the device is concerned. Since this is a scheduled message, the corresponding trigger message in DL can also indicate the amount of data the UE should send in the first UL message (i.e. MCS-like information as agreed by RAN1). Thus, we can assume that it is possible to indicate whether the device should include just a random number or more data as part of the first UL message.

Thus, the A-IoT device has to read from the memory and transmit the data. How much content of the device memory is read by the device and included in the first UL message is thus determined based on the contents of the trigger message in DL. For instance, if the trigger message contains an indication to transmit only an RN in the first UL message, the tag should simply read, for e.g. first 16 bits from the memory and transmit this in UL. If on the other hand, the trigger message indicates transmission of a larger payload, then the device should read more data and include all of this in the UL message.

From the perspective of the device, hence, there is no difference between 2-step and 3-step RACH for the transmission of the first UL message. All the device does is to read a given amount of data and transmit in UL. The 2-step RACH and 3-step RACH differentiation would hence mainly be visible only to the reader (since the reader has to determine the size of the first UL message and process the contents of the first UL message received from the device).

Proposal 1: MSG0 indicates whether MSG1 should include just random number (3-step RA) or random number + device ID (2-step RA)

Proposal 2: The first 16 bits in MSG1 contain the random number in all RACH options (applicable to 2-step/3-step RA, CFRA, CBRA)

RAN2 design should accommodate a minimum size for the payload in the first UL message which includes at least the random number (e.g. 16 bits – i.e. RN16) as this is also used in contention resolution subsequently. On the other hand, the maximum payload size in UL depends on a number of other considerations such as the coverage, total time the device can sustain the A-IoT transmit/receive session, number of other responding devices etc. Thus, the actual maximum size for the payload in the first UL message can be left to the implementation of the reader. However, in order to simplify the design, we could consider that for 2-step RA case, the payload size of MSG1 is simply the total size of RN + the upper layer device ID.

Proposal 3: The minimum size of MSG1 should be at least the size of the random number (i.e. 16 bits)

Proposal 4: The maximum size of MSG1 should be 16 bits (for the RN16) + the upper layer device ID size

Proposal 5: The decision between 2-step RA and 3-step RA is up to the reader

# Unified procedure for CFRA and CBRA

One further difference between various RACH procedures is whether CFRA or CBRA is used. CFRA is just a special case of CBRA, where there is a unique RACH resource indicated in the MSG0. Thus, if there is a one-to-one mapping between the scheduled UL resource and the scheduled devices (i.e. there is one unique time/freq/code resource for the given device), then the procedure can be seen as a CFRA procedure. On the other hand, if there is one-to-many mapping between the scheduled UL resources and the scheduled devices (i.e. the there is no unique time/freq/code resource for the given device), then it is a CBRA procedure. The number of devices scheduled per UL resource is up to the reader. From the device perspective, if there is a unique UL resource for the transmission of the first UL message, then this resource shall be chosen for transmission of the UL message. On the other hand, if there is more than one UL resource available, the device shall choose one of the UL resources randomly and transmit the first UL message. After choosing the resource for transmitting the first UL message, there should be no further difference between CFRA and CBRA from the device perspective.

Proposal 6: CFRA is supported as a special case of CBRA where there is a unique MSG1 resource for the device(s) indicated in MSG0

Proposal 7: Subsequent steps in CFRA (after MSG0) are same as CBRA

# Unified procedure for contention resolution

After transmitting the first UL message, the device should simply listen to the DL. For the purpose of contention resolution, the reader echoes the received information in the first UL message in the response message in DL. Thus, the contents of the response message in DL should match the contents of the UL message transmitted by the UE in the first UL message. In order to simplify the device procedure and to enable a unified procedure from the device perspective, the contents of the response message should be limited to the minimum payload size of the first UL message which is common to all types of RACH procedures as described above. This means that the contents of the response message should contain at least the random number and is copied from the first N bits (e.g. first 16 bits) of the first UL message from the device regardless of the payload size of the first UL message. If the contents of the response message match the first N bits (e.g. 16 bits) that the device transmitted, then the device considers the contention resolution as successful regardless of the 2-step/3-step RACH procedure (i.e. regardless of the payload size of the first UL message).

Finally, when two devices choose the same random number, then there will be contention and resolving this may need further discussion. In NR, the contention resolution is based on the network echoing part of the upper layer device ID and the UE will then initiate a new RACH procedure if this happens. In case of A-IoT, since the device cannot initiate a new RACH procedure anyway, designing a complex contention resolution mechanism for the rare case of the devices choosing same random number is unnecessary. Instead, when this happens, the reader may simply not schedule the colliding devices in the current paging round and use the next paging round to page these devices. Thus, no further optimization is needed for this case.

Proposal 8: After transmitting MSG1, the device listens to DL (for receiving MSG2) in all RACH options

Proposal 9: Regardless of the RACH option (2-step/3-step/CBRA/CFRA), contention resolution is deemed successful when MSG2 containing RN16 from MSG1 is received

Proposal 10: If two devices choose the same random number in MSG1, it is up to the reader to handle the collision (i.e. no further mechanism is specified to resolve this within the current paging round)

# Procedure for determining the access occasion for MSG1

The random-access procedure for A-IoT uses slotted Aloha. Once the device randomly selects the access occasion for A-IoT MSG1, the device then needs to count the slots so that the device can determine when it is allowed to transmit. Counting the slots can be either based on a counter maintained at the device side or by a trigger that is received from the reader. Using a device-based counter only works if the device can maintain a stable clock so that the slot boundaries are synchronized between the reader and the device and among other devices. However, the devices for A-IoT may not have a stable clock and hence a safer approach is to use a trigger from the reader for counting down.

Furthermore, the reader is aware of the level of contention based on the number of devices that are paged and number of devices that respond. So, the reader may send the trigger for counting down the slots at non-uniform intervals to ensure that the contention is minimized. This is also necessary since the time taken to transmit a given message from different devices may not be the same. Thus, a counter that is decremented autonomously at the device side is not suitable to determine the access occasion. In case of RFID a similar mechanism is used and the count-down procedure is based on explicit triggers from the reader.

Based on the above, it is suggested to introduce a subsequent DL trigger message after the paging message to trigger the countdown of the slot counter at the device.

Proposal 11: After the device randomly selects an access occasion for A-IoT MSG1 transmission, the device shall use a trigger from the reader to count down the slot counter to determine the actual transmit occasion

# Conclusion

The following proposals are made:

Proposal 1: MSG0 indicates whether MSG1 should include just random number (3-step RA) or random number + device ID (2-step RA)

Proposal 2: The first 16 bits in MSG1 contain the random number in all RACH options (applicable to 2-step/3-step RA, CFRA, CBRA)

Proposal 3: The minimum size of MSG1 should be at least the size of the random number (i.e. 16 bits)

Proposal 4: The maximum size of MSG1 should be 16 bits (for the RN16) + the upper layer device ID size

Proposal 5: The decision between 2-step RA and 3-step RA is up to the reader

Proposal 6: CFRA is supported as a special case of CBRA where there is a unique MSG1 resource for the device(s) indicated in MSG0

Proposal 7: Subsequent steps in CFRA (after MSG0) are same as CBRA

Proposal 8: After transmitting MSG1, the device listens to DL (for receiving MSG2) in all RACH options

Proposal 9: Regardless of the RACH option (2-step/3-step/CBRA/CFRA), contention resolution is deemed successful when MSG2 containing RN16 from MSG1 is received

Proposal 10: If two devices choose the same random number in MSG1, it is up to the reader to handle the collision (i.e. no further mechanism is specified to resolve this within the current paging round)

Proposal 11: After the device randomly selects an access occasion for A-IoT MSG1 transmission, the device shall use a trigger from the reader to count down the slot counter to determine the actual transmit occasion

# References

- RP-234058, New SID: Study on solutions for Ambient IoT (Internet of Things) in NR, RAN#102

- 3GPP TR 38848 V18.0.0 (2023-09), Study on Ambient IoT (Internet of Things) in RAN

- 3GPP TR 23.700-13 V0.2.0 (2024-03), Study on Architecture support of Ambient power-enabled Internet of Things