3GPP TSG-RAN2#128									         R2-2409787

Orlando, USA, 14 - 18 Nov, 2024

Source: 			ZTE Corporation, Sanechips

Title: 	DSR enhancements for XR

Agenda item:	8.7.4.2

Document for: 	Discussion and Decision

# Introduction

Rel-19 XR WI in RP-240791 has an objective for RAN2 to study and if justified specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity.

In RAN2#125bis meeting, the following related agreements has been made [2]:

- RAN2 will study whether/how to resolve the issue of data with low remaining time being delayed due to other data from LCHs with higher LCH priority when using the existing LCP procedure. At least the following alternatives will be studied: - Alternative 1: Enhance LCP restrictions/LCH selection. - Alternative 2: Enhance LCH prioritization. - RAN2 should consider potential impact on traffic from SRBs.

In RAN2#126 meeting, the following related agreements has been made [3]:

LCP enhancements – LCH prioritization - Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR. - The solution should consider impact on UE complexity (as already indicated in SI objective description) - For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - Use additional priority configured to LCHs in case of these LCHs with delay-critical data - FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH - We try to avoid RAN1 impacts. - RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way. - For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial - The solutions should not disallow non-delay critical data from using an UL grant. - LCP prioritization within a logical channel will not be considered in RAN2 discussions - FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR). DSR enhancements – multiple thresholds and grouping of data - Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. - FFS whether DSR triggering is impacted - FFS whether PDU set importance needs to be included

In RAN2#127 meeting, the following related agreements has been made [4]:

LCP Enhancement - RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR. - FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3). Enhanced DSR contents - Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. - For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

In RAN2#127bis meeting, the following related agreements has been made [5]:

- We do not change the definition of delay-critical data - For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) - Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. - For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - Buffer size of data volume in each portion - Shortest remaining time among PDCP SDUs buffered in each portion. - There is no need to include PSI in the enhanced DSR MAC CE. - A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. - FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

In this contribution we will further investigate the possible DSR enhancements per this objective.

# Discussion

It has been agreed in RAN2#127 meeting that network should be able to configure multiple remaining time thresholds for each LCG to report multiple pairs of remaining time and buffer sizes per LCG, but there will be a single triggering threshold. The value range of remainingTimeThreshold-r18 is INTEGER (1..64) ms. Very small remaining time (e.g. 1ms, 2ms etc ) are not useful as there is not enough time to report these such that network can receive the DSR, process this and respond with scheduling. Considering the useful range of the remaining times, at most three remaining time thresholds seem a reasonable choice of split and this. Only these configured remaining time thresholds along with the buffered data associated with these remaining time will be reported. For each of the portions reported, the eDSR should report the corresponding buffer size and the shortest remaining time among the PDCP SDUs in this portion, which is shown in the following figure. And for flexibility, variable number of remaining time thresholds (e.g.1, 2 or 3 thresholds) should be allowed. The total  number of remaining time thresholds to be used can be configurable.

Proposal 1: For DSR enhancement, at most three remaining time thresholds can be configured for one LCG, and variable number of remaining time thresholds are allowed.

Figure 1: Example for 3 pairs of remaining time and related Data volume in eDSR

It has been agreed in RAN2#127 meeting that there will be a single eDSR triggering threshold. In the last meeting, different opinions for setting eDSR triggering threshold are proposed, and it is agreed that companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. We can use the figure 1 as an example, if the triggering threshold is set to remaining time threshold 1, then only there is data with remaining time less than or equal to remaining time threshold 1, eDSR will be triggered, and eDSR will include three pairs of remaining time and related buffer size;

Observation 1: If the triggering threshold is set to the remaining time threshold 1(e.g. the smallest reporting threshold), eDSR will include three pairs of remaining time and related buffer size.

If the triggering threshold is set to remaining time threshold 3, then the eDSR will be triggered once there is data with remaining time less than or equal to remaining time threshold 3, and eDSR can only include one pair of remaining time and related buffer size(e.g. Remaining time 3 reported and buffer size 3).

Observation 2: If the triggering threshold is set to the remaining time threshold 3(e.g. the largest reporting threshold), eDSR can only include one pair of remaining time and related buffer size(e.g. Remaining time 3 reported and buffer size 3).

Extremely, if the triggering threshold is set to a value that is larger than all the reporting threshold, then when the eDSR is triggered, there is no data is to be reported.

In fact, once there is no data with remaining time less than or equal to the triggering threshold, the eDSR is triggered. Thus, if a reporting threshold is smaller then the triggering threshold, then there is no data with remaining time less than or equal to the reporting threshold, and the reporting threshold smaller than the triggering threshold will be useless.

Observation 3: The reporting threshold smaller than the triggering threshold is not useful.

In fact, the multiple reporting thresholds are used to provide more time information for data to be scheduled, and whether the data should be prioritized for scheduling depends on the remaining time threshold for LCP, the smallest reporting threshold should be used as the triggering threshold so that the eDSR can include more time information for data to be scheduled.

Proposal 2: The smallest reporting threshold is used as the triggering threshold to trigger the eDSR.

Considering that the data volume with remaining time larger than the largest reporting threshold can be reported by BSR, it is not necessary to report data volume with remaining time larger than the largest reporting threshold in the DSR.

Proposal 3: Not report data volume with remaining time larger than the largest reporting threshold in the DSR.

Considering that the data is maintained per LCH with FIFO, the data in one LCH will use same discardTimer to calculate the remaining time, the data with smaller remaining time should be ahead of the data with larger remaining time, it is not necessary to consider the case of non-delay critical data ahead of delay critical data in the buffer size calculation for eDSR.

Proposal 4: The non-delay critical data ahead of delay critical data is not included in the buffer size calculation for eDSR.

In RAN2 #127bis meeting, it has been agreed that a one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. If the bit is used to indicate whether further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG, then some unnecessary payload will be increased in some cases. E.g. when there are 3 reporting threshold, and there is data related to the smallest reporting threshold and the largest reporting threshold, but there is no data related to the in-between reporting threshold, if the bit should be set to indicate further pair of remaining time information and buffer size information is present, three pairs of remaining time and buffer size need be included in the eDSR, e.g. the pair of remaining time(what value to be set is not known) and buffer size 0 related to the in-between reporting threshold should be included, which will increase the eDSR payload. If the bit is used to indicate whether a certain pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG, then the only two pairs of remaining time and buffer size need be included in the eDSR, e.g. the pair related to the in-between reporting threshold need not be included, which can save eDSR payload.

Observation 4: If the bit is used to indicate whether further pair of remaining time information and buffer size information is present, some unnecessary payload will be increased in some cases. If the bit is used to indicate whether a certain pair of remaining time information and buffer size information is present, it can save eDSR payload.

Proposal 5: RAN2 confirm that the one-bit indication is used indicate whether a certain pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG.

To report with multiple pairs of remaining time and buffer size for one LCG, DSR MAC CE should be enhanced to report multiple pairs of remaining time and buffer size for one LCG. Considering that there is not enough “R” bits to extend the DSR MAC CE to indicate whether each pair of remaining time and buffer size is present for one LCG, a new DSR MAC CE can be defined as follows:

| R | LCG ID |  | R | F3 | F2 | F1 | Oct 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BT1 | R | Remaining Time 1 |  |  |  |  | Oct 2 |
| Buffer size1 |  |  |  |  |  |  | Oct 3 |
| BT2 | R | Remaining Time 2 |  |  |  |  | Oct 4 |
| Buffer size2 |  |  |  |  |  |  | Oct 5 |
| BT3 | R | Remaining Time 3 |  |  |  |  | Oct 6 |
| Buffer size3 |  |  |  |  |  |  | Oct 7 |

Enhanced DSR MAC CE for reporting multiple pairs of remaining time and buffer size

In which:

-	LCG ID: This field identifies the group of logical channel(s) whose DSR is being reported.

-	Fi: This field indicates whether the pair of remaining time i and buffer size i are present. Fi is set to 0 to indicate that the Oct including BTi, remaining time i and buffer size i are absent; Fi is set to 1 to indicate that the Oct including BTi, remaining time i and buffer size i are present;

-	Remaining Time i: This field indicates the shortest remaining value of running PDCP discardTimer among all PDCP SDUs that are buffered for an LCG, with remaining time less than the remaining time threshold i but equal to or larger than the remaining time threshold i-1, but have not been transmitted in any MAC PDU, at the time of the first symbol of the first PUSCH transmission that includes this DSR MAC CE. The length of this field is 6 bits. This field is present only if the buffer size indicated by the corresponding Buffer Size field is not zero; otherwise, this field is reserved and set to 0. If present, the value r in this field indicates a remaining time within the range of (r, r + 1] msec;

-	BT i: This field is present only if the corresponding LCG is configured with additionalBS-TableAllowed and the buffer size indicated by the corresponding Buffer Size field is not zero; otherwise, this field is reserved and set to 0. If present, the BT field set to 1 indicates that the buffer sizes with finer granularity are used to set the value of the Buffer Size field, while the BT field set to 0 indicates that the buffer sizes specified in legacy Table are used instead;

-	Buffer Size i: The Buffer Size i field indicates the total amount of UL data with remaining time less than the remaining time threshold i but equal to or larger than the remaining time threshold i-1 for an LCG after the MAC PDU has been built.

-	R: Reserved bit, set to 0.

With new DSR format, UE should report it’s capability on supporting enhanced DSR. Only if the UE reports it’s capability, multiple remaining time threshold for one LCG can be configured for the UE.

Proposal 6: To enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG, the following aspects are specified:

- Define new DSR MAC CE to report multiple pairs of remaining time and buffer.

- Introduce UE capability on support enhanced DSR.

In RAN2#127bis meeting, there is a FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds. This FFS depends on how to configure the multiple reporting thresholds. If additional reporting thresholds are configured, and the legacy threshold is always used as the smallest reporting threshold, then it can be based on UE implementation which DSR format is used. And if the multiple reporting thresholds are configured independent of the legacy threshold, then the new DSR should be used for the LCG when triggered by the new triggering threshold.

Proposal 7: RAN2 discuss how to configure the multiple reporting thresholds, e.g. additional reporting thresholds are configured or the multiple reporting thresholds are configured independent of the legacy threshold.

# Conclusion and proposals

The following observations/proposals are made:

Observation 1: If the triggering threshold is set to the remaining time threshold 1(e.g. the smallest reporting threshold), eDSR will include three pairs of remaining time and related buffer size.

Observation 2: If the triggering threshold is set to the remaining time threshold 3(e.g. the largest reporting threshold), eDSR can only include one pair of remaining time and related buffer size(e.g. Remaining time 3 reported and buffer size 3).

Observation 3: The reporting threshold smaller than the triggering threshold is not useful.

Observation 4: If the bit is used to indicate whether further pair of remaining time information and buffer size information is present, some unnecessary payload will be increased in some cases. If the bit is used to indicate whether a certain pair of remaining time information and buffer size information is present, it can save eDSR payload.

Proposal 1: For DSR enhancement, at most three remaining time thresholds can be configured for one LCG, and variable number of remaining time thresholds are allowed.

Proposal 2: The smallest reporting threshold is used as the triggering threshold to trigger the eDSR.

Proposal 3: Not report data volume with remaining time larger than the largest reporting threshold in the DSR.

Proposal 4: The non-delay critical data ahead of delay critical data is not included in the buffer size calculation for eDSR.

Proposal 5: RAN2 confirm that the one-bit indication is used indicate whether a certain pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG.

Proposal 6: To enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG, the following aspects are specified:

- Define new DSR MAC CE to report multiple pairs of remaining time and buffer.

- Introduce UE capability on support enhanced DSR.

Proposal 7: RAN2 discuss how to configure the multiple reporting thresholds, e.g. additional reporting thresholds are configured or the multiple reporting thresholds are configured independent of the legacy threshold.

# References

- RP-240791, Revised WID on XR (eXtended Reality) for NR Phase 3, RAN#103

- Draft_R2_125bis_Meeting_Report_v2

- Draft_R2_126_Meeting_Report_v2

- Draft_RAN2_127_Meeting_Report_v1

- Draft_RAN2_127b_Meeting_Report_v1