3GPP TSG-RAN WG2 Meeting #128	R2-2409790

Orlando, US, 18 – 22 November, 2024

Agenda Item:	8.7.4.1

Source: 	Canon Research Centre France

Title:	Discussion on Logical channel priority

Document for:	Discussion and decision

# Introduction

During RAN2#127bis in Hefei, the following agreements related to LCP were recorded in [1]:

Agreements on LCP enhancements - As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round. - FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation - Introduce an independent per-LCH remaining time threshold for applying delay-critical priority. - We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold.

# Discussion

In this paper, we discuss how the dual priority mechanism shall be designed so that it maintains the fairness of LCP algorithm.

## LCP

LCP is designed to achieve fairness. After LCH restrictions, LCP is performed in two steps. First, the granted resources are allocated to each LCH by priority order up to their bucket capacity. Then, in a second step, if there are remaining resources they are allocated to each channel by priority with no size limit.

In Fukuoka meeting, it was decided to enhance LCP to resolve the delay issue for low priority time critical data. Also, in Fukuoka meeting, it was decided to use an additional priority for a LCH in case there are time-critical data to be sent.

In our understanding, when this priority switching mechanism is implemented, each time an LCH has time critical data available for transmission, the additional priority will be used by the LCP algorithm. The additional priority being higher than the legacy priority, the time critical data will have a better chance to be selected by the LCP algorithm to be transmitted within the current configured grant.

Further, this means that high priority non-time critical data may have less chance to be transmitted within the current configured grant. And since the data are non-time critical, it is acceptable to delay the transmission until further grant. However, the LCP fairness being broken, the deprioritized high priority traffic is now prone to starvation.

Observation 1: When time critical data are prioritized, non-time critical data having higher priority could miss a transmission opportunity

This is particularly true in the case where the granted resources cannot accommodate for the bucket size of all channels. In that case it may happen that high priority channel cannot be allocated a transmission opportunity corresponding to the allowed bucket size.

It is therefore important that the LCP fairness can be restored at a further grant when there are no more time critical data to handle.

Proposal 1: Restore LCP fairness to prevent high priority non time critical data starvation.

Fairness restoration may be left to gNB implementation, it would be up to the gNB to provide more resources at next grant to accommodate for the resources lost by the deprioritized data. However, priority switching is an autonomous decision by the UE, so there are no means for the gNB to trigger fairness restoration unless additional signalling is implemented.

Observation 2: gNB is not aware of LCH priority switching.

When a further grant is received by the UE, if there are no time critical data to be sent, the UE can compensate the previously lost resources for the high priority LCH.

Proposal 2: Fairness restoration is implemented at the UE as a resource compensation mechanism

To implement fairness restoration, the UE shall first record the resource prejudice inflicted to the deprioritized LCH when a grant is received and time critical data on another channel is prioritized.

Then, when a further grant is received, if the granted resources cannot accommodate for the resources previously lost for the high priority channel, then the UE may redirect resources currently allocated to the lower priority channel to the higher priority channel as a compensation.

For example, if the received grant can accommodate for the bucket size of the two channels, then the UE may decide to allocate the bucket of the low priority channel, in addition to the bucket of the high priority channel to the high priority channel.

Proposal 3: Under certain conditions, redirect resources allocated to low priority channel to higher priority channels

# Conclusion

According to the analysis in section 2, it is proposed:

Observation 1: When time critical data are prioritized, non-time critical data having higher priority could miss a transmission opportunity

Proposal 1: Restore LCP fairness to prevent high priority non time critical data starvation.

Observation 2: gNB is not aware of LCH priority switching.

Proposal 2: Fairness restoration is implemented at the UE as a resource compensation mechanism

Proposal 3: Under certain conditions, redirect resources allocated to low priority channel to higher priority channels

# References

- R2-2409215	Report from session on R18 MBS, R18 QoE and R19 XR, Session chair (Huawei)