3GPP TSG-RAN WG2 Meeting #128	R2-2409794

Orlando, FL, US, 18 –22 Nov 2024

Agenda Item:	8.11.2

Source:	NEC

Title:	Random Access for SBFD Operation

Document for:	Discussion, Decision

# Introduction

In RAN2#127bis meeting, the following agreements were reached for the random access procedure for SBFD operation:

- RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact.

- From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

- Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network.

- For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

- The following two RACH configuration options are considered for SBFD based random access:

Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed.

Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration

- For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters.

- The RACH configuration for SBFD is transmitted via SIB1.

- FFS dedicated RRC signalling detail. FFS whether NW can provide both configurations.

This paper intends to further discuss the RAN2 impact of the random access procedures for SBFD operation.

# Discussion

## RACH occasions selection and switch

In the last RAN2 meeting, it is agreed that for the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. Meanwhile there is an FFS about the case when UE selects legacy ROs first.

In our opinion, if the UE selects legacy ROs first, the UE should be allowed to also switch to additional SBFD RACH occasions.

Proposal-1: the UE should be allowed to switch to additional SBFD RACH occasions after certain (configured) number of times of RACH attempt in legacy RACH occasions, if the UE selects legacy ROs first.

During the random access procedure, the UE may sweep the beams and measure their SSBs/CSI RSs in order to determine the best-associated beam with the best RSRP performance. In the meantime, in order to avoid random access delay, it may be desirable for the SBFD-aware UEs to select the first available RO among all of the RACH resources valid to the UE (including both legacy RACH resources and additional RACH resources) for its initial access. In this manner, the UE is expected to select the best suitable RO set or RO type (legacy RO or additional RO) to initiate random access.

Proposal-2: UE selects the first available RO type(legacy RO or additional RO) among all of the RACH resources valid to the UE for its initial access.

In order to support random access in SBFD symbols by UEs in RRC_IDLE/INACTIVE mode, RACH resource selection for SI request and RA-SDT should also be discussed. The same discussion for proposal 2 can be applied for RACH resource selection for SI request and RA-SDT. Whether the network can configure separated criteria for SI request and RA-SDT is FFS.

Proposal-3: For SI request and RA-SDT, the UE always selects the first available RO type(legacy RO or additional RO) among all of the RACH resources valid to the UE.

In the last RAN2 meeting, it is agreed that upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication is from network. The network may be able to command the UE to perform random access via PDCCH order, and the UE can select the RACH resource between legacy and additional RACH resources following the recommendation from the network within such PDCCH order. However, the detailed PDCCH order design in this case is subject to RAN1 discussion.

However, in case of no indication from the network, which type of conditions/prioritizations to apply to the UE during its selection of RO type is not clear. Meanwhile, there may be also cases where UE is required to select the RACH resources autonomously among the RACH resources configured to it, for example, when the UE loses the synchronization during RRC CONNECTED mode. It would be helpful to specify a criteria for the UE to choose the suitable RACH resource. We see the benefits of specifying certain conditions for the UE to switch its usage of RO between additional RACH resources and legacy RACH resources. As an example, the downgraded serving cell signal strength can be one of the certain conditions. In this case, the UE may transmit the random-access attempt using more power more than normal case and then there is a potential to cause self-interference within one gNB or inter-gNB CLI, using the ROs configured in SBFD symbols. Hence, it is important to consider that UE can switch to legacy RACH resources if serving cell’s signal strength is not good. So, we may need a way for SBFD-aware UEs to switch between additional RACH resources and legacy RACH resources based on the serving cell signal strength.

Proposal-4: The UE can switch its usage of RO between additional RACH resources and legacy RACH resources under certain conditions (e.g., based on the serving cell signal strength).

## RSRP threshold for SSB/CSI-RS selection

There is one more issue for the RACH resource decision. In general, the RSRP threshold for SSB/CSI-RS selection can be the same for both legacy RACH resources and additional RACH resources. However, in practice, the network may configure the UE with a different threshold for SSB/CSI-RS signal strength (e.g. RSRP) or threshold for the path loss that UE experiences for SSB/CSI-RS for RACH resources occurring during SBFD symbols.

Proposal-5: RAN2 to discuss if RSRP threshold for SSB/CSI-RS selection can be the same or different for legacy RACH resources and additional RACH resources

## RACH resource configuration

In the last meeting discussion, there is an FFS on dedicated RRC signalling detail for 2.3	RACH resource configuration and whether the network can provide both configurations (i.e., SIB1 and dedicated RRC signalling).

It should be beneficial to have both configuration, since the UE in RRC connected state may be configured via RRCReconfiguration message for the additional RACH resources.

Proposal-6: The additional RACH configuration can be provided via RRCReconfiguration message to support the SBFD-aware UEs in RRC CONNECTED mode.

Proposal-7: Both SIB1 based configuration and dedicated RRC signalling based configuration should be supported.

## Activation and deactivation of Additional ROs

In addition, when there is additional RACH configuration for SBFD-aware UEs, it would be beneficial to allow the deactivation and activation of all or a subset of such ROs. Usually less number of ROs configured in the system will improve the system throughput, since the additional resources can be used schedule the data transmission for the connected UEs. RAN2 can discuss the method on how to deactivate and activate additional RACH Occasions for SBFD-aware UEs after it is configured. RRC message or MAC CE based approach may be discussed in this context. Such message may also indicate the time duration for which the deactivation is applicable for. For example, this can be in form of DRX cycle configured by gNB where within the DRX-on duration (when gNB is in active state), gNB enables all ROs and for DRX-off duration (when gNB is in low power state), gNB can configure a set of ROs to be deactivated.

Proposal-8: RAN2 to discuss the method on how to deactivate and activate additional RACH Occasions for SBFD-aware UEs after it is configured.

## Support of both RACH configuration Option 1 and Option 2

RAN1 agreed that one UE does not have to support both RACH configuration option 1 and option 2 simultaneously for SBFD.

In the meantime, in the last RAN1 meeting, there is no consensus in RAN1 to support the following in Rel-19:

- Enabling both RACH configuration Option 1 and Option 2 for SBFD random access operation in a cell from network side.

In our understanding, following the RAN1 agreements above, it is beneficial to have both RACH configuration Option 1 and Option 2 for SBFD random access operation enabled within the same cell from network perspective, which may benefit different UE implementation on this feature. If only one option is enabled by the cell, there would be less UEs that can benefit from the feature.

Proposal-9: Support to enable both RACH configuration Option 1 and Option 2 for SBFD random access operation in a cell from network side.

# Conclusion and Proposal

We have the following proposals:

Proposal-1: the UE should be allowed to switch to additional SBFD RACH occasions after certain (configured) number of times of RACH attempt in legacy RACH occasions, if the UE selects legacy ROs first.

Proposal-2: UE always selects the first available RO type(legacy RO or additional RO) among all of the RACH resources valid to the UE.

Proposal-3: For SI request and RA-SDT, the UE always selects the first available RO type(legacy RO or additional RO) among all of the RACH resources valid to the UE.

Proposal-4: The UE can switch its usage of RO between additional RACH resources and legacy RACH resources under certain conditions (e.g., based on the serving cell signal strength).

Proposal-5: RAN2 to discuss if RSRP threshold for SSB/CSI-RS selection can be the same or different for legacy RACH resources and additional RACH resources

Proposal-6: The additional RACH configuration can be provided via RRCReconfiguration message to support the SBFD-aware UEs in RRC CONNECTED mode.

Proposal-7: Both SIB1 based configuration and dedicated RRC signalling based configuration should be supported.

Proposal-8: RAN2 to discuss the method on how to deactivate and activate additional RACH Occasions for SBFD-aware UEs after it is configured.

Proposal-9: Support to enable both RACH configuration Option 1 and Option 2 for SBFD random access operation in a cell from network side.

# Reference

- RAN1#118bis meeting minutes

- RP-241614 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)