3GPP TSG-RAN WG2 Meeting #128	R2-2409795

Orlando, FL, 18-22 Nov 2024

Agenda item:	8.3.5

Title:		Simulation assumptions for measurement event prediction

Source:		NEC

Document for:	Discussion and Decision

# 1. Introduction

For AI/ML mobility study, in RAN2#127bis meeting, RAN2 made progress on measurement event on definition, methodology, metrics and simulation assumption. The following agreements were reached at the previous RAN2 meetings:

Agreements@RAN2#127bis

For indirect measurement event prediction, the intermediate output (i.e., the output of RRM prediction model) is RSRP of serving/neighboring cells.  The final output is the expected occurrence time of a certain measurement event (ex. event A3).

The System level performance (e.g. HO performance) evaluation is optional (i.e. companies can bring results if they chose).

- System level performance for measurement event prediction can be prioritized by companies if they chose to do it.

RAN2 will prioritize discussions on intermediate KPI discussion before discussing system level performance for the corresponding use case.

Discussion on what (type of information)/how generalization study can take place in Nov. meeting

The SLS simulation assumption discussion is covered in the post#127bis email discussion by assuming:

- The simulation assumptions agreed for measurement event prediction and RLF prediction is taken as baseline for SLS in principle

- The HO model in 36.839 is taken as baseline

- The HO performance will be HOF and number of HO only and definition in 36.839 is taken as baseline

- The baseline of HO performance is R15 legacy measurement and HO procedure

This document discusses the scenarios of measurement event predictions and the inputs for the AI/ML model for measurement event prediction based on the agreements RAN2 made for this subtopic.

# 2. Discussion

## 2.1 Metrics for measurement event prediction

During the email discussion[3], the following metrics were discussed

Counter n0: refers to the case where no event is predicted and occur in baseline case. This is not used for F1 score calculation.

Counter n1: refers to the case where an event is predicted while it doesn’t occur in baseline case i.e. model’s prediction is kind false alarm

Counter n2: refers to the case where no event is predicted while it occurs in baseline case i.e., an real event is missed by the model

Counter n3: refers to the case where an event is predicted while it does occur in baseline case i.e. model predicts an event correctly.

However, even though an measurement event is predicted, it may be worthwhile to classify if the predicted event is a valid event or invalid event. When the UE is moving faster or cell coverages are not uniformly spread (e.g. cell coverages among neighbouring cells are complicated), a L3 HO based on those predicted measurement events might not work sufficiently well. For example, even though the HO is successfully performed, the UE may stay in the target cell for short period and then perform the HO again. The UE may continue this behaviour subsequently. The issue is known as “short time of stay” or frequent HO. One specific case of frequent HO is a ping-pong HO, where the UE comes back to the source cell right after the HO completion to the target cell. If a measurement event predication results a “short time of stay” or frequent HO, it can be counted as invalid measurement event predication. For the purpose of performance observation, it may be helpful to know the probability of invalid measurement event predications among all of the measurement event predications.

Proposal-1: Introduce one more metric to count the probability of invalid measurement event predications among all of the measurement event predications.

for RRM measurement predictions, there are discussions on how to define a prediction accuracy metric. The RSRP difference is assumed to be the prediction accuracy metric for RRM measurement. For measurement event prediction and mobility event prediction, we may use a bit different metric to weigh the prediction accuracy. Since measurement event or mobility event occurs at a certain timeframe, one of the possible metric can be considering the timing difference between the predicted occurrence of the measurement event or HO event and the real time of the occurrence of the event. At some extreme case, the predicted occurrence of the event may never happen, so then basically, the occurrence of the event should be also one of the metrics. A CDF curve may be used to express the trend of the timing difference for multiple instances of event predictions.

Proposal-2: The occurrence of the event, and the timing difference between the predicted occurrence of the event and the real time of the occurrence of the event can be defined as the prediction accuracy metric for measurement event (and mobility event) predictions.

Prediction Efficiency (PE) quantifies how early an a handover measurement event is predicted in advance of its occurrence, aiming to optimize resource management and minimize packet drops. This metric is crucial in ensuring that the system reacts promptly to avoid any disruptions or degraded QoS during handovers, particularly in dense and dynamic network environments. Mathematically, PE is the ratio of the time difference between when the A3 event is predicted and when it would naturally trigger based on conventional thresholds, to the time window within which the handover can be successfully executed without packet loss or service interruption.

PE= Tpredicted-TtriggerThandover_window

Where:

- ​Tpredicted is the time at which the A3 event is predicted by the AI/ML model.

- Ttrigger  is the actual time at which the A3 event is triggered based on the standard measurement report.

- Thandover_window  represents the time window during which a handover can be safely initiated and completed without packet drops or service degradation. The Thandover_window  includes TTT, Handover preparation time and handover execution time.

High PE means the system can predict the A3 event well in advance, providing sufficient time to manage resources and execute a handover smoothly. Low PE indicates late predictions, potentially causing packet drops or failed handovers due to insufficient time for preparation. This metric is valuable for evaluating the performance of AI/ML models used in handover management, particularly when optimizing user experience in dense urban scenarios.

Proposal-3: The ratio of the time difference between the measurement event prediction time and trigger time to the time window within which handover can be successfully executed without service interruption can be termed as prediction efficiency metric for measurement event predictions.

## 2.2 Simulation parameters

During the email discussion [3], the following simulation parameters were listed for the measurement event prediction based simulation based on case A.

- A3 event offset (db)

- TTT (ms)

- UE speed (km/h)

- OW length (ms)

- PW length (ms)

- Max ETD (ms, note1)

- Event occurrence Window Length

- Probability threshold

According to the email discussion, there are some controversial on the setting of UE speed, OW/PW length and TTT.

For mobility performance following legacy mechanisms, the setting of TTT is very critical to ensure the mobility performance. A long TTT will delay the execution of the handover decision and a short TTT may lead to the occurrence of ping-pong handover. Hence, sometimes, the length of TTT can seen as a tradeoff between the handover failure rate and the ping-pong handover rate in real network. From simulation perspective, it would be beneficial to see the performance of the measurement event prediction following the AI/ML model with the different setting of TTT value. We suggest to set low, middle and high values (i.e., 80ms, 160ms and 320ms) for TTT for the simulation.

Proposal-4: TTT value can be set as low, middle and high values (i.e., 80ms, 160ms and 320ms) in the simulation for measurement event predictions.

According to the observation submitted by the companies for RRM measurement prediction, the UE speed plays an important role in impacting the performance of measurement prediction, then for indirect approach based measurement event prediction, it may worthwhile to set different UE speed to observe the performance. Following simulation result submitted by the companies  at last meeting, the prediction for RRM measurement may benefit from having a long OW, but may not help by further increasing the OW when the UE speed is quite high. Hence we think that the setting of OW/PW length may be linked to the UE speed. A different setting of OW/PW length may apply to a different UE speed.

Proposal-5: Multiple UE speed (e.g., 60, 90 and 120 kmph) can be set in the simulation for measurement event predictions.

Proposal-6: A different setting of OW/PW length may apply to a different UE speed in the simulation for measurement event predictions(e.g., 400/400ms, 800/400ms, 800/800).

# 3. Conclusion

The following proposals are proposed following the discussion in this paper:

Proposal-1: Introduce one more metric to count the probability of invalid measurement event predications among all of the measurement event predications.

Proposal-2: The occurrence of the event, and the timing difference between the predicted occurrence of the event and the real time of the occurrence of the event can be defined as the prediction accuracy metric for measurement event (and mobility event) predictions.

Proposal-3: The ratio of the time difference between the measurement event prediction time and trigger time to the time window within which handover can be successfully executed without service interruption can be termed as prediction efficiency metric for measurement event predictions.

Proposal-4: TTT value can be set as low, middle and high values (i.e., 80ms, 160ms and 320ms) in the simulation for measurement event predictions.

Proposal-5: Multiple UE speed (e.g., 60, 90 and 120 kmph) can be set in the simulation for measurement event predictions.

Proposal-6: A different setting of OW/PW length may apply to a different UE speed in the simulation for measurement event predictions(e.g., 400/400ms, 800/400ms, 800/800).

# References

[1] RAN2#125bis meeting minutes

[2] R2-2404222 Target scenarios for measurement event prediction

[3] Draft Summary of [POST127bis][022][AI mobility] Simulation Assumption of measurement event/RLF prediction and SLS