3GPP RAN WG2 Meeting #128 	R2-2409799

# Orlando, USA, November 18th – 22nd, 2024

Agenda Item:	8.9.2

Source:	InterDigital, Inc.

Title:	Support of Store and Forward.

Document for:	Discussion

# Introduction

A revised WID was approved to further enhance the support of NB-IoT and eMTC [1]

The following objective is provided

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

In RAN2#127 the following was agreed:

Agreements:

1.	RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options)

2.	RAN2 will consider both single satellite pass and multiple satellite pass scenarios

3.	RAN2 will consider both MO and MT data within scope

4.	UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

5.	RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected.

7.	RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

In RAN2#127 the following was agreed:

Agreements:

1.	The dynamic indication that “the cell is operating in S&F mode” is called “S&F operation” indication (we can come back on the exact name when putting this in the spec, if needed)

2.	RAN2 assumes that if an indication that “the cell is operating in S&F mode” is not provided in a NTN cell, the UE should assume that the NTN cell is operating in real-time mode (i.e. default/normal/”not-S&F” mode). RAN2 assumes there should be no distinction between the case that the UE is served by a NTN cell that do not support S&F capability and the case that the UE is served by a NTN cell that does support S&F capability but indication that “the cell is operating in S&F mode” is not provided.

3.	An S&F explicit capability indication by the serving cell, conceived as a static indication of whether the S&F capability is supported or not by a specific satellite/NTN-cell, in addition to the indication that “the cell is operating in S&F mode”, is not needed

4.	If the “S&F operation” indication is not broadcast, a Rel-19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure

5.	When present, the “S&F operation” indication has two possible settings:

‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

# Discussion

## 2.1 S&F Operation mode

According to the agreements from last meeting:

5.	When present, the “S&F operation” indication has two possible settings:

‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

The behaviour described in the above agreement is identical to the behaviour used for feature specific barred/no barred indication. Idle mode UEs, when determining cell status and cell reservations according to 38.304 [3] section 5.3.1 may access the cell if the value is “not barred” and treat the cell as barred if the value is “barred”. UEs in RRC_CONNECTED don’t use the IE.

Observation: The agreed behaviour for “S&F operation” is identical to “cellBarred” and feature specific “cellBArred” IEs.

For consistency with all of the other IEs of this type, we propose that the IE is named in line with the other IEs of this type, and the behaviour in 38.304 is aligned with the other IEs of this type and captured in section 5.3.1 along with the others.

Proposal 1: To align with existing IEs of the same type and behaviour, “S&F operation” IE is named “cellBarredSandF” and has the value “barred” or “not barred”.

Typically, when a cell is barred (either using the legacy cellBarred IE or using a feature specific IE) then the UE is required to exclude the cell for cell selection/reselection for 300s. Note that there are some conditions whereby the UE “may” exclude the cell for up to 300s for power saving purpose, but these conditions are typically MIB or SIB1 reception failure, i.e. the error case. Conditions whereby the cell is explicitly barred result in the UE “shall” exclude the cell from cell selection/reselection for 300s (see 38.304 [3] section 5.3.1)

In most cases, cell barring occurs due to abnormal situations, and the status is not updated very frequently. Hence, UE does not need to re-check the status for 5 minutes to allow for power saving.

However, in the case of S&F operation, it is likely that the mode itself is likely to be updated more frequently than other cellBarred IEs. Applying 5 minutes barring may limit the amount of time a cell is usable, if the UE determined based on the status being barred and S&F operation being on. For example, the UE may determine that the cell is barred, but the gNB then changes S&F operation to normal operation sooner than 5 minutes. Similarly, if the gNB does not change the status for longer than 5 minutes, then the UE may need to perform SIB reception and determine the status again unnecessarily.

One additional reason for S&F mode to have an indicated timing other than fixed 5 minutes is related to the satellite movement. Due to the satellite movement, nature of the orbit and the number of satellites in the constellation, a satellite may likely go out of coverage for a UE in about 5 minutes. Thus, based upon the deployment, e.g., orbit and constellation, the operators may choose a suitable time after which the UE may need to check the S&F status/indication.

In previous meetings, there has been discussions and proposals related to informing the UE with a t-Service like parameter to indicate the remaining time of the current mode. In our opinion, a time based parameter could be suitable if there is any need to perform a cell reselection prioritization or to defer or start a service, based on the amount of time remaining in the current mode.

Without the time based parameter, then the UE would have no knowledge of how long is left in the current mode, and this might result in more frequent failures, depending on the service. It should be noted that this does not mean that S&F does not work at all, however this is an optimisation that could be used to improve the overall user experience. Reading the WID literally, RAN2 should not introduce such an optimisation in order to minimize UE impact. However, as this type of optimisation can improve user experience, it should not be precluded on the basis of this alone but the technical merits should be considered and sufficiently discussed.

Proposal 2: RAN2 to consider whether to introduce a parameter indicating the remaining time of the current mode of S&F, and for what purpose, e.g.

- To defer starting a connection based on the remaining time of the current mode

- To prioritise cells during cell selection/reselection

If the use of the parameter is simply to enable the UE to determine if and when to start a service, then only serving cell information is needed, and only an informative note would be needed to say what the UE may do with the information.

Proposal 3: If a parameter indicating the remaining time of the current mode of S&F is agreed, the UE may defer starting a connection if the remaining time in the current mode is not enough to complete the service.

If the parameter is also used to perform some cell reselection prioritisation then the information would obviously also be beneficial for neighbouring cells and normative requirements are needed.

Proposal 4: If a parameter indicating the remaining time of the current mode of S&F is agreed, and used for cell reselection prioritization, then this information is also broadcast for neighbour cells in addition to the current cell.

As discussed above, usually the UE excludes a barred cell for 5 minutes if it is barred. If a parameter indicating the remaining time of the current mode of S&F is agreed, then the fixed 5 minute timer may not be appropriate but rather the UE should bar according to the remaining time parameter so that it may check the status when the mode changes (In this case we may also need to discuss whether this applies to both cases - by checking the legacy cellBarred when the mode changes from S&F to normal, and by checking the cellBarredSandF when the mode changes from normal to S&F).

Otherwise, the UE should exclude the cell for cell selection for 5 minutes, as per the existing cell barring IEs.

Proposal 5: Depending on whether remaining time parameter is agreed:

Proposal 4a: If a parameter indicating the remaining time of the current mode of S&F is agreed, then if the UE is barred due “cellBarredSandF” = “barred” the UE uses the value of the time parameter to determine how long to bar the cell. FFS the case of changing from normal to S&F mode.

Proposal 4b: If a parameter indicating the remaining time of the current mode of S&F is NOT agreed, then if the UE is barred due “cellBarredSandF” = “barred” the UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds. (aligned to existing cell barred behaviour)

# Conclusion

In this contribution we discuss support of PWS and make the following proposals for discussion:

Observation: The agreed behaviour for “S&F operation” is identical to “cellBarred” and feature specific “cellBArred” IEs.

Proposal 1: To align with existing IEs of the same type and behaviour, “S&F operation” IE is named “cellBarredSandF” and has the value “barred” or “not barred”.

Proposal 2: RAN2 to consider whether to introduce a parameter indicating the remaining time of the current mode of S&F, and for what purpose, e.g.

- To defer starting a connection based on the remaining time of the current mode

- To prioritise cells during cell selection/reselection

Proposal 3: If a parameter indicating the remaining time of the current mode of S&F is agreed, the UE may defer starting a connection if the remaining time in the current mode is not enough to complete the service.

Proposal 4: If a parameter indicating the remaining time of the current mode of S&F is agreed, and used for cell reselection prioritization, then this information is also broadcast for neighbour cells in addition to the current cell.

Proposal 5: Depending on whether remaining time parameter is agreed:

Proposal 4a: If a parameter indicating the remaining time of the current mode of S&F is agreed, then if the UE is barred due “cellBarredSandF” = “barred” the UE uses the value of the time parameter to determine how long to bar the cell. FFS the case of changing from normal to S&F mode.

Proposal 4b: If a parameter indicating the remaining time of the current mode of S&F is NOT agreed, then if the UE is barred due “cellBarredSandF” = “barred” the UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds. (aligned to existing cell barred behaviour)

# References

- RP-242397 Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”

- 36300-i00 , “E-UTRA/E-UTRAN;Overall description;Stage 2”

- 36304-i30, “User Equipment (UE) procedures in idle mode”