3GPP TSG-RAN WG2 Meeting#128                                	R2-2409821

Orlando, USA, November 18th – 22nd, 2024

Agenda item:	8.8.2

Source: 	Google

Title: 	Discussion on the DL coverage enhancement at system level

Document for:	Discussion and Decision

# Introduction

One key research area (i.e., the area for improvement) in the Rel-19 NR-NTN (i.e., NR_NTN_Ph3) is the downlink coverage enhancement, where the enhancement can be considered from both the link level and the system level perspectives, as described in the following Rel-19 NR-NTN WID [1].

DL coverage enhancements can be considered at both - Link level to improve the link margin of selected physical channels in order to accommodate the EIRP reduction in FR1-NTN. A link margin improvement for physical channels (e.g. PDSCH and PDCCH) may be considered without impact on SSB design. - System level to support an efficient dynamic and flexible power sharing between beams or different beam pattern/size (i.e., wide or narrow) across the satellite foot print for FR1-NTN and FR2-NTN.

The Rel-19 NR-NTN discussion was kicked off in RAN2 since RAN2#125bits meeting, and some important progress (shown below) were made across several meeting [2][3] with regard to the DL coverage enhancement.

Agreements in RAN2#127: - From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects) - From RAN2 point of view, If the SSB periodicity is larger than 160ms, for example ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 may need to be extended. And the field description of nAndPagingFrameOffset may need to be enhanced to consider the SSB periodicity higher than 160ms. - RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity. Agreements in RAN2#127bis: - If it turns out that there is a need to bar UEs not supporting DL-CE, Rel-19 UEs not supporting DL-CE can be barred from accessing a cell operating with DL-CE using the existing NTN bar bit, in the same way as pre-Rel19 NTN UEs (this is an extension of the previous agreement to include also Rel-19 UE not supporting DL-CE). - If it turns out that there is a need to bar UEs not supporting DL-CE, then we need to introduce a barring mechanism to control access of UEs supporting Rel19 NTN DL-CE. FFS on the details. (This also implies that UEs supporting Rel19 NTN DL-CE will not consider the existing NTN barring bit) - (also depending on the details of the RAN1 solution) we can further consider methods to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE.

In this paper, we discuss the support of system-level DL coverage enhancement from the signaling perspective, including the necessity to bar the legacy UE and to extend the RRC parameters related to the SSB periodicity. Based on the discussion, several way forwards are proposed in this paper to facilitate the discussion in RAN2 while waiting for more inputs from RAN1.

# Discussion

## The necessity of extending the SSB periodicity

RAN1 has investigated the effect of increasing the SSB periodicity (as a consequence of introducing the beam hopping mechanism) on improving the coverage ratio, and observed that there might be a need to increase the SSB periodicity beyond the existing maximum value (160ms) to support the coverage ratio up to 96.8%. In addition, RAN1 also agreed to add a default value of 160ms (apart from the existing default value 20ms) regarding the SSB periodicity assumed by the UE during the initial access [4].

Agreement in RAN1#118bis: For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is at least 160ms. - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms

Note that the above agreement is about the UE assumed value for SSB periodicity during the initial access, and therefore has nothing to do with the actual RRC signaling. In fact, in the reply LS R1-2407538 [5], RAN1 indicated the extension of the SSB periodicity is still under RAN1’s discussion.

Answer to Q1 (Reply LS R1-2407538): According to the updated WID (RP-241667), SSB enhancement other than SSB periodicity extension is not considered. Thereby, RAN1 understanding is that enhancements to the existing SSB patterns, e.g. SSB position in burst, SSB index number, are not within the scope. The extension of the SSB periodicity is still under RAN1’s discussion.

The necessity of increasing the SSB periodicity is due to the introduction of the beam hopping mechanism. For example, the satellite illustrated in Figure 1 provides a number beam footprints (grouped into four clusters) on the ground to increase the coverage and activates only one of the four beam footprint clusters at one time due to the power constraint. Although the satellite is transmitting the SSB burst every 20 ms, the SSB burst is transmitted in turn among the four footprint clusters. Therefore, for each beam footprint (e.g., beam footprint i), 75% of the SSB bursts are absent because the satellite is transmitting SSB bursts in other footprint clusters. From the beam footprint perspective, it is equivalent that the SSB burst is transmitted once every 80 ms (i.e., ssb-periodicityServingCell = 80 ms). The actual SSB periodicity may go beyond 160 ms (the maximum value in ssb-periodicityServingCell), if there are more than eight footprint clusters provided by the satellite in the example.

Figure 1. The increased SSB periodicity due to the beam hopping among the four beam clusters.

In view of this, several companies proposed in RAN2#127 to extend the maximum value for ssb-periodicityServingCell, and also for other SSB related parameters such as the SMTC, mgrp, and nAndPagingFrameOffset. However, even without being informed of the extended SSB periodicity due to beam hopping (i.e., even with the original SSB periodicity), the UE should be able to know the presence/absence of an SSB burst based on the DTX configuration of the cell/beam that the UE is camping on. Figure 2 below illustrates the same satellite using the same beam hopping sequence as that in Figure 1, but the network additionally signals the DTX pattern for each beam footprint. In Figure 2, although ssb-periodicityServingCell is still 20 ms, with the aid of the DTX configuration, the UE is able to only receive the SSB during the DTX ON period while ignoring the SSBs in the DTX OFF period. The DTX configuration is almost the same for different beam footprints, except for the DTX offset value that is different for different beam footprints in different beam footprint clusters.

Figure 2. The absence/presence of an SSB burst is known to the UE based on the DTX configuration.

The increased SSB periodicity due to the beam hopping can be known to the UE based on the original SSB periodicity (i.e., the SSB periodicity before beam hopping) plus the DTX configuration.

Therefore, with the aid of the DTX configuration, the existing values for ssb-periodicityServingCell and for other SSB related parameters/IEs can remain the same, as the UE is able to figure out the actual periodicity of the SSB burst by itself.

The existing values for ssb-periodicityServingCell, SSB-MTC, SSB-MTC4, and mgrp can remain the same if the DTX pattern is signaled to support the beam hopping mechanism.

In addition, combining the SSB periodicity with the DTX configuration has the merit of offering more flexible beam hopping options, as illustrated in Figure 3. In Figure 3, the beam hopping is conducted between two footprint clusters, and each footprint cluster is activated for a duration of 40 ms followed by a deactivated duration of 40 ms. Such a beam hopping option can be easily configured by the DTX configuration with the DTX ON duration equal to 40 ms and with the DTX cycle length equal to 80 ms. However, it is not possible to use the extended SSB periodicity values to configure such a beam hopping option as the SSB periodicity is changing over time (i.e. , the SSB periodicity is {20ms, 60ms, 20ms, 60ms, …}).

Figure 3. The absence/presence of an SSB burst is known to the UE based on the DTX configuration.

The DTX configuration offers more flexible beam hopping options, due to the configurable ON duration in the DTX configuration.

If the SSB periodicity is to be extended, we may also need to extend the number of SMTCs for the purpose of neighbor cell measurement. Consider the example in Figure 1, since the SSB occurrence is shifted by 20 ms per beam footprint cluster, we need n SMTCs if there are n footprint clusters. However, if the UE is aware of the DTX configurations for it neighbor cells, there is no need to extend the existing SMTC list as the UE can adjust the SMTC for a neighbor cell based on the DTX configuration of the neighbor cell.

Based on the above observations, we think as long as the DTX configuration (of the service cell, the neighbor cell, or both) is provided to the UE, there is no need to extend the existing values for ssb-periodicityServingCell as well as for other SSB related parameters/IEs. There is also no need to extend the existing SMTC list due to the same reason.

RAN2 neither extends the values for ssb-periodicityServingCell (and other SSB related parameters) nor extends the SMTC list, but introduces the DTX configuration instead to support the system level DL coverage enhancement.

## The provision of the DTX configuration

The introduction of the N1 beam footprints (i.e., the number of the beam footprints not providing service) will have impact to the UE (both the legacy and Rel-19 UE) in the idle or inactive state. This is because the camped cell may not transmit anything to the UE at the time when the UE wakes up at its paging occasions. One may argue that the network can configure a paging cycle length and the number of paging frames (that are evenly distributed within the paging cycle) in a way that the paging frame always aligns with the occurrence of the SSB. However, as the maximum interval between two adjacent paging frames is now 160ms (according to nAndPagingFrameOffset), aligning the paging frame with the occurrence of SSB through the paging configuration is not possible when the DTX cycle (i.e., the actual SSB periodicity) is more than 160 ms.

The beam hopping mechanism may have the impact to the UE in idle/inactive state, as when the UE wakes up at its paging occasions, it may not detect anything including the SSB burst.

The WID pointed out that the Rel-18 Network Energy Saving (NES) techniques should be considered as the baseline for signaling the beam hopping pattern. However, the cell DTX/DRX configuration in Rel-18 NES cannot be directly employed here to signal the beam hopping pattern, as the cell DTX/DRX configuration in Rel-18 NES is only applicable to the UE in the connected state, and the cell will still transmit common control signals even in the non-active period of a DTX cycle. As the beam hopping mechanism may impact the legacy UE as well as the Rel-19 UE in the idle/inactive state, the cell DTX/DRX configuration shall be also applicable to the UEs in the idle/inactive state, and shall cover the state where the satellite beam does not transmit anything. Therefore, we think the beam hopping pattern (i.e., the cell or beam DTX configurations) can be provided in the system information, so that at least the Rel-19 UE is able to determine when to wake up and monitor the paging message based on the beam hopping pattern and the paging configuration.

The DTX configuration can be provided in the system information for idle/inactive UEs, and covers the Tx state where the beam/cell is not transmitting anything.

Regarding whether the DTX configuration is provided to the UE on a per-beam or a per-cell basis, we think this is relevant to how the network/operator maps the satellite beams into an NR beam or an NR cell. According to R1-2400303 [6], there are five possible options for mapping the satellite beams into an NR beam or NR cell.

- Single NR cell per satellite beam and single NR beam cell

- Multi-Satellite beam NR cell and single NR beam cell: Cell splitting

- Multi-Satellite beam cell and multi NR beam cell

- Multi NR cells per satellite beam

- Multi NR cells split across satellite beams

If the DTX configuration is provided on a per-cell basis, it works for Option 1 and Option 4 above, but does not work for the other options. On the other hand, if the DTX configuration is provided on a per-beam basis, it works for all the options except for Option 2, and may result in some duplicated signaling in Option 4 (i.e., not optimized for Option 4). In fact, the cell DTX configuration can be treated as a special case of the beam DTX configuration, in which all the NR beams within the same cell share the same beam DTX configurations.

Providing beam DTX configuration can support most of the mapping options between the satellite beam and the NR beam/cell, and the cell DTX configuration can be considered as a special case of the beam DTX configure.

Therefore, we think the provision of the DTX configuration on a per-beam basis can be considered as the baseline, where each DTX configuration can be associated to an SSB index. It can be further discussed whether the DTX configuration can be associated to a CSI-RS resource ID.

The DTX configuration can be a beam-specific configuration, and can be associated to an SSB index.

To address the issue of having duplicated DTX configurations while providing the beam-specific DTX configuration in Option 4, one possible handling is to provide a DTX configuration but not associate it with any beam index. The UE would consider such a DTX configuration (not associated with any beam index) begin applicable to all the beams within the same cell.

If a DTX configuration is not associated with an SSB index, it is applicable for the entire cell.

## The impact on the legacy UEs

It was discussed in the RAN2#127bis meeting that whether there is a need to bar the legacy UE or the UE not supporting DL-CE, from accessing the cell operating with DL-CE. We think that as the legacy UE or the UE not supporting DL-CE will perform the neighboring cell measurement solely based on the legacy SMTC configuration, most of the time these UEs will not detect the SSBs of the DL-CE cells while performing the measurement. Take Figure 4 as an example, a legacy UE is configured with the DRX cycle length equal to 1.28 seconds, and the default SMTC of the target frequency to be measured by the UE has a cycle length equal to 160 ms and a duration equal to 5 ms. If a DL-CE cell operating in that frequency has been detected by the UE before, without knowing the actual SMTC periodicity (i.e., 640 ms) of the DL-CE cell, the chance for the UE to capture two meaningful samples separated by at least (Tmeasure,NR_Inter/2) from the DL-CE cell within Tmeasure,NR_Inter is only 10%. The chance is even less if the default SMTC periodicity is less than 160 ms or if the UE is configured with other DRX cycle lengths (e.g., 2.56 s).

It is unlikely the legacy UE can obtain a meaningful measurement result from the cell operating with DL-CE per Tmeasure,NR_Inter or Tmeasure,NR_Intra.

Moreover, even if the legacy UE is lucky enough to obtain a meaningful measurement results from a DL-CE cell within a Tmeasure,NR_Inter or a Tmeasure,NR_Intra, the UE has to be continuously that lucky for a duration of Treselection in order to trigger the cell reselection, which is very unlikely to happen. Therefore, we do not think there is a need to bar the legacy UE from accessing a DL-CE cell, as the chance for the legacy UE to reselect the DL-CE cell is very small. If there is no need to bar the legacy UE, there is also no need to introduce a new barring mechanism to bar the Rel-19 UE supporting DL-CE (since the legacy barring mechanism can be used to bar all UEs including the Rel-19 UE supporting DL-CE). For the same reason, a new method for the UE not supporting DL-CE to down-prioritize the cell reselection to the DL-CE cell seems to be not justified as well.

Figure 4. Legacy UE blindly picks two among the eight SMTC windows in a DRX cycle to perform the measurement

RAN2 neither introduces a new barring mechanism to bar the Rel-19 UE supporting DL-CE, nor introduces a new method for the UE not supporting DL-CE to down-prioritize the cell reselection to cells operating with DL-CE.

## The impact on the RRC_CONNECTED UE

When the UE is in the connected state and the serving beam or the serving cell is under the OFF state (i.e., the state when the beam or cell is not transmitting anything), the UE is not required to monitor the PDCCH for saving energy. In order to further save the UE power and to prevent the UE from declaring the RLF or the beam failure while the beam/cell is not transmitting anything, we think the connected UE should refrain itself from performing the RLM and BFD, and may also refrain from performing the serving cell measurement during the beam/cell off periods.

The connected UE is not required to monitor the PDCCH, perform the RLM/BFD, and perform the serving cell measurement during the OFF periods configured by the DTX configuration.

As adopting P4 above may have some impact to the specification, an alternative approach to introduce minimum specification impact is to rely on the c-DRX configuration. Since the UE being configured with c-DRX would comply with the timing requirement (for measurement, RLM, and BFD) that is based on the c-DRX cycle length, the UE is able to avoid monitoring/measuring the RS during the beam/cell OFF period if every c-DRX on-duration would fall within or overlap with the DTX active duration. To ensure the c-DRX on-duration would align with the DTX active duration, a stage-2 description similar to the one for the Rel-18 NES might be needed. That is, the UE in the connected state is expected to receive a c-DRX configuration whose the cycle length is equal to one or multiple times of the cell/beam DTX cycle length.

The connected UE is expected to receive a c-DRX configuration whose the cycle length is equal to one or multiple times of the DTX cycle length.

# Conclusion

In this paper, we discussed several key issues regarding the DL coverage enhancement at the system level. Based on the discussion in this paper, we respectfully ask RAN2 to discuss and consider the following observations and proposals.

The necessity of extending the SSB periodicity:

Observation 1	The increased SSB periodicity due to the beam hopping can be known to the UE based on the original SSB periodicity (i.e., the SSB periodicity before beam hopping) plus the DTX configuration.

Observation 2	The existing values for ssb-periodicityServingCell, SSB-MTC, SSB-MTC4, and mgrp can remain the same if the DTX pattern is signaled to support the beam hopping mechanism.

Observation 3	The DTX configuration offers more flexible beam hopping options, due to the configurable ON duration in the DTX configuration.

Proposal 1	RAN2 neither extends the values for ssb-periodicityServingCell (and other SSB related parameters) nor extends the SMTC list, but introduces the DTX configuration instead to support the system level DL coverage enhancement.

The provision of the DTX configuration:

Observation 4	The beam hopping mechanism may have the impact to the UE in idle/inactive state, as when the UE wakes up at its paging occasions, it may not detect anything including the SSB burst.

Proposal 2	The DTX configuration can be provided in the system information for idle/inactive UEs, and covers the Tx state where the beam/cell is not transmitting anything.

Observation 5	Providing beam DTX configuration can support most of the mapping options between the satellite beam and the NR beam/cell, and the cell DTX configuration can be considered as a special case of the beam DTX configure.

Proposal 3	The DTX configuration can be a beam-specific configuration, and can be associated to an SSB index.

Proposal 4	If a DTX configuration is not associated with an SSB index, it is applicable for the entire cell.

The impact on the legacy UEs:

Observation 6	It is unlikely the legacy UE can obtain a meaningful measurement result from the cell operating with DL-CE per Tmeasure,NR_Inter or Tmeasure,NR_Intra.

Proposal 5	RAN2 neither introduces a new barring mechanism to bar the Rel-19 UE supporting DL-CE, nor introduces a new method for the UE not supporting DL-CE to down-prioritize the cell reselection to cells operating with DL-CE.

The impact on the RRC_CONNECTED UE:

Proposal 6	The connected UE is not required to monitor the PDCCH, perform the RLM/BFD, and perform the serving cell measurement during the OFF periods configured by the DTX configuration.

Proposal 7	The connected UE is expected to receive a c-DRX configuration whose the cycle length is equal to one or multiple times of the DTX cycle length.

# Reference

RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3.

Draft report of 3GPP TSG RAN WG2 meeting #127, ETSI MCC.

Draft report of 3GPP TSG RAN WG2 meeting #127bis, ETSI MCC.

Draft Report of 3GPP TSG RAN WG1 meeting #118bis, ETSI MCC.

R1-2407538, Draft Reply LS on DL coverage enhancements, China Mobile.

R1-2400303, Discussion on NR NTN Downlink coverage enhancements, Thales.