3GPP TSG-RAN WG2 Meeting #128	R2-2409840

Orlando, USA, Nov. 18th –22nd, 2024

Agenda item:	8.2.5

Source: 	Fujitsu

Title: 	Discussions on topology 2 related aspects

Document for:	Discussion and decision

1. Introduction

According to the New SID [1], the overall objective shall be to study a harmonized air interface design with minimized differences for Ambient IoT for Topologies 1 and 2.

Regarding Topology 2, RAN2 has made the following agreements [2][3]:

Agreements

- RAN2 is responsible for the interface between intermediate node (i.e. Reader) and RAN for topology 2.

- A-IoT air interface in topology 1 between A-IoT device and reader is fully reused in topology 2, i.e. topology is transparent to the A-IoT device and there is no impact on A-IoT device.

- RAN2 assumes that intermediate UE authorization is performed by upper layers. RAN2 impact can be studied after further SA/RAN3 progress.

- RAN2 will study the impacts to RAN2 based on the SA2 identified architecture options (i.e. no new AS layer architecture options will be studied)

- NW is in control of resources to be used by AIoT air interface.

- Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO).  May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

Agreements

- Dedicated resource configuration is only given to the UE reader via dedicated signalling.  Mechanisms for shared resource pool amongst readers are not considered in this release.

- The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control.    FFS how valid resource is determined – we will consider temporary out of connection scenarios.    FFS on resource validity across multiple cells.

- The AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc).

This contribution will provide more consideration on Ambient IoT Topology 2 on the following aspects:

- Resource allocation;

- Impacts to NR Uu interface.

2. Discussion

Resource Allocation

RAN2 has agreed that the radio resources used by A-IoT air interface between the A-IoT device and the UE reader are controlled by the network and dedicated resource configuration is only given to the UE reader via dedicated signalling. Also, mechanisms for shared resource pool amongst readers are not considered in this release.

To implement dedicated resource configuration, the granularity for resource allocation needs to be discussed.

One possibility is the radio resources can be allocated by the gNB when the UE is selected/identified as the reader, i.e. the resources are allocated per UE reader. For example, after the resources are pre-allocated per UE reader, the UE reader performs inventory/command procedure over A-IoT air interface when A-IoT upper layer information is received, and the radio resource(s) used for the inventory/command procedure are determined by the UE reader from the radio resources pre-allocated for the UE reader.

Alternatively, the radio resources used by A-IoT air interface can be allocated by the gNB when the gNB receives A-IoT upper layer information, e.g. inventory request or command. It means the resources can be allocated per A-IoT service request. Or the radio resources can be allocated per transmission. For example, the radio resources for R2D transmission(s) can be allocated by the gNB when the gNB receives A-IoT upper layer information from, e.g. the CN while the radio resources for D2R transmission(s) can be allocated by the gNB when the gNB receives request from the intermediate UE. In addition, the radio resources used by A-IoT air interface can be allocated per A-IoT paging (round), per access round, per D2R trigger, per R2D/D2R transmission pair.

Proposal 1: RAN2 to consider the following options for the granularity of radio resources used by the A-IoT air interface:

- per UE reader

- per A-IoT service request

- per A-IoT paging (round)

- per access round

- per D2R trigger

- per R2D/D2R transmission pair

- per transmission

RAN2 has agreed that the UE may perform the A-IoT procedure on A-IoT interface between the UE reader and the A-IoT device only if the resource configuration is valid. However, how valid resource is determined and whether the resource configuration is valid across multiple cell need more study. It is beneficial if the resource configuration is valid across multiple cell, e.g. in RAN area after the UE enters RRC_INACTIVE state. In this case, ongoing A-IoT procedure continues. Considering the timeline, we think both cases are considered in the SI and RAN2 can make decision on down-selection in WI phase. So, the resource configuration is valid in the cell where it is received or in the configured area, e.g. containing multiple cell.

Proposal 2: Radio resources used by the A-IoT air interface will be considered invalid when the UE moves out of the cell where the resources are configured or the configured area containing multiple cells.

Potential impacts/enhancements to NR Uu interface

Regarding NR Uu interface, it is observed that there are some impacts due to support of A-IoT and thus some enhancements are needed. For example, UE capability and reporting need to be considered so that the network can know e.g. if the UE can be selected as a UE reader. Also, mechanisms for transmission (new RRC message or specific DRB) of A-IoT data should be discussed after the protocol stacks are determined. If the radio resources used by A-IoT air interface can be requested by the UE, RAN2 needs to discuss mechanisms for UE request.

Considering the timeline, we think that these potential impacts/enhancements to NR Uu interface can be discussed and specified in WI phase.

Proposal 3: RAN2 to specify the following potential enhancements to NR Uu interface in WI phase:

- UE capability report

- Transfer of A-IoT upper layer information

- UE request for radio resource over A-IoT radio interface

3. Conclusion

In this contribution, we had some discussions on the Ambient IoT Topology 2 and made the following proposals:

Resource Allocation

Proposal 1: RAN2 to consider the following options for the granularity of radio resources used by the A-IoT air interface:

- per UE reader

- per A-IoT service request

- per A-IoT paging (round)

- per access round

- per D2R trigger

- per R2D/D2R transmission pair

- per transmission

Proposal 2: Radio resources used by the A-IoT air interface will be considered invalid when the UE moves out of the cell where the resources are configured or the configured area containing multiple cells.

Potential impacts/enhancements to NR Uu interface

Proposal 3: RAN2 to specify the following potential enhancements to NR Uu interface in WI phase:

- UE capability report

- Transfer of A-IoT upper layer information

- UE request for radio resource over A-IoT radio interface

Reference

- RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, RAN#103, March 18-21, 2024

- RAN2#127 chair notes

- RAN2#127bis chair notes