3GPP TSG-RAN WG2 Meeting #128	R2-2409843

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.6.4

Source: 	Fujitsu

Title:	Discussion on conditional Intra-CU LTM

Document for:	Discussion and Decision

- Introduction

In the RAN#105 meeting, it was agreed that conditional LTM would be in scope of Rel-19 as follows:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

This document discusses basic function of conditional LTM for non-DC case.

- Discussions

Preparation phase of conditional Intra-CU LTM

In Rel-18 LTM, L1 measurement and LTM are decoupled, i.e. either L1 measurement report or L3 measurement report can be used for LTM decision. In Rel-19, for conditional Intra-CU LTM, the same principle with CHO is applicable considering work load. Therefore, CHO conditions can be reused, e.g. due to UE capability of L1 measurement.

Proposal 1: For conditional Intra-CU LTM, CHO conditions can be reused for triggering the LTM. The execution condition is generated by the CU.

Regarding L1 measurement events, RAN2 has agreed that LTM event 3/5-like can be used for triggering the LTM. However, beam level measurement is used for the event evaluation of the LTM event 3/5. To avoid too frequent measurement reporting, TTT is applied for event-triggered L1 measurement reporting. For conditional Intra-CU LTM, a similar mechanism can be considered to avoid Ping-Pong for LTM. Alternatively, multiple beams or cell level measurement will be used for the event evaluation to trigger LTM.

Proposal 1bis: For conditional Intra-CU LTM, RAN2 to discuss whether TTT, multiple beams or cell level measurement is used for triggering LTM.

Early synchronization of conditional Intra-CU LTM

RAN2 has agreed that early RACH towards candidate cell for TA acquisition can be supported. In Rel-18, early RACH towards candidate cell is triggered by the network via PDCCH order and valid TA for target cell can be included in the LTM cell switch command. The trigger of early RACH towards candidate cell can be reused for conditional LTM. For TA provision, one possibility is RAR-based TA acquisition, as discussed in Rel-18 and Rel-19 inter-CU LTM. However, if the RAR is received from candidate cell, it may incur more interruption. We think that TA value for candidate cell is provided by the serving cell via RAR or TA command MAC CE if early RACH towards candidate cell for TA acquisition is used. Currently, CFRA with 4-step RA type and CFRA without NW response with 4-step RA type are specified for PDCCH triggered RA. If RAR is used, existing procedures cannot be used without modifications. It means that the existing CFRA procedure with 4-step RA type needs be enhanced. If TA command MAC CE is used, it is straightforward to apply the TA command for target cell.  Considering the complexity, TA command MAC CE is the baseline.

Proposal 2: The UE obtains the TA from the serving cell. TAC MAC CE is baseline.

In Rel-18, TA = 0 or TA with same value with current TAG can be supported by including corresponding TA value in the LTM cell switch command. For conditional Intra-CU LTM, no LTM cell switch MAC CE will be used and thus whether these cases will be supported or not needs more discussion.

Proposal 2bis: RAN2 to discuss if TA = 0 or TA same with current TAG will be supported for conditional intra-CU LTM.

Execution and completion of conditional Intra-CU LTM

For RACH-based conditional LTM, as legacy, TX/RX beam can be selected by the UE during RACH according to corresponding L1 RSRP even if there is no LTM cell switch command indicating the TCI state(s) from the network. The selected beams can be used during the LTM cell switch and after the cell switch until a new TCI state is activated/indicated by the network.

In case of conditional RACH-less LTM, how to determine TX/RX beam needs more discussions. RAN2 has agreed that Rel-18 TCI state activation/deactivation for candidate cells can be used for conditional Intra-CU LTM. Based on this, one question is whether and how to determine the beam used during and after the LTM execution, e.g. best activated beam for the target cell. The discussion can be in the scope of RAN1. So, RAN1 needs to be involved.

Proposal 3: For beam determination during RACH-less conditional Intra-CU LTM, wait for RAN1.

Conditional Intra-CU LTM cell switch execution can be performed with or without RACH. In Rel-18, intra-CU LTM cell switch execution without RACH is performed if the UE has valid TA. We think that the same principle can be reused for the determination.

Proposal 4: Conditional RACH-less LTM is performed if TA is valid; Conditional RACH-based LTM is performed if TA is invalid.

If TA for candidate cell is maintained by the UE, the UE needs to check whether the TA value for target cell is valid or not when conditional LTM cell switch is triggered. For UE based TA measurement, TA value is considered as valid if the network configures it. However, for TA indicated by the network, more RAN2 discussions are needed. For example, TA timer or RSRP change can be used.

Proposal 4bis: RAN2 to discuss how to determine if the TA is valid or not.

In Rel-18, in case of RACH-less LTM, the gNB considers the LTM completed when the first UL transmission is received, and the UE considers the LTM completed when the UE confirms that the first UL transmission is received at the gNB. RAN2 has agreed that the same mechanism is reused for conditional Intra-CU LTM.

The first UL transmission can be dynamically scheduled by the gNB in Rel-18 LTM. However, there are some concerns for conditional Intra-CU LTM. Because the target cell or the network cannot know when the UE will perform cell switch and will not send PDCCH to schedule UL resources for the first UL data. Some enhancements are needed. We don’t see any significant benefit to support dynamic scheduling of first UL transmission considering CG-based approach can be used for the transmission. So, no DG-based approach is preferred.

Proposal 5: For conditional RACH-less LTM, we will not support DG-based approach for the first UL transmission at target cell.

Signalling procedure for conditional Intra-CU LTM

In Rel-18, early synchronization can be supported by L3 measurement. In case of no L1 measurement report, the source CU can make decision on TCI state activation/deactivation and the triggering of early RACH towards candidate LTM cell for TA acquisition based on L3 measurement report and indicate to source DU. We think that it can be reused for conditional Intra-CU LTM. Based on these discussions and signalling procedure for Rel-18 intra-CU LTM, the signalling procedure for conditional LTM can be captured in the figure below:

Figure 1: Signalling procedure for conditional LTM

Proposal 6: For conditional Intra-CU LTM, RAN2 to agree signalling procedure in Figure 1.

- Conclusion

This document discussed basic function of conditional LTM for non-DC case and made the following proposals:

Preparation phase of conditional Intra-CU LTM

Proposal 1: For conditional Intra-CU LTM, CHO conditions can be reused for triggering the LTM. The execution condition is generated by the CU.

Proposal 1bis: For conditional Intra-CU LTM, RAN2 to discuss whether TTT, multiple beams or cell level measurement is used for triggering LTM.

Early synchronization of conditional Intra-CU LTM

Proposal 2: The UE obtains the TA from the serving cell. TAC MAC CE is baseline.

Proposal 2bis: RAN2 to discuss if TA = 0 or TA same with current TAG will be supported for conditional intra-CU LTM.

Execution and completion of conditional Intra-CU LTM

Proposal 3: For beam determination during RACH-less conditional Intra-CU LTM, wait for RAN1.

Proposal 4: Conditional RACH-less LTM is performed if TA is valid; Conditional RACH-based LTM is performed if TA is invalid.

Proposal 4bis: RAN2 to discuss how to determine if the TA is valid or not.

Proposal 5: For conditional RACH-less LTM, we will not support DG-based approach for the first UL transmission at target cell.

Signalling procedure for conditional Intra-CU LTM

Proposal 6: For conditional Intra-CU LTM, RAN2 to agree signalling procedure in Figure 1.