3GPP TSG-RAN WG2 Meeting #128	R2-2409846

Orlando, USA, November 18th – 22nd, 2024

Agenda item:	8.7.4.2

Source: 	Fujitsu

Title: 	Discussions on DSR enhancements

Document for:	Discussion and decision

Introduction

On DSR retransmission enhancement in XR, RAN2 has made the following agreements in RAN2#127bis meeting [1]:

Agreements on DSR enhancements - We do not change the definition of delay-critical data - For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) - Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. - For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - Buffer size of data volume in each portion - Shortest remaining time among PDCP SDUs buffered in each portion. - There is no need to include PSI in the enhanced DSR MAC CE. - A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. - FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

In this contribution, we would like to discuss further details on the potential DSR enhancements for XR.

Discussion

Constraints on threshold configuration

In Rel-18, a threshold, i.e., remainingTimeThreshold, is defined for triggering a DSR for an LCH within an LCG. This threshold is configured for an LCG and is also used in the definition of delay-critical data. That is to say, a single threshold is used for both DSR triggering and delay-critical data definition, as shown in the following [2][3] respectively.

DSR triggering condition in the Rel-18 MAC specification:

5.4.9 Delay status reporting The Delay Status Reporting (DSR) procedure is used to provide the serving gNB with delay status of LCGs. This delay status for an LCG includes remaining time, which is the smallest remaining value of the running PDCP discardTimers among PDCP SDUs that are buffered for the LCG but have not been transmitted in any MAC PDU as specified in clause 7.3 in TS 38.323 [4], and the total amount of delay-critical UL data for the LCG according to the data volume calculation procedure specified in clause 5.5 in TS 38.322 [3] and clause 5.15 in TS 38.323 [4] for the associated RLC and PDCP entities, respectively. RRC controls the DSR procedure by configuring the following parameter: - remainingTimeThreshold (per LCG): the threshold on remaining time for triggering a DSR for a logical channel within an LCG. If an LCG is configured for delay status reporting, the MAC entity shall for each logical channel within the LCG: 1> if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and 1> if there is no DSR pending for the logical channel: 2> trigger a DSR for the logical channel.

Delay-critical PDCP SDU definition in the Rel-18 PDCP specification:

Delay-critical PDCP SDU: if pdu-SetDiscard is not configured, a PDCP SDU for which the remaining time till discardTimer expiry is less than the remainingTimeThreshold. If pdu-SetDiscard is configured, a PDCP SDU belonging to a PDU Set of which at least one PDCP SDU has the remaining time till discardTimer expiry less than the remainingTimeThreshold.

In Rel-19, based on the RAN2 agreements on the enhanced DSR, there is a single DSR triggering threshold, but there can be multiple remaining time thresholds for reporting for an LCG. In RAN2#127 meeting, there is an FFS about whether there are any constraints on how the NW configures DSR triggering and reporting thresholds.

Based on the discussions in the last meeting, for the configuration constraints on DSR triggering threshold and reporting thresholds for an LCG, there can be following options:

- Option 1: The triggering threshold is equal to or higher than the largest reporting threshold.

- Option 2: The triggering threshold is lower than the largest reporting threshold.

In option 1, when a DSR is triggered based on the triggering threshold, all the data with remaining time below the reporting thresholds would be considered as delay-critical data for the LCG. Hence, only the delay information of the delay-critical data will be reported in the enhanced DSR MAC CE, and the current specification of DSR cancellation related behaviors can be reused. On the other hand, DSR may be easily triggered since the triggering threshold is relatively high, so the signalling overhead due to DSR transmission in option 1 could be a little higher than that in option 2.

In option 2, when a DSR is triggered based on the triggering threshold of an LCG, the delay information of the data in the LCG with remaining time higher than the triggering threshold but lower than or equal to the reporting threshold(s) will also be reported in the enhanced DSR MAC CE. As RAN2 agrees that the definition of delay-critical data is not modified in the last meeting, it means that the delay information of the non-delay critical data can also be reported in the enhanced DSR MAC CE. In the current specification, DSR cancellation is related to delay-critical data only but not non-delay critical data. Therefore, how to handle the DSR trigger or cancellation for the reported non-delay critical data may need to be further discussed. From this perspective, the standard impact is big. On the other hand, the signalling overhead from DSR transmission is lower since the triggering threshold is relatively low in option 2 compared with option 1.

The comparison of the above 2 options and the impact on delay-critical data definition is shown in the following Table 1:

Table 1 Comparison of option 1 and option 2

| Option | Advantages | Disadvantages |
| --- | --- | --- |
| Option 1: The triggering threshold is equal to or higher than the largest reporting threshold | Smaller standard impact with reusing current specification of delay-critical data. | Signalling overhead with more easily triggered DSR. |
| Option 2: The triggering threshold is lower than the largest reporting threshold | Low signalling overhead. | Bigger standard impact on specifying non-delay critical data impact on DSR triggering and cancellation. |

From the comparison, we prefer option 1 due to its simplicity and smaller impact on the standard. Moreover, there is no need to configure the triggering threshold higher than the largest reporting threshold, since it may happen that DSR is triggered based on the higher triggering threshold but no buffer size can be reported in the enhanced DSR MAC CE when the remaining time of the data volume is higher than the largest reporting threshold and lower than the triggering threshold.

Proposal 1: DSR triggering threshold is configured to be the largest reporting threshold.

Inclusion of non-delay critical data ahead of delay-critical data

In RAN2#127 meeting, there is an FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR. The scenario raised by the proponent company is that the UE buffer is not ordered according to the discard timer and the UE may send non-delay critical data before delay-critical data. In our opinion, although RAN2 has agreed not to do intra-LCH prioritization, UE implementation can differentiate the delay-critical data from non-delay critical data and can transmit delay-critical data properly with the LCP enhancements. Therefore, there is no need to include non-delay critical data ahead of delay-critical data in the buffer size calculation for DSR.

Proposal 2: Non-delay critical data ahead of delay-critical data is not included in the buffer size calculation for DSR.

Enhanced DSR MAC CE format

In the last meeting, RAN2 agrees that a one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. In our opinion, to be simple, the format of the legacy DSR MAC CE can be reused with the R bit used as the one-bit indication. For example, the format of the enhanced MAC CE is shown in Figure 1:

Figure 1: Format of the enhanced DSR MAC CE

In figure 1, the E bit is used to indicate whether the further pair of remaining time information and buffer size information present in the enhanced DSR MAC CE is for the same LCG. For example, if the value of E bit is 0, the further pair of remaining time information and buffer size information present in the enhanced DSR MAC CE is for the same LCG; otherwise, if the value of E bit is 1, the further pair of remaining time information and buffer size information present in the enhanced DSR MAC CE is for the next reported LCG with the value of LCGi field set to 1.

Proposal 3: The format of the enhanced DSR MAC CE as shown in Figure 1 is adopted.

Another issue that can be discussed is the order of the multiple pairs of remaining time information and buffer size information present for one LCG. In our opinion, the multiple pairs can be present in ascending order of the reporting thresholds of the remaining time, which is very straightforward.

Proposal 4: For a reported LCG, the multiple pairs of remaining time information and buffer size information present are present in ascending order of the reporting thresholds.

Legacy DSR and enhanced DSR coexistence

In the last RAN2 meeting, there is an FFS issue on whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds. In our understanding, we can always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds. For the LCG configured with multiple reporting thresholds but with only one pair of remaining time and buffer size be reported, or for the LCG configured with only one reporting threshold, the enhanced DSR format can include just one pair of remaining time information and buffer size information for this LCG. This is much simpler compared with determining whether legacy DSR or enhanced DSR to be reported based on buffer size or number of reporting threshold(s) of each LCG.

Observation 1: For the LCG configured with multiple reporting thresholds but with only one pair of remaining time and buffer size to be reported, or for the LCG configured with only one reporting threshold, the enhanced DSR format can include just one pair of remaining time information and buffer size information for this LCG.

Proposal 5: Enhanced DSR is always used in case there is at least one LCG configured with multiple reporting thresholds.

Conclusion

In this contribution, we have discussed the further details of the DSR enhancements. We have the following proposals:

Proposal 1: DSR triggering threshold is configured as the maximum reporting threshold.

Proposal 2: Non-delay critical data ahead of delay-critical data is not included in the buffer size calculation for DSR.

Proposal 3: The format of the enhanced DSR MAC CE as shown in Figure 1 is adopted.

Proposal 4: For a reported LCG, the multiple pairs of remaining time information and buffer size information present are present in ascending order of the reporting thresholds.

Observation 1: For the LCG configured with multiple reporting thresholds but with only one pair of remaining time and buffer size to be reported, or for the LCG configured with only one reporting threshold, the enhanced DSR format can include just one pair of remaining time information and buffer size information for this LCG.

Proposal 5: Enhanced DSR is always used in case there is at least one LCG configured with multiple reporting thresholds.

References

- R2-2409215, Report from session on R18 MBS, R18 QoE and R19 XR,	Session chair (Huawei), RAN2#127bis meeting

- TS38.321 v18.3.0

- TS38.323 v18.3.0