3GPP TSG-RAN WG2 Meeting #128                                                                    R2-2409855

Orlando, USA, 18th – 22nd Nov 2024

Source:	CATT

Title:	Consideration on LCP enhancement

Agenda Item:	8.7.4.1

Document for:	Discussion and Decision

Introduction

In this contribution, we will further discuss the issues on LCP enhancement including LCP additional priority configuration and when/how to apply the additional priority, our views are also given in this paper.

Discussion

Agreements on LCP enhancements - As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round. - FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation - Introduce an independent per-LCH remaining time threshold for applying delay-critical priority. - We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold. - For now, we will use “LCH priority-adjusted data”

In RAN2#127 meeting, it is agreed that RAN2 no longer consider the enhancement of the LCP restriction as one of the candidate solutions for LCP enhancements in Rel-19 XR. For LCP priority, based on above agreements in RAN2#126 meeting[1], it has agreed that the priority of LCH will be adjusted based on configuration and applied in the LCP procedure, more details are discussed here:

How to configure the additional priority?

Generally, there are two options to configure the additional priority:

- Option 1: Pre-configuration.

- Option 2: Configured by RRC signalling.

For Option 1, the priority to LCH with LCH priority-adjusted data could be preconfigured as a fix value, such as the highest priority. This option is not flexible. Furthermore, in some cases, it is not reasonable. For example, if there is important MAC CE or other data which is more important than the delay-critical data, it is not reasonable to always prioritize the LCH priority-adjusted data. Hence, it is preferable to let it configurable as in Option 2. The network can configure the additional priority for the LCH by RRC signaling, the specific value of additional priority could be considered as network implementation.

Proposal 1: Additional logical channel priority and its corresponding remaining time threshold can be configured by network via dedicated RRC signaling.

Whether the priority of LCH can be changed during one LCP procedure?

According to legacy procedure as below, the logical channels selected for the UL grant are allocated resources in a decreasing priority order with Bj >0 (step 1), and Bj will be updated (step 2), then logical channels selected are allocated remaining resources in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first (step 3).

5.4.3.1.3 Allocation of resources Before the successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall not select the logical channel(s) corresponding to non-DAPS DRB(s) for the uplink grant received in a Random Access Response or the uplink grant for the transmission of the MSGA payload. The source MAC entity shall select only the logical channel(s) corresponding to DAPS DRB(s) during DAPS handover. The MAC entity shall, when a new transmission is performed: 1> allocate resources to the logical channels as follows: 2> logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s); 2> decrement Bj by the total size of MAC SDUs served to logical channel j above; 2> if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally. NOTE 1: The value of Bj can be negative.

If two priorities are configured for one LCH, a new issue rasied. That is, if the resource allocated considering PBR in step 1 can accommodate all the LCH priority-adjusted data, when performs resource allocation in step 3, whether the legacy priority or additional priority can be used since there is no LCH priority-adjusted data. RAN2 needs to discuss whether the LCH priority will fallback to the legacy value to avoid allocating remaining resource to logical channel with an incorrect high priority. Considering the LCP procdure is very tighy in timeline, the re-evaluation to LCH prority in second round resource allocation is only associated to the LCH with additional priority in first round resrouce allocaiton, and to evaluate whether any of LCH priority-adjusted data is remained, it can be seen that this re-evaluation to LCH prority procedure could be executed in MAC without any time tight issue and there is no UE complexity issue. If we leave this option as UE implementation, it is not sure whether the UE apply the legacy priority or not, it is unfair to the other LCH in the second round resource allocation.

Proposal 2: To keep the fairness amongst different LCHs, MAC entity could re-evaluate whether the legacy priority or additional priority of an LCH can be used in the second round resource allocation based on whether there is still UL LCH priority-adjusted data without resource allocation.

Whether to enhance the PBR for LCH using additional priority

Based on above description to resource allocation,  the logical channel with Bj > 0 will be selected in the first round of resource allocation. Generally, the MAC entity will stop to allocate resource to a LCH if its PBR could be met when performing resource allocation. However, there may be still UL delay-critical data without resource after the PBR is met, if this type of data is not allocated resource, maybe it will become obsolete data. One option is to increase the PBR, such as setting it as infinity. If going to this otpion, based on leagcy procedure, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel, which will also inlcude the not priority-adjusted data, such as not delay-critcal data. So, it can be seen that this option is not fair to the other LCH with higher priority than the legacy priority of the LCH.Even if there is remained delay-critical datain the first round of resource allocation, it is better to allocate the resource based on the priority order to keep the fairness among different LCHs in the first round.

Proposal 3: Enhancement for the first round resource allocation during LCP (e.g., resource allocation is not only based on Bj, but also consider the amount of UL delay-critical data) is not considered in Rel-19.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: Additional logical channel priority and its corresponding remaining time threshold can be configured by network via dedicated RRC signaling.

Proposal 2: To keep the fairness amongst different LCHs, MAC entity could re-evaluate whether the legacy priority or additional priority of an LCH can be used in the second round resource allocation based on whether there is still UL LCH priority-adjusted data without resource allocation.

Proposal 3: Enhancement for the first round resource allocation during LCP (e.g., resource allocation is not only based on Bj, but also consider the amount of UL delay-critical data) is not considered in Rel-19.

Reference

- RAN2#126 meeting, Report from session on R18 MBS, R18 QoE and R19 XR, Huawei