3GPP TSG-RAN WG2 Meeting #128                                                                    R2-2409856

Orlando, USA, 18th – 22nd Nov 2024

Source:	CATT

Title:	Consideration on DSR Enhancement

Agenda Item:	8.7.4.2

Document for:	Discussion and Decision

Introduction

In this contribution, we will further discuss the issues on DSR enhancement including the remaining time design, the case of non-delay critical data ahead of delay critical data, DSR MAC CE and the co-existing of new DSR and legacy DSR, our views are also shown in this paper.

Discussion

The enhanced DSR agreements in RAN2#127bis meeting report [1] are as follows, more details on the remaining issues are discussed in this section.

Agreements on DSR enhancements - We do not change the definition of delay-critical data - For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) - Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. - For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - Buffer size of data volume in each portion - Shortest remaining time among PDCP SDUs buffered in each portion. - There is no need to include PSI in the enhanced DSR MAC CE. - A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. - FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

The remaining time design

According to legacy description on DSR as below, the Remaining Time in DSR MAC CE is the shortest remaining time, anyway, this shortest remaining time is naturally less than the remainingTimeThreshold which is used to trigger the DSR.

TS 38.323 Delay-critical PDCP SDU: if pdu-SetDiscard is not configured, a PDCP SDU for which the remaining time till discardTimer expiry is less than the remainingTimeThreshold. If pdu-SetDiscard is configured, a PDCP SDU belonging to a PDU Set of which at least one PDCP SDU has the remaining time till discardTimer expiry less than the remainingTimeThreshold. TS 38.321 RRC controls the DSR procedure by configuring the following parameter: - remainingTimeThreshold (per LCG): the threshold on remaining time for triggering a DSR for a logical channel within an LCG. …. The fields in the DSR MAC CE are defined as follows: - LCGi: This field indicates the presence of delay information (i.e. the Remaining Time and Buffer Size fields) for the LCG i. The LCGi field set to 1 indicates that the delay information for the LCG i is reported. The LCGi field set to 0 indicates that the delay information for the LCG i is not reported; - Remaining Time: This field indicates the shortest remaining value of running PDCP discardTimer (described in clause 7.3 in TS 38.323 [4]) among all PDCP SDUs that are buffered for an LCG but have not been transmitted in any MAC PDU, at the time of the first symbol of the first PUSCH transmission that includes this DSR MAC CE. The length of this field is 6 bits. This field is present only if the buffer size indicated by the corresponding Buffer Size field is not zero; otherwise, this field is reserved and set to 0. If present, the value r in this field indicates a remaining time within the range of (r, r + 1] msec;

In R19 enhanced DSR, the basic and original purpose to enhance DSR is to give buffer size to a remaining time with finer granularity to help network allocating UL grant, so, for delay-critical data, it is reasonable to assume that the reporting threshold in enhanced DSR is also less than the triggering threshold (i.e., agreed as legacy remainingTimeThreshold) for enhanced DSR, otherwise, if the reporting threshold for a remaining time is higher than the triggering threshold, it is not clear how the network uses this buffer size information and its non-delay critical remaining time, its benefit is limited, and consequently, the PDCP and RLC layers have to define another procedure similar to legacy triggering threshold to notice MAC the data within this higher remaining time threshold is available.

On DSR cancellation, the legacy procedure is as follows:

TS 38.321 After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR.

Based on above legacy procedure, the DSR will be cancelled when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR. The legacy PDCP SDU associated with the DSR is delay-critical, so it is reasonable to cancel the DSR if all the associated PDCP SDUs are discarded or transmitted. However, if triggering threshold to value is lower than largest reporting threshold for new DSR, some non-delay critical data will be indicated in the new DSR. The legacy two conditions will not be applied or enhanced to give a clear and updated definition on all the the PDCP SDUs discarded or transmitted in the MAC PDU, more spec impact will be introduced. Considering the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, we prefer that the reporting thresholds are less than the triggering threshold.

Proposal 1: The reporting remaining time thresholds need to be less than the triggering threshold in enhanced DSR to offer finer granularity about the delay status.

In order to reduce the impact to network restriction, it is suggested to set the reporting thresholds as the percentage of triggering threshold.

Proposal 2: It is suggested to set the reporting threshold as the percentage of triggering threshold.

The issue on non-delay critical data ahead of delay critical data

In RAN2#127 meeting, some companies proposed that the non-delay critical data could be ahead of delay critical data in the buffer. If the UE needs to schedule the data in buffer in order, i.e., the non-delay critical data ahead of delay critical data should be scheduled firstly than the data after it, then to let network have a correct understanding to UE buffer, below two options could be considered:

- Option 1: the buffer size of non-delay critical data could be given separately from the buffer size of delay-critical buffer size in enhanced DSR, the order of non-delay critical/delay critical buffer size could follow the order of data in buffer size, besides, the remaining time is only reflected the remaining time for delay critical data rather than the non-delay critical data.

- Option 2: the buffer size of non-delay critical data could be added with the buffer size of associated delay critical data, the total buffer size will be reported to network, the remaining time is only reflected the remaining time for delay critical data rather than the non-delay critical data.

For above two options, option 1seesmly more clearly and could give network a full picture of buffer size for different delay status, but considering that the UE will not perform the intra-LCH prioritization, which means that the UE will put the data to the resource according the their order in the buffer, so it is not necessary to let network know how many non-delay critical data before the delay-critical data in buffer queue, anyway, the UE requires the network to give UL grant to schedule delay critical data together with the non-delay critical data ahead of as soon as possible,  so option 2 is preferable, option 2 will also spend less signaling overhead compared to option 1.

Proposal 3: For non-delay critical data ahead of delay critical data, the buffer size of non-delay critical data could be added with the buffer size of delay critical data, the total buffer size is reported to network, and the associated remaining time is computed based on the remaining time of delay critical data.

The DSR MAC CE

The legacy DSR MAC CE is as follows, it is not easy to extend legacy DSR to include buffer size information for different remaining time since the number of remaining time of each LCG may be various, it has been agreed to introduce a new MAC CE to enhance DSR. For the remaining time, the maximum time is 64ms for legacy discardTimer in ASN.1 coding, so the maximum reporting threshold is at least 64ms for enhanced DSR. Besides, it is suggested to set the number of reporting threshold to 2 or 4, the more reporting thresholds may be not needed for scheduling.

TS 38.321 figure 6.1.3.72-1: DSR MAC CE

Proposal 4: For the enhanced DSR MAC CE, the number of reporting thresholds for one LCG could be configured as 2 or 4.

In last meeting, one-bit indication in legacy Reserved bit is used to indicate whether a certain/further pair of remaining time information and buffer size information is present for the associated LCG. In data buffer, considering the data associated to a reporting threshold may be empty, it is preferable to introduce an indication to indicate whether a further pair of remaining time information and buffer size information is present, which could make the enhanced DSR report more flexible from the view of bits saving.

Proposal 5: The introduced indication is to indicate whether a further pair of remaining time information and buffer size information is present in the enhanced DSR MAC CE.

The co-existing of new DSR and legacy DSR

The legacy DSR is reported per LCG based on the remainingTimeThreshold configured per LCG, so the new DSR is also configured per LCG. It is possible that network could configure some LCG with legacy DSR inlcuding only one remaining time threshold and configure other LCG with new DSR including multiple reporting thresholds, which depends on the network requirement to the delay granularity for each LCG.

Proposal 6: It is network implementation to configure either legacy remainingTimeThreshold for legacy DSR or new reporting threshold(s) for new DSR to the same LCG.

For UE side, if only the LCG(s) configured with legacy remainingTimeThreshold triggers the delay status reporting, the legacy DSR MAC CE will be used. If the LCG(s) configured with legacy remainingTimeThreshold and the LCG(s) configured with the new reporting threshold(s) trigger the delay status report simultaneously, considering the new DSR could cover the report of only one pair of remaining time information and buffer size information for legacy DSR, it is preferable to use the new DSR for all the triggered LCH. In the new DSR, for the LCG configured with remainingTimeThreshold, the introduced indication will set based on new DSR rule, then network could interpret the new DSR with same options and get the delay status information for the LCG without any mistake.

Proposal 7: The UE will use new DSR in case that at least one LCG configured with new reporting threshold(s) needs to report the delay status.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: The reporting remaining time thresholds need to be less than the triggering threshold in enhanced DSR to offer finer granularity about the delay status.

Proposal 2: It is suggested to set the reporting threshold as the percentage of triggering threshold.

Proposal 3: For non-delay critical data ahead of delay critical data, the buffer size of non-delay critical data could be added with the buffer size of delay critical data, the total buffer size is reported to network, and the associated remaining time is computed based on the remaining time of delay critical data.

Proposal 4: For the enhanced DSR MAC CE, the number of reporting thresholds for one LCG could be configured as 2 or 4.

Proposal 5: The introduced indication is to indicate whether a further pair of remaining time information and buffer size information is present in the enhanced DSR MAC CE.

Proposal 6: It is network implementation to configure either legacy remainingTimeThreshold for legacy DSR or new reporting threshold(s) for new DSR to the same LCG.

Proposal 7: The UE will use new DSR in case that at least one LCG configured with new reporting threshold(s) needs to report the delay status.

Reference

- RAN2#127bis meeting, Report from session on R18 MBS, R18 QoE and R19 XR, Huawei