3GPP TSG-RAN WG2 Meeting #128                                                        R2-2409858

Orlando, USA, 18th – 22nd November 2024

Source:	CATT

Title:	Discussion on XR Rate Control

Agenda Item:	8.7.6

Document for:	Discussion and Decision

Introduction

In RAN2#127bis meeting, regarding to XR rate control, the below agreements were agreed [1]:

Agreements on XR rate control

- FFS if the indication is per DRB or per QoS flow. Companies should analyse the impact on QoS enforcement, interworking with L4S etc.

- RAN2 to consider the following approaches to provide recommended bit rate values better fitting XR applications:

- Extend the Bit Rate field

- Define a new bit rate table to provide sufficient granularity for XR traffic

- Introduce new values for the bitRateMultiplier

- Send LS to SA4 asking about range/granularity which is required

In this contribution, we focus on the following key issues on XR rate control:

- Whether the indication is per DRB or per QoS flow?

- Bit rate values indication enhancements.

- Whether bit rate query procedure is needed?

- Is there any enhancement needed for Uu uplink interface to let the gNB aware the UL congestion?

Discussion

Whether the indication is per DRB or per QoS flow

Following the guidance from the RAN2#127bis online discussion, we discuss this issue from below two aspects:

Interworking with L4S:

There are many discussions in RAN plenary for the relationship between L4S and RAN awareness based rate control. The common understanding is that the current L4S mechanism is capable of identifying congestion and implementing congestion control, which is applicable to both UE and tethered UE. As a general-purpose network traffic control technology, L4S typically operates between a pair of sending and receiving hosts (or end nodes). Given the variability in traffic patterns across hosts rather than being homogeneous, it is plausible that L4S might suffer from a too rough granularity. L4S can generally provide an IP layer granularity, as RFC 9330 shows:

L4S packet identifier (protocol): Once there are at least two treatments in the network, hosts need an identifier at the IP layer to distinguish which treatment they intend to use.

The key point is congestion control performed per QoS flow/DRB is superior to per IP/MAC address in providing an adequate response to these variations and ensuring effective network resource management. But there is no clear evidence shows that RAN must support per QoS flow indication is the best RAN awareness based rate control mechanism.

Observation 1: L4S can provide IP layer granularity while RAN awareness based rate control can provide finer granularity. There is no interworking issue with L4S and no evidence shows that per QoS flow indication in AS layer is necessary while using L4S.

QoS enforcement:

The proponent of QoS flow indication raised that gNB is responsible for QoS requirements enforcement so it should be up to NW which flow to throttle. But from gNB vender’s point of view, for legacy approach is using DRB level is more natural for gNB to control. gNB only cares about limiting the resources usage. Besides, MAC is not aware of QoS flows. The application will obviously control per flow. And how to map bitrate to apps should be up to UE. Besides, the need to introduce per QoS flow indication is not clear enough, hence, we propose:

Proposal 1: RAN2 doesn’t introduce per-QoS flow granularity indication in Rel-19 XR rate control.

Bit rate values indication enhancements

Considering the LS to SA4 [2] asking about range/granularity is on the air, the details about the approaches to provide recommended bit rate values better fitting XR applications should be waited until SA4’s reply.

Proposal 2: RAN2 waits for SA4 LS reply before further discussing bit rate values indication enhancements.

Whether bit rate query procedure is needed

The RAN-assisted codec adaptation is introduced during Rel-14. It provides a means for the eNB to send codec adaptation indication with recommended bit rate to assist the UE to select or adapt to a codec rate for MMTEL voice or MMTEL video. The RAN-assisted codec adaptation mechanism supports the uplink/downlink bit rate increase or decrease. The recommended bit rate for UL and DL is conveyed as a MAC CE from the eNB to the UE. The whole mechanism had been adopted for NR. Besides recommended bit rate MAC CE, recommended bit rate query MAC CE is also introduced. It can be considered as an enhancement to the recommended bit rate MAC CE. Based on the recommended bit rates received from the gNB, the UE may initiate an end-to-end rate adaptation with its peer UE. The peer UE may then send a query message to its local gNB to check if the recommended bit rate can be provided. This is to avoid ping pong in tuning where the increase of the bitrate in one end of the connection is immediately followed by a request from the other end to reduce the bit rate.  Furthermore, to limit UE sending frequent query messages, a prohibit timers can be configured by the gNB.

For Rel-19 XR rate control, it is just only related between the UE and gNB. Besides, the intention of introducing this new recommended bit rate MAC CE is to solve the UL congestion, the suggested bitrate from gNB to UE should be lower than normal transmission (non-UL congestion case). From this point of view, it is not convinced to introduce query message for Rel-19 XR. Further, no prohibit timer is needed either.

Proposal 3: Query message and related prohibit timer is not supported in Rel-19 XR rate control.

Is there any enhancement needed for Uu uplink interface to let the gNB aware the UL congestion

This issue had been discussed in Rel-18 XR[3], and concluded as below:

1.It is feasible for RAN to estimate the congestion information (e.g. traffic latency) per-QoS flow and per-DRB in downlink and uplink directions. 2.It is feasible for RAN to estimate the congestion information (e.g. traffic latency) per-QoS flow and per-DRB in uplink without UE impacts.

With the above green marked part, RAN2 can depend on legacy method to let the gNB aware the UL congestion is happening without any UE impacts no matter which granularity (per-QoS flow or per-DRB) is selected for the congestion awareness. In other words, any assistance information/trigger condition at UE/QoS flow to DRB mapping information is not needed in Rel-19 XR topic. Besides in the current WID, it is also limited to specify in MAC layer XR rate control signalling over downlink. Hence, it is obvious that there is no UE specification impact for assisting RAN estimates UL congestion per QoS flow and/or per DRB.

Observation 2: There is no UE specification impact for letting the gNB aware the UL congestion is happening.

Proposal 4: For XR uplink rate control, RAN2 does not introduce any UE specification impact for assisting RAN estimates UL congestion per QoS flow and/or per DRB.

Besides, RAN2 should confirm that with the new RAN rate recommendation message from the gNB to the UE, the gNB may provide the UE with up-to-date knowledge of the recommended bit rate. However, RAN2 will not specify the conditions and timing for when the gNB sends the new RAN rate recommendation message.

Proposal 5: For XR uplink rate control, RAN2 does not specify the conditions and timing for when the gNB sends the new RAN rate recommendation MAC CE.

Conclusion

According to the analysis in section 2, it is proposed:

Observation 1: L4S can provide IP layer granularity while RAN awareness based rate control can provide finer granularity. There is no interworking issue with L4S and no evidence shows that per QoS flow indication in AS layer is necessary while using L4S.

Proposal 1: RAN2 doesn’t introduce per-QoS flow granularity indication in Rel-19 XR rate control.

Proposal 2: RAN2 waits for SA4 LS reply before further discussing bit rate values indication enhancements.

Proposal 3: Query message and related prohibit timer is not supported in Rel-19 XR rate control.

Observation 2: There is no UE specification impact for letting the gNB aware the UL congestion is happening.

Proposal 4: For XR uplink rate control, RAN2 does not introduce any UE specification impact for assisting RAN estimates UL congestion per QoS flow and/or per DRB.

Proposal 5: For XR uplink rate control, RAN2 does not specify the conditions and timing for when the gNB sends the new RAN rate recommendation MAC CE.

Reference

- R2-2409215	 Report from session on R18 MBS, R18 QoE and R19 XR Session chair (Huawei)

- R2-2409390	 Reply on FS_XRM_Ph2	SA4	LS out	Rel-19	NR_XR_Ph3-Core

- R2-2212189  Reply LS to SA2 on XR, vivo, Huawei