3GPP TSG-RAN WG2 Meeting #128	R2-2409864

Orlando, USA, 18 - 22, Nov, 2024

Agenda Item:	8.1.2.2

Source:	Xiaomi

Title:	Discussion on LCM for UE-sided model for BM

Document for:	Discussion

Introduction

The following aspects are studied in LCM:

-	Data collection

-	Model training

-	Functionality/model identification

-	Model delivery/transfer

-	Model inference operation

-	Functionality/model selection, activation, deactivation, switching, and fallback operation.

-	Including: Decision by the network (either network initiated or UE-initiated and requested to the network), decision by the UE (event-triggered as configured by the network, UE’s decision reported to the network, or UE-autonomous either with UE’s decision reported to the network or without it)

-	Functionality/model monitoring

-	Model update

-	UE capability

In this contribution, we discuss the LCM for UE-sided model except for data collection and model transfer/delivery.

Discussion

Applicable functionality report

It’s aimed to merge the proactive and reactive report. Following signalling diagram is proposed,

Fig 1. Applicable functionality report flow

We understand NW may provide two types of NW side additional condition for different purpose in step 3,

Type 1: Current NW side additional condition, which reflects current NW status. UE use this to determine whether a functionality is applicable.

Type 2: NW side additional condition for filtering, which may not reflect current NW status. Because NW may only be able to change the NW side additional condition among limited states. Only the functionalities whose expected NW side additional condition is align with the limited states are useful to NW. It’s a waste of signalling for UE to report other functionalities. Because NW would never activate these functionalities, since their expected NW side additional condition would never deployed by NW. Such filtering is useful to avoid UE to report the functionalities which NW would never activate.

Proposal 1: In step 3, NW can provide NW side additional condition(s) for report filtering, which may not reflect current NW status. UE only report the functionalities whose expected NW side additional condition is matched with the indicated NW side additional conditions.

UE would check the functionality applicability. It’s agreed a functionality is applicable if all following conditions are met,

- NW side additional condition is met

- At least one model is available.

- UE side additional condition is met

Any condition not met would result in functionality not applicable. It’s not clear what is reported in step 4. At least UE shall report the applicable functionality. Furthermore, if a functionality is not applicable, it may be beneficial for NW to know the cause. Because NW may be able to enable the applicable functionality based on the cause to non-applicable. If NW side additional condition is not met, NW may change NW side additional condition. If model is not available, NW may transfer the model to UE, if capable, depending on the model delivery solution.

To facilitate the NW handling, UE shall report the cause of the functionality not applicable, i.e. NW side condition not met, UE side additional condition not met or no available model.

Proposal 2: In step 4, UE can report the non-applicable functionalities and the cause of non-applicable, i.e. NW side condition not met, UE side additional condition not met or no available model

In last meeting, it’s agreed UAI is supported and RRCReconfigurationComplete can be used to report applicable functionality. It’s not clear how to select the message during the report. One straightforward solution is left for UE to choose the message to report. Even if UE receives RRCReconfiguration message in step 3, UE shall also be allowed to use UAI to report applicable functionality. Because UE may need time to check the functionality applicability. The delay of RRCReconfigurationComplete message would impact other configurations.

Proposal 3: It’s up to UE to use UAI or RRCReconfiguraitonComplete message to report applicable functionality.

It’s agreed UE can trigger the report upon applicable AI functionality becomes non-applicable. It’s not clear whether this is only applied for activated functionality. If multiple functionalities are configured, the applicability change may occur frequently. It’s desired to avoid the frequent report. However, NW may switch the functionality, i.e. deactivate activated functionality and activate candidate functionality. So, NW may also need to know the applicability change of candidate functionality. The applicability change includes applicable to non-applicable or non-applicable to applicable. Therefore, UE shall also trigger the report upon candidate AI functionality applicability change.

Proposal 4：UE shall trigger the report upon activated functionality becomes non-applicable.

Proposal 5: UE shall trigger the report upon candidate functionality applicability change, i.e. applicable to non-applicable or non-applicable to applicable. The candidate functionality is configured by NW.

Data collection initiation

It’ agreed the data collection initiation and configuration for data collection is under network control. For UE side model, the data collection is triggered by UE or UE side server. The desired data may be associated with certain configuration, e.g. set A or set B. If the desired data can be collected in current configuration, UE can collect the data without the indication to NW. But if the desired data can’t be collected in current configuration, UE can send request NW to provide desired configuration. If NW can provide the desired configuration, UE can collect the data accordingly.

Proposal 6: For UE side data collection, UE can request NW to provide desired configuration for data collection if current configuration is not aligned with the desired configuration. Otherwise, UE doesn’t need to send request to NW.

Functionality identification

As first step, UE and NW need to perform functionality identification to reach common understanding on the functionality. During SI, it’s agreed each AI model is associated with meta data. It’s not clear whether functionality is associated with meta data. We understand the meta data is also needed for each functionality. Because NW/UE need to know some basic info about AI functionality to make LCM decision or determine whether AI functionality is applicable.

Observation 1: Meta data of functionality is essential for NW to make LCM decision and for UE to determine the applicable functionality.

Proposal 7: Each functionality is associated with meta data.

We would further discuss the content of meta data. NW Additional condition and UE side additional are essential for UE to determine the applicable functionality.  The UE side additional condition can be minimum required computing or storage capability.

Proposal 8: Meta info of AI functionality includes following info,

- Expected NW condition;

- Expected UE condition, e.g. Minimum required computing and storage capability.

In last meeting, it’s agreed that supported AI/ML-enabled Features/FGs and supported functionalities are included in UE capability. It’s not clear the granularity and number of functionalities. NW may not activate all supported AI functionalities. Similar issue exists in legacy capability report on band combination. NW can indicate the specific band/band combination and UE only reports the capability on corresponding band/band combination. We understand similar solution can be reused in AI capability report framework. The difference is AI functionality capability request may not per band/band combination. NW may request UE to report the capability of certain use case or additional condition. Existing message can be reused.

Proposal 9: NW can request UE to report supported AI functionality of certain use cases or additional condition to reduce the capability report signalling.

Conclusion

Based on the discussion in section 2, we have following proposals:

Applicable functionality report

Proposal 1: In step 3, NW can provide NW side additional condition(s) for report filtering, which may not reflect current NW status. UE only report the functionalities whose expected NW side additional condition is matched with the indicated NW side additional conditions.

Proposal 2: In step 4, UE can report the non-applicable functionalities and the cause of non-applicable, i.e. NW side condition not met, UE side additional condition not met or no available model

Proposal 3: It’s up to UE to use UAI or RRCReconfiguraitonComplete message to report applicable functionality.

Proposal 4：UE shall trigger the report upon activated functionality becomes non-applicable.

Proposal 5: UE shall trigger the report upon candidate functionality applicability change, i.e. applicable to non-applicable or non-applicable to applicable. The candidate functionality is configured by NW.

Data collection initiation

Proposal 6: For UE side data collection, UE can request NW to provide desired configuration for data collection if current configuration is not aligned with the desired configuration. Otherwise, UE doesn’t need to send request to NW.

Functionality identification

Observation 1: Meta data of functionality is essential for NW to make LCM decision and for UE to determine the applicable functionality.

Proposal 7: Each functionality is associated with meta data.

Proposal 8: Meta info of AI functionality includes following info,

- Expected NW condition;

- Expected UE condition, e.g. Minimum required computing and storage capability.

Proposal 9: NW can request UE to report supported AI functionality of certain use cases or additional condition to reduce the capability report signalling.

Reference

[1] R2-2311720	Reply LS on Data Collection Requirements and Assumptions
[2] TR 38.843