3GPP TSG-RAN WG2 Meeting #128	R2-2409882

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:		8.7.5

Source:		Samsung

Title:			RLC Enhancements for XR

Document for:	Discussion & Decision

# 1 Introduction

In this paper, we discuss RLC enhancements for XR viz. Timely RLC AM retransmissions and Unnecessary RLC AM retransmission avoidance.

# 2 Discussion

## 2.1 Timely RLC AM Retransmissions

In RAN2#127bis, following agreements were made:

Agreements on RLC timely retransmissions - RAN2 confirm that existing mechanisms are insufficient to resolve the timely RLC retransmission problem and RLC enhancements for timely RLC retransmission are investigated in Rel-19. - Exclude enhanced status reporting. - Focus the discussion on autonomous retransmission and polling enhancements, e.g. we need to understand how each option affects the capacity and packet delay

We further analyze the autonomous retransmisison and polling enhacements. Table I presents a comprehensive comparison of these two approaches to enable timely RLC AM retransmissions.

TABLE I

| Approach | Attribute / Condition | Pros | Cons |
| --- | --- | --- | --- |
| Autonomous Retransmissions | Blind retransmission | - Quick retransmission | - In-efficient and huge impact on capacity i.e. resource wastage |
|  | Upon failure of a number of HARQ transmissions | - Early determination & Local NACK | - Not straight-forward to map a RLC PDU and HARQ TB - Involves complicated and cumber-some cross-layer interactions - Large specification impact |
|  | Upon remaining time of the PDU becomes lesser than a threshold | - Pre-emptive retransmission | - Unnecessary - same can be achieved with a suitable configuration of polling timers |
|  | Upon availability of spare transmisison resources | - Opportunistic approach | - Not full-proof and non-deterministic solution |
| Enhanced polling | Dedicated polling thresholds and associated counters for delay critical RLC SDUs | - Polling enhancements for delay-critical SDUs - Differentiated treatment for delay critical SDUs - Standardized and regulated approach - Network configuration and control | - New configurations needed for delay critical but manageable |

Autonomous retransmission implies a transmitter can speed up retransmission without having to wait for receiving a status report from the receiver. Though the proposal looks attractive; however, there can be drastic impacts on the capacity as a large number of unnecessary and unregulated retransmissions could take place. As also described in Table I, it is not easy to regulate such autonomous transmissions based on specific conditions (e.g. HARQ transmisions, timers or spare transmission resources).

Observation 1: With autonomous retransmission, there can be drastic impacts on the capacity as a large number of unnecessary and unregulated retransmissions could take place. It is not easy to regulate such autonomous transmissions based on specific conditions (e.g. HARQ transmissions, timers or spare transmission resources).

Proposal 1: No enhancements are pursued for autonomous RLC AM retransmissions.

Further, we discuss the enhanced polling approach to facilitate timely RLC AM retransmissions.

As shown in Excerpt 1, there are three cases in legacy RLC AM to trigger a polling at the transmitting side:

- Case 1: the number of initially transmitted RLC SDUs or the bytes thereof, have been accumulated up to a certain thresholds (pollPDU or pollByte), respectively, whichever comes first

- Case 2: there is no data to (re)transmit

- Case 3: transmitting window stalling happens

Excerpt 1:

38.322 5.3.3 Polling ... Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall: - increment PDU_WITHOUT_POLL by one; - increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU; - if PDU_WITHOUT_POLL >= pollPDU; or - if BYTE_WITHOUT_POLL >= pollByte: - include a poll in the AMD PDU as described below. Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU; or - if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling); - include a poll in the AMD PDU as described below. ...

Among the three cases, a special care can be taken for Case 1, in reducing the polling latency, especially, when there are delay-critical RLC SDUs involved. E.g., dedicated thresholds (i.e., pollPDU and pollByte) for delay-critical RLC SDUs and the associated counters can be introduced, in order to reduce the polling latency for delay-critical RLC SDUs.

Observation 2: Differentiated treatment for delay-critical RLC SDUs can be achieved with introducing dedicated polling thresholds and the accosiated counters for delay-critical RLC SDUs. The approach also enables standardized and regulated means with network based configuration and control (unlike autonomous retransmissions).

Proposal 2: Dedicated polling thresholds and the associated counters are introduced for delay-critical RLC SDUs.

## 2.2 Unnecessary RLC AM Retransmission Avoidance

In RAN2#127bis, following agreements were made:

Agreements on avoiding unnecessary retransmissions - RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does. - RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified - FFS if some C-PDU handling is needed to avoid C-PDU discard - FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed)

Since RLC AM is designed for loss-less delivery, legacy RLC AM will not work if discarding an RLC SDU already submitted to lower layer, causes RLC SN gap, i.e., window stalling will occur at both Tx and Rx. In this case, Rx needs to detect the SDU abandonment and such detection would likely be delayed as it would rely on a local timer expiry and received SN gap occurrence.

To overcome these drawbacks and provide an effective “combined” approach, RAN2 should consider introducing an indication from Tx RLC to inform Rx RLC about the stop of the outdated SDU transmission. This can also help avoid the C-PDU discard issue at the Rx RLC.

Proposal 3: For the combined approach, adopt an indication from the Tx RLC to the Rx RLC about the stop of the outdated SDU transmission.

Now, for the indication from Tx RLC to Rx RLC, following options are considerable

- RLC control PDU carrying information about outdated SDU(s). It would be feasible and simple to organize this information as first outdated SDU SN and an optional bitmap for following SDUs.

- RLC data PDU header carrying information or just an indication about preceding outdated SDU(s).

Proposal 4: RAN2 to discuss and decide among the following options for an indication from Tx RLC to Rx RLC about stopping transmission of outdated SDU.

- RLC control PDU carrying information about outdated SDU(s) e.g. first outdated SDU SN and an optional bitmap for following SDUs.

- RLC data PDU header carrying information or indication about preceding outdated SDU(s).

# 3 Conclusion

Based on the discussion above, RAN2 is requested to consider the following observations and proposals:

Observation 1: With autonomous retransmission, there can be drastic impacts on the capacity as a large number of unnecessary and unregulated retransmissions could take place. It is not easy to regulate such autonomous transmissions based on specific conditions (e.g. HARQ transmissions, timers or spare transmission resources).

Proposal 1: No enhancements are pursued for autonomous RLC AM retransmissions.

Observation 2: Differentiated treatment for delay-critical RLC SDUs can be achieved with introducing dedicated polling thresholds and the accosiated counters for delay-critical RLC SDUs. The approach also enables standardized and regulated means with network based configuration and control (unlike autonomous retransmissions).

Proposal 2: Dedicated polling thresholds and the associated counters are introduced for delay-critical RLC SDUs.

Proposal 3: For the combined approach, adopt an indication from the Tx RLC to the Rx RLC about the stop of the outdated SDU transmission.

Proposal 4: RAN2 to discuss and decide among the following options for an indication from Tx RLC to Rx RLC about stopping transmission of outdated SDU.

- RLC control PDU carrying information about outdated SDU(s) e.g. first outdated SDU SN and an optional bitmap for following SDUs.

- RLC data PDU header carrying information or indication about preceding outdated SDU(s).