3GPP TSG-RAN WG2 Meeting #128                                     R2-2409892

Orlando, USA, Nov. 18th -22nd, 2024

Source: 			OPPO

Title: 	Discussion on paging procedure for Ambient IoT

Agenda item:		8.2.3

Document for: 	Discussion and Decision

- Introduction

In the last RAN2#127-b meeting, regarding paging procedure for Ambient IOT, following agreement has been achieved.

Agreements:

1 The A-IOT paging message can include information to avoid duplicate response from the device to a reader. This information should be short and simple. FFS how to indicate. Wait for further information and requirements from other WGs and make this decision in normative phase. Based on this information the device determines whether to skip sending the response to paging.

2. From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs. RAN2 will determine the need for this after considering aspects related to RAN2 multiplexing and other WGs.

3. If multiple devices ID in single paging is supported, if A-IOT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA

4. For all use cases, device determines the random access type from the paging message. FFS whether it is explicitly or implicitly. FFS whether we will down selection for all RA types (2step, 3step, CFRA) or try to unify the design for 2step, 3step

In this contribution, we aim to provide our views on the paging procedure for the A-IOT system.

2. Discussion

- 2.1 avoiding duplicate response from devices

There are two scenarios that multiple/subsequent A-IOT paging messages are associated with the same request from the CN:

- The same CN request is sent towards more than one reader for enhancing the possibility of reaching the target A-IOT devices

- The reader sends more than one initial A-IOT paging messages at different time to avoid the case that A-IOT devices miss a particular paging message due to low battery level.

In order to let A-IOT device do not respond to the repeated paging messages to save energy and radio resource, an ID identifying the CN request may be needed to be included in the paging message. Some companies claimed that the terminology paging area could be defined, and the device will only respond to the A-IOT paging message originated from the current camping paging area. But this solution could only deal with the first bullet. The A-IOT device still needs to respond to consecutive paging messages from the same reader corresponding to the same CN request. In addition, the A-IOT device needs to be aware of its current paging area information, which complicates the A-IOT device implementation. As a result, we propose to include an ID identifying the CN request in the A-IOT paging message.

Observation 1: repeated A-IOT paging messages corresponding to the same CN request are possible in two scenarios. Firstly, the CN request is sent towards more than one reader for enhancing the coverage of CN request. Secondly, A-IOT paging messages are sent consecutively to avoid the case that A-IOT devices miss a particular paging message due to low battery level.

Proposal 1: include an ID identifying the CN request in the A-IOT paging message to avoid A-IOT device making duplicated responding to A-IOT paging messages corresponding to the same CN request.

- 2.2 How the network configures the RA type

- 2.2.1 Contention-based or contention-free?

It should be noted that there should one-to-one mapping relationship between each A-IOT device and access occasion, when multiple device IDs or group/mask ID is included in the paging message. The access occasion order could be just determined by the order of the device IDs in the message or the ascending/descending order when the mask ID is applied. On the other hand, for the contention-based RACH procedure, the access occasions allocated to the targeted devices may not be sufficient, there are collisions between different devices for performing accessing towards the network. In such cases, a such parameter, functioning as the Q to randomize the slot counter for the RFID system, should be included in paging message to let the targeted A-IOT devices to randomize the access occasion indices. It can be concluded that absence or presence of such parameter implies that it is contention-free or contention-based RACH procedure being configured by the network, which is aligned with the agreement we made in RAN2#126 meeting that AIOT paging message indicates information from which the device can determine resources to be used for response (D2R message).

Proposal 2: RAN2 to agree that presence (absence) of the parameter for randomizing the devices’ access indices implies the contention-based (-free) RACH procedure

- 2.2.2 3-step or 2-step?

For the 2-step RACH procedure, only the device generated RN16 needs to be transmitted towards the network. But for the 3-step RACH procedure, the A-IOT device needs to transmit data/upper layer ID towards the network also. Therefore, the TB size allocated to devices for msg1 transmission in these two scenarios are totally different, which could be leveraged by A-IOT devices to differentiate whether to apply the 2-step or 3-step RACH procedure.

Observation 2: the TB size allocated to devices for msg1 transmission in these two scenarios are totally different, which could be leveraged by A-IOT devices to differentiate whether to apply the 2-step or 3-step RACH procedure.

Proposal 3: RAN2 to agree the device could determine to apply 2-step or 3-step RACH procedure by investigating the TBS allocated to the msg1 transmission.

- 2.3 Resource allocated for msg1 transmission

- 2.3.1 paging message containing an ID of a single A-IOT device

In such case, the paged A-IOT device could access to the network immediately, after reception of the A-IOT paging message though subject to the restriction of any potential message processing delay. The paging message should just indicate the D2R message scheduling information, i.e., the time-frequency radio resource for the A-IOT device to send msg1, considering that RAN1 is discussing multiple-access scheme for the A-IOT device.

Observation 2: when receiving of a A-IOT paging message containing only one ID of a single A-IOT device, the A-IOT device could access to the network immediately, subject to the restriction of any potential message processing delay.

Proposal 4: to trigger one-device RACH procedure, A-IOT paging message should indicate the D2R message scheduling information for msg1 transmission.

In addition, since only one device is paged, the access occasion allocated should be dedicated to this device, and therefore CFRA procedure shall be applied.

Proposal 5: to confirm that when only one device ID is included in the A-IOT paging message, CFRA procedure shall be applied.

- 2.3.2 paging message containing more than one A-IOT device IDs

When more than one A-IOT devices are included in the A-IOT paging message, though the number of access occasions could be less than the number of A-IOT devices, still one-to-one mapping between the accession occasion and D2R resource should be ensured.

In RAN1, D2R scheduling information is being discussed. Generally, it consists of time domain resources and frequency domain resources, i.e., both TDM and FDM are in consideration, as indicated as follows:

RAN1#118 Agreement

For D2R scheduling, the following information potentially can be explicitly/implicitly indicated to the device via corresponding PRDCH:

-	Time domain resources

-	Frequency domain resources

-	MCS-like information

-	Chip duration

-	ID associated with device(s)

-	Repetitions

FFS: other information

FFS: For each information, whether higher-layer signaling and/or L1 R2D control signaling is used

As a result, we think that it is necessary to indicate the mapping relationship between each access occasion and the time-frequency resource in the A-IOT paging message, for the convenience of the A-IOT to know in which time-frequency resource to perform msg1 transmission. An example is provided as follows:

Figure 1: an example of mapping relationship between access occasion and time-frequency resource

Proposal 6: include the mapping relationship between the time-frequency resources and access occasions in the A-IOT initial paging message, of which details depends on RAN1 further progress.

- 2.4 How to let the A-IOT device know its access occasion index when multi-device CFRA is applied?

To trigger multi-device CFRA procedure, A-IOT paging message should also indicate the mapping relationship between device IDs and access occasion indices, and accordingly the device could find the access occasion corresponding to its ID.

If a group/mask ID is used to trigger multiple A-IOT devices to perform CFRA procedure, then a targeted device’s access occasion index could be determined by the position of its ID relative to the group/mask ID. However, if the targeted devices are not in one group, in other words, individual device IDs need to be included in the A-IOT paging message, then one-to-one mapping between device ID and access occasion index may be needed to be included in the A-IOT paging message.

Observation 3: If a group/mask ID is used to trigger multiple A-IOT devices to perform CFRA procedure, then a targeted device’s access occasion index could be determined by the order of its ID relative to the group/mask ID included in the A-IOT paging message.

One exampling implementation is indicated in the following figure, where the access occasion of each ID is indicated explicitly. For this kind of implementation, of course, large overhead is needed for the paging message to indicate the access occasion explicitly for each device ID. On the other hand, some may argue that the access occasion of one targeted device ID could be determined by the device ID position in the list. For example, ID x in the figure 3 is in the beginning of the list, and therefore could be allocated with the first access occasion. But in practice, an A-IOT device may miss some segments of the R2D paging message and therefore could not obtain the complete ID list and cannot derive the access occasion.

Observation 4: additional space in the A-IOT paging message is needed to accommodate the access occasion index explicitly for each targeted device ID, if the access occasion of each individual device ID is required to be included in the message.

Observation 5: determination of the access occasion of one targeted A-IOT device from the device ID position/order in the targeted device ID list may suffer from the problem of missing some paging message segments including device ID information over the Au air interface.

Figure 2: A-IOT device needs to scan the device ID list included in the paging message, until it finds its own device to know its access occasion index

Another way is to consider using the PF/PO-like solution to allocate the access occasion for each device ID. A targeted A-IOT device is distributed to the initial access occasion matched with the remainder of {the device ID MOD a new parameter defining the scale of the access occasions in one period}. In such way, the collision is restricted to A-IOT devices sharing the same ID modulo operation result. Hence, the device shall only need to find its order in the group of A-IOT devices with same modulo operation result and accordingly adjust its modulo operation result somehow to derive its true access occasion.

An example is provided as follows. If the divisor is set to 10, A-IOT devices ID modulo results will be distributed in the range of [0,9]. Assuming that devices with ID 1, 11, 21 are the target devices, their ID modulo results are the same. To avoid collision, each of them needs to adjust the result with a different offset quantity, respectively. Such offset could be derived from the knowledge of accumulated offset and the order of each device within the list sharing the same modulo result. Hence, the ID 1, 11, 21 will be distributed to the 1st, 2nd, and 3rd access occasion, respectively. Subsequently, the reader could update the accumulated offset to 2, considering the ID 11 and 22 occupy the 2rd and 3rd accession occasion. Accordingly, the devices with ID 2 and 12, will be distributed to the 4th to 5th access occasion, and so on. The overall procedure is illustrated in the following figure.

Figure 1: using PF/PO similar approach to compute the access occasion indexes for the target A-IOT devices

With this method, the A-IOT device only needs to know the list of device IDs sharing the same modulo result with itself, and the scale of the access occasions in one period, the devisor for the modulo operation, from the initial A-IOT paging message. Then, according to the accumulated offset updated by the reader’s subsequent R2D message, an A-IOT device could determine its access occasion. It could be obviously found that only part of device ID list is required at A-IOT device with this method, which could mitigate the need of successful reception of complete device ID list included in the A-IOT initial paging message.

Observation 6: when the PF/PO-like access index allocation approach is applied, the A-IOT device only needs to know the list of device IDs sharing the same modulo result with itself, and the number of the access occasions in one period (the devisor for the modulo operation) from the initial A-IOT paging message.

Proposal 7: to trigger A-IOT devices to find their correct access occasions in A-IOT CFRA procedure, three options of different kind of information can be discussed to be embedded in the A-IOT paging message:

- One-to-one mapping relationship between the A-IOT device ID and the access occasion index.

- Mask ID defining the group of targeted A-IOT devices and the ID for the initial access occasion and indication of the ascending or descending order of the IDs for the subsequent access occasions

- modulo operation related parameters such as the divisor and lists of targeted A-IOT devices.

- Conclusion and proposals

In the contribution, following proposals and observations are made:

Avoiding duplicate response from devices:

Observation 1: repeated A-IOT paging messages corresponding to the same CN request are possible in two scenarios. Firstly, the CN request is sent towards more than one reader for enhancing the coverage of CN request. Secondly, A-IOT paging messages are sent consecutively to avoid the case that A-IOT devices miss a particular paging message due to low battery level.

Proposal 1: include an ID identifying the CN request in the A-IOT paging message to avoid A-IOT device making duplicated responding to A-IOT paging messages corresponding to the same CN request.

How the network configures the RA type:

Proposal 2: RAN2 to agree that presence (absence) of the parameter for randomizing the devices’ access indices implies the contention-based (-free) RACH procedure.

Observation 2: the TB size allocated to devices for msg1 transmission in these two scenarios are totally different, which could be leveraged by A-IOT devices to differentiate whether to apply the 2-step or 3-step RACH procedure.

Proposal 3: RAN2 to agree the device could determine to apply 2-step or 3-step RACH procedure by investigating the TBS allocated to the msg1 transmission.

Resource allocated for msg1 transmission:

Observation 2: when receiving of a A-IOT paging message containing only one ID of a single A-IOT device, the A-IOT device could access to the network immediately, subject to the restriction of any potential message processing delay.

Proposal 4: to trigger one-device RACH procedure, A-IOT paging message should indicate the D2R message scheduling information for msg1 transmission.

Proposal 5: to confirm that when only one device ID is included in the A-IOT paging message, CFRA procedure shall be applied.

Proposal 6: include the mapping relationship between the time-frequency resources and access occasions in the A-IOT initial paging message, of which details depends on RAN1 further progress.

How to let the A-IOT device know its access occasion index when multi-device CFRA is applied:

Observation 3: If a group/mask ID is used to trigger multiple A-IOT devices to perform CFRA procedure, then a targeted device’s access occasion index could be determined by the order of its ID relative to the group/mask ID included in the A-IOT paging message.

Observation 4: additional space in the A-IOT paging message is needed to accommodate the access occasion index explicitly for each targeted device ID, if the access occasion of each individual device ID is required to be included in the message.

Observation 5: determination of the access occasion of one targeted A-IOT device from the device ID position/order in the targeted device ID list may suffer from the problem of missing some paging message segments including device ID information over the Au air interface.

Observation 6: when the PF/PO-like access index allocation approach is applied, the A-IOT device only needs to know the list of device IDs sharing the same modulo result with itself, and the number of the access occasions in one period (the devisor for the modulo operation) from the initial A-IOT paging message.

Proposal 7: to trigger A-IOT devices to find their correct access occasions in A-IOT CFRA procedure, three options of different kind of information can be discussed to be embedded in the A-IOT paging message:

- One-to-one mapping relationship between the A-IOT device ID and the access occasion index.

- Mask ID defining the group of targeted A-IOT devices and the ID for the initial access occasion and indication of the ascending or descending order of the IDs for the subsequent access occasions

- modulo operation related parameters such as the divisor and lists of targeted A-IOT devices.