3GPP TSG-RAN WG2 Meeting #128                                R2-2409893

Orlando, USA, 18th – 22rd November, 2024

Source: 			OPPO

Title: 	Discussion on providing MBS service area in NTN network

Agenda item:		8.8.4

Document for: 	Discussion and Decision

- Introduction

Agreements:

- For each MBS service we include one or more intended service area IDs into MCCH. FFS whether the list of the intended service areas (and related IDs) is also included in MCCH or if it is provided in a new or existing SIB. We will consider possible enhancements (including enhancements left up to UE implementation) to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service.

In the last RAN2#128 meeting, agreements on the support of broadcast service in the NTN network have been achieved as follows:

In this paper, we would like to further present our opinions on the topic.

- Discussion

- 2.1 how to indicate the service area information in the Uu signalling?

Firstly, in the last RAN2 meeting, we observe that the current SIB20 may not be large enough to accommodate the complete intended area information without segmentation. Therefore, to avoid complexity, we need to exclude this option. In addition, if the intended service area is included in the new existing SIB, the UE needs to read from both of the MCCH and the SIB for obtaining the complete information of the intended service area for each MBS service. To achieve this, the UE needs to monitor the scheduled information of both the SIB20 and new SIB to read the MCCH scheduling information and intended service area information. If they are scheduled differently in time domain to some extent, large latency is expected for the UE to obtained the complete area information, which is unnecessary at all. As a result, we propose to include complete intended service area information in the MCCH.

Observation 1: if the intended service area is included in the new existing SIB, the UE needs to read from both of the MCCH and the SIB for obtaining the complete information of the intended service area for each MBS service, which is not efficient.

Proposal 1: include the list of the intended service areas also in the MCCH.

- 2.2 Avoiding UE MCCH re-acquisition when unnecessary

When the UE firstly camps on the NTN cell, the UE could obtain the intended service area related information. Then if the UE is interested in MBS service but outside the intended area, the UE could just skip MCCH re-acquisition procedure, if the intended service area is found to be not updated or at least not updated regularly. But if the intended service area is updated quite often, the UE may need to update the intended service area accordingly. As a result, we think it is Ok to leave to UE implementation to decide whether or not to re-acquire the MCCH information. For example, if the UE finds that the intended area information is not updated after it finishes re-acquisition several times, then the UE could start skipping the MCCH re-acquisition for quite a long time. Anyway, it could depend on the UE decision.

Observation 2: if the UE could avoid MCCH re-acquisition depends on whether or not the intended service area is refined or if it is refined quite often.

Proposal 2: leave to UE implementation to decide whether or not to re-acquire the MCCH information when outside the intended service area.

- 2.3 Service continuity

In the TS 38.331, SIB21 contains information of the MBS Frequency Selection Area Identities for both of the current and/or neighbouring carrier frequencies. According to TS 23.247, the MBS FSA ID is used for broadcast MBS session to guide the frequency selection of the UE, and the MBS FSA ID(s) of a MBS session will be communicated in the service announcement towards the UE. In this way, the UE could prioritize the frequency on which its interested MBS services is provided when performing the cell reselection, and can continue the MBS service when it leaves the current cell. For the NTN cell, such mechanism could be reused.

Observation 3: the legacy mechanism of prioritizing the frequency on which its interested MBS services is provided when performing the cell reselection could be reused in the NTN cell.

However, the reason why the intended service area within each neighbour cell is provided for each MBS service session in MBS broadcast service session, or for each FSAI in the SIB21 is not clear. Assuming the coverage of two neighbour cells are overlapped, only if the intended service areas for the interested MBS service are set different in their coverage, UE should consider to prioritize reselect to the cell whose intended service area is expected to traversed by the UE. On example is given in the Figure 1, the intended service area for the interested MBS service of the neighbour cell #1 is closer to the UE, so the UE should prioritize to reselect to the neighbour cell #1. However, we doubt the scenario why the intended service area of two overlapped neighbour cells are set differently. In the Figure 1, the reasonable setting is to let the neighbour cell #2 also provide the MBS service in the intended services area highlighted will green colour. As a result, we propose not to consider to indicate the information of the intended service area of the neighbour cell in either the MBS configuration or the SIB21

Observation 4: it is doubtful why the intended service area of two overlapped neighbour cells are set differently, which force the UE to prioritize the one whose intended service area is closer to the UE.

Proposal 3: RAN2 to not to consider to indicate the information of the intended service area of the neighbour cell in either the MBS configuration or the SIB21.

Figure 1: the intended service areas for one MBS is different for two neighbour cells

- 2.4 MBS interest indication

According to TS 38.331, the purpose of this procedure is to inform the network that the UE in RRC_Connected is receiving or is interested to receive MBS broadcast service(s) and/or to inform the network about the priority of MBS broadcast versuse unicast and multicast MRB reception. Even if the UE is not entering any intended service area, the UE could still perform the MBS interest indication procedure towards the network. If the network would like to provide the MBS service the UE no matter where the UE is, the network could just send the MBS content towards the UE via unicast. Otherwise, if the network would like to provide the service only if the UE is in the intended service area, the network could trigger the localization towards the UE to check whether or not the UE is in the intended service area. Also, the network could trigger the MBS service broadcast if it knows that there is UE interested in the MBS service. As a result, we think the legacy mechanism of MBS interest indication could be followed by the NTN cell, and no other additional enhancement such as UE indicates the MBS interest indication only when the UE enters one intended service area is needed.

Observation 5: after the UE sends the MBS interest indication, if the network would like to provide the MBS service towards the UE no matter where the UE is, the network could just send the MBS content towards the UE via unicast; Otherwise, if the network would like to provide the service only if the UE is in the intended service area, the network could trigger the localization towards the UE to check whether or not the UE is in the intended service area. Also, the network could trigger the MBS service broadcast if it knows that there is UE being interested in particular MBS services.

Proposal 4:  not to introduce any additional restriction on when to initialize the MBS interest indication on the top of the legacy UE behaviour.

- Conclusion and proposals

In this paper, following observations and proposals have been made by us:

Observation 1: if the intended service area is included in the new existing SIB, the UE needs to read from both of the MCCH and the SIB for obtaining the complete information of the intended service area for each MBS service, which is not efficient.

Proposal 1: include the list of the intended service areas also in the MCCH.

Observation 2: if the UE could avoid MCCH re-acquisition depends on whether or not the intended service area is refined or if it is refined quite often.

Proposal 2: leave to UE implementation to decide whether or not to re-acquire the MCCH information when outside the intended service area.

Observation 3: the legacy mechanism of prioritizing the frequency on which its interested MBS services is provided when performing the cell reselection could be reused in the NTN cell.

Observation 4: it is doubtful why the intended service area of two overlapped neighbour cells are set differently, which force the UE to prioritize the one whose intended service area is closer to the UE.

Proposal 3: RAN2 to not to consider to indicate the information of the intended service area of the neighbour cell in either the MBS configuration or the SIB21.

Observation 5: after the UE sends the MBS interest indication, if the network would like to provide the MBS service towards the UE no matter where the UE is, the network could just send the MBS content towards the UE via unicast; Otherwise, if the network would like to provide the service only if the UE is in the intended service area, the network could trigger the localization towards the UE to check whether or not the UE is in the intended service area. Also, the network could trigger the MBS service broadcast if it knows that there is UE being interested in particular MBS services.

Proposal 4: not to introduce any additional restriction on when to initialize the MBS interest indication on the top of the legacy UE behaviour.