3GPP TSG-RAN WG2 Meeting #128                                     R2-2409896

Orlando, USA, Nov. 18th -22nd, 2024

Source: 			OPPO

Title: 	Discussion on supporting PWS in IOT-NTN network

Agenda item:	8.9.4

Document for: 	Discussion and Decision

- Introduction

In the last RAN2#127-bis meeting, agreements on the support of PWS in the IOT-NTN network have been achieved as follows:

Agreements:

- RAN2 confirms the understanding that this WID objective covers all PWS services, including ETWS and CMAS.

- The support for PWS introduced for NB-IOT NTN can be made applicable for NB-IOT in TN, if this does not require additional NB-IOT TN specific changes.

- For NB-IOT NTN PWS, introduce the following new SIBs, taking the content within the corresponding LTE SIBs as a baseline (but also checking whether we can have some optimization/skip some unnecessary fieleds):

  - SystemInformationBlockType10-NB for primary ETWS notification;

  - SystemInforamtionBlockType11-NB for secondary ETWS notification;

  - SystemInformationBlockType12-NB for CMAS notification

- Add the following PWS indication in direct indication information for NB-IOT (Come back in the next meeting on whether this is also added in Paging-NB):

  - Etws-indication

  - cmas-indication

- Upon receiving the PWS notification from NB-IOT cell, the PWS-capable NB-IOT UE acquires the corresponding PWS message immediately (can come back to clarify further what immediately means)

- Clarify in the spec (and at least in 36.300) that for NB-IOT in the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g., when the UE is in eDRX (can come back to further clarify the specific cases).

In this paper, we would like to further present our opinions on the supporting PWS in IOT-NTN network.

- Discussion

- 2.1 Adding PWS indication in paging message

In the last RAN2#127-b meeting, it was agreed to add the PWS indication in direct indication information for NB-IOT. However, in the current TS 38.331, for the normal UE, paging message also include etws-Indication and cms-Indication IE, as indicated as follows:

Paging ::=					SEQUENCE {

pagingRecordList				PagingRecordList					OPTIONAL,	-- Need ON

systemInfoModification			ENUMERATED {true}					OPTIONAL,	-- Need ON

etws-Indication					ENUMERATED {true}					OPTIONAL,	-- Need ON

nonCriticalExtension			Paging-v890-IEs						OPTIONAL

}

Paging-v890-IEs ::=			SEQUENCE {

lateNonCriticalExtension		OCTET STRING						OPTIONAL,

nonCriticalExtension			Paging-v920-IEs						OPTIONAL

}

Paging-v920-IEs ::=			SEQUENCE {

cmas-Indication-r9				ENUMERATED {true}					OPTIONAL,	-- Need ON

nonCriticalExtension			Paging-v1130-IEs					OPTIONAL

}

We think the mechanism for the normal LTE UE should be followed by the NB-IOT also, and hence propose to include in etws-Indication and cmas-Indication in the paging-NB message.

Proposal 1: adding etws-Indication and cmas-Indication in the Paging-NB message.

- 2.2 UE behaviour when receiving PWS notification

In the TS 36.331, it is clearly stated that change of system information only occurs at specific radio frames, i.e., the concept of a modification period is used, other than for ETWS and CMAS. So it is obviously that when emergency event occurs, the event should be reflected in the PWS related SIBs immediately, i.e., the UE do not need to wait for the next SIB modification period coming. We think that the intention is to minimize the time between the rise of an emergency event and the user recognizing the event to let the user take necessary actions as soon as possible. As a result, we think that for NB-IOT UE, such principle should be followed.

Observation 1: the intention of change of PWS-related system information could occur at any radio frame, i.e., the concept of modification period is not applied, is to minimize the time between the rise of an emergency event and the user recognizing the event to let the user take necessary actions as soon as possible.

Proposal 2: when receiving the PWS notification from the network, the NB-IOT UE should acquire the PWS message immediately

- 2.3 Introduction of new capability

Currently, in the TS 36.306, two capabilities regarding support of the ETWS and CMAS are defined as follows:

- ETWS

It is optional for UE to support ETWS reception as specified in TS 36.331 [5].

- CMAS

It is optional for UE to support CMAS reception as specified in TS 36.331 [5]. It is optional for a CMAS-capable UE to support Geofencing information (warningAreaCoordinates-r15) as specified in TS 36.331 [5].

We think the same capabilities should be introduced for NB-IOT UE also.

Proposal 3: introduce capabilities of support of ETWS and CMS services for the NB-IOT UE.

- Conclusion and proposals

In this paper, following observations and proposals have been made by us:

Proposal 1: adding etws-Indication and cmas-Indication in the Paging-NB message.

Observation 1: the intention of change of PWS-related system information could occur at any radio frame, i.e., the concept of modification period is not applied, is to minimize the time between the rise of an emergency event and the user recognizing the event to let the user take necessary actions as soon as possible.

Proposal 2: when receiving the PWS notification from the network, the NB-IOT UE should acquire the PWS message immediately

Proposal 3: introduce capabilities of support of ETWS and CMS services for the NB-IOT UE.