3GPP TSG-RAN WG2 Meeting #128	 R2-2409903

Orlando, USA, Nov 18-22, 2024

Agenda Item:	8.4.3

Source: 	NEC

Title:  	LP-WUS measurement relaxation and offloading

Document for:	Discussion and Decision

- 1 Introduction

In last RAN2#127b meeting, the following agreements are made:

- Working assumption

For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’.

FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

In this contribution, we further discuss LP-WUS specific measurement relaxation and measurement offloading.

- 2 Discussion

- 2.1 Background

Based on RAN4 latest agreement, only case#1 and case#3 are agreed and discussed:

Background:

Case #1 and #3 have been agreed

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

Based on RAN4 understanding, during the UE is monitoring LP-WUS, two RRM measurement cases should be considered, case#1 is MR measurement offloading including serving cell and/or neighbour cell while case#3 is MR measurement relaxation including both serving cell and neighbour cell. With this in mind, we need to further discuss the details about MR measurement offloading and MR measurement relaxation for both serving cell and neighbour cell.

- 2.2 Measurement relaxation

Neighbour cell MR measurement relaxation

RAN2 has agreed that not-at-cell-edge is reused for neighbour cell measurement relaxation, based on this understanding, we need to further discuss the relationship between this measurement relaxation criterion and entry/exit condition of LP-WUS monitoring.

There are two scenarios:

Scenario-1: the threshold of not-at-cell-edge is lower than entry condition of LP-WUS monitoring

Scenario-2: the threshold of not-at-cell-edge is greater than entry condition of LP-WUS monitoring

Note that in this contribution, we use “LP-WUS specific measurement relaxation” to stand for the new introduced relaxation by LP-WUS feature, to easily differentiate from the legacy measurement relaxation.

Fig.1 relationship between not-at-cell-edge and entry condition

For scenario-1 (i.e., not-at-cell-edge is lower than entry condition), it means the LP-WUS capable UE can start LP-WUS specific neighbour cell MR measurement relaxation without entering LP-WUS monitoring mode of which the UE behaviour is just similar to the legacy Rel-16 measurement relaxation. But based on discussion, companies may think the LP-WUS specific neighbour cell MR measurement relaxation should be performed more relaxed, i.e., the relaxed factor K should be more than the legacy value (i.e., K=3). We understand it may be feasible but it is related to RAN4 requirement, i.e., a further neighbour cell measurement relaxation (e.g., K > 3) will be introduced for the UE just fulfilling not-at-cell-edge but without entering LP-WUS monitoring.

Observation-1: for scenario-1 (i.e., not-at-cell-edge is lower than entry condition), the non-LP-WUS monitoring UE may perform further relaxed neighbour cell measurement (i.e., K >3) without entering LP-WUS monitoring mode (compared with legacy Rel-16 measurement relaxation).

Proposal-1: for the scenario-1 (i.e., the threshold of not-at-cell-edge is lower than that one of LP-WUS monitoring entry condition), RAN2 to confirm that it is feasible that the UE may perform LP-WUS specific measurement relaxation without entering LP-WUS monitoring mode, as like Rel-16 measurement relaxation behaviour but maybe with further relaxed factor which is determined by RAN4.

For scenario-2 (i.e., not-at-cell-edge is greater than entry condition), it means the LP-WUS monitoring UE may have not started LP-WUS specific neighbour cell MR measurement relaxation. To our understanding, the most important thing is that as long as the UE is monitoring LP-WUS, the MR transition should not be performed frequently (otherwise, power saving is lost). In this scenario, based on RAN4 requirement (taking intra-frequency as example), the LP-WUS monitoring UE shall perform “normal” neighbour cell MR measurement at least every Tmeasure,NR_Intra and the typical value is 1280ms. Considering the MR transition duration may be around 400/800ms, technically speaking, it still can work but we are concerning that there may be no too much power saving gain in this scenratio-2.

Observation-2: for scenario-2 (i.e., not-at-cell-edge is greater than entry condition), the LP-WUS monitoring UE may perform normal neighbour cell MR measurement (e.g., measurement period may be 1280ms), which means MR transition is needed based on this measurement period, therefore the power saving gain is marginal.

Based on the analysis, the wise idea should be the UE perform LP-WUS monitoring and neighbour cell MR measurement relaxation simultaneously. Having said this, since we have already agreed to reuse not-at-cell-edge as LP-WUS specific neighbour cell MR measurement relaxation, it is possible to decouple LP-WUS monitoring and neighbour cell MR measurement relaxation.

Proposal-2: for scenario-2 (i.e., the threshold of not-at-cell-edge is greater than that one of LP-WUS monitoring entry condition), RAN2 to confirm that although it is feasible that the LP-WUS monitoring UE can perform normal neighbour cell MR measurement, the power saving gain may be marginal.

Serving cell MR measurement relaxation

Since RAN4 has agreed serving cell MR measurement relaxation case, we still need to consider this case in additional to serving cell measurement offloading. Based on the assumption that not-at-cell-edge is reused as neighbour cell MR measurement relaxation criterion. One question is that do we still consider the same not-at-cell-edge criterion also for serving cell MR measurement relaxation. Basically there are two solutions:

Solution-1: both serving cell and neighbour cell MR measurement relaxation share the same criterion.

Solution-2: serving cell MR measurement relaxation criterion is independent from that one of neighbour cell.

For solution-1, it may not work. Reason is that when it comes to scenario-1 (i.e., not-at-cell-edge is lower than entry condition), it means the non-LP-WUS monitoring UE can perform serving cell relaxation which is never discussed before. In our understanding, serving cell measurement is really important for RRC_IDLE/INACTIVE mobility, any serving cell relaxation should not be perform without LR starting to perform serving cell measurement. In fact there was hot discussion in legacy Rel-16 measurement relaxation that whether serving cell should be relaxed or not and the agreement is NO. Furthermore, it become even worse for scenario-2 ((i.e., not-at-cell-edge is greater than entry condition), it means the LP-WUS monitoring UE may perform normal serving cell MR measurement, which may be every I-DRX cycle (the typical value could be 320/640ms), it is not acceptable that a LP-WUS monitoring UE should turn on MR every 320/640ms.

Observation-3: if reusing not-at-cell-edge as serving cell MR measurement relaxation, for scenario-1 (i.e., not-at-cell-edge is lower than entry condition), the non-LP-WUS monitoring UE may perform relaxed serving cell MR measurement, which is not promising.

Observation-4: if reusing not-at-cell-edge as serving cell MR measurement relaxation, for scenario-2 (i.e., not-at-cell-edge is greater than entry condition), the LP-WUS monitoring UE may perform normal serving cell MR measurement (of which measurement period is every I-DRX cycle), which is not acceptable.

Therefore, we think for serving cell MR measurement relaxation criterion, it better should be reusing entry/exit condition of LP-WUS monitoring. That is to say, as long as the UE enter LP-WUS monitoring mode, the UE start to perform serving cell MR measurement in a relaxed way, so that the frequency of MR transition could be reduced. It is also noted that LP-SS measurement is already started in this case.

Proposal-3: for serving cell MR measurement relaxation, consider entry/exit condition of LP-WUS monitoring as the criterion.

- 2.3 Measurement offloading

Here we discuss the relationship between serving cell and neighbour cell measurement offloading, based on RAN4 understanding, it is assumed that both should be performed simultaneously, but one detail is that RAN4 case#1 does not preclude an intermediate state (i.e., serving cell MR measurement is OFF, FFS condition of neighbour cell MR measurement offloading). In last section, we consider different criterion for each serving cell and neighbour cell measurement relaxation due to not-at-cell-edge is agreed. But here, things are a bit different.

The reason why the UE need to perform neighbour cell measurement is to ensure the mobility, i.e., cell re-selection in RRC_IDLE/INACTIVE. Our understanding is that once the UE fulfil serving cell measurement offloading criterion, it means the signal strength/quality is already good enough, in this scenario, there will be no mobility issue. As serving cell MR measurement has been stopped, let along neighbour cell MR measurement. Therefore neighbour cell MR measurement offloading should also be performed.

Proposal-4: once the agreed criteria for serving cell measurement offloading is met, then both serving cell and neighbour cell MR measurements are offloaded.

- 3 Conclusions

Observation-1: for scenario-1 (i.e., not-at-cell-edge is lower than entry condition), the non-LP-WUS monitoring UE may perform further relaxed neighbour cell measurement (i.e., K >3) without entering LP-WUS monitoring mode (compared with legacy Rel-16 measurement relaxation).

Observation-2: for scenario-2 (i.e., not-at-cell-edge is greater than entry condition), the LP-WUS monitoring UE may perform normal neighbour cell MR measurement (e.g., measurement period may be 1280ms), which means MR transition is needed based on this measurement period, therefore the power saving gain is marginal.

Observation-3: if reusing not-at-cell-edge as serving cell MR measurement relaxation, for scenario-1 (i.e., not-at-cell-edge is lower than entry condition), the non-LP-WUS monitoring UE may perform relaxed serving cell MR measurement, which is not promising.

Observation-4: if reusing not-at-cell-edge as serving cell MR measurement relaxation, for scenario-2 (i.e., not-at-cell-edge is greater than entry condition), the LP-WUS monitoring UE may perform normal serving cell MR measurement (of which measurement period is every I-DRX cycle), which is not acceptable.

Proposal-1: for the scenario-1 (i.e., the threshold of not-at-cell-edge is lower than that one of LP-WUS monitoring entry condition), RAN2 to confirm that it is feasible that the UE may perform LP-WUS specific measurement relaxation without entering LP-WUS monitoring mode, as like Rel-16 measurement relaxation behaviour but maybe with further relaxed factor which is determined by RAN4.

Proposal-2: for scenario-2 (i.e., the threshold of not-at-cell-edge is greater than that one of LP-WUS monitoring entry condition), RAN2 to confirm that although it is feasible that the LP-WUS monitoring UE can perform normal neighbour cell MR measurement, the power saving gain may be marginal.

Proposal-3: for serving cell MR measurement relaxation, consider entry/exit condition of LP-WUS monitoring as the criterion.

Proposal-4: once the agreed criteria for serving cell measurement offloading is met, then both serving cell and neighbour cell MR measurements are offloaded.

- 4 References