3GPP TSG-RAN WG2 Meeting #128			         R2-2409925

Orlando, USA, Nov 18th – 22th, 2024

Agenda Item:	8.4.3

Source: 	LG Electronics Inc.

Title:         	RRM relaxation and RRM offloading

Document for: 	Discussion and Decision

Introduction

Regarding RRM relaxation and RRM offloading, RAN4 agreed to support RRM measurement case #1 (RRM offloading) and case #3 (RRM relaxation) as follows:

RAN4#111

- Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #4

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |
| #4 Relaxed case c | Off | On, FFS the condition and the details | ON |

RAN4#112

- Cases/states to be considered for RRM relaxation and serving cell measurement offloading

  - Support case #3

In this contribution, we discuss entry/exit conditions for case#1 and case#3, and the impact of RRM offloading/relaxation on cell re-selection.

Discussion

- RRM offloading (case#1)

RAN2 agreed followings as conditions for RRM offloading and relaxation:

- For serving cell measurement offloading (i.e., there is no serving cell measurement by MR):

  - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

  - The exit condition is based on the LR measurement results.

- For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’.

During RRM measurement offloading, the UE cannot acquire any MR measurement results, so as we already agreed, the decision to stop RRM offloading should be based on the LR measurement results. If the entry condition for RRM measurement offloading is only based on MR measurement, ping-pong may happen due to the gap between the MR measurement and LR measurement, Since RAN4 agreed not to support combination of MR measurement and LR measurement, ping-pong between MR and LR measurement hinders the UE from obtaining stable measurement results for the serving cell. The entry condition for RRM offloading based on MR and LR measurements will be effective in acquiring the stable measurement results.

Proposal 1	[For case #1] The entry condition for RRM offloading is ‘if the serving cell quality measured by MR is greater than MR threshold and the LR measurement result is greater than LR threshold’.

LR can measure only serving cell and the RRM offloading to LR should not affect the mobility performance. Currently, no matter how good the quality of the saving cell quality is, UE is required to measure higher priority cells for cell re-selection. If UE does not measure any neighbor cell during RRM offloading, the mobility performance would be degraded. Therefore, UE should perform neighbor cell measurements according to the existing measurement rules, i.e. based on SIntraSearchP and SnonIntraSearchP. No change is needed in the measurement rules for cell re-selection.

Proposal 2	[For case #1] If the entry condition for RRM offloading is met, UE performs serving cell measurements using LR only (as agreed by RAN4).

Proposal 3	[For case #1] Even if the condition for RRM offloading is met, UE performs neighbor cell measurements using MR according to the existing measurement rules for cell re-selection.

- RRM relaxation (case#3)

When UE performs relaxed measurements using MR in case 3, the MR measurement results for serving cell is available, hence the exit condition for RRM relaxation does not need to rely on the LR measurements and UE can determine whether to stop the RRM relaxation based on the MR measurements. If the exit condition is based on MR measurements, we don’t need to define separate entry/exit condition for RRM relaxation. As in R16 RRM relaxation, the UE can stop RRM relaxation if the entry condition is no longer met without separate exit condition. Therefore, we propose followings:

Proposal 4	[For case #3] The entry condition for RRM relaxation is ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. The LR measurement is not considered.

Proposal 5	[For case #3] No separate exit condition for RRM relaxation. If the entry condition for RRM relaxation is not met, UE stops the RRM relaxation, i.e. performs legacy measurements.

RAN4#112bis:

- For case 3, whether to combine MR and LP-WUR samples when serving cell measurement when results from both MR and LP-WUR are available

  - From RAN4 requirement perspective, combine measurement results from different receivers shall not be considered

Since RAN4 concluded that combine measurement results from different receivers shall not be considered, UE does not need to perform LR measurement when the serving cell is measured by MR. Basically, the mobility decision, i.e. cell re-selection, should be based on the more accurate MR measurement. In case 3, LR does not need to be On at least for measurement purpose.

Proposal 6	[For case #3] If the condition for RRM relaxation is met, UE performs serving cell measurements using only MR with relaxed requirement.

RAN4 will define new relaxed measurement requirements for serving cell and neighbor cells. If the condition for RRM relaxation is met, UE will change the measurement requirement used for serving cell and neighbor cell measurements to the new relaxed measurement requirements. Although the measurement requirement is changed, the measurement target is not changed, i.e. which neighbor cells to measure depending on the measurement results of the serving cell. That is, UE still needs to select measurement target according to the existing measurement rules for cell re-selection.

Proposal 7	[For case #3] If the condition for RRM relaxation is met, UE performs neighbor cell measurements using MR with relaxed requirement according to the existing measurement rules for cell re-selection.

Some companies have argued that the UE should perform using LP-WUS and RRM offloading/relaxation under the same conditions. However, as we already agreed, RRM offloading is not suitable to use for UEs at cell edge. In contrast, LP-WUS can have the same coverage as cell and in such case, UEs at cell edge also can use LP-WUS. The LP-WUS coverage where UE can reliably receive LP-WUS may be larger or smaller than case1/3 coverage.

If the same entry condition is applied to using LP-WUS and RRM offloading/relaxation, although UE can receive LP-WUS reliably, it will stop using LP-WUS outside the case1/3 coverage, or it cannot perform RRM offloading/relaxation outside the LP-WUS coverage. UE still can save power by using LP-WUS without RRM offloading/relaxation or by performing RRM offloading/relaxation without using LP-WUS. The unified condition for using LP-WUS and RRM offloading/relaxation degrades the power saving gain, so we propose to decouple conditions for using LP-WUS form the conditions for RRM offloading/relaxation.

Proposal 8	The condition for RRM offloading/relaxation is not associated with the conditions for using LP-WUS.

- Impact of RRM offloading/relaxation on cell re-selection

In RRC_IDLE/INACTIVE state, the UE periodically performs serving cell and neighbour cell measurements for cell reselection according to measurement rules. The measurement interval for serving cell varies depending on the SMTC periodicity, DRX cycle, and UE’s power class. When the interval is shortest, the serving cell measurement should be performed at least once every 2.56 seconds. In contrast, the minimum measurement period for neighbour cells on the intra-frequency is 10.24 seconds, and for inter-frequency neighbour cells, it is 15.36 seconds. Therefore, if serving cell measurements are offloaded to LR, MR can maintain ultra-deep sleep for at least 10 seconds.

The target cell that the UE needs to measure depends on the quality of the serving cell, and the measurement cycle varies according to the frequency priority of the target cells. Hence, the duration that MR can sleep without neighbour cell measurements varies based on the quality of the serving cell.

Table 1 Expected power saving gain and impact of measurement offloading on cell reselection depending on the serving cell quality

| Serving cell quality | Measurement target | Maximum MR sleep duration | cell-ranking criterion Rs |
| --- | --- | --- | --- |
| If Srxlev > SIntraSearchP and Squal > SIntraSearchQ | Serving cell + Higher priority cells | 60 seconds | Does not need to be modified. |
| If Srxlev < SIntraSearchP or Squal < SIntraSearchQ | Serving cell + Higher priority cells + intra-frequency neighbour cell (+ inter-frequency with equal or lower priority) | 10.24~30.72 seconds | Needs to be modified to support LR measurement cell-ranking |

Even if the quality of the serving cell is excellent (i.e., if the serving cell fulfills Srxlev > SIntraSearchP and Squal > SIntraSearchQ), MR should periodically measure higher priority cells. However, once a higher priority cell has been measured but not reselected, the UE does not need to measure that cell for 60 seconds thereafter. Therefore, when serving cell quality is good enough to meet the condition for skipping intra-frequency measurement, a great power saving can be achieved through the serving cell measurement offloading to LR.

In the cell reselection to a higher priority frequency, the UE selects the highest ranked cell on the highest priority frequency by comparing the cell-ranking of higher priority cells measured by MR. Therefore, UE is not required to perform ranking using LR measurements though the serving cell measurement is offloaded to LR.

Observation 2	If the serving cell measurement is offloaded to LR when the condition for skipping intra-frequency measurement is met,

- a great power saving can be achieved by allowing MR to sleep for 60 seconds, and

- it does not affect the cell-ranking criterion Rs.

If the serving cell does not fulfil Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE should perform measurements on identified cells on the intra-frequency using MR every 10.24 to 30.72 seconds. Consequently, MR needs to wake up much more frequently than when UE is not required to perform intra-frequency measurements, and the UE power saving gain achievable through the serving measurement offloading to LR is significantly reduced.

The intra-frequency and equal priority inter-frequency cell reselection is performed based on comparing the ranking of serving cell and neighbour cells on intra-/equal priority inter-frequency. This means the UE needs to calculate ranking of serving cell using LR measurements and compare it with MR measurements based ranking of neighbour cells to select the highest ranked cell among them. If the target cell for reselection is selected based on the relatively inaccurate LR measurement results, the UE may perform incorrect cell reselection.

Observation 3	If the serving cell measurement is offloaded to LR when the condition for skipping intra-frequency measurement is not met,

- the power saving gain is significantly reduced, compared to the case when the condition for skipping intra-frequency measurement is met, and

- the highest ranked cell is selected based on LR measurements and it may lead to incorrect cell reselection.

The mobility performance should not be compromised for power saving, and the mobility decision should be made based on reliable MR measurements. Therefore, when UE initiates (or before initiating) intra-frequency measurements, the serving cell measurement offloading to LR should be halted, and MR should measure serving cell.

Proposal 9	RAN2 assumes that the offloading threshold is always higher than the threshold for skipping intra-frequency measurement.

Proposal 10	Do not support cell-ranking based on LR measurements.

Conclusion

Regarding RRM offloading (case#1):

Proposal 1	[For case #1] The entry condition for RRM offloading is ‘if the serving cell quality measured by MR is greater than MR threshold and the LR measurement result is greater than LR threshold’.

Proposal 2	[For case #1] If the entry condition for RRM offloading is met, UE performs serving cell measurements using LR only (as agreed by RAN4).

Proposal 3	[For case #1] Even if the condition for RRM offloading is met, UE performs neighbor cell measurements using MR according to the existing measurement rules for cell re-selection.

RRM relaxation (case#3)

Proposal 4	[For case #3] The entry condition for RRM relaxation is ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. The LR measurement is not considered.

Proposal 5	[For case #3] No separate exit condition for RRM relaxation. If the entry condition for RRM relaxation is not met, UE stops the RRM relaxation, i.e. performs legacy measurements.

Proposal 6	[For case #3] If the condition for RRM relaxation is met, UE performs serving cell measurements using only MR with relaxed requirement.

Proposal 7	[For case #3] If the condition for RRM relaxation is met, UE performs neighbor cell measurements using MR with relaxed requirement according to the existing measurement rules for cell re-selection.

Proposal 8	The condition for RRM offloading/relaxation is not associated with the conditions for using LP-WUS.

Impact of RRM offloading/relaxation on cell re-selection

Proposal 9	RAN2 assumes that the offloading threshold is always higher than the threshold for skipping intra-frequency measurement.

Proposal 10	Do not support cell-ranking based on LR measurements.