3GPP TSG-RAN WG2 Meeting #128			         R2-2409926

Orlando, USA, Nov 18th – 22th, 2024

Agenda Item:	8.5.2

Source: 	LG Electronics Inc.

Title:         	On-demand SSB SCell operation

Document for: 	Discussion and Decision

Introduction

RAN1 has made some progress on the on-demand SSB operation and shared it with RAN2 through the LS [1]. In this contribution, we will analyse the impact of the on-demand SSB on L3 measurement reporting and propose MAC CE signalling aligned with the RAN1 progress.

Discussion

According to TS38.331, whenever the UE has a measurement configuration, the UE shall perform RSRP and RSRQ measurements for each serving cell for which servingCellMO is configured, and include the measurement results of the serving cell in every measurement report message.

In the previous meeting, RAN2 agreed for Case #1 (no always-on SSB on the cell) that UE does not expect to measure SSB when on-demand SSB transmission is deactivated. If servingCellMO is configured for the OD-SSB SCell, the RAN2 agreement conflicts with the current UE behaviour. To resolve the contradiction between the current UE behaviour and the RAN2 agreement, two solutions can be considered as follows:

- Option 1. Do not configure servingCellMO for OD-SSB SCell

- Option 2. Change the procedural text to meet the RAN2’s agreement

If servingCellMO is not configured for OD-SSB SCell, the measurement results of the OD-SSB SCell will not be reported even when the OD-SSB transmission is activated, unless the measurement reporting condition associated with OD-SSB SCell is met. Since the SCell requires tight management though the measurement results frequently reported by UE, option 1 does not seem acceptable. Therefore, it seems more desirable to modify the procedural text related to the servingCellMO.

UE is not required to include the SSB based measurement results of the SCell configured with servingCellMO in the measurement report, if the SSB based measurement results of the SCell are not available. Therefore, even if the SSB transmission is deactivated, it will not impact the L3 measurement reporting.

Proposal 1	The servingCellMO can be configured for the OD-SSB SCell.

Proposal 2	Even if the servingCellMO is configured for the OD-SSB SCell, UE does not derive the SSB based measurement results of the OD-SSB SCell when the on-demand SSB transmission is deactivated.

According to RAN1’s agreement, RAN2 should do the signaling support for the on-demand SSB and the always-on SSB based L3 measurements. To analyze the impact of them on the existing measurement configuration, we should first consider whether the existing measurement configuration can be re-used for OD-SSB/always-on SSB based measurements.

The characteristic of the always-on SSB is it is transmitted with a long periodicity. Although RAN1 agreed that the always-on SSB is SSB supported in Rel-18 specifications, considering that the primary measurement timing configuration, i.e. smtc1, is used also for measurements on neighbor cells, it does not seem suitable to configure the always-on SSB using the existing smtc1.

The measurement object configuration, i.e. MeasObjectNR, contains also smtc2, smtc3list, and smtc4list, to support the secondary measurement timing which is different from the primary measurement timing for the listed cells. For always-on SSB based SCell measurements, similar signaling can be used, e.g. smtc5. The new measurement timing configuration for always-on SSB is applicable only to an SCell which is associated with the measurement object, so the cell list is not needed.

Proposal 3	Introduce smtc5 in MeasObjectNR associated with the OD-SSB SCell to configure the measurement timing of always-on SSB.

RAN1 also agreed that multiple periodicities should be support for on-demand SSB. It is already possible in the measurement object configuration to configure multiple measurement timing for particular cell using smtc3list or smtc4list. The usage of them is limited to IAB-MT and NTN cells, respectively. Therefore, it seems reasonable to introduce smtc6list to configure multiple measurement timing configuration for on-demand SSB.

Proposal 4	Introduce SSB-MTC6List in MeasObjectNR associated with the OD-SSB SCell to configure the measurement timing of on-demand SSB.

Conclusion

Proposal 1	The servingCellMO can be configured for the OD-SSB SCell.

Proposal 2	Even if the servingCellMO is configured for the OD-SSB SCell, UE does not derive the SSB based measurement results of the OD-SSB SCell when the on-demand SSB transmission is deactivated.

Proposal 3	Introduce smtc5 in MeasObjectNR associated with the OD-SSB SCell to configure the measurement timing of always-on SSB.

Proposal 4	Introduce SSB-MTC6List in MeasObjectNR associated with the OD-SSB SCell to configure the measurement timing of on-demand SSB.