# 3GPP TSG RAN WG2 Meeting #128      	                           R2-2409941
Orlando, US, 18th – 22th Nov., 2024

Agenda item:	8.5.3

Source:	Apple

Title:	Discussion on on-demand SIB1

WID/SID:	Netw_Energy_NR_enh-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3]

  - Specify procedures and signaling method(s) for Case 2 [RAN1/2]

    - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell

    - Triggering method by UL WUS using PRACH

  - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3]

  - Note 1: No modification of SSB will be discussed under this objective

  - Note 2:

    - UL WUS: Uplink wake-up signal

    - Cell A: A cell that is periodically transmitting at least its own SIB1

    - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

  - Note 3:

    - RAN1 strives to minimize impact to legacy UE

    - RAN1 specification impact to support this feature should be minimized

The Rel-19 WID of Network Energy Savings (WID RP-242354) was updated in RAN#105 [1]. The study objective on on-demand SIB1 (OD-SIB1) for UEs in IDLE/INACTIVE state becomes normative WI objective as copied:

In this contribution, we share our view on essential issues from below aspects:

- Access restriction for legacy UEs

- Detailed procedure of OD-SIB1 acquisition

- UL WUS configuration and signaling

# 2 Discussion

## 2.1 Access restriction for legacy UEs

The following agreements on legacy UE barring were agreed in RAN2#127b [2]:

Agreements on OD-SIB1

- Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

- NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

We discuss RAN2 impacts of these two agreements respectively.

### 2.1.1 Issue 1: Whether “no SIB1 indication” can bar legacy UEs

We first discuss whether “no SIB1 indication” in MIB can bar legacy UEs based on latest RAN1 agreements. In RAN1#118b [3], the following related agreements were made:

Agreement For further work of on-demand SIB1 in idle/inactive mode, it is assumed that: - SSB transmitted in the NES cell supporting OD-SIB1 is with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster - UE determines a cell supporting OD-SIB1 based at least on UL WUS configuration from Cell A. Agreement For type 0 PDCCH monitoring occasions of on demand SIB1, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from UL WUS configuration if SSB on NES cell is on sync raster and K_SSB is not equal to 30 in FR1 or 14 in FR2.

According to above agreements, our understanding is:

- If NES cell’s SSB is transmitted in sync raster, its K_SSB will take value K_SSB > 23 for FR1 and K_SSB >11 for FR2.

- Whether SSB of NES cell can be transmitted off sync raster is FFS in RAN1.

We further provide our analysis on 1) in Table.1 based on existing TS 38.213 [4] and TS 38.304 [5].

|  | K_SSB Value on NES cell | Legacy UE behavior according to existing spec | Rel-19 NES UE behavior according to existing spec and RAN1/RAN2 status |
| --- | --- | --- | --- |
| Sub-case 1 | K_SSB <= 23 for FR1 and K_SSB <= 11 for FR2 | - Regard it is CD-SSB and detect SIB1 (38.213) - Camp in the NES cell (38.304) | - Regard it is CD-SSB and detect SIB1 (38.213) - Camp in the NES cell (38.304) |
| Sub-case 2 | 23 < K_SSB < 30 for FR1 and 11 < K_SSB < 14 | - Regard it is NCD-SSB, and detect another SSB according to PDCCH-ConfigSIB1 (38.213) - May regard the cell is barred up to 300s (38.304) | - Regard it is NCD-SSB, and detect another SSB according to PDCCH-ConfigSIB1 (38.213) - NW implementation may indicate SSB info of Cell A in PDCCH-ConfigSIB1. - May regard the cell is barred up to 300s (38.304) |
| Sub-case 3 | K_SSB = 30 for FR1 and K_SSB = 14 for FR2 | - Regard it is NCD-SSB, and no further behavior is specified (reserved K_SSB value in 38.213) - May regard the cell is barred up to 300s (38.304) | - Regard it is NCD-SSB (38.213) - RAN1 may define new UE behavior and repurpose PDCCH-ConfigSIB1. - May regard the cell is barred up to 300s (38.304) |
| Sub-case 4 | K_SSB = 31 for FR1 and K_SSB = 15 for FR2 | - Regard it is NCD-SSB and no SIB1 (38.213) - May regard the cell is barred up to 300s (38.304) | - Regard it is NCD-SSB and no SIB1 (38.213) - May regard the cell is barred up to 300s (38.304) |

Table.1 Analysis of legacy UE and Rel-19 UE behavior when NES cell takes different value of K_SSB

RAN1 agreed K_SSB value are sub-cases 2/3/4. According to Table.1, legacy UEs may bar the NES Cell up to 300s in all these sub-cases. Meanwhile, Rel-19 UE may be specified with new behavior (highlighted in red font) which is being discussed in RAN1. RAN2 can just wait RAN1 and then discuss whether any RAN2 spec change. However, irrespective of whether RAN1 agree Rel-19 UE new behavior, the legacy UE barring behavior is not impacted.

Observation 1: According to existing 38.213 and 38.304, legacy UEs may bar the NES Cell in all the K_SSB value range agreed by RAN1 (i.e. K_SSB > 23 for FR1 and K_SSB >11 for FR2 when SSB is transmitted in sync raster). Meanwhile, Rel-19 UE may be specified with new behavior which is being discussed in RAN1.

Although whether SSB of NES cell can be transmitted off sync raster is FFS in RAN1, we think it will not change legacy UE barring NES cell because cell reselection is based on SSB in sync raster according to TS 38.300 [6]:

9.2.1.2	Cell Reselection

A UE in RRC_IDLE performs cell reselection. The principles of the procedure are the following:

- Cell reselection is always based on CD-SSBs located on the synchronization raster (see clause 5.2.4).

Observation 2: Irrespective of whether RAN1 conclude SSB of NES cell can be transmitted off sync raster, it will not change legacy UE barring NES cell because cell reselection is based on SSB in sync raster.

Thus, based on Observation 1 and 2, we think “no SIB1 indication” in MIB can bar legacy UEs based on latest RAN1 agreements, and no separate barring bit is needed.

Proposal 1: RAN2 confirm that “no SIB1 indication” in MIB of NES cell can bar legacy UEs according to latest RAN1 agreements on K_SSB value, and thereby no separate barring bit is needed.

### 2.1.2 Issue 2: How to allow NES cell to reselect to NES cell

Then, we discuss the detailed solution on how to implement below agreement, which is intended to introdce solutions to allow Rel-19 NES UEs to reselect to NES cells which has barred legacy UEs via “no SIB1 indication” in its MIB.

- NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

We understand “e.g..” part provides two solutions:

- Solution 1 (via new NES dedicated excluded cell list): PCI and frequency of NES cells are included in legacy intra/interFreqExcludedCellList in SIB3/4 of Cell A and introduce a new NES dedicated intra / interFreqExcludedCellList for NES UEs only.

- Solution 2 (via new NES dedicated reselection priorities): introduce a dedicated reselection priority for NES UEs only.

On Solution 1, we think it is an optimization, given that “no SIB1 indication” in MIB can already bar legacy UEs according to Proposal 1. The benefit of this solution is to reduce UE power consumption on performing measurements towards NES cells if they are included in legacy excluded cell list. However, we don’t think it is necessary to introduce a new NES dedicated intra / interFreqExcludedCellList for NES UEs only. Please note that NES cells’ frequency and PCI informatin are already agreed to be included in UL WUS configuraiton of Cell A according to RAN1#118b agreement [3]:

Agreement

For the purpose of “to which cell does the configuration applies”, at least the following parameters are included in the UL-WUS configuration.

| Purpose | Parameters |  |
| --- | --- | --- |
| To which cell does the configuration applies | NES-CellId | PhysCellId |
|  |  | ARFCN-ValueNR |

Note: ARFCN-ValueNR is used to indicate the absolute radio frequency channel number (ARFCN) for SSB of NES cell.

With this agreement, the Rel-19 NES UEs can already identify NES cells from UL WUS config of Cell A.

Observation 3: Because NES cells’ frequency and PCI information are agreed to be included in UL WUS configuration of Cell A, the Rel-19 NES UEs can already identify NES cells from UL WUS config of Cell A.

Then, RAN2 can just specify that NES UEs ignores the legacy intra/interFreqExcludedCellList (i.e. regard them as candidate target cells) if these NES cells are in the excluded cell list. And the new NES dedicated excluded cell list is not needed. Thus, we propose:

    - Proposal 2: After the Rel-19 UE obtains UL WUS configuration, it considers all the NES cells included in the UL WUS configuration as candidates of cell reselection, even if any of them are in legacy excluded cell list.

Proposal 3: No need to introduce a new dedicated NES excluded cell list because the PCI and frequency information of NES cells in UL WUS configuration are sufficient for Rel-19 UE to identify NES cells.

On Solution 2, we think it is useful to provide NW flexibility to steer NES UEs and legacy UEs towards different frequencies. Specifically, when NES frequency deploys mixed NES cells and legacy cells, Rel-19 UE and legacy UEs need use different inter-frequency priority for the frequencies with NES cell:

- For NES UEs, it may be preferred by NW to prioritize camping in the frequency with NES cells because NES cell can only server NES UEs and NES UEs should fully utlize them for loading balancing.

- For legacy UE, it may be preferred by NW to deprioritize camping in the frequency with NES cells because NW may deploy few legacy cells in the frequency to avoid interference to NES cell.

Observation 4: When NES frequency deploys mixed NES cells and legacy cells, Rel-19 UE and legacy UEs may need different inter-frequency priority for the frequencies with NES cell:

- For NES UEs, it may be preferred by NW to prioritize camping in the frequency with NES cells because NES cell can only server NES UEs and NES UEs should fully utlize them for loading balancing.

- For legacy UE, it may be preferred by NW to deprioritize camping in the frequency with NES cells because NW may deploy few legacy cells in the frequency to avoid interference to NES cell.

Thus, we propose to introduce NES dedicated reselection priorities.

Proposal 4: To provide NW flexibility to steer NES UEs and legacy UEs towards different frequencies, introduce a NES dedicated reselection priority.

## 2.2 Procedure of OD-SIB1 acquisition

### 2.2.1 Trigger condition of OD-SIB1 acquisition

The related study agreements in RAN2#126b [7] can be found below:

Triggering condition of WUS transmission

- If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

Meanwhile, whether a new RSRP threshold is needed was also discussed in RAN2#126b [7], but no conclusion was made:

Proposal 9: In addition to the already agreed conditions (i.e. UE doesn’t have a valid SIB1, the cell doesn’t broadcast SIB1 and supports on-demand SIB1) introduce a new RSRP triggering condition for the NES UE to be allowed to send WUS to trigger on-demand SIB1. FFS what is the triggering condition and whether the triggering condition is a new threshold.

[Samsung, Xiaomi, OPPO, NEC, Lenovo]: Think the existing cell reselection criterion is enough. [Nokia]: May be useful, but reluctant to agree with them now. We can discuss it in stage 3. [ZTE]: Support the proposal. It is good for NW is allowed to configure separate threshold than cell reselection criterion threshold. [LG]: Current cell reselection criterion is not absolute threshold, it’s relative to the serving cell’s radio condition. Support the proposal. [IDC]: If we have new threshold and the threshold is not met, anyway the existing cell reselection criterion should be used to trigger WUS transmission. [Vivo]: With the new threshold, it will make UE behaviour more complicated. We need to consider many combination cases, e.g. new threshold is {met, not met} + existing cell reselection criterion is {met, not met}. [Samsung]: Rather discussing whether new threshold is required or not, it’s better to discuss whether we’ll allow earlier or later WUS transmission triggering (than cell reselection criterion). The UE may achieve it w/o new threshold. [Apple]: Seems each company has different understanding on the use of the threshold. It’s good to see overall companies’ understanding on how the threshold is used or works.

We think the benefit to introduce a new RSRP threshold is not clear. Furthermore, considering legacy cell reselection criterion already uses RSRP threshold (e.g. ThreshX, HighP, ThreshX, LowP), new RSRP threshold will bring more complicated UE behavior during cell reselection (e.g. if new threshold is met but legacy threshold is not). Thus, we prefer to reuse legacy cell reselection criterion as OD-SIB1 trigger condition.

Thus, we propose:

Proposal 5: Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

### 2.2.2 How to check OD-SIB1 broadcasting status

RAN2#127b agreed that the UE needs to check if OD-SIB1 is currently being broadcasted or provided on-demand before triggering of OD-SIB1 acquisition procedure.

Agreements on OD-SIB1

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

We understand that it is a UE requirement rather than UE implementation. However, existing MIB has only 1 spare bit which is assumed to use only if serious legacy issue was identified in real deployment. So, it is impossible to use it for checking broadcasting status of OD-SIB1. The checking can only work if some bits in MIB of NES can be repurposed for it. However, because MIB design is within RAN1 expertise, we suggest to postpone this discussion until RAN1 conclude whether there are spare bits in MIB of NES cell can be repurposed.

Proposal 6: RAN2 postpone the discussion on how the UE checks if SIB1 is currently being broadcasted in NES cell after RAN1 conclude whether any spare bits in MIB of NES cell can be repurposed for it.

### 2.2.3 Failure case and UE behaviour

RAN2#126b made below agreements [7]:

UE behaviour on RACH failure of OD-SIB1 REQ

- As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation.

This issue was further discussed in RAN2#127b [2] and below agreement was made:

Agreements on OD-SIB1

- In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

- The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

- The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

We prefer to specify UE behavior rather than leaving to UE implementation. In legacy OD-SIB procedure, it is up to UE implementation when to retry the SI request because it is not essential to acquire other SIB(s). However, SIB1 is most essential information, and thereby RAN2#126b has agreed that the UE bars the NES cell if the procedure fails. As the cell barring will cause significant impact to system performance, the UE behavior should be clearly specified.

Observation 5: In legacy OD-SIB procedure, it is up to UE implementation when to retry the SI request because it is not essential to acquire other SIB(s). However, SIB1 is most essential information, and RAN2#126b has agreed that the UE bars the NES cell if the procedure fails. Thus, the UE behavior should be clearly specified.

Meanwhile, we also understand that some company prefer that the UE can still re-acquire OD-SIB1 even if RACH procedure is failed. To make progress, we suggest another way-forward that the UE may consider the cell as barred:

Proposal 7: The MAC layer will indicate the RACH failure for OD-SIB1 request to upper layers. After that the UE upper layer may consider the cell as barred.

### 2.2.4 Paging interruption during OD-SIB1 acquisition

This issue was raised by some company contribution in study phase [8], but it was not treated. In details, after the UE sends OD-SIB1 request in NES cell, it can’t monitor paging in NES Cell until OD-SIB1 is acquired. This is an extra paging interruption compared with legacy cell reselection. We think it is a valid issue because TS 38.133 has specified requirement of paging interruption during cell reselection (i.e. “The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms”) [9]:

4.2.2.6	Maximum interruption in paging reception

UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. In addition, when the UE is configured with eDRX_IDLE cycle, the UE shall not miss any paging in a PTW provided the paging is sent in at least 2 DRX cycles before the end of that PTW.

At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. Ttarget_cell_SMTC_period is the periodicity of the SMTC occasions configured for the target NR cell. If the target cell is in the PCI list of smtc2-LP, the SMTC periodicity follows smtc2-LP; otherwise, the SMTC periodicity follows smtc.

At inter-RAT cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels for paging reception of the target inter-RAT cell. For NR to E-UTRAN cell re-selection the interruption time must not exceed TSI-EUTRA + 55 ms.

TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell.

TSI-EUTRA is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 36.331 [16] for an E-UTRAN cell.

These requirements assume sufficient radio conditions, so that decoding of system information can be made without errors and does not take into account cell re-selection failure.

Observation 6: TS 38.133 has specified requirement of paging interruption during cell reselection (i.e. “The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms”).

We think there are two alternatives:

- Alt-1: leave it to UE implementation. But RAN2 need to send LS to RAN4 to check whether the paging interruption requirement needs change.

- Alt-2: include PCCH-Config of NES cell in UL WUS configuration provided by Cell A, so that Rel-19 UEs can monitor paging from NES cell during OD-SIB1 acquisition.

Since RAN2 has not discussed this issue before, we suggest RAN2 to discuss these 2 alternatives.

Proposal 8: On paging interruption issue during OD-SIB1 acquisition, RAN2 discuss the following 2 solutions:

- Alt-1: leave it to UE implementation. RAN2 send LS to RAN4 to check whether the paging interruption requirement during cell reselection specified in TS 38.133 needs change.

- Alt-2: include PCCH-Config of NES cell in UL WUS configuration provided by Cell A, so that Rel-19 UEs can monitor paging from NES cell during OD-SIB1 acquisition.

## 2.3 UL WUS configuration and signalling

In RAN1#118b [4], RAN1 further discussed detailed UL WUS configurations. We summarize RAN1 confirmed UL WUS configuration in Table.2.

| Purpose | Parameters |  |  |
| --- | --- | --- | --- |
| To which cell does the configuration applies | NES-CellId | PhysCellId |  |
|  |  | ARFCN-ValueNR |  |
| WUS transmission | SIB1-RequestConfig | ss-PBCH-BlockPower |  |
|  |  | SSB-positionInBurst |  |
|  |  | tdd-UL-DL-ConfigurationCommon |  |
|  |  | rach-OccasionsSIB1 | Prach-ConfigurationIndex |
|  |  |  | msg1-FDM |
|  |  |  | msg1-FrequencyStart |
|  |  |  | zeroCorrelationZoneConfig |
|  |  |  | preambleReceivedTargetPower |
|  |  |  | preambleTransMax |
|  |  |  | powerRampingStep |
|  |  |  | ra-ResponseWindow |
|  |  |  | ssb-perRACH-Occasion |
|  |  | sib1-RequestPeriod |  |
|  |  | sib1-RequestResources | ra-PreambleStartIndex |
|  |  |  | ra-AssociationPeriodIndex |
|  |  |  | ra-ssb-OccasionMaskIndex |
| WUS transmission | rsrp-ThresholdSSB |  |  |
|  | prach-RootSequenceIndex |  |  |
|  | msg1-SubcarrierSpacing |  |  |
|  | restrictedSetConfig |  |  |
|  | frequencyBandList |  |  |
|  | absoluteFrequencyPointA |  |  |
|  | offsetToCarrier |  |  |
|  | p-Max |  |  |
|  | frequencyShift7p5khz (working assumption)ULSubCarrierSpacing |  |  |

Table.2 Summary of RAN1 confirmed UL WUS configuration

On top of it, we think:

- n-TimingAdvanceOffset is missed because it is necessary for the UE to calculate initial TA.

- On BWP info of initial UL BWP: it is intended for the UE to identify PRACH frequency location because PRACH is configured per BWP. However, as RAN1 has clarified: “Msg1-FrequencyStart is with respect to the first RB of the UE carrier determined by offsetToCarrier and absoluteFrequencyPointA”, we think it is not needed.

Thus, we propose:

Proposal 9: On top of RAN1 agreed UL WUS configuration, n-TimingAdvanceOffset is also required.

Although discussion of UL WUS configuration is not finalized, we think the overhead of UL WUS configuration is already very large based on Table. 2. The following Table.3 provides an estimation:

| Purpose | Approximate payload size analysis | Note | Total payload size |
| --- | --- | --- | --- |
| Identify NES cell(s) | PhysCellId (10bit) ARFCN-ValueNR (22bit) | This is only value of one NES cell. Please note that RAN2 agreed multiple NES cells’ UL WUS config can be included in on Cell A. | 32bit |
| RACH resource parameter for SIB1 request | rach-ConfigGeneric (40bit) - prach-ConfigurationIndex (8bit) - msg1-FDM (2bit) - msg1-FrequencyStart (9bit) - zeroCorrelationZoneConfig (4bit) - preambleReceivedTargetPower (8bit) - preambleTransMax (4bit) - powerRampingStep (2bit) - ra-ResponseWindow (3bit) ssb-perRACH-Occasion (3bit) si-RequestPeriod (3bit) SI-RequestResources (14bit) - ra-PreambleStartIndex (6bit) - ra-AssociationPeriodIndex (4bit) - ra-ssb-OccasionMaskIndex (4bit) Other RACH config (22bit) - msg1-SubcarrierSpacing (3bit) - prach-RootSequenceIndex (10bit) - rsrp-ThresholdSSB (7bit) - n-TimingAdvanceOffset (2bit) - TDD-UL-DL-ConfigCommon (~30bit) - ss-PBCH-BlockPower (7bit) - restrictedSetConfig (1bit) | - Assume up to 275 PRBs. - n-TimingAdvanceOffset is missed. | 120bit |
| UL frequency info | FrequencyInfoUL-SIB (~133bit) - MultiFrequencyBandListNR (80bit) - absoluteFrequencyPointA (22bit) - scs-SpecificCarrierList (~24bit) - p-Max (6bit) - frequencyShift7p5khz (1bit) | - It is necessary to obtain carrier information of PRACH transmission and related setting (e.g. UL power). - Assume up to 8 bands - Assume only 1 SCS. | 133bit |
| SIB1 reception | pdcch-ConfigSIB1 (8bit) | It seems conflicted with latest RAN1 agreement | 8bit (TBD) |
| RAR reception | RA-SearchSpace (6bit) monitoringSlotPeriodicityAndOffset (16bit) duration (12bit) monitoringSymbolsWithinSlot (14bit) nrofCandidates (6bit) | - Assume CORESET#0 is used for RAR reception (i.e. no extra common CORESET configuration). - Assume only one search space is configured for RAR. | 52bit |
| Total payload size: 349bit×Num of NES Cell (e.g. If 8 NES Cells’ UL WUS config are included, 2792 bit is required) * * Optimization of area-specific configuration is not considered, i.e. one NES cell has one UL WUS config in this analysis. |  |  |  |

Table.3 Analysis on required payload size of UL WUS configurations

As can be observed from Table.3, at least 349 bit is required if only one NES cell’s UL WUS configurations is included, and it can be up to 2792 bit if 8 NES cells’ UL WUS configurations are included. This number may increase because some configurations under discussion (e.g. window to detect OD-SIB1 in RAN1) are not included.

However, according to TS 38.331 [10], SIB1 or a SI message is limited to 2976 bit.

NOTE 2:	The physical layer imposes a limit to the maximum size a SIB can take. The maximum SIB1 or SI message size is 2976 bits.

Observation 7: At least 349bit is required if only one NES cell’s UL WUS configurations is included, and it can be up to 2792 bit if 8 NES cells’ UL WUS configurations are included. However, according to TS 38.331, SIB1 or a SI message is limited to 2976 bit.

To resolve the issue of large overhead of broadcasting UL WUS configuration, we think it is necessary to introduce RRC release message to include UL WUS configuration:

- It can reduce overhead of broadcasting UL WUS configuration for multiple NES cells.

- Extra RAN2 spec impact is to add WUS configuration in RRC release message. The spec impact is minor and within RAN2 expertise.

Observation 8: The benefit of using RRC release message to include UL WUS configuration is to reduce overhead of broadcasting UL WUS configuration for multiple NES cells. And the spec impact is minor and within RAN2 expertise.

Thus, we propose:

Proposal 10: UL WUS configuration can be included in RRC release message in both Cell A and NES cell.

# 3 Conclusion

In this contribution, we discuss on-demand SIB1 for IDLE/INACTIVE UEs. Our observations are:

Observation 1: According to existing 38.213 and 38.304, legacy UEs may bar the NES Cell in all the K_SSB value range agreed by RAN1 (i.e. K_SSB > 23 for FR1 and K_SSB >11 for FR2 when SSB is transmitted in sync raster). Meanwhile, Rel-19 UE may be specified with new behavior which is being discussed in RAN1.

Observation 2: Irrespective of whether RAN1 conclude SSB of NES cell can be transmitted off sync raster, it will not change legacy UE barring NES cell because cell reselection is based on SSB in sync raster.

Observation 3: Because NES cells’ frequency and PCI information are agreed to be included in UL WUS configuration of Cell A, the Rel-19 NES UEs can already identify NES cells from UL WUS config of Cell A.

Observation 4: When NES frequency deploys mixed NES cells and legacy cells, Rel-19 UE and legacy UEs may need different inter-frequency priority for the frequencies with NES cell:

- For NES UEs, it may be preferred by NW to prioritize camping in the frequency with NES cells because NES cell can only server NES UEs and NES UEs should fully utlize them for loading balancing.

- For legacy UE, it may be preferred by NW to deprioritize camping in the frequency with NES cells because NW may deploy few legacy cells in the frequency to avoid interference to NES cell.

Observation 5: In legacy OD-SIB procedure, it is up to UE implementation when to retry the SI request because it is not essential to acquire other SIB(s). However, SIB1 is most essential information, and RAN2#126b has agreed that the UE bars the NES cell if the procedure fails. Thus, the UE behavior should be clearly specified.

Observation 6: TS 38.133 has specified requirement of paging interruption during cell reselection (i.e. “The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms”).

Observation 7: At least 349bit is required if only one NES cell’s UL WUS configurations is included, and it can be up to 2792 bit if 8 NES cells’ UL WUS configurations are included. However, according to TS 38.331, SIB1 or a SI message is limited to 2976 bit.

Observation 8: The benefit of using RRC release message to include UL WUS configuration is to reduce overhead of broadcasting UL WUS configuration for multiple NES cells. And the spec impact is minor and within RAN2 expertise.

Based on observations, our proposals are:

Access restriction for legacy UEs

Proposal 1: RAN2 confirm that “no SIB1 indication” in MIB of NES cell can bar legacy UEs according to latest RAN1 agreements on K_SSB value, and thereby no separate barring bit is needed.

    - Proposal 2: After the Rel-19 UE obtains UL WUS configuration, it considers all the NES cells included in the UL WUS configuration as candidates of cell reselection, even if any of them are in legacy excluded cell list.

Proposal 3: No need to introduce a new dedicated NES excluded cell list because the PCI and frequency information of NES cells in UL WUS configuration are sufficient for Rel-19 UE to identify NES cells.

Proposal 4: To provide NW flexibility to steer NES UEs and legacy UEs towards different frequencies, introduce a NES dedicated reselection priority.

Procedure of OD-SIB1 acquisition

Proposal 5: Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

Proposal 6: RAN2 postpone the discussion on how the UE checks if SIB1 is currently being broadcasted in NES cell after RAN1 conclude whether any spare bits in MIB of NES cell can be repurposed for it.

Proposal 7: The MAC layer will indicate the RACH failure for OD-SIB1 request to upper layers. After that the UE upper layer may consider the cell as barred.

Proposal 8: On paging interruption issue during OD-SIB1 acquisition, RAN2 discuss the following 2 solutions:

- Alt-1: leave it to UE implementation. RAN2 send LS to RAN4 to check whether the paging interruption requirement during cell reselection specified in TS 38.133 needs change.

- Alt-2: include PCCH-Config of NES cell in UL WUS configuration provided by Cell A, so that Rel-19 UEs can monitor paging from NES cell during OD-SIB1 acquisition.

UL WUS configuration and signaling

Proposal 9: On top of RAN1 agreed UL WUS configuration, n-TimingAdvanceOffset is also required.

Proposal 10: UL WUS configuration can be included in RRC release message in both Cell A and NES cell.

# 4 References

[1] RP-242354, Revised WID for Enhancements of network energy savings for NR, Ericsson.

[2] RAN2#127b, Chair Notes.

[3] RAN1#118b, Chair Notes.

[4] TS 38.213-v18.0.0, Physical layer procedures for control, 2023-12.

[5] TS 38.304-v18.0.0, User Equipment (UE) procedures in Idle mode and RRC Inactive state, 2023-12.

[6] TS 38.300-v18.0.0, NR and NG-RAN Overall Description; Stage 2, 2023-12.

[7] RAN2#126b, Chair Notes.

[8] R2-2406470, Remaining details to enable on-demand SIB1, Intel Corporation.

[9] TS 38.133-v18.4.0, Requirements for support of radio resource management, 2024-1.

[10] TS 38.331-v18.0.0, Radio Resource Control (RRC) protocol specification, 2023-12.