# 3GPP TSG RAN WG2 Meeting #128      	                           R2-2409944
Orlando, US, 18th – 22th Nov., 2024

Agenda item:	8.1.2.3

Source:	Apple

Title:	Further discussion on LCM procedure of AI/ML based positioning

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

The updated Rel-19 WID of AI/ML for air interface (WID RP-242399) was agreed in RAN#105 [1]. The WI objective on general LCM framework, including applicable functionality, is led by RAN2 which is copied below:

LCM procedure for UE-sided model for AI positioning case 1 was first discussed in RAN2#126 [2]. And then in RAN2#127b [3], its baseline LCM procedure was agreed:

Agreements: 1: The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. Step 5: The LMF requests the inferred location information using the LPP request location information message. Step 6: UE reports the inferred location using LPP provide location information message. 2: Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress). 3: Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress). 4: FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively). FFS the signalling details. 5: RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

In this contribution, we further discuss remaining issues on the LCM procedure of AI/ML based positioning.

# 2 Discussion

Based on RAN2#127b agreement [3], the baseline LCM procedure can be illustrated in Figure. 1. We highlight the FFSs which are discussed in this contribution.

Figure.1: Baseline LCM procedure of UE-sided model for AI/ML positioning Case 1

## 2.1 FFS 1: need of NW-side additional condition and its contents

In this section, we address the FFS in below RAN2#127b agreement [3]:

Agreements

3: 	Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress).

Please note that RAN1#118b made below agreement [4]:

Agreement

For AI/ML positioning Case 1, regarding the assistance data provided from LMF to UE, for ensuring consistency between training and inference,

- for each of the existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD, study whether it should be: (a) explicitly indicated, (b) implicitly indicated and/or (c) other;

- Companies can provide inputs on further enhancements of existing assistance data, including new information

- Note: this does not mean that training and inference phases are mentioned in assistance data.

Table. Existing assistance data (supported up to Rel-18) that may be transferred from LMF to UE in UE-based DL-TDOA [1] or UE-based DL-AoD [2], as applicable.

|  | Information | UE-based DL-TdoA | UE-based DL-AoD |
| --- | --- | --- | --- |
| 1 | Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement |  |  |
| 2 | Timing relative to the serving (reference) TRP of candidate NR TRPs |  |  |
| 3 | DL-PRS configuration of candidate NR TRPs |  |  |
| 4 | Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation |  |  |
| 5 | SSB information of the TRPs (the time/frequency occupancy of SSBs) |  |  |
| 6 | Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB |  |  |
| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |  |  |
| 8 | Fine Timing relative to the serving (reference) TRP of candidate NR TRPs |  |  |
| 9 | PRS-only TP indication |  |  |
| 10 | The association information of DL-PRS resources with TRP Tx TEG ID |  |  |
| 11 | LOS/NLOS indicators |  |  |
| 12 | On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation |  |  |
| 13 | Validity Area of the Assistance Data |  |  |
| 14 | PRU measurements together with the location information of the PRU |  |  |
| 15 | Data facilitating the integrity results determination of the calculated location |  |  |
| 16 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) |  |  |
| 17 | Expected Angle Assistance information |  |  |
| 18 | PRS priority list |  |  |

[1] Table 8.12.2.1.0-1 in 38.305, Use equipment (UE) positioning in NG-RAN (Release 18), v18.3.0

[2] Table 8.11.2.1.0-1 in 38.305, Use equipment (UE) positioning in NG-RAN (Release 18), v18.3.0

As this RAN1 agreement clearly indicates the assistance data is used for “ensuring consistency between training and inference”, we understand that it already means that at least part of the existing assistance data summarized in the table of RAN1 agreement can be regarded as NW-side additional conditions. The remaining issue is whether any of them are precluded as NW-side additional conditions.

Observation 1: RAN1#118b has agreed that at least part of the existing assistance data can be regarded as NW-side additional conditions. The remaining issue is whether any of them are precluded as NW-side additional conditions.

Because it is up to NW implementation to include which assistance data in the LPP message, we see no reason to preclude any of them as NW-side additional conditions. If NW doesn’t prefer to indicate one assistance data to the UE as NW-side additional condition, it may just not transmit it in LPP ProvideAssistanceData message. It doesn’t make sense to restrict NW implementation. Thus, we propose:

Proposal 1: All the existing assistance data in UE-based DL-TDOA and UE-based DL-AoD can be explicitly included in LPP ProvideAssistanceData message in step 3 as NW-side additional conditions.

Next issue is whether associated ID is also applicable to AI/ML based positioning and can be included in step 3 as NW-side additional conditions. In RAN2#127, some company contributions challenged its necessity [5]. Their two main arguments are listed below:

- Some NW-side additional conditions (e.g. TRP/ARP locations, DL-PRS beam information, synchronization information) can be regarded to be already included in existing LPP message ProvideAssistanceData. Additional Assistance Data ("network-side additional conditions") can be easily added to the existing framework, if needed.

- Any associated ID for the "Network-side Additional Conditions" would imply a "hard-decision" and thus, suboptimal. A simple "associated ID" change indication does not provide any information to what degree the information has changed, and therefore, corresponds to a "hard decision" not to use a trained model for inference in the given scenario. Providing the (normal) assistance data as "Network-side Additional Conditions" leaves the decision on whether a trained model is still suitable for inference in a given situation up to implementation.

We are not convinced by these two arguments:

- For argument 1), we agree that some NW-side additional conditions are already included in existing LPP ProvideAssistanceData. However, existing Assistance Data may not cover all NW-side additional conditions required for AI/ML positioning. For example, the association between PRS configuration used for data collection and PRS configuration used for inference. We disagree that these new NW-side additional conditions can be easily added to existing framework because each new assistance information needs to be agreed and then specified in 3GPP. If some assistance information has risk of exposure of NW implementation, they are difficulty to be agreed in 3GPP. At least, such standardization work will unnecessarily delay the deployment of AI/ML positioning. On the contrary, associated ID is a solution with better forward compatibility because the NW implementation behind associated ID is transparent to the UE.

Observation 2: It is not always easy to add new NW-side additional condition(s) to LPP Assistance Data. At least, such standardization work will unnecessarily delay the deployment of AI/ML positioning. On the contrary, associated ID solution has better forward compatibility as NW implementation behind is transparent to the UE.

- For argument 2):

  - Firstly, we disagree that associated ID always implies a "hard-decision". According to below agreement of RAN2#127 [6]:

UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration).  FFS  how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

it allows the UE to determine applicable functionality based on its internal information (i.e. UE-side additional condition) which already leaves room for flexible UE implementation.

  - Secondly, we don’t think associated ID and legacy way are mutual exclusive. In our understanding, if associated ID is provided in LPP ProvideAssistanceData, the UE can still request new DL PRS configuration via LPP message LPP RequestAssistanceData when it determines current functionality is not applicable.

Observation 3: According to RAN2#127 agreement, applicability reporting based on associated ID already leaves room for flexible UE implementation, i.e. it is not “hard-decision”.

Thus, we propose that associated ID is also applicable to AI/ML positioning Case 1. But it doesn’t make sense to use associated ID to represent existing explicit assistance data because they are already there and more accurate.

Proposal 2: RAN2 assume that associated ID can also be included in LPP ProvideAssistanceData as NW-side additional conditions which are not covered by the existing assistance data in UE-based DL-TDOA and UE-based DL-AoD.

Proposal 3: If associated ID is provided in LPP ProvideAssistanceData, the UE can still request new DL PRS configuration via LPP RequestAssistanceData when it determines current functionality is not applicable.

## 2.2 FFS 2: whether LMF controls the UE applicability reporting

In this section, we address the FFS in below RAN2#127b agreement [3]:

Agreements

4: 	FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively).  FFS the signalling details.

As captured in TS 37.355 [7], existing LPP capability signaling supports both solicited reporting (i.e. UE reports reactively upon LMF request) in Section 5.1.1 and unsolicited reporting (i.e. UE reports proactively) in Section 5.1.2.

Observation 4: In TS 37.355, existing LPP capability signaling supports both solicited reporting (i.e. reactive reporting upon LMF request) in Section 5.1.1 and unsolicited reporting (i.e. proactive reporting) in Section 5.1.2.

We understand that the solicited reporting and unsolicited reporting are exactly matched with the reactive reporting and proactive reporting agreed in LCM procedure of AI/ML based beam management.

Agreements on procedures - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc

Thus, the applicable functionality reporting in step 4 can also support both solicited reporting and unsolicited reporting.

Proposal 4: On applicable functionality reporting in step 4, support both reactive reporting (i.e. solicited information transfer as response to LPP RequestCapabilities) and proactive reporting (i.e. unsolicited information transfer).

In case of unsolicited information transfer, legacy LPP procedure doesn’t need LMF control whether the UE can report. And please note that existing TS 38.305 has clearly captured such principle [8]:

7.1.2.5	Sequence of procedures

LPP procedures are not required to occur in any fixed order, in order to provide greater flexibility in positioning. Thus, a UE may request assistance data at any time in order to comply with a previous request for location measurements from the LMF; an LMF may instigate more than one request for location information (e.g., measurements or a location estimate) in case location results from a previous request were not adequate for the requested QoS; and the target device may transfer capability information to the server at any time if not already performed.

Despite the flexibility allowed by LPP, it is expected that procedures will normally occur in the following order:

1. Capability Transfer;

2. Assistance Data Transfer;

3. Location Information Transfer (measurements and/or location estimate).

Observation 5: Legacy LPP unsolicited information transfer procedure doesn’t need LMF control whether the UE can report. And TS 38.305 clearly captures that the target device may transfer capability information to the server at any time.

We see no reason to challenge this principle of legacy LPP in AI/ML based positioning. Thus, we propose:

Proposal 5: As legacy LPP, no need to introduce LMF control on the UE sending unsolicited LPP provide capabilities in step 4 for applicable functionality reporting.

## 2.3 FFS 3: how the UE determines applicable functionality in step 4

In RAN2#127 [6], it was agreed that the UE determines applicable functionalities:

Agreements on procedures - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

With our clarification on NW-sided additional conditions in Proposal 1, we think the same agreement can be reused for LCM of AI/ML based positioning.

Proposal 6: In UE-side model of AI/ML positioning Case 1, UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

## 2.4 FFS 4: order of inference config and applicability reporting

In this section, we address the highlighted FFS in RAN2#127 post-meeting email discussion summary [9]:

Proposal 7: For positioning Case 1, UE can report a change in applicable functionality by sending applicable functionalities upon change via LPP signaling (unsolicited information transfer). UE can also provide applicable functionality reporting as a response to LMF request (solicited information transfer). FFS on the order of inference configuration and applicable functionality reporting, FFS on functionality activation.

Please note that existing TS 38.305 has captured below principle for LPP procedures [8]:

7.1.2.5	Sequence of procedures

LPP procedures are not required to occur in any fixed order, in order to provide greater flexibility in positioning. Thus, a UE may request assistance data at any time in order to comply with a previous request for location measurements from the LMF; an LMF may instigate more than one request for location information (e.g., measurements or a location estimate) in case location results from a previous request were not adequate for the requested QoS; and the target device may transfer capability information to the server at any time if not already performed.

Despite the flexibility allowed by LPP, it is expected that procedures will normally occur in the following order:

1. Capability Transfer;

2. Assistance Data Transfer;

3. Location Information Transfer (measurements and/or location estimate).

We see no reason to challenge above principle of LPP in AI/ML based positioning. Thus, we propose:

Proposal 7: Following existing principle on sequence order of LPP procedures, no need to specify fixed step order between inference configuration and applicable functionality reporting.

## 2.5 FFS 5: need of functionality activation

The existing LPP Location Information Transfer procedure can be found in Section 5.3.1 of 37.355 [7]:

### 5.3.1 Location Information Transfer procedure The Location Information Transfer procedure is shown in Figure 5.3.1-1. Figure 5.3.1-1: LPP Location Information transfer procedure 1. The server sends a RequestLocationInformation message to the target to request location information, indicating the type of location information needed and potentially the associated QoS. 2. The target sends a ProvideLocationInformation message to the server to transfer location information. The location information transferred should match or be a subset of the location information requested in step 1 unless the server explicitly allows additional location information. If step 3 does not occur, this message shall set the endTransaction IE to TRUE. 3. If requested in step 1, the target sends additional ProvideLocationInformation messages to the server to transfer location information. The location information transferred should match or be a subset of the location information requested in step 1 unless the server explicitly allows additional location information. The last message shall include the endTransaction IE set to TRUE.

As we can see, the UE immediately sends response via ProvideLocationInformation to LMF upon reception of RequestLocationInformation. Thus, it can be regarded that the functionality is activated when the LPP message RequestLocationInformation (i.e. configuration) is received, i.e. no extra activation procedure after reception of configuration.

Observation 6: In existing LPP Location Information Transfer procedure, the UE activates the response procedure when LPP message RequestLocationInformation is received, i.e. no extra activation procedure after reception of configuration.

We see no reason to introduce a new activation procedure on top of the existing procedure for AI/ML based positioning. Thus, we propose:

Proposal 8: For UE-side model of AI/ML positioning Case 1, the functionality is activated upon reception of LPP message RequestLocationInformation as legacy LPP Location Information Transfer procedure, i.e. no need to introduce extra procedure of functionality activation.

# 3 Conclusion

In this contribution, we further discuss LCM procedure of AI/ML based positioning. Our observations are:

Observation 1: RAN1#118b has agreed that at least part of the existing assistance data can be regarded as NW-side additional conditions. The remaining issue is whether any of them are precluded as NW-side additional conditions.

Observation 2: It is not always easy to add new NW-side additional condition(s) to LPP Assistance Data. At least, such standardization work will unnecessarily delay the deployment of AI/ML positioning. On the contrary, associated ID solution has better forward compatibility as NW implementation behind is transparent to the UE.

Observation 3: According to RAN2#127 agreement, applicability reporting based on associated ID already leaves room for flexible UE implementation, i.e. it is not “hard-decision”.

Observation 4: In TS 37.355, existing LPP capability signaling supports both solicited reporting (i.e. reactive reporting upon LMF request) in Section 5.1.1 and unsolicited reporting (i.e. proactive reporting) in Section 5.1.2.

Observation 5: Legacy LPP unsolicited information transfer procedure doesn’t need LMF control whether the UE can report. And TS 38.305 clearly captures that the target device may transfer capability information to the server at any time.

Observation 6: In existing LPP Location Information Transfer procedure, the UE activates the response procedure when LPP message RequestLocationInformation is received, i.e. no extra activation procedure after reception of configuration.

Based on observations, our proposals can be found below:

FFS related to NW-side additional condition and its contents in Step 3

Proposal 1: All the existing assistance data in UE-based DL-TDOA and UE-based DL-AoD can be explicitly included in LPP ProvideAssistanceData message in step 3 as NW-side additional conditions.

Proposal 2: RAN2 assume that associated ID can also be included in LPP ProvideAssistanceData as NW-side additional conditions which are not covered by the existing assistance data in UE-based DL-TDOA and UE-based DL-AoD.

Proposal 3: If associated ID is provided in LPP ProvideAssistanceData, the UE can still request new DL PRS configuration via LPP RequestAssistanceData when it determines current functionality is not applicable.

FFS related to applicable functionality reporting in Step 4

Proposal 4: On applicable functionality reporting in step 4, support both reactive reporting (i.e. solicited information transfer as response to LPP RequestCapabilities) and proactive reporting (i.e. unsolicited information transfer).

Proposal 5: As legacy LPP, no need to introduce LMF control on the UE sending unsolicited LPP provide capabilities in step 4 for applicable functionality reporting.

Proposal 6: In UE-side model of AI/ML positioning Case 1, UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

FFS related to inference configuration and functionality activation in Step 5

Proposal 7: Following existing principle on sequence order of LPP procedures, no need to specify fixed step order between inference configuration and applicable functionality reporting.

Proposal 8: For UE-side model of AI/ML positioning Case 1, the functionality is activated upon reception of LPP message RequestLocationInformation as legacy LPP Location Information Transfer procedure, i.e. no need to introduce extra procedure of functionality activation.

# 4 References

[1] RP-242399, New WID: New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm.

[2] RAN2#126, Chair Notes.

[3] RAN2#127b, Chair Notes.

[4] RAN1#118b, Chair Notes.

[5] R2-2406643, On LCM for UE-sided models, Qualcomm Incorporated.

[6] RAN2#127, Chair Notes.

[7] 3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[8] 3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[9] R2-2406381, Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung)_Phase 2, Intel Corporation.