3GPP TSG-RAN WG2 Meeting #128	R2-2409958

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.7.5

Source:	Apple

Title:	Discussions on Fast RLC Retransmission

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion / Decision

# Introduction

For Rel-19 XR Phase 3, we have the following objective [1]:

-	Specify the following user plane enhancements [RAN2]

-	RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

According to the agreements made in RAN2 #125bis, it is confirmed that RAN2 will consider the following directions:

RAN2 #125bis Agreements: RAN2 will analyse solutions to ensure timely RLC retransmission(s) for XR RAN2 will analyse how to avoid unnecessary retransmissions (e.g. to avoid reTx of out-dated packets)

For timely RLC retransmission, the following agreements have been made during RAN2 #126 and RAN2 #127bis [2,3]:

RAN2 #126 Agreements on Timely RLC Retransmissions: To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling - FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism Impact on capacity should be considered RAN2 focuses on the enhancements for UL traffic RAN2 #127bis Agreements on Timely RLC Retransmissions: RAN2 confirm that existing mechanisms are insufficient to resolve the timely RLC retransmission problem and RLC enhancements for timely RLC retransmission are investigated in Rel-19. Exclude enhanced status reporting. Focus the discussion on autonomous retransmission and polling enhancements, e.g. we need to understand how each option affects the capacity and packet delay

This paper aims to provide some of our views relating to RLC-AM enhancements for faster retransmissions.

# Discussions

## Analysis of Autonomous Retransmission and Polling Enhancements

### Autonomous Retransmission

To achieve faster retransmission, several companies have proposed that transmitter RLC entity may autonomously perform retransmission even if no feedback is received from the receiver (namely autonomous retransmission). Specifically, it has been suggested that the transmitter can autonomously consider a packet for retransmission once it becomes delay-critical (i.e. when the remaining time of a packet becomes smaller than a threshold). From the latency perspective, such scheme may be beneficial as a more urgent RLC SDU could be retransmitted more proactively and quickly. Nonetheless, some concerns on impacts to the system capacity have also been brought up in the discussions, especially in uplink scenarios. For instance, the UE may not know the network status in general, blind retransmission may cause further congestions to other traffic flows and/or users.

When packets start to become delay-critical, implicitly some degree of congestion may be present in the network, and therefore the radio availability may be scarce. If we recklessly trigger more retransmissions without status feedback from the receiver under such situations, congestions may become even more severe. It is worth highlighting that, regardless of how frequently or how quickly a packet is considered for retransmission, if there is no radio resource available, the retransmission of this packet still cannot be performed over the air interface. For UL, since the network does not know when (and how many) autonomous RLC retransmissions will be triggered, the gNB may not be able to schedule appropriate uplink resource on time. Thus, we cannot guarantee that autonomous retransmission based on remaining time can always achieve the desired latency.

Furthermore, for XR use cases we may have to cope with PDU Sets, wherein a large number of packets of a PDU Set may trigger autonomous retransmission concurrently (due to their similar arrival times). With such a surge of data volume for retransmission, it is difficult to ensure that the gNB can allocate sufficient radio resources to accommodate both initial transmissions and retransmissions that have been triggered by the UE autonomously. Since RLC-AM shall always prioritize retransmissions over initial transmissions, in such cases any UL resource allocated by the network may be largely consumed by the retransmissions, which causes delay for the new transmission. Consequently, the initial transmissions cannot be performed immediately even if the radio resource is available, and they may subsequently become delay-critical as well. This eventually leads to a vicious cycle, and therefore triggering autonomous retransmission in an impetuous manner will only worsen the congestion.

Observation 1: It is questionable how the network can timely allocate sufficient UL resource for autonomous RLC retransmission, especially when the UE concurrently triggers autonomous retransmission for a large number of packets (e.g. packets of a PDU Set).

Observation 2: Regardless of how quickly/frequently autonomous RLC retransmission is triggered, if there is no available/sufficient radio resource, we cannot ensure retransmission can be performed immediately, so the benefits in terms of latency cannot be guaranteed.

Observation 3: Triggering autonomous retransmission in an impetuous manner (e.g. directly consider a RLC SDU for retransmission as soon as its remaining time becomes smaller than a threshold) may make the congestion even more severe.

Based on these observations, we think if Autonomous Retransmission is to be adopted, it should only be used as a last resort, i.e. when the window is stalled and when the transmitter cannot receive timely status feedback. In such situations, since new transmission cannot be performed anyway due to window stalling, the transmitter can endeavor to expedite the retransmissions.

Proposal 1: If Autonomous Retransmission is to be adopted, it should only be applied as a last resort, i.e. when the transmitting window is stalled, and status feedback cannot be received on time.

### Polling Enhancements

For Polling enhancements, the triggering conditions of polling can be extended by taking delay-criticality of RLC SDUs into account. The transmitter can therefore trigger a poll faster when there is a critical need. Some examples of new triggering conditions for polling may include:

- When a delay-critical RLC SDU is present in the queue,

- When the number of RLC PDUs corresponding to delay-critical RLC SDUs sent since the last status report is received satisfies a threshold,

- When the number of bytes RLC PDUs corresponding to delay-critical RLC SDUs sent since the last status report is received satisfies a threshold.

Since polling can be transmitted whenever there is an urgent need, theoretically the receiver can provide the status report in a timely manner. For UL cases where the receiver is a gNB, we believe it is up to network implementation to send the status report. Thus, as long as the gNB is properly implemented, we think faster RLC retransmission can be achieved. Nevertheless, one could argue that we cannot guarantee the status report is always received on time. Specifically, how quickly the transmitter can receive the status report may be constrained by the instantaneous DL channel quality and DL congestion status.

Observation 4: Faster RLC Retransmission can be achieved with Polling Enhancement in UL, as long as the gNB can provide status report in a timely manner. However, this cannot always be guaranteed.

With this in mind, we think a complementary mechanism may be introduced to deal with such situations. In particular, we can allow the transmitter to perform retransmission of delay-critical RLC SDUs when t-PollRetransmit expires and when the transmitting window is stalled. In fact, this is very aligned with an existing procedure. According to Clause 5.3.3.4 of TS 38.322, the specification has already provided some guidelines about if a UE can trigger retransmission when t-PollRetransmit expires, and which RLC SDU can be selected to be considered for retransmission. Hence, in our opinions, reusing such framework makes a lot of sense in terms of performance benefits and specification impacts.

It is worth noting that, although the UE also triggers retransmission autonomously without feedback in this case, there will be less concerns on impacts to capacity as compared to Autonomous Retransmission schemes that RAN2 has been discussing. This is because the t-PollRetransmit itself can naturally act as a prohibit timer to refrain the UE from triggering retransmission excessively. Besides, the retransmission mechanism based on t-PollRetransmit expiration has been in the specification for several releases already, we believe this does not impact the existing gNB/UE implementations too much.

An exemplary text proposal below shows how simple the specification change is for this complementary mechanism. Basically, only one additional sentence is needed in Clause 5.3.3.4 of TS 38.322 (Note that the number of retransmissions that can be triggered can be further discussed, it does not have to be N>=1):

#### 5.3.3.4 Expiry of t-PollRetransmit Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements); or - if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling): - consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission; or - consider N>=1 delay-critical RLC SDUs which have not been positively acknowledged for retransmission, if delayCriticalReTx is configured; or - consider any RLC SDU which has not been positively acknowledged for retransmission. - include a poll in an AMD PDU as described in clause 5.3.3.2.

Observation 5: As a complementary mechanism to Polling Enhancement, the transmitter can trigger retransmissions of delay-critical RLC SDUs upon expiration of t-PollRetransmit.

## Comparison of Candidate Solutions

Since RAN2 has agreed to examine the impacts to capacity and packet delay for the candidate solutions, we have compiled a table to compare the candidate solutions analyzed in this paper:

| Solution | Analysis of Impacts to Capacity | Analysis of Impacts to Packet Delay |
| --- | --- | --- |
| Autonomous Retransmission triggered by packet remaining time | - Huge impacts to capacity as autonomous retransmission may be triggered for a large number of packets simultaneously (e.g. for a PDU Set) - There may be a need to constrain or limit how frequently autonomous retransmission can be triggered (e.g. based on a new prohibit timer), which means extra specification efforts. | - If the network knows how many packets are considered for retransmission, the resource may be appropriately allocated to enable fast retransmission. Nonetheless, it is questionable how the network can know about this in a timely manner, so benefits of packet delay cannot be guaranteed. |
| Polling Enhancements with retransmission of delay-critical RLC SDUs upon expiration of t-PollRetransmit | - No much impacts to capacity, as the transmitter merely include a polling bit in the data PDU when there is an urgent need. | - Ideally, status report can be received more quickly. Even if status report is not received, the transmitter can retransmit delay-critical RLC SDUs when t-PollRetransmit is expired. |

Based on the analysis, we think RAN2 should consider Polling Enhancement that includes a complementary mechanism of triggering retransmission for delay-critical RLC SDUs upon expiry of t-PollRetransmit.

Proposal 2: RAN2 adopts the following Polling Enhancements to enable faster RLC retransmission:

- New Poll Triggering conditions based on delay-criticality of packets; and

- Retransmission of delay-critical RLC SDUs upon expiration of t-PollRetransmit.

# Conclusions

In this paper, we have presented our views on RLC-AM enhancements, with the following observations and proposals:

Observation 1: It is questionable how the network can timely allocate sufficient UL resource for autonomous RLC retransmission, especially when the UE concurrently triggers autonomous retransmission for a large number of packets (e.g. packets of a PDU Set).

Observation 2: Regardless of how quickly/frequently autonomous RLC retransmission is triggered, if there is no available/sufficient radio resource, we cannot ensure retransmission can be performed immediately, so the benefits in terms of latency cannot be guaranteed.

Observation 3: Triggering autonomous retransmission in an impetuous manner (e.g. directly consider a RLC SDU for retransmission as soon as its remaining time becomes smaller than a threshold) may make the congestion even more severe.

Observation 4: Faster RLC Retransmission can be achieved with Polling Enhancement in UL, as long as the gNB can provide status report in a timely manner. However, this cannot always be guaranteed.

Observation 5: As a complementary mechanism to Polling Enhancement, the transmitter can trigger retransmissions of delay-critical RLC SDUs upon expiration of t-PollRetransmit.

Proposal 1: If Autonomous Retransmission is to be adopted, it should only be applied as a last resort, i.e. when the transmitting window is stalled, and status feedback cannot be received on time.

Proposal 2: RAN2 adopts the following Polling Enhancements to enable faster RLC retransmission:

- New Poll Triggering conditions based on delay-criticality of packets; and

- Retransmission of delay-critical RLC SDUs upon expiration of t-PollRetransmit.

# References

- RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur), Sept. 2024, Melbourne, Australia.

- 3GPP RAN2 #126 Chair’s Notes, Fukuoka, Japan, May 2024.

- 3GPP RAN2 #127bis Chair’s Notes, Hefei, China, October 2024.