3GPP TSG-RAN WG2 Meeting #128													     R2-2409970

Orlando, USA, Oct 18th – 22nd, 2024

Agenda item:	8.10.2

Source:	Apple

Title:	SON for LTM

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we provide or views on the remaining details that need to be discussed on SON for LTM.

# 2   	Discussion

### 2.1		Background

RAN2#127-bis we’ve made the following agreements related to SON for LTM:

Agreements - The UE shall log cell IDs such as reestablishment cell ID, source, failed, reconnect cell ID, following the same principle as RLF, HOF and successful recovery, incl. the time between UE executing the LTM command and the failure. - If RA-based LTM failure happens the UE logs and reports RACH info in the RLF report. Additional information is TBD. - Unless RAN3 defines a NW-based solution: The UE logs and reports whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined). - Include an explicit indicator in SHR whether the successful LTM execution was RACH-less or RACH-based. Can sort out the details during stage-3 implementation.

Agreements - Unless RAN3 defines a NW-based solution: Introduce an explicit indication in RLF-Report to indicate whether a neighbour cell is an LTM candidate cell. - UE logs available L1 measurement results for the serving cell, the target cell and other LTM candidate cells when a successful LTM cell switch triggers SHR.

All in all the agreements so far seem to address what’s needed for LTM and the only thing that’s remaining is to agree on the details, which is the focus on the present contribution.

### 2.2		Cel IDs

On the agreement “The UE shall log cell IDs such as reestablishment cell ID, source, failed, reconnect cell ID, following the same principle as RLF, HOF and successful recovery…” there were comments made that this information is already included in the existing signaling, so before proceeding we need indeed to take a look at what’s already available.

RLF-Report IE already contains:

- c-RNTI: This field indicates the C-RNTI used in the PCell upon detecting radio link failure or the C-RNTI used in the source PCell upon handover failure.

- previousPCellId: This field is used to indicate the source PCell of the last handover (source PCell when the last executed RRCReconfiguration message including reconfigurationWithSync was received). For intra-NR handover nrPreviousCell is included and for the handover from EUTRA to NR eutraPreviousCell is included.

- failedPCellId: This field is used to indicate the PCell in which RLF is detected or the target PCell of the failed handover. For intra-NR handover nrFailedPCellId is included and for the handover from NR to EUTRA eutraFailedPCellId is included. The UE sets the ARFCN according to the frequency band used for transmission/ reception when the failure occurred.

- reconnectCellId: This field is used to indicate the cell in which the UE comes back to connected after connection failure and after failing to perform reestablishment, or to indicate the suitablecell in which the UE reconnects after failure in performing MobilityFromNRCommand for voice fallback (without initiating re-establishment procedure). If the UE comes back to RRC CONNECTED in an NR cell then nrReconnectCellID is included and if the UE comes back to RRC CONNECTED in an LTE cell then eutraReconnectCellID is included.

As we can see, the existing IEs already cover all the agreed information of “reestablishment cell ID, source, failed, reconnect cell ID”. The only thing that’s missing is an explicit reference to LTM, as the current field descriptions refer to “handover”, “RRCReconfiguration message”, etc.

Similarly, the procedural text (in TS 38.331 5.3.10.5) does not explicitly refer to LTM and therefore needs modifications. For example, examining the text which indicates how the UE sets previousPCellId we can see that it only says “f an RRCReconfiguration message including the reconfigurationWithSync was received…”. The situation is similar for other IEs.

Observation 1: the existing IE in RLF-Report cover all the agreed cell id information, the only thing that’s missing is reference to LTM in the field description and the procedural text.

Proposal 1: the field description and the procedural text for c-RNTI, previousPCellId, failedPCellId and 	reconnectCellId need to be extended to cover LTM; other than that the existing IEs can be re-used.

### 2.3	Time

On the agreement “The UE shall log … the time between UE executing the LTM command and the failure”, we further note that RLF-Report already contains the timeConnFailure IE defined as follows:

- timeConnFailure: This field is used to indicate the time elapsed since the last HO execution until connection failure. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.

Similarly to what we’ve observed for cell id IEs, this IE can be easily re-used to carry the LTM-related indication with simple text modifications.

Observation 2: the existing IEs in RLF-Report cover the agreed time between the LTM command and the failure indication, the only thing that’s missing is reference to LTM in the field description and the procedural text.

Proposal 2: the field description and the procedural text for timeConnFailure need to be extended to cover LTM; other than that the existing IE can be re-used.

### 2.4	RA information

Finally, regarding the agreement “UE logs and reports RACH info in the RLF repot” we note that for this an existing ra-InformationCommon IE can also be re-used. Similarly to the previously discussed information, all that’s needed is some modifications to the field description and the procedural text.

Observation 3: the existing IE in RLF-Report cover the agreed RACH info information for LTM, the only thing that’s missing is reference to LTM in the field description and the procedural text.

Proposal 3: the field description and the procedural text for ra-InformationCommon need to be extended to cover LTM; other than that the existing IE can be re-used.

There is also the “Additional information is TBD.” agreement. There’ve been several proposals for additional information, but the justifications brought up were rather weak.

For example, there were proposals to differentiate between the following cases:

- UE doesn’t attempt RACH-less LTM but rather attempts only RACH-based LTM

- UE attempts RACH-less LTM, it fails and if the UE has been configured to try LTM after LTM failure, the UE attempts RACH-based LTM (which may result in a failure)

While the case described above is indeed possible, it is hardly a common case but rather a somewhat corner case scenario. Furthermore, as always with SON case, the intention is not necessarily to “catch” all failure cases every time they occur (the network/the operator are unlikely to take mitigation actions on a single report anyway), but rather ensure that sufficient number of such cases is indeed logged and reported, because the network eventually would process those statistics and attempt (automatic or manual) mitigations only when enough cases have been reported.

With this in mind, we think that a simple solution where the inclusion of ra-InformationCommon indicates that at least one RACH-based LTM was attempted and failed is quite sufficient as over time it would provide enough statistical information for the network to mitigate all the issues.

Proposal 4: the inclusion of ra-InformationCommon indicates that at least one RACH-based LTM was attempted and failed.

Proposal 5: no additional RACH info is needed for LTM.

# 3	Proposals

### 3.1 	Observations

Observation 1: the existing IE in RLF-Report cover all the agreed cell id information, the only thing that’s missing is reference to LTM in the field description and the procedural text.

Observation 2: the existing IEs in RLF-Report cover the agreed time between the LTM command and the failure indication, the only thing that’s missing is reference to LTM in the field description and the procedural text.

Observation 3: the existing IE in RLF-Report cover the agreed RACH info information for LTM, the only thing that’s missing is reference to LTM in the field description and the procedural text.

### 3.1 	Proposals

Proposal 1: the field description and the procedural text for c-RNTI, previousPCellId, failedPCellId and 	reconnectCellId need to be extended to cover LTM; other than that the existing IEs can be re-used.

Proposal 2: the field description and the procedural text for timeConnFailure need to be extended to cover LTM; other than that the existing IE can be re-used.

Proposal 3: the field description and the procedural text for ra-InformationCommon need to be extended to cover LTM; other than that the existing IE can be re-used.

Proposal 4: the inclusion of ra-InformationCommon indicates that at least one RACH-based LTM was attempted and failed.

Proposal 5: no additional RACH info is needed for LTM.

# 5	References

[1]		RAN2#127 and RAN#126-bis and RAN2#127-bis meeting minutes