3GPP TSG RAN WG2 #127-bis	R2-2409982

Orlando, Florida, USA, November 18th – 22nd, 2024

Source:	Panasonic

Title:	Improvements to LTE-TN-to-NTN mobility

Agenda Item:	8.8.6     LTE to NR-NTN mobility

Document for:	Discussion, decision

# 1. Introduction

The Work Item Description for LTE to NR-NTN mobility is this, see [2]:

The [RAN2] work item aims at specifying support for the following objectives:

Idle mode mobility based on cell re-selection from E-UTRA-TN to NR-NTN, where E-UTRA-TN provides satellite information for NR-NTN neighbour cells in a System Information Block.

Furthermore, we reached the following agreements regarding agenda item “8.8.6 LTE to NR-NTN mobility” during the two most recent RAN2 meetings #127 and #127-bis – see [3] and [4]:

Agreements:

1.	Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33 (can come back to this if we find serious issue with this).

2.	The ntn-PolarizationalDL is optional.

3.	Reuse the SatelliteId-r18 to identify either an NR- or an IoT-NTN satellite.

4.	Consider a solution that avoids repeating the ephemeris for a satellite which provides both IoT- and NR-NTN cells.

5.	maxSat-r17 (4) is reused for the maximum number of NR satellites.

6.	RAN2 will not do further work to introduce multiple SMTCs in LTE.

7.	NR-NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA-TN to NR-NTN cell.

8.	RAN2 confirms that measurements of NR-NTN cells for a UE in E-UTRAN-TN RRC_INACTIVE are supported, with the understanding that UE moves to RRC idle upon selecting the NR-NTN cell.

9.	Introduce a new UE capability without signalling for LTE-TN to NR-NTN mobility.

Table 1: List of RAN2#127 agreements regarding LTE to NR-NTN mobility

Agreements:

10.	The following signalling design is agreed, to avoid ephemeris duplication for the same satellite providing both IoT-NTN and NR-NTN:

The satelliteId-r19 in an entry of NR-NTN assistance info list (i.e. neighSatelliteInfoListNR) can be set equal to a satelliteID value included in IoT-NTN assistance info list (i.e. neighSatelliteInfoList) and thus refers to the ephemeris data of IoT-NTN identified by this specific Satellite ID, in which case the ephemeris data (i.e. ephemerisInfo-r19) in that entry of neighSatelliteInfoListNR can be absent.

11.	Regarding satellite ID assignment across IoT-NTN and NR-NTN satellite assistance info lists, RAN2 thinks this can be left to NW implementation to handle.

12.	RAN2 sticks to the agreement that only duplication of ephemeris data needs to be avoided in the case of same satellite supporting both NR NTN and IoT NTN.

Table 2: List of RAN2#127-bis agreements regarding LTE to NR-NTN mobility (agreements renumbered for unambiguous cross-referencing)

This paper considers the agreements above and proposes related improvements.

# 2. Discussion

# 2.1 Background

# For cell reselection purposes, it is possible to refer from a terrestrial E-UTRAN cell to NTN neighbour cells – be it NR-NTN cells or (NB-)IoT-NTN cells. This task is overtaken by SIB33(-NB) [1] that is provided in the terrestrial E-UTRAN cell in question – see table 3 below outlining the existing coding being part of [1].

-- ASN1START

SystemInformationBlockType33-r18 ::= SEQUENCE {

neighSatelliteInfoList-r18	NeighSatelliteInfoList-r18			OPTIONAL,	-- Need OR

neighValidityDuration-r18		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240, s900}

OPTIONAL,	-- Need OP

lateNonCriticalExtension		OCTET STRING							OPTIONAL,

...

}

NeighSatelliteInfoList-r18 ::=	SEQUENCE (SIZE(1..maxSat-r17)) OF NeighSatelliteInfo-r18

NeighSatelliteInfo-r18 ::=	SEQUENCE {

satelliteId-r18				SatelliteId-r18,

ephemerisInfo-r18				CHOICE {

stateVectors-r18				EphemerisStateVectors-r17,

orbitalParameters-r18			EphemerisOrbitalParameters-r17

},

nta-CommonParameters-r18		SEQUENCE {

nta-Common-r18					INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r18			INTEGER (-261935..261935)	OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r18	INTEGER (0..29479)			OPTIONAL	-- Need OP

},

epochTime-r18					SEQUENCE {

startSFN-r18					INTEGER (0..1023),

startSubFrame-r18				INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Mac-r18						INTEGER (1..512)				OPTIONAL,	-- Need OP

t-ServiceStartNeigh-r18		TimeOffsetUTC-r17				OPTIONAL	-- Need OR

}

-- ASN1STOP

Table 3: SIB33(-NB) coding

A single satellite is able to provide multiple beams and herewith also multiple cells. It is obvious that there are satellite-specific attributes and beam/cell-specific attributes.
Although the same satellite is able to provide both, IoT-NTN and NR-NTN cells, SIB33(-NB) doesn’t distinguish between the named cell types (and RATs, in fact). That distinction might be done elsewhere or it might be added to SIB33(-NB). The current agreement deploying different types of NeighSatelliteInfoLists for IoT-NTN and NR-NTN – and herewith introducing the distinction between the different cell types in question on satellite level – might be improved in terms of structure and coding efficiency as described in the remainder of this paper.

The way forward consists of a clear separation between satellite-specific and beam/cell-specific assistance information. Because a particular satellite can’t necessarily be identified as an IoT-NTN or NR-NTN satellite, it hardly makes sense to make that type assignment on, say, the satellite level – although duplication of satellite-specific assistance information as agreed in Hefei with the different types of NeighSatelliteInfoLists (NeighSatelliteInfoList and NeighSatelliteInfoListNR) – would enable the distinction in a costly manner.

Our proposal is to add a beam/cell-related loop to SIB33(-NB). Alternatively, beam/cell-specific information could be signalled with a new SIB, e.g. SIB34. This paper covers the first solution as the one coming along with minimal specification impact.

Figure 1 below illustrates the different multi-beam situations. On the left hand side the two beams are covering the same area, on the right hand side the two beams are covering separate areas. It doesn’t matter in the first place, if the beams/cells are of the IoT-NTN or NR-NTN type. An indication of the beam/cell type (and RAT, in fact) – IoT-NTN or NR-NTN – can be added to the beam loop or can be signalled elsewhere.

Figure 1: Satellite emitting two beams covering the same (left) or different area(s) (right)

Observation 1:	SIB33(-NB) mainly features satellite-specific fields, but also beam/cell-specific ones. That structure leads to a significant signalling inefficiency when it comes to multi-beam satellites, because the entire NeighSatelliteInfoList needs to be repeated – one such info list per beam.

Observation 2:	SIB33(-NB) does not yet consist of an optional polarization indication for each DL beam as required with agreement 2 listed – see table 1. Furthermore, an indication of the RF coverage region would be a useful addition – also in conjunction with intended service area indications.

# 2.2 Proposal for SIB33(-NB) extension

# 2.2.1 Adding a beam loop to IE SIB33(-NB)

The proposed way forward mainly consists of the addition of a beam loop to SIB33(-NB). That loop assigns a beam identifier – beamId-r19 – to each beam emitted by the satellite identified with satelliteId-r18 and consists of the field t-ServiceStartNeigh-r19. The latter field is defined in the same way as its Rel.18 counterpart already.
Also an optional ntn-PolarizationDL field becomes part of the named loop, because in the absence of that indication the UE may need more time to search the SSB of the satellite. Since it is still possible to find the SSB, that field can be an optional one. Also agreement 2 from the Maastricht meeting #127 requires the addition of that field in optional form.
Furthermore, the geographical area illuminated by the beam in question is described with a circular area consisting of the latitude and longitude of a related reference location as well as the radius of the circle.

Observation 3:	For efficient coding, a beam-related loop featuring the characteristics of multiple beams needs to be added to SIB33(-NB).

Proposal 1:	Add a beam-related loop – enabling the indication of the characteristics of multiple beams – to SIB33(-NB).

# Coding

For the exact coding of the new fields see table 4 – basically an excerpt from SIB33(-NB) coding described with table 3 above plus the added and highlighted new fields.

-- ASN1START

SystemInformationBlockType33-r18 ::= SEQUENCE {

neighSatelliteInfoList-r18	NeighSatelliteInfoList-r18			OPTIONAL,	-- Need OR

neighValidityDuration-r18		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240, s900}

OPTIONAL,	-- Need OP

NeighSatelliteBeamList-r19 ::=	SEQUENCE (SIZE(1..maxBeam-r19)) OF NeighSatelliteBeams-r19

NeighSatelliteBeams-r19 ::=	SEQUENCE {

satelliteId-r18					SatelliteId-r18,

beamId-r19						BeamId-r19,

t-ServiceStartMBNeigh-r19		TimeOffsetUTC-r17			OPTIONAL	-- Need OR

ntn-polarizationDL-r17			ENUMERATED{rhcp,lhcp,linear}OPTIONAL	-- Need R

footprintInfo-r19			SEQUENCE {

referencePoint-r17		SEQUENCE {

longitude-r17			INTEGER (-131072..131071),

latitude-r17			INTEGER (-131072..131071)

} 														OPTIONAL,	-- Need OR

radius-r17					INTEGER (1..256)

},

lateNonCriticalExtension		OCTET STRING					OPTIONAL,	...

}

NeighSatelliteInfoList-r18 ……
……

Table 4: beamId loop and fields therein (SIB33(-NB))

satelliteID:	The satellite ID applicable to a specific cell, used to associate with the satellite assistance information for neighbour cells.

beamId:		The beam ID applicable to a specific cell illuminated by the satellite identified with satelliteID, used to associate with the beam assistance information for neighbour cells.
Optionally, the beamId could be extended by an indication for either the RAT in place or a flag indicating IoT-NTN or NR-NTN.

t-ServiceStartMBNeigh:	Indicates the earliest time when the area covered by the current serving cell is going to be covered by the neighbour cell(s) served by the satellite beam indicated by satelliteId and beamId, see 5.5.3.1, 5.5.8 and 36.304 [4].

ntn-PolarizationDL:	If present, this parameter indicates polarization information for downlink transmission on service link: including Right hand, Left hand circular polarizations (RHCP, LHCP) and Linear polarization.

latitude:	Latitude of the reference point. Unit in degree. Step of 360/262144 degree. Actual value = field value * (360/262144).

longitude:		Longitude of the reference point. Unit in degree. Step of 360/262144 degree. Actual value = field value * (360/262144).

radius:		Distance between the reference point and the edge of the satellite or beam coverage. Unit in km. Step of 10 km. Actual value = field value * 10.

Observation 4:	A fully backwards-compatible extension of SIB33(-NB) for indicating the characteristics of multiple beams is feasible.

Proposal 2:	Adopt the coding proposal outlined with clause 2.2.2 of this document R2-2408257 for an efficient coding of multiple beam characteristics as part of SIB33(-NB) – a fully backwards-compatible extension.

3. Others

As far as agreement 11 from meeting #127-bis is concerned (see table 2), uniqueness of satelliteIDs emitting different beams/cells is automatically given, because there is only a single NeighSatelliteInfoList-r18 per satellite.

Observation 5:	Due to the single occurrence of a NeighSatelliteInfoList per satellite, uniqueness of satelliteIDs is given automatically.

Proposal 3:	Adopt the improved signalling structure realizing the Hefei agreements with clear separation between satellite-specific assistance information and beam/cell-specific assistance information.

# 4. Conclusions

In this document, we discussed how to improve the agreements made so far in conjunction with LTE-TN-to-NR-NTN mobility. The proposals made also cover the LTE-TN-to-IoT-NTN case – as mandated.

The following observations and proposals were made:

Observation 1:	SIB33(-NB) mainly features satellite-specific fields, but also beam/cell-specific ones. That structure leads to a significant signalling inefficiency when it comes to multi-beam satellites, because the entire NeighSatelliteInfoList needs to be repeated – one such info list per beam.

Observation 2:	SIB33(-NB) does not yet consist of an optional polarization indication for each DL beam as required with agreement 2 listed – see table 1. Furthermore, an indication of the RF coverage region would be a useful addition – also in conjunction with intended service area indications.

Observation 3:	For efficient coding, a beam-related loop featuring the characteristics of multiple beams needs to be added to SIB33(-NB).

Observation 4:	A fully backwards-compatible extension of SIB33(-NB) for indicating the characteristics of multiple beams is feasible.

Observation 5:	Due to the single occurrence of a NeighSatelliteInfoList per satellite, uniqueness of satelliteIDs is given automatically.

Proposal 1:	Add a beam-related loop – enabling the indication of the characteristics of multiple beams – to SIB33(-NB).

Proposal 2:	Adopt the coding proposal outlined with clause 2.2.2 of this document R2-2408257 for an efficient coding of multiple beam characteristics as part of SIB33(-NB) – a fully backwards-compatible extension.

Proposal 3:	Adopt the improved signalling structure realizing the Hefei agreements with clear separation between satellite-specific assistance information and beam/cell-specific assistance information.

- References

[1]	3GPP TS 36.331 V18.3.1 (2024-09)				3rd Generation Partnership Project; Technical
                    	Specification Group Radio Access Network;
                    	Evolved Universal Terrestrial Radio Access (E-
                    	UTRA); Radio Resource Control (RRC);
                    	Protocol specification (Release 18)

[2]	RP-240776			Revised WID on Non-Terrestrial Networks
                       (NTN) for NR-NTN Phase 3

[3]	R2-127 NR-NTN-IoT-NTN (Sergio)_ EOM		NTN Session Chair’s minutes of meeting #127
                                                                                  in August 2024

[4]	R2-127bis NR-NTN-IoT-NTN (Sergio)_ EOM	NTN Session Chair’s minutes of meeting #127-
                                                                                         bis in October 2024