3GPP TSG-RAN WG2 Meeting #127bis	R2-2409983

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.10.2

Source:	Nokia

Title:	MRO for CHO with candidate SCG

WID/SID:	NR_ENDC_SON_MDT_Ph4-Core

Document for:	Discussion and Decision

# 1	Introduction

The following agreements were reached on the topic of MRO for CHO with candidate SCG with regard to information to be logged in various SON reports:

RAN2#126 - RAN2 to enhance the RLF report with additional information regarding the state of the two execution conditions. We see later if we also can enhance the SCGFailureInformation report.

RAN2#127 - UE includes following information in RLF report:

b.        Time information regarding condition fulfilment for CHO with candidate SCGs. Details are FFS. We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled.

c.        Measurement results of PCells and PSCells.

RAN2#127bis

UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), and the first met execution condition (as agreed by RAN3), for a failed CHO with candidate SCGs. Details FFS.

Include the elapsed time between the point in time of the first fulfilled condition and RLF in RLF report. Details FFS.

This paper discusses further details on information logging in SON reports for MRO for CHO w/ candidate SCG.

# 2	Enhancements of SON reports for MRO for CHO with candidate SCGs

The agreements have been reached by RAN2 only cover the scenarios where an RLF is detected on the MCG leg before, during or shortly after the execution of CHO w/ candidate SCG, namely:

Scenario A: RLF on MCG leg is detected before execution of CHO w/ candidate SCG.

Scenario B: HOF on MCG leg during handover execution of CHO w/ candidate SCG.

Scenario C: RLF on the MCG leg is detected shortly after successful execution of CHO w/ candidate SCG.

In addition to the already agreed information to be logged in RLF report (time gap between first and second met condition and the indication of the first met condition), it may be beneficial to collect more information by the UE to aid the network in better understanding the root cause of the failure and decide on appropriate corrective actions.

If only one execution condition was met before the RLF on MCG was detected, the network may benefit from knowing how close the second execution condition was to fulfilment at that time, for example if time to trigger (TTT) was running or not for the second condition. This additional information helps the network in deciding which and to what degree the execution conditions’ parametrization needs to be adapted.

Proposal 1: In case RLF on MCG is detected before execution of CHO w/ candidate SCG, the RLF report should be enhanced with additional information on the state of the execution event that was not triggered, e.g. whether TTT was running for this event or not.

SCG failure may also be detected (instead of MCG failure) in the same scenarios as described above and corresponding to subcase b) in the failure scenarios agreed by RAN3. In this case, only SCGFailureInformation will be generated, not RLF report. The current content of SCGFailureInformation only includes PSCell measurements but no PCell measurements. The MN may not have up to date PCell measurements reported by the UE either. This makes difficult for the network to know or deduce which execution condition (CHO or CPAC or both) is causing the stalling or the failure in the execution of CHO w/ candidate SCG.

Observation 1: in case SCG failure is detected before, during or shortly after the execution of CHO w/ candidate SCGs, the only report generated is the SCGFailureInformation, which only contains PSCell measurements.

In order to aid the network in determining which execution condition/event needs reconfiguration to avoid failures in the future, the SCGFailureInformation message could also be enhanced with the same information as the RLF report in this case.

Proposal 2: In case RLF is detected on the SCG before, during or shortly after successful completion of CHO w/ candidate SCG, the SCGFailureInformation message should be enhanced with the following information:

- An indication of the first fulfilled event/execution condition (CHO or CPAC)

- Additional information on the state of the execution event that was not triggered, e.g. was TTT running for this event or not

- Time duration between the two execution conditions being fulfilled when both have been fulfilled

# 3	Conclusion

This document has made the following observation and proposals:

Proposal 1: In case RLF on MCG is detected before execution of CHO w/ candidate SCG, the RLF report should be enhanced with additional information on the state of the execution event that was not triggered, e.g. whether TTT was running for this event or not.

Observation 1: in case SCG failure is detected before, during or shortly after the execution of CHO w/ candidate SCGs, the only report generated is the SCGFailureInformation, which only contains PSCell measurements.

Proposal 2: In case RLF is detected on the SCG before, during or shortly after successful completion of CHO w/ candidate SCG, the SCGFailureInformation message should be enhanced with the following information:

- An indication of the first fulfilled event/execution condition (CHO or CPAC)

- Additional information on the state of the execution event that was not triggered, e.g. was TTT running for this event or not

- Time duration between the two execution conditions being fulfilled when both have been fulfilled