3GPP TSG-RAN WG2 Meeting #128	R2-2409984

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.10.2

Source:	Nokia

Title:	MRO for LTM

WID/SID:	NR_ENDC_SON_MDT_Ph4-Core

Document for:	Discussion and Decision

# Introduction

The following agreements were reached on the topic of MRO for LTM at and RAN2#126:

- RAN2#125bis

- RAN2 considers SHR, RA report and RLF for MCG LTM SON.

- RAN2#127

RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch. FFS details, e.g. if explicit or implicitly signalled.

RA-based access will not be a new triggering condition for the SHR report.

HO interruption time will not be a new triggering condition for the SHR report.

Unless RAN3 defines a NW-based solution: The UE logs and reports whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined).

Include an explicit indicator in SHR whether the successful LTM execution was RACH-less or RACH-based. Can sort out the details during stage-3 implementation.

This paper discusses details regarding the optimization of early TA acquisition and possible SHR enhancements.

# 2	Early TA acquisition optimization

The main rational behind and strength of LTM is the RACH-less cell change, which is only possible with knowing the timing advance (TA) when cell switch is triggered. One critical part of the LTM is therefore the phase called early TA acquisition, which the UE may be instructed to perform before receiving the cell switch command. If the ETA is successful, the interruption time during the cell switch will be significantly reduced as RA can be skipped. If the UE received the cell switch command that does not include a valid TA previously acquired for the target cell, the UE can still access the cell via regular RA procedure, but the benefits of LTM compared to the legacy L3 handover types are lost.

In case of the fully defined network-triggered early TA acquisition, the two trigger criteria for early TA acquisition and cell switch command need to be properly synced for a successful RACH-less LTM. Firstly, the TA once acquired may become invalid because of changed UE position or changed propagation conditions with respect to the candidate cell and the network will need to trigger early TA acquisition again. Secondly, the early TA acquisition procedure should not be performed too often as it is associated with some interruption time.

Observation 1: Timely and correct early TA acquisition is a crucial part of LTM to guarantee the benefits of LTM, namely a RACH-less cell switch for minimum handover interruption time.

For example, the network should avoid unnecessary early TA acquisition (e.g. obtain the TA for cells that are then not used for cell switch) and also identify instances where the TA was acquired but became invalid by the time cell switch command was sent.

Proposal 1: RAN2 to log additional information regarding TAs that were unnecessarily acquisitioned.

In other cases, the TA may be obtained and included in the cell switch command but upon UE’s attempt to use it, access to the target cell fails. The UE then has to switch RACH based access via LTM recovery/re-establishment procedure when new TA value is provided from network in RAR message. For such cases, reporting of the information about the difference between the TA reported in the Cell switch command for which the RACH-less LTM failed, and TA received consequently in RAR for recovery/re-establishment cell may help to optimize the wrong cell switch timing.

Proposal 2: RAN2 to discuss adding the following information into RLF report: TA difference between TA provided by network in Cell Switch Command for which the RACH-less LTM failed, and TA received in RAR for recovery/re-establishment cell.

# 3	 Enhancement of SHR

Successful HO report (SHR) was introduced in Rel-17 to capture near failure events that happen during a successful HO. SHR is triggered based on running timers (T310/T312/T304) exceeding a preconfigured threshold. In some cases, additional SHR triggering conditions have been defined. For example, in case of DAPS HO, SHR is also generated if the UE experiences an RLF on the source link during the DAPS HO as this is not the desired behaviour.

One optimization aspect with respect to LTM is the unnecessary configurations. As a reminder, in LTM, the UE may receive multiple configurations but will only execute one of them once it received the cell switch command. Moreover, the UE does not release the configuration and the prepared candidate PCells after LTM cell switch execution or LTM recovery execution. This enables fast triggering of cell changes and reduces the signalling overhead that is associated with the preparation of the LTM configurations.

There is currently however no way for the network to detect any unnecessary configurations and implicitly blocked resources. Information that would help the network to identify these cases could be added to the SHR.

Proposal 3: RAN2 to enhance SHR with new information and triggering criteria that allow the network to identify unnecessary LTM configurations.

Currently, no new LTM specific SHR triggers were agreed in RAN2 and some like RACH based access and handover interruption time were even excluded. However, we believe that, in case of RACH-less the ACK for the first UL transmission taking longer than expected would constitute an undesired behaviour (near failure scenario). The LTM handover will still be successful (e.g. by means of fallback to RACH-based access or since the number of RLC retransmissions remain below the maximum admissible number) but the interruption time would be significantly increased.

Observation 2: The ACK for first UL transmission taking longer than expected during an LTM HO represents an undesired behaviour.

Proposal 4:  RAN2 to define a threshold related to number of RLC retransmissions of the first UL transmission in RLC AM for RACH-less LTM as new SHR trigger.

# 4	Conclusion

This document has made the following observations and proposals:

Observation 1: Timely and correct early TA acquisition is a crucial part of LTM to guarantee the benefits of LTM, namely a RACH-less cell switch for minimum handover interruption time.

Proposal 1: RAN2 to log additional information regarding TAs that were unnecessarily acquisitioned.

Proposal 2: RAN2 to discuss adding the following information into RLF report: TA difference between TA provided by network in Cell Switch Command for which the RACH-less LTM failed, and TA received in RAR for recovery/re-establishment cell.

Proposal 3: RAN2 to enhance SHR with new information and triggering criteria that allow the network to identify unnecessary LTM configurations.

Observation 2: The ACK for first UL transmission taking longer than expected during an LTM HO represents an undesired behaviour.

Proposal 4:  RAN2 to define a threshold related to number of RLC retransmissions of the first UL transmission in RLC AM for RACH-less LTM as new SHR trigger.