3GPP TSG RAN WG2 Meeting #128			R2-2410037

Orlando, US, Nov. 18th – 22nd, 2024

Agenda item:	8.3.2

Title:	Discussion on cluster based RRM measurement prediction

Source:	Beijing Jiaotong University (BJTU)

Document for:	Discussion

1. Introduction

In the 3GPP TSG-RAN WG2 #126 meeting, [1] proposed cluster approach for RRM measurement prediction and introduced its relationship between model input and output in temporal and spatial prediction. According to the summary of the 126th meeting [2], there are two main discussion points for cluster approach.

- whether we should also report some information relevant to cluster approach. Apart from two company (Huawei, Apple), other companies are fine with proposal from NTT docomo. Since RAN2 agreed at last meeting that “To define cluster approach at least based on the number of input cells, number of output cells and their relationship in temporal domain, spatial domain and frequency domain”. So rapporteur recommend to let company report as part of the “model input” and “model output”, if any.

- Huawei and ZTE raise the point that some of the parameters hidden in the Note 4/5/6 under table 1 should be listed in the template explicitly to align among company. Rapporteur recommend some of them on the reflector, where apart from Apple there is no negative comment is received.

Previous meetings have determined that evaluation scenario combination of FR2 to FR2 intra-frequency temporal domain case A has a high priority. And in RAN2 #127 meeting, the definition of the intra-frequency temporal domain case A was completed.

- In case A, once the measurement results in prediction window are predicted, the observation window and prediction window move forward with either one sampling period (L1/L3 filtering option 1) or one measurement period (L1/L3 filtering option 2).

3GPP TSG-RAN WG2 #127-bis meeting had a heated discussion on cluster-based and cell-specific RRM prediction methods.

-	Oppo thinks it is more complex and lower performance so it should be considered with lower priority.

-	Samsung agrees that it is more complex but if this complex model is trained well it can outperform the cell specific one. There is tradeoff between complexity and performance.

-	Mediatek disagrees with Oppo and thinks that cluster can outperform.

-	NTT Docomo thinks that even the model is more complex we can obtain more outputs in one shot and cell specific we have to run it multiple times t get those outputs.

-	Qualcomm thinks that it should be up to the companies whether they use cluster based approach to make better prediction.  It is very useful especially for mobility.

-	Ericsson thinks that it really depends on which cells we combine, correlated cells will give us better results. Nokia agrees and it hasn’t been fully studied, also simulation results have been agreed.

-	ZTE and Apple thinks it is too early to conclude as not many companies have provided results.

-	Apple thinks that cluster based approach it means different things to people, multiple input/output, so companies should clarify which flavor they are using.

-	CATT has only adopted the cluster based approach as it is more useful and practical.

-	Mediatek thinks that the current table captures input/out and you can deduct the cluster based approach from that information.

In this paper, in order to solve complexity challenges of AI models in cluster-based approaches, we propose a novel cluster-based RRM measurement prediction method for FR2 to FR2 intra-frequency temporal domain case A. We evaluate the performance of this algorithm from two distinct perspectives: L3 RSRP difference and system-level KPI.

2. Motivation

Currently, there are two main AI-based RRM measurement prediction methods for network triggered L3-based handover. One is cell-specific/per-cell approach. The AI-based cell handover approach is to train a model for each cell to predict the future RSRP of that cell, and then deploy multiple trained models to the network side. This AI-based cell handover prediction method has a simple idea, but its shortcomings lie in the high deployment and management overhead of AI models, and it is difficult to inherit the trained models for different terminal locations, so it will encounter great obstacles in practical application. Another approach which we call cluster-based approach is to use an AI model to predict the future RSRP of multiple adjacent cells based on cell-level/cluster-level RSRP measurement information. This approach significantly reduces the deployment and management overhead associated with AI models while enhancing prediction accuracy by leveraging the spatial correlation of channels across different cells. However, as the number of predicted cells grows, the complexity of AI models and the beam measurement overhead increase substantially. To address this, we propose a novel cluster-based temporal prediction approach, which effectively reduces the frequency of beam measurements and the computational complexity of AI models.

3. Scenarios description

Fig. 1 The scenario of mobility management

Without loss of generality, we consider a FR2 mmWave wireless communication scenario with a user equipment (UE) and seven base stations (BS) shown in Fig.1. Each BS is equipped with three half-wavelength spaced uniform plane arrays (UPAs) of 32 antennas with sector orientation of 30, 150 and -90 degrees, respectively, and we consider a downlink communication system with analog beamforming architecture for each UPA. Meanwhile, the UE is equipped with two half-wavelength spaced antennas without beamforming and moves according to the city street model. The distance between base stations is set to 200m, and the channel model is referenced to 3GPP TR 38.901 UMa for LOS propagation.

Table 1 Simulation setting

| Parameters | Values |
| --- | --- |
| Channel | UMa |
| Bandwidth | 80MHz |
| Subcarrier bandwidth | 120KHz |
| Number of subcarriers | 64 |
| Center frequency | 30GHz |
| Antenna setup of BS | （M, N, P, Mg, Ng）=（4, 8, 1, 1, 1） |
| Height of BS | 25m |
| Antenna setup of UE | （M, N, P, Mg, Ng）=（2, 1, 1, 1, 1） |
| Height of UE | 1.5m |
| UE distribution | 100%outdoor |
| UE speed | 60km/h、120km/h、350km/h、500km/h |
| UE motion trajectory | City street |
| Transmitting power of BS | 40dbm |
| Number of sites | 7 |
| Number of sectors in a site | 3 |
| Number of UE | 1 |
| Sampling interval | 40ms |
| Distance between BSs | 200m |

4 The cluster-based temporal prediction approach

At the RAN2 125 meeting, the following three cases are agreed for cell level measurement prediction:

- Case 1: To predict beam level results, then generate cell level results based on the predicted beam results.

- Case 2: To directly predict cell level results based on cell level results.

- Case 3: To directly predict cell level results based on beam level results

Different cases have different AI model’s input and output, and have different advantages and disadvantages. For case 1, using L1-RSRP as both the input and the output of the AI models. The AI model output is then filtered to derive the L3-RSRP, which is used to execute cell handovers. For this option, the output L1-RSRP of AI model can be applied to both beam-level and cell-level mobility management. Nevertheless, L1-RSRP encompasses extensive information that exceeds the requirements for handover, while the correlation between the input and output L1-RSRP is intricate, thereby amplifying the learning challenge for the AI models. For case 2, the input for AI models is the L3-RSRP of each cell at historical instances, with the output being either the L3-RSRP of each cell or the L3-RSRP of the top K cells at future TTT instances. The dimension of L3-RSRP information is lower than that of L1-RSRP. However, acquiring L3-RSRP, which requires averaging data over multiple time slots, introduces extra latency which is not desired in wireless communications. For case 3, leveraging L1-RSRP to forecast L3-RSRP via AI can be an advantageous strategy. This method reaps the benefits of the above two approaches, making use of the abundant data in L1-RSRP to enhance the AI model's prediction accuracy, while also streamlining the mapping between the model's input and output, which is good for mobility management.

Here gives a detailed description of the AI model’s input and output of case 3. In literature, the input into AI models is the RSRP of beam pairs of all cells, leading to significant measurement overhead and computational complexity. To this end, we consider two approaches to reduce the measurement overhead for cluster based RRM measurement prediction. For the first approach, all beams from partially selected cells are used to predict the L3-RSRP for all cells. For the second approach, partially selected beams of each cell are used to predict the L3-RSRP for all cells. We utilize the partial L1-RSRP from the past six instances to predict the L3-RSRP in the next four instance.

We employ CNN-based and LSTM-based models as shown in Fig.2. The LSTM-based model comprises four LSTM layers and each layer has 168 hidden units. The features extracted by the LSTM network undergo further processing via an FC layer and a sigmoid function for feature mapping. To enhance the performance of the LSTM network, we adopt four identical networks to predict L3-RSRP for four future instances separately. For the CNN-based model, a set of regular 2D convolution layers with 12, 24 and 32 kernels are employed before feeding it into two FC layers. To avoid overfitting during the training process, max pooling layers are incorporated after each CNN layer.

Fig. 2 The architecture of cluster based RRM prediction[3].

In the end, we will introduce the training methods of AI models for the above method. To enhance the prediction performance of the LSTM-based model, the output of this neural network is the predicted RSRP of all cells at a future moment, thus 4 networks need to be trained separately to predict 4 moments in the future. For CNN-based model, only one network needs to be trained to output RSRP of all cells in upcoming 4 moments.

Proposal #1: For cluster-based temporal prediction approach, consider using case 3 (To directly predict cell level results based on beam level results) as AI models’ input and output for mobility management.

5 Simulation and results

The channel used in our experiments is generated by QuaDRiGa. We generate four datasets corresponding to various UE speeds of 60 km/h, 120 km/h, 350 km/h, and 500 km/h. Each dataset comprises 50,000 samples, which are split into training, validation, and test sets in a 3:1:1 ratio. For the proposed two approaches with equidistant sampling, L1-RSRP of 50% beams in each cell or all beams in 50% cells are selected as input to the AI model. The batch size is 256 and the maximum epoch is 800. The learning rate dynamically changes from10-3 to 10-8, utilizing the adaptive moment estimation optimizer. The RSRP difference is one evaluation metric of prediction accuracy for cell-level mobility management, which reflects the disparity between the predicted L3-RSRP and the actual L3-RSRP of all cells. In addition, we also consider the RLF rates as the system-level evaluation metric to illustrate the overall handover performance of the communication system.

The RSRP difference between the AI predictions and the actual measurements is shown in Fig. 3. The experimental results show that both CNN-based and LSTM-based models show excellent cluster-level temporal prediction performance. Table 2 shows the significantly reduced RLF rates in AI-assisted cell handover processes. Remarkably, the proposed two approaches to reduce the measurement overhead labeled as Part_Cell and Part_Beam, achieve comparable RLF performance with the traditional approach using all beam measurements of all cells, labeled as All_Beam_Cell, while we can save 50% beam measurement overhead. More detailed network architecture and simulation experiment results are introduced in [3]

Table 2 Comparison of the RLF rates and complexity of AI-based and Non-AI cell handover methods [3]

Fig. 3 RSRP difference of AI models [3]

Proposal #2: Consider using all beams from partially selected cells or partially selected beams of each cell as AI models’ input for cluster-based temporal prediction approach.

6 Conclusion

In this paper, we introduce a cluster-based temporal prediction approaches for L3 handovers, focusing on the input, output, network structure and training methods of AI/ML models. Based on the discussion above, we propose the following proposals:

Proposal #1: For cluster-based temporal prediction approach, consider using case 3 (To directly predict cell level results based on beam level results) as AI models’ input and output for mobility management.

Proposal #2: Consider using all beams from partially selected cells or partially selected beams of each cell as AI models’ input for cluster-based temporal prediction approach.

7. References

[1] R2-2404700, ‘Discussion on RRM measurement prediction’.

[2] R2-2406085, ‘Summary of [POST126][031][AIMob] Simulations (OPPO)’.

[3] W. Li, W. Chen, S. Wang, Y. Zhang, M. Matthaiou, and B. Ai, “Ai-based beam-level and cell-level mobility management for high speed railway communications,” 2024. [Online]. Available: https: //arxiv.org/abs/2407.04336