3GPP TSG-RAN WG2 Meeting #128                                    	R2-2410048

Orlando, USA, Nov 18th – 22th, 2024

Agenda item:	8.8.2

Source:	Qualcomm Incorporated

Title:	Discussion on cell DTX

Document for:	Discussion and Decision

# Introduction

In RAN#127, following agreements were made on DL coverage enhancements.

Agreements:

1.	From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects)

2.	From RAN2 point of view, If the SSB periodicity is larger than 160ms, for example ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 may need to be extended. And the field description of nAndPagingFrameOffset may need to be enhanced to consider the SSB periodicity higher than 160ms.

3.	RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity

4.	If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

In RAN2#127bis, following agreements were made.

Agreements:

1.	If it turns out that there is a need to bar UEs not supporting DL-CE, Rel-19 UEs not supporting DL-CE can be barred from accessing a cell operating with DL-CE using the existing NTN bar bit, in the same way as pre-Rel19 NTN UEs (this is an extension of the previous agreement to include also Rel-19 UE not supporting DL-CE)

2.	If it turns out that there is a need to bar UEs not supporting DL-CE, then we need to introduce a barring mechanism to control access of UEs supporting Rel19 NTN DL-CE. FFS on the details. (This also implies that UEs supporting Rel19 NTN DL-CE will not consider the existing NTN barring bit)

3.	(also depending on the details of the RAN1 solution) we can further consider methods to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE.

In this document, we cell DTX aspect for NTN.

# Discussion

Due to power budget limitation at satellite, all beams may not be possible to be active at the same time. It is also possible that which beam needs to be turned off at a time t1 may depend on what is the traffic load around or before time t1. As discussed in [1], TDM pattern is an effective way to enable flexible power sharing among satellite beams and it allows the options of using multiple patterns.

In scenarios where beams must be alternated to manage power effectively, it becomes crucial to implement a dynamic and adaptable system. This ensures the optimal utilization of power resources while maintaining the integrity and performance of NTN. The TDM pattern's ability to accommodate multiple configurations provides a level of flexibility that is essential for addressing the variable traffic demands and power constraints inherent in satellite operations.

Figure 1 Cell DTX for different UEs.

RAN2 received LS response from RAN1 in [2] and has clarified following.

Question4 : RAN2 would like to remind RAN1 that satellite beams are currently not visible to UEs and any decision about different beam status (i.e. "off", "common messages only" and "active traffic") will likely have to relate to beams visible to the UE (e.g. SSB beams). RAN2 would also like to know whether RAN1 intends to define beam status for beams not visible to the UE or to define new beam status for beams visible to the UE.

Answer to Q4:

Beam footprint status in terms of "off", "common messages only" and "active traffic” in one of the RAN1 agreements is only defined for the sake of system-level evaluation methodology.

Therefore, it may be better RAN2 to assume the support of the case where NTN cell DTX is complete cell off, i.e., all broadcast signals are off and cell completely “on” pattern. This can be revisited if needed based on RAN1 progress.

RAN2 assume that new NTN cell DTX with ON and OFF pattern of the beam is supported.

As mentioned earlier, a single semi-static cell DTX pattern may not be efficient. A dynamic cell DTX pattern based on traffic load of the beams can be more efficient. For this, dynamic DCI based cell DTX activation/deactivation or multiple cell DTX patterns can be considered.

Rel-18 NES supports a single DTX pattern per serving cell, however, it already supports DCI-based activation and deactivation of the DTX pattern. This mechanism can also be extended for NTN scenario. Dynamically one beam may be deactivated with cell DTX or activated with a new cell DTX pattern depending on traffic load and power sharing management. We can consider the DCI-based activation/deactivation command for both cases when the cell configures UE with multiple DTX pattern or just a single DTX pattern. For IDLE mode, similar mechanism defined for PEI in wakeup signal can be considered. UE can monitor the DCI in its paging occasion. How to enable the activation/deactivation for UEs in RRC_IDLE or RRC_INACTIVE can be further discussed.

RAN2 assumes dynamic DCI-based activation/deactivation of NTN cell DTX patterns depending on beam/cell traffic load is supported. FFS on details.

RAN2 should also consider impact of cell DTX on legacy UEs, neighbor cell measurement and initial access delay. For UEs in RRC_IDLE, the DTX pattern should minimize the delay in initial access. Cell DTX pattern can also affect the neighbor cell measurements specially in RRC_IDLE. It can also affect the system information update procedure.

RAN2 study the impacts of cell DTX on system information change, UE measurements and initial access delay.

# Conclusion

Following proposals are made.

Proposal 1	RAN2 assume that new NTN cell DTX with ON and OFF pattern of the beam is supported.

Proposal 2	RAN2 assumes dynamic DCI-based activation/deactivation of NTN cell DTX patterns depending on beam/cell traffic load is supported. FFS on details.

Proposal 3	RAN2 study the impacts of cell DTX on system information change, UE measurements and initial access delay.

# References

[1] R1-2403211, “Downlink coverage enhancement for NR NTN”, Changsha, China, April 15th – 19th, 2024.

[2] R2-2407919, “Reply LS on DL coverage enhancements”, Hefei, China, 14th - 18th October 2024.