3GPP TSG-RAN WG2 Meeting #128                                    	R2-2410051

Orlando, USA, Nov 18th – 22th, 2024

Agenda item:	8.9.4

Source:	Qualcomm Incorporated

Title:	Discussion on PWS in NB-IoT NTN

Document for:	Discussion and Decision

- Introduction

Following agreements were made in RAN2#127bis meeting.

Agreements:

1.	RAN2 confirms the understanding that this WID objective covers all PWS services, including ETWS and CMAS

2.	The support for PWS introduced for NB-IoT NTN can also be made applicable for NB-IoT in TN, if this does not require additional NB-IoT TN specific changes.

3.	For NB-IoT NTN PWS, introduce the following new SIBs, taking the content within the corresponding LTE SIBs as a baseline (but also checking whether we can have some optimization/ skip some unnecessary fields):

- SystemInformationBlockType10-NB for primary ETWS notification;

- SystemInformationBlockType11-NB for secondary ETWS notification;

- SystemInformationBlockType12-NB for CMAS notification.

4.	Add the following PWS indication in direct indication information for NB-IoT (Come back in the next meeting on whether this is also added in Paging-NB):

- etws-Indication;

- cmas-Indication.

5.	Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE acquires the corresponding PWS message immediately (can come back to clarify further what immediately means)

6.	Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases)

In this document, we provide details on remaining aspects of PWS in NB-IoT.

- Discussion

Currently NB-IoT UEs do not support “acceptable cell”, “Camped on Any cell state” and “emergency call” functionalities. NB-IoT UEs cannot also read broadcast notification message in RRC_CONNECTED. When PWS is supported in a cell (i.e., in NB-IoT RAT), the UE and its user need to be aware of the network capability.

## 4.4 NB-IoT functionality in Idle Mode This specification is applicable to NB-IoT, except for the following functionality which is not applicable to NB-IoT: - Acceptable cell - Accessibility measurements - Access Control based on ACDC categories - Camped on Any cell state - CSG, including support for manual CSG selection and CSG or Hybrid cell related functionality in PLMN selection, or HNB name (SIB9), Cell selection and Cell reselection. - Emergency call - E-UTRAN Inter-frequency Redistribution procedure - Inter-RAT Cell Selection and Reselection including measurements in other RATs - Logged measurements - Mobility History Information - Mobility states of a UE - Priority based reselection - Public warning system including CMAS, ETWS, PWS.

In NB-IoT, cell reselection is based on ranking regardless of frequency priorities. If a UE is PWS capable and camps in a cell that does not support PWS while there are neighbor cells supporting PWS, then UE implementing such important feature will not be able to take benefit and is going to miss the warning message. Therefore, we think UE needs to be aware of the PWS capability of the cell to prioritize cell search and PLMN search.

Introduce a new indication that a NB-IoT cell supports PWS feature.

Currently, the eMTC UE supports the direct indication for ETWS/CMAS notification. This is missing for NB-IoT. For NB-IoT, the reserved bits 3 and 4 can be used for ETWS and CMAS indications.

| Table 6.6-1: Direct Indication information using P-RNTI for non-NB-IoT Bit; Direct Indication information. 1; systemInfoModification. 2; etws-Indication. 3; cmas-Indication. 4; eab-ParamModification. 5; systemInfoModification-eDRX. 6; uac-ParamModification. 6, 7, 8; Not used, and shall be ignored by UE if received.. | Table 6.7.5-1: Direct Indication information for NB-IoT Bit; Field in Direct Indication information. 1; systemInfoModification. 2; systemInfoModification-eDRX. 3, 4, 5, 6, 7, 8; Not used, and shall be ignored by UE if received. |
| --- | --- |

For eMTC, the PWS notification can also be provided via paging message and it can be discussed whether this needs to be added in paging message for NB-IoT since reading PDSCH for paging can add additional delay and resource. For NB-IoT, Direct Indication information may be sufficient.

For PWS notification, indications in paging message, i.e., in Paging-NB is not considered.

Currently Cat M UEs and NB-IoT UEs are not required to detect the system information change or monitor paging in RRC_CONNECTED. In Rel-16, PWS notification (ce-ETWS-CMAS-RxInConn-r16) in RRC_CONNECTED was introduced but it was mainly intended for high end non-BL UEs operating in CE mode where the UE is able to monitor SI-RNTI in RRC_CONNECTED to receive the ETWS-CMAS notification.  For Cat M and NB-IoT UEs, network can also release the UEs to IDLE mode if there is emergency message.

#### 4.3.29.13 etws-CMAS-RxInConnCE-ModeA-r16 This field indicates whether the UE supports ETWS/CMAS indication reception in RRC_CONNECTED state when the UE is operating in coverage enhancement mode A as specified in TS 36.331 [5]. A UE indicating support of etws-CMAS-RxInConnCE-ModeA-r16 shall also indicate support of ce-ModeA-r13. This feature is only applicable if the UE supports a UE Category other than Category M1 and M2. #### 4.3.29.14 etws-CMAS-RxInConnCE-ModeB-r16 This field indicates whether the UE supports ETWS/CMAS indication reception in RRC_CONNECTED state when the UE is operating in coverage enhancement mode B as specified in TS 36.331 [5]. A UE indicating support of etws-CMAS-RxInConnCE-ModeB-r16 shall also indicate support of ce-ModeB-r13. This feature is only applicable if the UE supports a UE Category other than Category M1 and M2.

In our view, the WID objective scope is limited only to RAN2. NB-IoT UEs monitoring additional RNTIs in connected mode may require RAN1 work as new common search space may be needed. If NB-IoT supports it in connected mode, we think this can be applicable to cat M1 and M2 UEs, therefore, this issue should be addressed for both NB-IoT and Cat M UEs together.

ETWS/CMAS in RRC_CONNECTED is not considered in Rel-19.

In TS 36.306, currently it is specified in section 6.2 that PWS reception is optional for UE without UE capability signaling. We think this text should be applicable for NB-IoT NTN and no further specification change is expected in TS 36.306.

## 6.2 PWS features ### 6.2.1 ETWS It is optional for UE to support ETWS reception as specified in TS 36.331 [5]. ### 6.2.2 CMAS It is optional for UE to support CMAS reception as specified in TS 36.331 [5]. It is optional for a CMAS-capable UE to support Geofencing information (warningAreaCoordinates-r15) as specified in TS 36.331 [5]. ### 6.2.3 KPAS It is optional for UE to support KPAS reception as specified in TS 36.331 [5]. The Korean Public Alert System (KPAS) uses the same AS mechanisms as defined for CMAS. Therefore a KPAS-capable UE shall support all behaviour that is included in TS 36.331 [5] and TS 36.304 [14] for a CMAS-capable UE. ### 6.2.4 EU-Alert It is optional for UE to support EU-Alert reception as specified in TS 36.331 [5]. The Europearn Union Warning System EU-Alert uses the same AS mechanisms as defined for CMAS. Therefore a EU-Alert-capable UE shall support all behaviour that is included in TS 36.331 [5] and TS 36.304 [14] for a CMAS-capable UE.

It is optional for NB-IoT UEs to support PWS feature without UE capability signaling. Add PWS (clause 6.2) as applicable in NB-IoT NTN in TS 36.306.

- Conclusion

Following proposals are made:

Proposal 1	Introduce a new indication that a NB-IoT cell supports PWS feature.

Proposal 2	For PWS notification, indications in paging message, i.e., in Paging-NB is not considered.

Proposal 3	ETWS/CMAS in RRC_CONNECTED is not considered in Rel-19.

Proposal 4	It is optional for NB-IoT UEs to support PWS feature without UE capability signaling. Add PWS (clause 6.2) as applicable in NB-IoT NTN in TS 36.306.