3GPP TSG-RAN2 Meeting #128	R2-2410086

Orlando, US, 18 - 22 November 2024	Revision of R2-2409059

Agenda Item:	8.4.3 RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Source: 	Ericsson

Title:  	LP-WUS and RRM measurements

Document for:	Discussion and Decision

# Introduction

In this contribution LP-WUS and RRM measurements are discussed further.

For the RAN2 and RAN4 agreements the reader is referred to the Annex.

# Discussion

## MR serving cell measurements offloading and relaxation

RAN4 agreed to study different use cases for MR offloading and relaxation:

Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |
| #4 Relaxed case c | Off | On, FFS the condition and the details | ON |

The WID objective includes further MR serving and neighbour cell measurement relaxation (RP-241824):

- Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

RAN2 agreed that from RAN2 perspective MR serving cell measurements offloading/relaxation is only supported when the UE supports WUR:

RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs.

First RAN2 should clarify and agree what fully offloading exactly means:

When the LR entry condition is fulfilled and the LR performs serving cell measurements, then the MR is not required to perform serving cell measurements (i.e. RAN4 scenario #1 is supported).

When the MR serving cell measurements can be “fully” offloaded to the LR, then there is no strong need for “partial” offloading, i.e. to support “relaxed MR serving cell measurements” while the LR performs continuous serving cell measurements:

When the LR entry condition is fulfilled, the LR performs serving cell measurements and the MR does not perform relaxed neighbour cell measurements, then the MR does not perform relaxed serving cell measurements (RAN4 scenario #2 is not supported).

RAN4 agreed that MR and LR serving cell measurements cannot be combined:

Agreement:

- From RAN4 requirement perspective, combine measurement results from different receivers shall not be considered

This limits the need to support RAN4 case #3, i.e. when the entry condition is fulfilled and the LR performs serving measurements then the MR performs serving cell measurements are when it performs relaxed neighbour cell measurements. In case #3 the MR is performing relaxed neighbour and serving cell simultaneously, however:

- This is possible when the MR performs relaxed intra-frequency measurements, i.e. serving cell measurements come for free in such case. This is not the case for relaxed inter-frequency neighbour cell measurements.

- The MR serving cell measurements cannot be combined with LR serving cell measurements, and it is unclear how the UE shall then use these relaxed MR serving cell measurements that are not performed every DRX cycle.

There can be use cases where the MR and LR have to perform serving cell measurements simultaneously. For example when MR measurements are used to retune the LR timing/frequency e.g. when the LR quality is bad [1]. But in such cases the MR performs serving cell measurements continuously every DRX cycle, i.e. they are not relaxed. And perhaps relaxed MR serving cell measurements performed during intra-frequency neighbour cell measurements can be used to assist the LR exit condition, but how to use these MR serving cell measurements can be left to UE implementation. Furthermore it is assumed that cell reselection is performed based on MR measurements only and for cell reselection both serving and neighbour cell measurements are required. But we do not see a need for “partial” serving cell offloading in addition to “fully” offloading:

RAN2 to discuss whether partial MR serving cell measurement offloading needs to be supported.

Perhaps it can be left to UE implementation to use MR serving cell measurements when the MR has to perform intra-frequency neighbour cell measurements for the LR exit condition:

When the LR entry condition is fulfilled and the LR performs serving cell measurements, then it is up to UE implementation (if available) to use MR serving cell measurements for the LR exit condition.

## Entry/exit condition for LR serving cell measurements

RAN2 agreed for the entry/exit condition for LP-WUS monitoring:

Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed.

Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds.

The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ.

The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s).

The LPWUS monitoring exit condition does not include MR measurements.

RAN2 agreed for the entry/exit condition for MR serving cell measurement fully offloading:

For serving cell measurement offloading (i.e., there is no serving cell measurement by MR):

    - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

    - The exit condition is based on the LR measurement results.

There is no power saving gain when the entry condition for offloading is fulfilled but the entry condition for LP-WUS monitoring is not fulfilled, i.e. the MR still have to wake-up every DRX cycle to monitor paging. And it is also not motivated why the entry/exit condition for offloading and LP-WUS monitoring should be different, because the same type of physical signal is used (LP-SS and LP-WUS):

The same entry/exit condition is used for serving cell measurement offloading and LP-WUS monitoring.

Details can be further discussed during stage 3 when the ASN.1 structures are discussed.

RAN2 has not agreed yet on what the UE is required/allowed to do when the LR exit condition is fulfilled:

When the LR exit condition is fulfilled, then the UE shall stop using LR measurements and the MR is required to start serving cell measurements.

In case of LR entry, the UE should not perform MR and LR measurements in sequence, i.e. start LR measurements when the MR is above the MR entry threshold. This would further delay the LR entry. It is beneficial when the MR and LR measurements are performed in parallel:

If an LP-SS threshold is configured, a threshold below the MR serving cell threshold of the entry condition can be configured where the UE is required to start LP-SS measurements.

To prevent continuous LR measurements it is be left to UE implementation for how long to continue the LR measurements when the LR entry condition is not fulfilled, and when to start LP-SS measurements again.

## MR neighbour cell measurements

RAN2 agreed to use the Rel-16 relaxation criteria as a baseline for neighbour cell relaxation:

RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

### Legacy neighbour cell measurement rules

One interesting question is whether LP-WUS can be deployed using the legacy measurement rules for intra-frequency (Sintrasearch) and inter-frequency (SnonIntraSearch) measurement threshold. Typically the UE has to perform neighbour cell measurements in the complete cell and inter-frequency measurements are only triggered when the UE is near the cell border:

The cell border is practically determined by when the UE performs intra-frequency cell reselection, e.g. a neighbour becomes 4 dB better than the serving cell. In case the UE does not find a better intra-frequency neighbour and ends up in poor coverage, then the UE is triggered to perform inter-frequency measurements and find a suitable cell on another frequency. When the UE finds a suitable cell on a higher priority frequency during background measurements, then the UE can leave the serving cell at any time.

It is preferred that the UE performs intra-frequency cell reselection (shorter interruption and missed paging probability) and that inter-frequency measurements (which take longer time and consume more power) are deferred after intra-frequency measurement attempts are “exhausted”. Typically these inter-frequency measurements are triggered when the UE is in poor coverage and a better intra-frequency neighbour has not been found.

In urban environments the intra-frequency (Sintrasearch) cannot be configured close to the inter-frequency (SnonIntraSearch) measurement threshold, because due to the radio propagation conditions a stronger intra-frequency neighbour cell may be detected close to the cell center:

In urban environments the radio conditions may be also change rapidly and dramatically, e.g. when you turn a corner:

There can be scenarios where the UE has to measure intra-frequency neighbour cells in most of the cell and these intra-frequency measurements cannot be relaxed.

This implies that LP-WUS usage may be limited to a small part of the cell. In case of multi-beam operation the cell coverage may also be beam dependent, i.e. beams of the same cell provide different quality. This may be another reason why intra-frequency cell reselection is triggered:

In some scenarios the UE is required to perform intra-frequency measurements in most of the cell and these intra-frequency measurements cannot be relaxed.

For this reason it is beneficial to discuss neighbour cell measurements performed by OFDM-based LR, i.e. the LR also performs also intra-frequency measurements and ranking and the MR is only woken up when cell reselection is initiated:

RAN2 to discuss the need for intra-frequency neighbour cell measurements performed by OFDM-based LR in scenarios where the intra-frequency measurements cannot be relaxed.

Alternatively Rel-16 relaxation when both low mobility and not-at-cell-edge condition are fulfilled can be used, i.e. can prevent the need for LR neighbour cell measurements. This would mean that neighbour cell measurements by LR is only needed when neighbour cell relaxation is required for a mobile UE.

### Rel-16 relaxed neighbour cell measurements

Using the legacy measurements thresholds (SIntraSearch, SNonIntraSearch) and Rel-16 threshold (Not-at-cell-edge) different areas can be identified with different measurement requirements, that also depend on the UE mobility (lowMobility):

Note: the thresholds in the example above depend on NW configuration.

The LP-WUS coverage is independent from the neighbour cell measurements areas depicted above, i.e. LP-WUS can cover the complete cell or only a small portion in the cell centre.

When a UE supporting LP-WUS is outside LP-WUS coverage or LP-WUS is not configured in the cell, then such UE should behave the same as a UE not supporting LP-WUS:

The same Rel-16 RRM relaxation requirements apply when the UE is inside or outside LP-WUS coverage.

The LP-WUS coverage and the areas where the UE is allowed apply Rel-16 RRM relaxation are decoupled.

The OFDM-based serving cell measurements using PSS/SSS are quite similar in latency and performance compared to the MR serving cell measurements, if not identical. In this case the LR can continue to do serving cell measurements using the Rel-16 criteria:

For OFDM-based WUR the LR serving cell measurements can be used to determine when the Rel-16 RRM relaxation criteria are fulfilled.

FFS for OOK-based WUR whether separate Rel-16 neighbour cell relaxation criteria (values) need to be configured.

When the MR is triggered to perform neighbour cell measurements then this is for the purpose of cell reselection and the neighbour cell measurements may have to be compared with the serving cell measurements e.g. check if the intra-frequency neighbour is Q-hyst (db) better than the serving cell:

RAN2 to discuss whether cell reselection is based on MR measurements only, or whether LR measurements can also be used.

### Rel-19 further relaxed neighbour cell measurements

RAN4 started to discuss scaling factors for neighbour cell measurements larger than 3 used in Rel-19. For example scaling factors ≥ 8, which allow the MR to enter ultra-deep sleep.

In the figures below it is exemplified how Rel-19 relaxation fits into the legacy and Rel-16 measurement requirements, for the case when Rel-19 relaxation is allowed above and below the inter-frequency measurement threshold:

It is not clear what the added value of Rel-19 further relaxation because:

When the UE is stationary and above the not-at-cell-edge threshold (Rel-16) then the UE is not required to measure intra- or inter-frequency neighbour cells, i.e. the scaling factor is infinite/much better than the Rel-19 scaling factor.

And in certain scenarios further relaxation may not be feasible. For example in an urban environment where the coverage is very unpredictable and irregular and the propagation conditions can suddenly dramatically change. In such an environment relaxation may not be suitable. Or only allowed for a stationary UE, that while it was mobile did not find a better intra-frequency neighbour cell:

There is a risk that paging/cell reselection is impacted when a moving UE does not measure neighbour cells for more than 10-20 seconds (scaling factor between 8 and 16 with DRX cycle 1.28 sec).

It is not clear why neighbour cell measurements can be relaxed for such a long time, except that such a long time is needed to support ultra-deep sleep and for LP-WUS to be beneficial. Thus LP-WUS should not require Rel-19 further neighbour cell relaxation and the configuration of Rel-19 relaxation should be up to network implementation:

It is up to the network implementation to configure LP-WUS with or without Rel-19 further neighbour cell relaxation. The Rel-19 scaling factor is further discussed in RAN4.

RAN4 is also discussing when the Rel-19 relaxation applies e.g. when the LR entry condition is fulfilled or when the UE is above a new Rel-19 threshold. Rel-19 relaxation should be coupled with LP-WUS and not allowed for a normal UE. The LR entry condition can be viewed as “another MR threshold” which is configurable by the network. In case the network wants to deploy the Rel-19 relaxation conservatively it is possible that the LP-WUS signal can provide a larger coverage then the intended Rel-19 relaxation area. It can be left to network implementation to adjust the LP-WUS configuration, if needed:

If configured, the Rel-19 further neighbour cell relaxation only applies when the LR entry condition for LP-WUS is fulfilled.

Higher priority frequency measurements are essential for load balancing, and they can be relaxed up to 1 hour with Rel-16 relaxation:

Higher priority frequency measurements are not further relaxed.

## RAN2 and RAN4 work split

And RAN4 assumes that RAN2 will discuss offloading/relaxation criteria:

Agreement:

- Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility.

We agree that RAN2 should discuss the offloading/relaxation criteria and that RAN4 should discuss the relaxed measurement requirements. During Rel-16 relaxed RRM measurements in idle/inactive a similar work split was envisioned. However the relaxation criteria, i.e. the possible configuration options and the possible combinations of fulfilment of low mobility and/or not-at-cell-edge were captured in both 38.304 and 38.133. Furthermore the way this was captured was different in 38.304 and 38.133 which made it difficult to compare them:

This duplication has led to many LS exchanges between RAN2 and RAN4, and still possible discrepancies between RAN2 and RAN4 specification are still being discussed [2].

It should be discussed how to avoid duplication of the relaxation criteria in RAN2 and RAN4 specification. One option is that only the “not at cell edge” and “low mobility” criterion are specified in RAN2 specification. And that the different use cases, that depend on what is configured and which criteria are fulfilled is captured in RAN4 specification:

RAN2 captures the relaxation criteria (e.g. “not at cell edge” and “low mobility”) while RAN4 captures the different uses that can be identified based on the configuration and which criteria are fulfilled.

## LP-SS measurement accuracy and latency

In legacy the UE has to take the average of 2 RSRP/RSRQ measurement samples made during consecutive DRX cycles:

Thus the DRX cycle determines the latency in which the UE detects when the serving cell exceeds or drops below a threshold.

In case of OOK-based WUR and LP-SS the measurement latency is independent of the DRX cycle. The assumption is that the UE is required measure 5 samples of LP-SS measurements that are spaced 320 ms apart:

This means that if the DRX cycle is 320 ms, then the LR measurements are 2x slower compared to the MR measurements. RAN1 agreed to support 320 ms periodicity, and other periodicities are FFS. And RAN4 agreed that the measurement delay depends on the LP-SS periodicities:

Agreement

For LP-SS periodicity, support at least 320ms.

- FFS: 80ms,160ms, 640ms,1280ms, 2560ms, 5120ms, 10240ms

Agreement:

Measurement delay requirements for LP-WUR based on LP-SS in IDLE/Inactive mode shall be defined based on LP-SS periodicity.

RAN4 has been discussing how to capture the LP-SS measurement accuracy requirements, e.g. as dedicated accuracy requirements, potentially including test cases, or as a cell reselection margin (see legacy 3dB margin below) to the core requirements:

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined in TS38.304 [1] within Tevaluate,NR_Intra when Treselection = 0 as specified in table 4.2.2.3-1, table 4.2.2.3-2, table 4.2.2.3-3, table 4.2.2.3-4 or table 4.2.2.3-5 provided that:

when rangeToBestCell is not configured:

-	the cell is at least 3dB better ranked in FR1 or 4.5dB better ranked in FR2.

In legacy the UE is required to be able to determine that an intra-frequency neighbour cell is 3 dB better within Tevaluate,NR_Intra in FR1. For LP-WUS a larger margin would need to be defined, and this margin may depend on the LP-SS periodicity. A larger cell reselection margin implies that cell reselection is delayed:

RAN2 to discuss the impact of an increased cell reselection margin when LP-SS measurements or Rel-19 further RRM relaxation are configured.

# Summary

RAN2 is kindly asked to discuss LP-WUS and RRM measurements:

Observation 1	There can be scenarios where the UE has to measure intra-frequency neighbour cells in most of the cell and these intra-frequency measurements cannot be relaxed.

Observation 2	When the UE is stationary and above the not-at-cell-edge threshold (Rel-16) then the UE is not required to measure intra- or inter-frequency neighbour cells, i.e. the scaling factor is infinite/much better than the Rel-19 scaling factor.

Observation 3	There is a risk that paging/cell reselection is impacted when a moving UE does not measure neighbour cells for more than 10-20 seconds (scaling factor between 8 and 16 with DRX cycle 1.28 sec).

Proposal 1	When the LR entry condition is fulfilled and the LR performs serving cell measurements, then the MR is not required to perform serving cell measurements (i.e. RAN4 scenario #1 is supported).

Proposal 2	When the LR entry condition is fulfilled, the LR performs serving cell measurements and the MR does not perform relaxed neighbour cell measurements, then the MR does not perform relaxed serving cell measurements (RAN4 scenario #2 is not supported).

Proposal 3	RAN2 to discuss whether partial MR serving cell measurement offloading needs to be supported.

Proposal 4	When the LR entry condition is fulfilled and the LR performs serving cell measurements, then it is up to UE implementation (if available) to use MR serving cell measurements for the LR exit condition.

Proposal 5	The same entry/exit condition is used for serving cell measurement offloading and LP-WUS monitoring.

Proposal 6	When the LR exit condition is fulfilled, then the UE shall stop using LR measurements and the MR is required to start serving cell measurements.

Proposal 7	If an LP-SS threshold is configured, a threshold below the MR serving cell threshold of the entry condition can be configured where the UE is required to start LP-SS measurements.

Proposal 8	In some scenarios the UE is required to perform intra-frequency measurements in most of the cell and these intra-frequency measurements cannot be relaxed.

Proposal 9	RAN2 to discuss the need for intra-frequency neighbour cell measurements performed by OFDM-based LR in scenarios where the intra-frequency measurements cannot be relaxed.

Proposal 10	The same Rel-16 RRM relaxation requirements apply when the UE is inside or outside LP-WUS coverage.

Proposal 11	The LP-WUS coverage and the areas where the UE is allowed apply Rel-16 RRM relaxation are decoupled.

Proposal 12	For OFDM-based WUR the LR serving cell measurements can be used to determine when the Rel-16 RRM relaxation criteria are fulfilled.

Proposal 13	FFS for OOK-based WUR whether separate Rel-16 neighbour cell relaxation criteria (values) need to be configured.

Proposal 14	RAN2 to discuss whether cell reselection is based on MR measurements only, or whether LR measurements can also be used.

Proposal 15	It is up to the network implementation to configure LP-WUS with or without Rel-19 further neighbour cell relaxation. The Rel-19 scaling factor is further discussed in RAN4.

Proposal 16	If configured, the Rel-19 further neighbour cell relaxation only applies when the LR entry condition for LP-WUS is fulfilled.

Proposal 17	Higher priority frequency measurements are not further relaxed.

Proposal 18	RAN2 captures the relaxation criteria (e.g. “not at cell edge” and “low mobility”) while RAN4 captures the different uses that can be identified based on the configuration and which criteria are fulfilled.

Proposal 19	RAN2 to discuss the impact of an increased cell reselection margin when LP-SS measurements or Rel-19 further RRM relaxation are configured.

# References

- R4-2412505, Discussion on LP-WUS RRM requirement, Ericsson, DISC, RAN4#112

- R2-2407341, Correction to Relaxed measurement, LG, CR 38.304, Rel-16, RAN2#127

# Annex: Agreements

## RAN2 agreements

Serving cell measurements related

For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell.

Neighboring cell measurements related

RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise.

RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

General aspects

RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs.

Serving cell measurment offloading (i.e., there is no serving cell measurement by MR)

For serving cell measurement offloading (i.e., there is no serving cell measurement by MR):

    - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

    - The exit condition is based on the LR measurement results.

RRM relaxation for serving and neighbor cell

- Working assumption

For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’.

FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

## RAN4 agreements (selection)

Agreement:

From RAN4 requirement perspective, combine measurement results from different receivers shall not be considered.

Agreement

RAN4 specifies relaxed UE MR measurement requirements for both serving cell and neighbour cell measurements at idle/inactive mode for UE supporting LP-WUR

Agreement:

RAN4 will further discuss whether any technical issues regarding the LS from RAN1.

From Rel-19 RAN4 RRM requirement of MR offloading and relaxation perspective, RAN4 assumed LR and MR are operating on the same carrier frequency as baseline. FFS for the case of MR and LR working on different carrier frequencies if it is supported in RAN1/2.

Issue for further discussion based on contribution driven:

For WUS monitoring assumption, further discuss whether or not the same carrier frequency is assumed for LR and MR.

Agreement

Single Rx is the baseline for LR-WUR RRM measurement.

Agreement:

Measurement delay requirements for LP-WUR based on LP-SS in IDLE/Inactive mode shall be defined based on LP-SS periodicity.

Issue 1-1-5: Criteria (entry/exit conditions) for fully offloading case and MR RRM measurement relaxation

Issue 1-1-6: Criteria (entry/exit conditions) for LP-WUS monitoring

Recommendations:

For both issue 1-1-5 and 1-1-6, criteria are up to RAN2’s consideration

Issue 1-1-13: LP-SS frequency domain relation with SSB

Proposals

P1: For LP-SS and SSB FDMed multiplex, RAN4 to wait RAN1’s progress (Ericsson)

Recommendations:

No issue from RAN4 requirement perspective identified, suggest to close this issue

Issue 1-1-14: Impact on specification

Recommendations:

Postpone the discussion