3GPP TSG-RAN WG2 Meeting #128												R2-2410092

Orlando, USA, November 18th – 22nd, 2024

Agenda Item:	8.7.5

Source: 	Sharp

Title:  	Issues on RLC Enhancements

Document for: 	Discussion

# Introduction

In RAN2#127bis meeting, RAN2 made the following agreements:

Agreements on RLC timely retransmissions - RAN2 confirm that existing mechanisms are insufficient to resolve the timely RLC retransmission problem and RLC enhancements for timely RLC retransmission are investigated in Rel-19. - Exclude enhanced status reporting. - Focus the discussion on autonomous retransmission and polling enhancements, e.g. we need to understand how each option affects the capacity and packet delay Agreements on avoiding unnecessary retransmissions - RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does. - RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified - FFS if some C-PDU handling is needed to avoid C-PDU discard - FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed)

This document discusses open issues on RLC enhancements.

# Discussion

## RLC Timely Retransmission

In RAN2#127bis meeting, enhanced status reporting has been excluded for enhancements. Now there are autonomous retransmission and polling enhancements on the table. Both candidate solutions aim to accelerate retransmissions to guarantee timely transmission. Thus, the impacts to capacity are almost similar. The only difference is how fast timely retransmission can be performed.

Enhanced polling would mean that when the remaining time until the required delivery time is short (e.g., delay-critical data), the transmitter triggers a polling early to trigger the status report early and the UE performs retransmission early. This mechanism may need to modify only polling procedure. However, actual early retransmission will have delay from setting a poll until status report reception at the UE. It is not appropriate for XR traffic which has delay constraint. Moreover, if the status report experiences congestion in downlink, the early retransmission might not be possible. Also, the polled PDU may experience congestion or retransmissions. Thus, it is not considered as a good solution.

- Retransmission based on enhanced polling has delay until the reception of the status report and may experience congestion.

Autonomous retransmission without status report is a more direct way than enhanced polling. Also, autonomous retransmission without status report is useful when either uplink or downlink is experiencing congestion. The only expected spec impact would be that the autonomous retransmission is triggered in RLC.

One may argue that the autonomous retransmission wastes the uplink resource. However, the autonomous retransmission is triggered only for urgent data to reduce performance degradation. Excessive resource waste does not occur for most of the time, and RLC ACK usually prohibits the unnecessary retransmission.

- Autonomous retransmission without status report is useful for congestion scenario and potential resource waste can be under network control.

Based on the observations above, we see that RAN2 should introduce an RLC autonomous retransmission mechanism without status report.

- RAN2 to introduce an RLC autonomous retransmission mechanism without status report.

## Avoiding Unnecessary Retransmission

In RAN2#127bis meeting, RAN2 agreed a combined approach that both TX and RX independently detect obsolete SDUs. An FFS point is whether an indication from TX to RX is necessary.

In RX side, there is some delay until at least one out-of-order delivery is detected. Figures 1 and 2 compares detection based on TX-to-RX indication and standalone RX operation, respectively. Figure 1 shows the detection based on TX-to-RX indication and Figure 2 shows the detection based on t-reassembly timer of RX. The figures show that standalone RX operation has unnecessary delay until the variable update. Also, the buffered delay at the TX side cannot be taken into account. It is reasonable to set multiple expiries of t-reassembly timer should be used for the detection of outdated packet. In this case, the first expiry may not trigger to set the reception status as ACKed, then the delay may increase. In contrast, obsolete SDU detection based on TX-to-RX indication can be an instant procedure.

A TX-to-RX indication of obsolete SDUs can be configurable.

Figure 1. Immediate detection of obsolete SDU based on the indication

Figure 2. Delayed detection of obsolete SDU based on RX-side timer

## Discussion on AL-FEC Awareness

SA2 sent an LS to ask RAN2 to answer to two questions on AL-FEC. The first question is about DL PDU discard in RLC AM, as follows:

Question 1: Please provide feedback on feasibility of use of content ratio information for discarding of DL PDUs during congestion for RLC AM mode based on the above principles. Please also provide feedback on applicability of RLC AM for XR services, also considering the RAN-level enhancements of RLC Acknowledged Mode (RLC AM) with small packet delay budget as per RP-240791.

RLC AM guarantees 100 percents successful delivery. Even if gNB knows content ratio information, it cannot discard the SDU until it receives an acknowledgement for the SDU. Otherwise, receiver window cannot move. The current RLC AM procedure will have a critical reception problem. Moreover, RAN has a lot of reliability enhancement mechanisms such as robust MCS, PDSCH/PUSCH repetition, HARQ retransmission, RLC AM, and PDCP duplication. Upper-layer redundancy is not necessary.

One could argue that Rel-19 enhancement for avoiding unnecessary retransmission may help RX-side window shift. However, the agreed combined approach allows the RX to decide whether the SDU is obsolete or not. RX’s discard status could be different from that of TX-side. It means that TX side may not have accurate information of RX status. Due to the mismatch, TX may not be sure whether an SDU can be just discarded.

RAN2 to respond that due to 100 percents successful delivery of RLC AM, content ratio information cannot be used to discard the packet immediately.

The second question is about AL-FEC feasibility in RLC UM, as follows:

With respect to RLC UM, RAN indicated in R2-2405781 that "RAN2 does not think it is possible for NG-RAN to determine reliably whether a PDU has been successfully delivered over an unacknowledged-mode data bearer." After sending the previous LS in S2-2405625, SA2 clarified the principles for potential use of content ratio information for discarding of PDUs during congestion as listed above. Question 2: Please provide feedback on feasibility of use of content ratio information for discarding of DL PDUs during congestion for RLC UM based on the above principles.

Compared to the previous RAN2 conclusion captured in the RAN2 LS (R2-2405781), this question is specific to congestion scenario. However, the situation is the same. We see that the conclusion “it is possible for NG-RAN to determine reliably whether a PDU has been successfully delivered over an unacknowledged-mode data bearer.” is valid irrespective of congestion scenario. Thus, the answer should be the same.

RAN2 to respond that content ratio information in RLC UM is not feasible due to unreliable HARQ feedback.

# Conclusions

Based on the discussion given above, RAN2 is requested to discuss and agree the following proposals:

- RAN2 to introduce an RLC autonomous retransmission mechanism without status report.

A TX-to-RX indication of obsolete SDUs can be configurable.

RAN2 to respond that due to 100 percents successful delivery of RLC AM, content ratio information cannot be used to discard the packet immediately.

RAN2 to respond that content ratio information in RLC UM is not feasible due to unreliable HARQ feedback.