3GPP TSG-RAN WG2 Meeting #128	R2-2410099

Orlando, USA, Nov. 18th-22nd, 2024

Agenda item:	8.4.4

Source:	China Telecom

Title:	LP-WUS in CONNECTED mode

Document for:	Discussion and Decision

- Introduction

In the RAN2#127-bis meeting, option 1-2 was deeply discussed, and we have achieved following agreements [1].

For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2. In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring. - When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected.

In this contribution, we will further discuss the RAN2 impacts on RRC_CONNECTED mode with LP-WUS/WUR.

- Discussion

For Option 1-2, there is an FFS that if the legacy drx-onDurationTimer is started or not if the new timer is configured in the last meeting. As agreed in RAN1, “UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS”, we understand it means that PDCCH monitoring is an independent process, which is not affected by the C-DRX cycle. Therefore, to avoid missing the downlink transmission, UE should monitor the LP-WUS within the legacy drx-onDurationTimer. Upon receiving LP-WUS, UE starts a new timer to monitor PDCCH.

Proposal 1: For Option 1-2, LP-WUS monitoring is also performed at the legacy drx-onDurationTimer.

RAN2 has agreed that the timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring for Option 1-2. The specific configuration method for the time offset can be discussed by RAN2. The simplest method is through SIB to indicate the time offset from the RAN2 perspective. MR may select its sleep state according to the configured time offset. Moreover, it can also configured by special RRC signalling.

Proposal 2: RAN2 to support that the time offset can be configured by SIB or RRC signalling for Option 1-2.

To reasonably configure the above time offset, UE may report its supported minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. RAN1 has the following relevant agreement in RAN1#118-bis meeting [2].

Agreement - For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported candidate value - UE can indicate its preference for configured time offset between LP-WUS and PDCCH reception using UAI mechanism. FFS details. UAI is optional. - FFS: A single time offset is configured for a UE at a given time

From the RAN2 perspective, we need to determine the UE capability IE carrying the message of minimum time gaps, i.g. defining a new UE capability IE or reusing a legacy UE capability IE. Then, after receiving the reported values, the network should configure the value of time offset to the UE. In this way, UE can select sleep states accordingly to achieve the purpose of energy saving.

Proposal 3: RAN2 needs to determine the UE capability IE for reporting the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring.

As for the activation and deactivation of LP-WUS monitoring, RAN1 has reached the following agreements in RAN1#116 meeting [3]:

Agreement - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

Based on the above agreement, in case LP-WUS monitoring is enabled, there would exist at least one mechanism for activation/deactivation of LP-WUS monitoring, which can improve LP-WUS monitoring efficiency on the condition of the poor channel. In our view, Option 2 is more flexible. It can be a new signalling or existing signalling to activate/deactivate LP-WUS monitoring. For Option 3, if the channel quality is not good enough to transmit LP-WUS when the timer is still running, it may be invalid for UE to monitor the LP-WUS. Thus, this option is not more suitable than Option 2. Option 4 is also feasible as discussed in the SI stage, we can prioritize supporting Option 2 as a baseline.

Proposal 4: In RRC_CONNECTED mode, option 2 is more flexible for the activation/deactivation of LP-WUS monitoring.

Moreover, we have a view that PDCCH skipping can also be used for LP-WUS referring to the design of Rel-17. It can indicate the end of PDCCH monitoring. When UE receives the PDCCH skipping indication, it should stop the PDCCH monitoring and start the LP-WUS monitoring.

Proposal 5: PDCCH skipping indication can be considered to stop the PDCCH monitoring triggered by LP-WUS.

- Conclusion

Based on the above discussion, we have the following proposals.

Proposal 1: For Option 1-2, LP-WUS monitoring is also performed at the legacy drx-onDurationTimer.

Proposal 2: RAN2 to support that the time offset can be configured by SIB or RRC signalling for Option 1-2.

Proposal 3: RAN2 needs to determine the UE capability IE for reporting the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring.

Proposal 4: In RRC_CONNECTED mode, option 2 is more flexible for the activation/deactivation of LP-WUS monitoring.

Proposal 5: PDCCH skipping indication can be considered to stop the PDCCH monitoring triggered by LP-WUS.

- References

- RAN2#127-bis chairman notes.

- RAN1#118-bis chairman notes.

- RAN1#116 chairman notes.