3GPP TSG-RAN WG2 Meeting #128	           R2-2410101

Orlando, US, Nov 18th – Nov 22nd, 2024

Agenda item:	8.1.2.2

Source:	China Telecom

Title:	LCM for UE-sided model for BM

Document for:	Discussion

- Introduction

RAN2 has made some further progress on LCM for UE-sided model for beam management use case during RAN2#127bis meeting, including applicable/non-applicable functionality report, and data collection for model training. On the other hand, during RAN1#118bis meeting in Hefei, RAN1 discussed some of the questions in the LS on applicable functionality reporting for beam management UE-sided model sent by RAN2. It seems that the discussion is not easy. We would like to further discuss LCM for UE-sided model for BM in this paper, based on the progress up to now.

- Discussion

RAN2 has identified a general applicable functionality report procedure for BM cases as the figure 1 below.

Figure 1

According to RAN2#127bis meeting agreements, there are some left issues, such as the applicability reporting content, whether the inference configuration can be signaled in step3, how to indicate non-applicable functionality and so on.

1. UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality. We should aim to align the design on how the applicable functionality are signaled. FFS on the applicability reporting content. 2. FFS if inference configuration can be signalled in step3. 3. UE can report to the network when an applicable AI functionality becomes non-applicable. FFS how this is signaled (e.g. explicitly/implicitly). Consider different scenarios, whether it is regarding an active functionality) 4. Data collection initiation and configuration for data collection is under network control. FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server)

On the other hand, RAN1 has the following agreements about the questions from the LS on applicable functionality reporting for BM. Regarding step 3~step 5, RAN1 listed three options for the further study.

Agreement Answer to Q2 in R1-2407604 as below: RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list.. Agreement RAN 1 further study the following options for applicability for inference for UE-side model: Option 1: - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2)+3) NW configures one or more CSI-ReportConfig for inference configuration, where the associated ID may be configured in CSI framework as working assumption applied. - FFS on whether some IEs in the CSI report configuration can be removed or modified - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - In Step 4, UE reports applicability(ies) of the above CSI-ReportConfig - FFS on one or more of the above CSI-ReportConfig to be reported - FFS on activation (including when/how) of inference report after obtaining the applicability from UE Step 4 - FFS: whether Step 5 is needed, Option 2: - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - NW configures one set or multiple sets of inference related parameters - Note: the set of inference related parameters is not configured by CSI-ReportConfig - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - The associated ID(s) may be configured - wherein the associated ID(s) may be - FFS: a) part of one set of the inference related parameters, or - FFS: b) independently from the one set of the inference related parameters. - In Step 4, UE reports applicability of the above one or multiple sets of inference related parameters, where the associated ID information may be associated. - In Step 5, NW configures configuration(s) for CSI report for inference Option 3: - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2) The associated ID(s) may be provided to UE, e.g., a new RRC parameter. - In Step 4, UE reports by UAI - the applicable one or multiple sets of inference related parameters may be included. - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - Note: not applicable may also be replied by UE - Note: if the inference related parameters are not supported for reporting, only the applicability(ies) or not is reported in Step 4. - the associated ID(s) may be included - FFS: a) as part of the inference related parameters, or - FFS: b) independently from the set of the inference related parameters. - In Step 5, NW configures configuration(s) for CSI report for inference. Note: There is no impact of configuring CSI report configuration for non-AI beam management in RRCReconfiguration. Agreement For UE-side model, for beam management, for inference report, support periodic CSI report, aperiodic CSI report, and semi-persistent CSI report. Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Agreement Incorporating below text into the general part of the LS In RAN1’s discussion of RAN 2 terminologies on beam management, - The concept/terminology “functionality“ of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-specific FGs - The concept/terminology “ functionality“ of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters or information/parameters indicated by UE - The Activated functionalities may be enabled based on CSI framework. Therefore, the meaning and the granularity of “functionality“for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same, depends on certain option in RAN1, and the discussion is still ongoing Agreement Answer to Q1 in R1-2407604 as below, In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-specific FGs (including components and corresponding value ranges). These AI/ML-specific UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI..

The three options involve the step in which the inference configuration is sent, inference configuration content, inference related parameters, and so on. In our understanding, option1 implies that the inference configuration is configured in step 3 with/without step 5. Option 2 implies that inference configuration is configured in step 5, while inference related parameters can be sent to the UE in step 3 by the network. Option 3 implies that that inference configuration is configured in step 5, not step 3. Thus whether the inference configuration can be signaled in step 3 is up to which option is selected, and the discussion is still ongoing in RAN1. Moreover, the inference configuration content is also up to RAN1’s discussion. From this point, we suggest to postpone the discussion related to inference configuration for UE-sided model for BM cases in RAN2.

Proposal 1: postpone the discussion related to inference configuration for UE-sided model for BM cases in RAN2.

Regarding applicability reporting content (step 4), the three options above imply different applicability reporting content. In option 1, the applicability reporting content may indicate the applicable CSI-ReportConfig, e.g. reportConfigId of the applicable CSI-ReportConfig. In option 2, the applicability reporting content may include e.g. the applicability indicator of one or multiple sets of inference related parameters sent in step 3, the associated ID information. In option 3, the applicability reporting content may include the applicable one or multiple sets of inference related parameters, the associated ID information. Thus it can be seen that the applicability reporting content is also up to which option is selected. Therefore, we suggest to postpone the discussion on applicability reporting content for UE-sided model for BM cases in RAN2.

Proposal 2: postpone the discussion on applicability reporting content for UE-sided model for BM cases in RAN2.

Regarding non-applicable functionality reporting, RAN2 has agreed that UE can report to the network when an applicable AI functionality becomes non-applicable. However, the conditions for the UE to report non-applicable functionality need to be further discussed. Some companies mentioned whether it is regarding an active functionality. In our understanding, it may be an active functionality or may not. If the UE has reported an applicable functionality to the network before, and the functionality becomes non-applicable, the UE can report to the network, otherwise the network will still regard it as applicable and ready for inference. In addition, we think that non-applicable functionality signalling is related to the design of applicability reporting content. Thus it can be discussed later.

Proposal 3: If the UE has reported an applicable functionality to the network before, and the functionality becomes non-applicable, the UE can report to the network, regardless of whether the functionality is active or not.

3	Conclusions

Based on the discussion above, we have the following proposals.

Proposal 1: postpone the discussion related to inference configuration for UE-sided model for BM cases in RAN2.

Proposal 2: postpone the discussion on applicability reporting content for UE-sided model for BM cases in RAN2.

Proposal 3: If the UE has reported an applicable functionality to the network before, and the functionality becomes non-applicable, the UE can report to the network, regardless of whether the functionality is active or not.

4	References

- [1] Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China.

- [2] Draft Report of 3GPP TSG RAN WG1 #118b v0.2.0, Hefei, China.