# 3GPP TSG RAN WG2 Meeting #128      	                        R2-2410102
Orlando, USA, 18 - 22 Nov 2024

Agenda item:	8.1.3

Source:	China Telecom

Title:	Discussion on NW-sided data collection for AI/ML based beam management

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

In RAN2#127bis meeting [1], NW-side data collection was discussed, and the following agreements were made with several FFSs.

Agreements (RAN2#127b)

1. 	Periodic logging is supported for training data collection procedure in R19

2. 	Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

3. 	Periodic reporting of logged data is not supported.

4. 	On-demand reporting of the logged measurements will be specified

5. 	UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.  FFS of details of the message

6. 	The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network.

7. 	Low priority SRB will be used.    FFS new SRB or use of SRB4

8. 	For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress.

In this contribution, we further discuss NW-sided data collection for AI/ML based beam management.

# 2 Discussion

Based on the agreements from the previous meetings, RAN2 agreed that the on-demand reporting for AI/ML training data will be specified, and UEInformationRequest/UEInformationResponse will be used for the on-demand reporting. Additionally, the UE can indicate the availability of logged data to the network to assist network to trigger UEInformationRequest. But the trigger/definition of data availability indication and how to send the indication from UE to network are still FFS.

- What is the definition of data availability indication?

The data availability indication is used to indicate whether the UE stores the available measurement result. However, the exact meaning of an “available measurement result” and the conditions under which a measurement result is considered “available” remain unclear. We propose two options below to clarify these points:

- Option 1 (OP1): The availability indication is based on the measurement configuration provided by the network, determining whether the measurement result meets the network’s measurement configuration. For example, if the UE is configured with a specific data amount that the network requests, the data becomes available once the UE has collected enough to meet this threshold.

- Option 2 (OP2): The availability indication is determined based on the UE’s buffer. For instance, if the UE’s buffer limit is reached, that data is considered available.

According to OP1, we would like to propose:

Proposal 1: For NW-side data collection with data logging, network should configure the data reporting configuration to the UE, e.g. data volume threshold.

- How does the UE trigger the data availability indicator?

When a measurement result becomes available, we need to determine when the UE should send the data availability indicator to the network. Based on the previous proposals:

- If the availability indication is determined based on the network’s measurement configuration, the UE can send the data availability indicator when it meets the network's specified measurement request.

- If the availability indication is based on the UE’s buffer, the UE can send the indicator once it reaches its buffer limit.

Therefore, we have the following proposal:

Proposal 2: For NW-side data collection with data logging, the UE may send the data availability indicator either when the network’s measurement configuration is fulfilled or when the UE’s buffer limit is reached.

- How does the UE send the data availability indicator to network?

In legacy logged MDT, the UE indicates the availability of logged measurements within RRC Reconfiguration/Setup/Resume/Re-establishment complete messages. For instance, considering a handover case, if the network's data collection configuration is kept in the UE, the UE must inform the new gNB of its data availability. This notification allows the new gNB to request retrieval of the UE’s logged data as needed.

Therefore, we propose:

Proposal 3: The legacy mechanism (e.g., RRC complete messages) can be easily reused to indicate the availability of logged data for the training of a NW-sided model.

Moreover, as the data availability indication information is UE side assistance information which may be helpful to network, so we assume UAI can be also used to deliver this information to network.

Proposal 4: UAI message can be also used to indicate the availability of logged data for the training of a NW-sided model.

# 3 Conclusion

In this contribution, we further discuss NW-side data collection for AI/ML based beam management. We have following proposals:

Proposal 1: For NW-side data collection with data logging, network should configure the data reporting configuration to the UE, e.g. data volume threshold.

Proposal 2: For NW-side data collection with data logging, the UE may send the data availability indicator either when the network’s measurement configuration is fulfilled or when the UE’s buffer limit is reached.

Proposal 3: The legacy mechanism (e.g., indication in RRC complete messages) can be easily reused to indicate the availability of logged data for the training of a NW-sided model.

Proposal 4: UAI message can be also used to indicate the availability of logged data for the training of a NW-sided model.

# 4 References

[1] RAN2#127b, Chair Notes.